-------------------------------------------

|                                         |

|  FUZZY LOGIC INVERTED PENDULUM CONTROL  |

|                                         |

|               release 1.1               |

-------------------------------------------

FLIPC11.EXE is a freeware fuzzy controller software applied to a simulation of

the behaviour of an inverted pendulum.

-------------------------------------------------------------------------------

FEATURES:

- To run the simulation                                ("Simulation" Menu')

- To change the range value of each I/O variable       ("Change MFs" Menu')

- To change the Shape of each Membership Function      ("Change MFs" Menu')

- To change the number of rules you want to use        ("Change Rules" Menu')

- To redo the simulation with old values               ("Statistics" Menu')

- To create a complete table of the output value       ("Create Table" Menu')

-------------------------------------------------------------------------------

HOW DOES FLIPC SIMULATION WORK?

The input parameters are:

the interval between two iterations ("Timestep") in milliseconds;

the "Weight" of the rod in Kg;

the "Height" of the rod in meters;

the value of  Theta ("Angle") in bit;

the value of Dtheta ("Speed") in bit;

the value of  "Motor" in bit;

the Inference-Computation method.

Since there are five Membership Functions each input variable, the maximum

number of rules is 5^2 = 25. In the "Modify Rules" Menu, you can choise which

rules you want to use to run the simulation. Of course the unused rules don't

fire the system. In this case, the output crisp value will be undefined. When

this happens, the output value will be the same of the precedent iteration one:

Output t0 = Output t-1

This is displayed in the simulation graph (the color of the pixel changes).

Since I've considered the force applied to the motor with a costant value,

there might be the possibility that the simulation doesn't work properly.

This is due in particular to the values of the parameters  "Height of the rod"

and "Weight of the rod". If the "height" value is LOW and/or the "weight" one

is HIGH the motor is not always able to move the cart and consequently the rod

falls down. If the "height" value is HIGH and/or the "weight" one is LOW the

motor moves the cart so rapidly that the rod rebound at the limit falling down.

-------------------------------------------------------------------------------

FLIPC SIMUALTION is designed to run the simulation using both the Intel and

FP3000 (Omrom) processor. In this latter case you need the FB30-AT Omron board.

If you want to use FLIPC SIMULATION with the Omron board, you have to create

the "NAMEFILE.bin" file (object code generated by the FS10-AT Omron Design

Software) with the same Membership Functions of the FLIPC and the values of

the variables expressed in bit (0-4095).

-------------------------------------------------------------------------------

SUPPORT FILES

- "NAMEFILE.sin" is used if you want to redo the simulation with the FLIPC or

with the FS-10AT Omron design Software.

- "stat.txt"     is updated for every simulation with the input parameters and

the number of iterations necessary to stabilize the pendulum.

You can select the record to redo the simulation.

- "first.beg"    contains the input default values.

- "limits.mfs"   contains the four points of every MF.

- <"rules.rls">  is used to determine which rules will be activated.

- <"NAMEFILE.txt">  contains all the output crisp values.

-------------------------------------------------------------------------------

CREATE THE TABLE (Menu')

This Menu' allows you to print in a "Namefile.txt" the output crisp value

for all the combinations of Theta value, from Lower Limit (LL) to the Upper

Limit (UL), and Dtheta, from LL to UL.

T H E T A

| LL     LL+1   LL+2   ...           ...   UL-2   UL-1    UL

-----|------------------------------------------------------------

UL   | Out    Out    Out    Out    Out    Out    Out    Out    Out

|

UL-1 | Out    Out    Out    Out    Out    Out    Out    Out    Out

|

D    ...  | Out    Out    Out    Out    Out    Out    Out    Out    Out

|

T     1   | Out    Out    Out    Out    Out    Out    Out    Out    Out

|

H     0   | Out    Out    Out    Out    Out    Out    Out    Out    Out

|

E    LL   | Out    Out    Out    Out    Out    Out    Out    Out    Out

|

T    LL+1 | Out    Out    Out    Out    Out    Out    Out    Out    Out

|

A    ...  | Out    Out    Out    Out    Out    Out    Out    Out    Out

|

-2   | Out    Out    Out    Out    Out    Out    Out    Out    Out

|

-1   | Out    Out    Out    Out    Out    Out    Out    Out    Out

if(Out<0) Out=abs(Out)

else      Out+=UL

-------------------------------------------------------------------------------

FLIPC SOFTWARE LICENSE

I grant to you the right to use and to reproduce and distribute all or part

of the FLIPC ("Software") provided that the Software is not distributed for

profit; the Software may NOT be modified; all copyright notices are maintained

on the  Software.

The Software is owned by Michelangelo Sebastiani.

You may not remove the copyright notice from any copy of the Software or any

copy of the written materials, if any, accompanying the Software.

NO WARRANTY.  ANY USE OF THE SOFTWARE IS AT YOUR OWN RISK.

THE SOFTWARE IS PROVIDED FOR USE ONLY WITH PC-DOS.

THE SOFTWARE IS PROVIDED FOR USE "AS IS" WITHOUT WARRANTY

OF ANY KIND.  TO THE MAXIMUM EXTENT PERMITTED BY LAW,

I DISCLAIM ALL WARRANTIES OF ANY KIND, EITHER EXPRESS

OR IMPLIED, INCLUDING, WITHOUT LIMITATION, IMPLIED WARRANTIES

OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE.

-------------------------------------------------------------------------------

FB30-AT (Fuzzy Inference Board) and FS-10AT (Fuzzy Developing Software)

are trademarks of "Omron Corporation".

-------------------------------------------------------------------------------

If you have any questions concerning this FLIPC softaware, or if you desire to

contact me for any reason, please e-mail to:

msebast@cs.unitn.it

------------------------------------------------------------------------------