Name

dial

Purpose

To repeatedly dial a list of telephone numbers until one of

them answers by issuing commands for hayes-modems. The dial

process is executed in 'background' so that the user can use

other programs while waiting for the dial to complete.

Syntax

dial [-i<initstr>][-k<comb>][-w<secs>][-b<secs>]

<port> <speed><parity><data><stop> <tel> [tel]..

dial -q

Parameters

<port> is the com port number on which the modem is attached. Ports

from 1 to 4 are supported.

<speed><parity><data><stop> specifies the communication parameters

as follows :

<speed>  :  communication speed (bps). Legal values are 300, 1200

2400, 4800, 9600, 14400, 19200 and 38400.

<parity> :  parity, one of N (none), O (odd), E (even)

<data>   :  number of data bits (7 or 8)

<stop>   :  number of stop bits (1 or 2)

<tel> [tel] .. specifies the numbers to call. Since dial 'glues'

the telephone numbers directly to the ATD command, all the dial

modifiers (such as 'P' for pulse dial or 'T' for tone dial) can

be used here.

Options

The options can be preceeded by either the '-' character

or the ms-dos default '/' character. They may appear in

any order, but always before the arguments.

The options are :

-i :  Specify an initialization string fed to the modem

before issuing first dial command. The string is

fed to the modem 'as is', so the AT-prefix must

preceede a command here. Refer to your modem's

manual for information about the commands.

-k :  Specify an abort key combination. When these keys

are held down for about one second, a beep is emitted

from the pc speaker and dial is aborted.

<comb> is made up of r, l, c and a, where :

r : right shift

l : left shift

c : ctrl

a : alt

A valid combination must contain at least two keys.

By default there is no abort key combination, because

such a combination might interfere with the use of

other programs.

-w :  Specify a 'hard' wait time of the lenght of <secs>

seconds. After this time has passed since issuing

the dial command the dial proceeds to the next number.

Otherwize, whatever message from the modem (such as

'BUSY') causes dial to proceed to the next number.

By default this wait time is 30 seconds. The maximum

for this time is 300 seconds.

-b :  Specify a wait time between dials of the length of

<secs> seconds. This is useful, if you'd like to

give a change to someone calling to you to get

trough while dial is running. By default there is

no wait time between calls (except that for technical

reasons). The maximum for this time is 300 seconds.

If a wait time between the calls is specifed, dial

is aborted if the result code 'RING' is sent by

the modem while waiting. The abortion is signaled

by a beep emitted from the pc speaker.

-q :  Abort a previously started dial. A beep is emitted

from the pc speaker as aborting.

Comments

Dial issues hayes dial commands to the modem, until one of the

numbers in the list answers causing a string starting with

'CONNECT' to be sent from the modem. Dial can also be terminated

by user request (dial -q or a hotkey sequence) during the dial.

At this point, dial releases all the memory reserved by it and

restores the captured interrupt vectors essential for the

background operation. At the time dial is assumed to be terminated

successfully, a series of beebs is emitted from the pc speaker.

In case of premature termination a single beep is emitted instead.

Before the actual dialing starts, an initialization string is

fed to the modem (if one is specified). After this, the modem

is fed the commands needed to turn echo from the modem off and

to get word response codes.

The following table lists the assumed io base addresses and

interrupts for each port :

port     io base (hex.)    interrupt (hex.)

1        3f8h              0ch               (std.)

2        2f8h              0bh               (std.)

3        3e8h              0ch               (may vary)

4        2e8h              0bh               (may vary)

Dial does not have a possibility to store certain sets of

numbers and settings permanently in files, but this can

be accomplished by writing the dial command itself as a

.bat -file.

Examples

dial 2 1200N81 12345 67890 13579

Repeatedly dial numbers 12345, 67890 and 13579 using com2 at

1200 bps, no parity, 8 data bits, 1 stop bit.

dial 2 1200N81 t12345 t67890 t13579

As above, but enforce tone dialing for all numbers.

dial -w45 -b60 -klca 2 2400E71 12345 67890

Repeatedly dial numbers 12345 and 67890 using com2 at

2400 bps, no parity, 7 data bits, 1 stop bit. Wait

up to 45 seconds for carrier. Be idle 60 seconds

after each number.  Abort, if users keeps left shift-,

ctrl- and alt-keys depressed for about a second.

dial -iATM0 3 9600N81 12345 13579

Repeatedly dial numbers 12345 and 13579 using com3 at

9600 bps, no parity, 8 data bits, 1 stop bit. Issue

hayes 'speaker off' -command before starting to dial.

Return value

If successfully started, dial returns 0 and remains resident

in the memory. In other case 1 is returned to signal failure

(e.g. missing arguments or dial already running).

Bugs and caveats

Actually, there is no such thing as multitasking in the plain

MS-DOS environment. Therefore, this program may not work with

all configurations or with all programs. (Once again) this TSR

is required to be loaded as the last one to ensure proper

'unloading' and operation.

In case some other program has blocked the int 0x28, even a

terminated dial may still be resident in memory, causing

dial to fail if re-executed. In this case, run dial as 'dial

-q'. This always generates an int 0x28, thus providing the

resident dial a change to terminate.

The program may not work at the higher communication speeds.

It has been reported to work at speeds up to 9600 bps. Even

higher speeds are provided, however, because there is no

known reason why they certainly wouldn't work.

The wait time specified by the -w option has no effect if it

is longer than that the modem itself will wait for a call to

complete. Use the -i option to set the modem's wait time if

needed.

-----------------------------------------------------------------------------

Dial.exe was written using Borland's Turbo C by Jouni Leppjrvi (C) 1990, 1991

(jml@stekt.oulu.fi). It and the documentation provided with it may be used

and copied freely on a non-commercial basis. The program and the documentation

are provided 'as is' with no warranty of any kind. Comments and bug reports

are wellcome. (v 1.1, March 4, 1991)

-----------------------------------------------------------------------------