COPYRIGHT

See the "copying" file

THIS FILE:

Set the tab value to 3 for correct formatting of this file.

INTRODUCTION

Fsperf is a benchmarking tool designed to measure filesystem's

performance. It can perform a variety of tests reflecting typical

application's filesystem access patterns. Fsperf should be fairy

portable across different hardware and software platforms. This document

gives a brief survey of the issues related to the tuning of filesystems

performance, followed by the description of the program itself.

CONTENTS

1.    PERFORMANCE LIMITING FACTORS

1.1     CONTROLLERS

1.2     CACHES AND CACHES

1.2.1     Hardware caches

1.2.2     Software caches

1.3     OTHER HARDWARE AND SOFTWARE FACTORS INFLUENCING DISK PERFORMANCE

1.3.1   Access time and transfer rate

1.3.2     Software issues

2.    TEST METHODOLOGY

2.1       Multifile test

2.2       Write and Read tests

3.    THE PROGRAM

3.1       Synopsis

4.    EXAMPLES

5.    KNOWN BUGS & PROBLEMS

1. PERFORMANCE LIMITING FACTORS

Please note that fsperf is designed to measure FILESYSTEM'S performance,

hence it can be used to benchmark filesystems located on any possible

media (hard drive, CD-ROM, floppies, ...), and even filesystems which

are not bound to any physical media (ramdrives, and tmpfs type

filesystems). Since most of the people are really interested in

performance of filesystems located on hard-drive type devices the

following discussion will mostlt cover this particular case.

1.1 CONTROLLERS, BUS SPEED AND BUS CONGESTION

To use a mass storage device with a computer system we need a controller

interfacing the device to system's memory bus. The controller is not

necessarily directly connected to the memory bus. A hierarchy of buses

may exist in a given system. Generally the "closer" the controller is to

the processor-memory bus the higher performance can be expected. A good

example might be AT-BUS vs. LocalBus controllers in typical PC systems.

The controller itself may interface to many external devices: hard

drives, tape units, scanners, etc., hence, it may happen that

controller-device bus becomes congested by multiple devices trying to

transmit data at the same time. In fact the performance limiting

congestion may be caused either by to slow controllers or to slow

controller-memory interface. For example a theoretical 10 MB/sec speed

of a SCSI device will never be accomplished on an ordinary ISA

controller (about 3.5 MB/sec maximal throughput).

1.2 CACHES AND CACHES

In a modern computer system it is very hard to determine the actual

performance of a "bare" harddrive. Hardware and software caches existing

between the drive and your application can bias various benchmarks, so

it is vital to understand how the cache mechanisms work.

The basic idea of a disk (filesystem) cache is to exploit locality of

reference. Hence the cache memory buffers read blocks in case they will

be soon needed again, it also delays writes, in case the same block of

data will become modified soon. Note, that the idea of a filesystem

cache is quite similar to the concept of a memory cache, which we

encounter in almost all modern computer systems.

1.2.1 Hardware caches

First, we can have cache memory integrated into drive electronics -

typically 16 - 512 KB of memory, sometimes with "intelligent"

electronics managing the cache. This solution is most common with IDE

and Enchanced IDE (PC standard) drives.

Cache memory can also reside on the drive controller, typical size of

controller cache is 64kB - 1MB, but larger caches, up to 16MB are not

uncommon. Caches integrated into specialized RAID systems are a separate

issue. More expensive RAID systems have expandable caches, with up to

64MB of memory.

1.2.2 Software Caches

Software disk caching appears on the operating system level. It is

implemented in various ways depending on the actual operating system.

For example: in Unix type systems the kernel manages a filesystem buffer

pool which in fact acts like a cache, in MS-DOS disk caching is an

option provided by a "memory resident" program smartdrive, in Windows NT

caching is performed by a special driver located in the driver chain.

Naturally, the size of the system software cache is limited by the size

of physically available RAM memory.

1.3 OTHER HARDWARE AND SOFTWARE FACTORS INFLUENCING DISK PERFORMANCE

1.3.1 Access time and transfer rate

From the point of view of disk drive manufacturer the "Average Disk Head

Seek Time" (AST) together with the "Date Transfer Rate" are two most

important performance factors. Average seek time, expressed in

milliseconds, is the average time it takes to move the disks read/write

head from one track to another. In state-of- the-art disks today AST is

slightly below 10ms (11 - 14 ms is a typical value). Note that AST is

the average time of moving the head from a random track to another

randomly chosen track. A second parameter called "Track to Track Seek

Time" reflects the time it takes to move the head to an adjacent track.

TtTST is around 2 - 4 ms in typical disk drives of today.

While AST is a purely mechanical factor, entirely dependent on disk's

physical design, the "Transfer Rate" (TR) is a much more complex

parameter. Expressed in KB/sec or MB/sec it reflects the speed of

transfer of data from the drive to the controller. Naturally, we are

more interested in the effective transfer rate, i.e., the speed of data

transfer between the disk and the system memory. Disk's technical specs

often give only the value of maximal possible transfer rate (e.g.

occurring when one disk sector is read over and over again), more

demanding users should be interested in average transfer rate, or a

sequential/random, read/write, etc., transfer rates. Transfer rate is

also dependent on the amount of data moved in one atomic (from the point

of view of the application) filesystem access. To summarize: the actual

transfer rate is dependent on many factors: the type of access, the size

of data moved, the filesystem design, the OS overall performance, etc.

The technical data provided with the disk should be treated skeptically,

as the actual transfer rate occurring in a given hardware and software

environment may be orders of magnitude lower. Internal (i.e., non

sustainable) physical transfer rates of low-end PC (IDE) drives are

around 3 MB/s, good IDE drives approach 7 MB/s. Good SCSI drives should

obtain theoretical transfer rate of 10MB/s, while a theoretical limit

for fast SCSI is 20MB/s. Transfer rates measured in actual complex

applications (data bases, spreadsheets, etc.) vary between 300 KB/sec -

1.5 MB/s. Note that the above figures do not relate when techniques such

as stripping and interleaving implemented either in the operating system

or in dedicated (RAID) hardware are used.

1.3.2 Software issues

In a multiprocessing system there exist many "random" factors that can

influence overall filesystem's performance in many unpredictable ways.

In a typical case a process is one of many using a particular

filesystem, and hence a particular drive media and drive controller. The

observed drive's performance (from the point of view of an application)

may vary almost unpredictably. Consider a drive, which part is used as a

swap file (paging file) - the throughput observed by applications using

files located on the same drive may vary as system's paging activity

changes.

The filesystem's design is a separate issue. E.g.: in some versions of

the Unix operating system parameters such as fragment size, block size,

cylinder group size, etc., can be set up during filesystem's

initialization. Various combinations of these parameters can result in

different performance values for various types of file systems access.

For example one setup may be optimal for filesystem containing graphic

images (relatively small amount of large files), other for handling a

large amount of small files (program sources).

2. TEST METHODOLOGY

Fsperf currently supports five performance test types:

1) Multifile

2) Sequential Write

3) Random Write

4) Sequential Read

5) Random Read

The tests were designed to simulate (in a sort of condensed way) real

application's behavior. Hence, for example, there is no test which reads

the same file block over and over again - such a benchmark has no

practical meaning, besides, the results of such test would be probably

biased by some sort of cache between the harddrive and fsperf.

Here is a brief description and discussion of the tests fsperf

can perform:

2.1 Multifile test

Multifile creates and removes some number of "short files". The exact

methodology is as follows:

1) create file with the open() system call

2) optionally write() to the file using exactly one system call

3) unlink() the file

repeat the above steps

This test also has a "don't unlink" variant where all test files are

unlinked only after the time measuring phase is finished.

Not surprisingly this test measures the rate at which the filesystem can

create files. Creating new files is a more complicated task than writing

or reading, since file creation involves a lot of maintenance

information overhead. An interesting fact about file creation is that

many OSes do it synchronously. The reason is that we want the file

system more or less stable all the time in case a  temporary hardware

failure such as for example power out happens. Some OSes (Linux, I

believe is among them) do their file I/O almost completely

asynchronously therefore being faster (as far as many-short-files

operations are concerned). Where can we expect this type of performance

factor to play role? A good example is unpacking of a large archive file

(e.g., a tar archive) containing many relatively short files - for

example program sources.

The results of the multifile test are heavily dependent on the size of

the buffer written. You should not notice big difference in files/sec

creation ratio for file sizes below some basic storage blocks size

(cluster, block, etc.). For larger file sizes the write time will begin

to dominate, decreasing the overall file creation ratio. As a rule of

thumb: set the file size below 4-8KB limit.

2.2 Write and Read tests

The next four tests are in fact very similar. The methodology is

as follows:

1) create the "long file", start the timer

2) perform a given number of reads or writes, in a random

or sequential matter

3) stop the timer

4) unlink the file

Note, that all the tests are done on existing, fixed length file. The

size of the elementary read/write transfer is fixed (but it is possible

to perform a sequence of tests with growing transfer buffer sizes). The

very testfile creation should be time efficient, because the file is not

actually written to, all fsperf does is: creat(), lseek() to the desired

position, one byte write() and close().

In a random access test a given number of records is read/written, in a

sequential test the whole file is read/written.

The most important test parameter here is the transfer buffer size. You

can expect the performance to grow as the transfer buffer size

increases. Note that physically there is a basic data block (namely - a

disk sector) that is always transferred no matter how small your

transfer buffer is. Typical disk sector size is 512 B.

As you will increase data transfer buffer size the transfer rate will

grow. At some transfer block size it will become optimal. It may also

happen that you will observe a local maximum, with transfer rate

dropping as the transfer buffer size in increased beyond the optimal.

To perform a good benchmark you should try to learn what is the basic

transfer block size of your critical application, and only after that

execute the benchmark suite. Sadly most applications use a rather small

read/write buffer size (say 512B) while the optimal can be 8 or 16 KB.

You can also use the automatic transfer buffer size growing feature to

determine the optimal transfer block size and tune your application

accordingly.

You should also determine what is the size of your filesystem software

cache and consider using test files larger than the maximal cache size.

Note that some chaches are able to adjust their size dynamically, as the

amount of free system RAM changes.

3 THE PROGRAM

3.1 Synopsis:

diskperf M | L[SRsr] [ -?U ] ] [-d work_dir] [-v level] [-s size[k|m]] [-

n nfiles] [-R|W nrecords] [-r rsize] [-w wsize] [-l] [-f file

] [-S nsteps]

Options:

M           -         Multi-file test

L...        -         One-file test

LS          -         sequential write test

LR          -         random write test

Ls          -         sequential read test

Lr          -         random write test

The tests can be mixed in one program run. It is possible to perform any

combination of M and L tests. The order of tests is fixed: Multifile,

seq. write, random write, seq. read, random read.

-U           -         unlink test files only after test finished

-d work_dir  -         set work dir (and drive on MS-DOS systems)

-v level     -         set verbosity level. The verbosity levels are:

1: outputs major statistics

2: outputs all statistics

4: displays some additional warning

6: reports actions as they are performed

-t           -         display the results in a table

works only with L... type tests

-s size[k|m] -         sets file size for all types of tests,

option: kilo or megabytes

-n nfiles    -         create nfiles files, works only with the "M" test

-W nrecords  -         write nrecords during random access test

-R nrecords  -         read nrecords during random access test

-r rsize     -         set read-record size

-w wsize     -         write-record size

-l           -         don't unlink the test file after tests finished

-f file      -         use previously created file as the work file

-S nsteps    -         do "L" tests nsteps times doubling record size

-?           -         prints the help message

4. EXAMPLES

Create 100 files, 1KB each (default), unlink files only after test

finished:

C:\FSP>fsperf M -U

Total 100 files created, total 102400 bytes written to file(s)

Transfer rate:  16.3578 [KB/sec]

File create/write/(unlink) time:  15.9744 [files/sec]

Total file creation time:    6.260

Create 100 files, 16KB each:

C:\FSP>fsperf M -n 100 -s 16k

Total 100 files created, total 1638400 bytes written to file(s)

Transfer rate: 712.3478 [KB/sec]

File create/write/(unlink) time:  43.4783 [files/sec]

Total file creation time:    2.300

Random access and sequential write test, test file size is 4MB,

300 random and sequential (600 total) write operations are performed,

write buffer size is 8KB (default):

C:\FSP>fsperf LSR -s4m -W 300

4194304 byte file created

Random write transfer rate: 326.3745 [KB/sec]

Sequential write transfer rate: 820.8031 [KB/sec]

Long file sequential write time    5.110

Long file random write time    7.530

Random access and sequential write test, test file size is 4MB,

300 random and sequential (600 total) write operations are performed,

write buffer size is 1KB:

C:\FSP>fsperf LSR -s4m -W 300 -w1024

4194304 byte file created

Random write transfer rate:  57.1004 [KB/sec]

Sequential write transfer rate: 537.7313 [KB/sec]

Long file sequential write time    7.800

Long file random write time    5.380

Full (random-access/sequential, read/write) test. Test file size is

32MB, 1024 random access operations are done. Read/write buffer size

starts from 16 bytes, 4*13 tests, each with the buffer size doubled are

performed (the results below were obtained from a Unix system):

% fsperf LSRsr -s32m -R 1024 -W 1024 -r 16 -w 16 -S 13

Sector Size [B]       -->     Transfer Rate [KB/sec]   <--  Error []

Write       Read        WS        WR        RS        RR

16         16        132.63      1.49    252.29      3.28     20.00

32         32        241.40      2.98    453.44      6.55     20.00

64         64        414.25      5.96    699.05     10.92     16.67

....

32768      32768       1398.10   1082.40    838.86    621.38      4.17

65536      65536       1458.89   1048.58   1048.58    849.48      4.35

5.    KNOWN BUGS & PROBLEMS

- The time measuring code is rather primitive, the clock's resolution is

0.18 sec under MS-DOS, 1 sec under Unix. To obtain accurate results the

number of records read/written, the file size or the number of files in a

multifile test should be sufficiently high.