This is an evaluation copy of the TTirisMR component for Delphi 1.0

-------------------------------------------------------------------

Purpose

-------

The TIRIS (Texas Instruments Registration and Information System)

micro Reader, known as RI-STU-MRD1 can be used attached to a PC.

This Delphi 1.0 component controls the serial port and the protocol used

to communicate with the MR. This beta version supports the ReadOnly

and ReadWrite transponders.

The component allows two handshake signals to be read and two

handshake signal to be written. They can be used

e.g. to open a door, start a process, whatever, monitor the

end of a process etc. Only 3 wires are used to communicate with

the MR: Tx, Rx and Gnd. The handshakelines are free to use.

Installation

============

Copy the files TIRISMR.DCU and TIRISMR.DCR to your components

directory. Choose Options, Install Components, Add, point to

TIRISMR.DCU, click OK, again OK and your COMPLIB.DCL will be

rebuild, including the tirismr component on the TIRIS-page.

Properties:

===========

DTR                  True/False sets the state of the DTR

handshakeline. Can be used to open a door.

FBCC                 Who calculates the FBCC character?

Interval             If in single-mode and program 0, this is

the time beween Single-shot-commands to the

MR. It is also the time at which monitoring

in program 2 takes place

Mode                 Sets the mode of the component:

cmNone

cmShow

cmTrack

cmProgram

cmSingle

cmLine

cmNormal

cmVersion

See below for a description.

Name                 Normal component Property

PBurst1              Chargetime for normal read-operation (ms)

PBurst2              Chargetime for read after write-operation (ms)

Port                 Com-port to be used. Must be free, with its own

IRQ and known and correctly set in Windows

Configuration. All MR operate at 9600 Bd, 8N1,

which is set by the component.

PPause               If used, PauseTime. Read MR-doc for explanation.

I don't use it.

Refresh              If idle, this is the time in which a refreshing

command is sent to the MR. It takes care of immediate

operation after a power-down of the MR.

RTS                  Sets the state of the RTS handshakeline.

Can be used to open a door or start a process.

SpecialWriteTimings  If True, the Timings can be set in TOffL,TOffH,

TOnL, TOnH. A warning is given if set to True and

timings are not within range.

Tag                  Normal component property

TrackTimeOut         If Mode=cmTrack, this is the number of

Interval periods in which the same transponder

must be seen. If this time expires or if another

transponder is detected, an event fires.

Ignored in other modi than cmTrack.

TOff/On/H/L          Timings that are used if SpecialWriteTimings

is True.

WireLessSync         Can be set to True to indicate the use of WireLessSync

if more than one MR is used and intereference can be

expected.

WriteTimeOut         The number of Interval-periods that must

be used to try to write a R/W transponder.

Methods:

========

MRStart              Opens the Comm-device. 0 if succesfull,

otherwise ask for GetError.

MRStop               Closes the Comm-device if open. MRStop is

also called in the components destructor.

SendDatagram         Sends a datagram to the MR. Should only

be used in mode cmNone or if timers-interval

are zero.

GetError             Returns the last error-explanation.

Events:

=======

BeforeTX             Fires just before a datagram is sent

to the MR. Has the datagram in a string

as var parameter. You may want to monitor

or change it.

OnCTSChanged         Fires when the CTS handshake input

changes state.

OnDSRChanged         Fires when the DSR handshake input

changes state.

OnRX                 Fires on correct reception of data

parameter = TMRData in modes cmSingle,

cmLine, cmNormal, cmVersion and cmNone.

OnShow               Fires in mode cmShow when a transponder

is read.

OnTrackLost          Fires in mode cmTrack if last read transponder

has not been read for Timout times interval

ms OR if another transponder is read.

onProgram            Fires in mode cmProgram if programming

is succesful OR has timed out.

Mode:

=====

cmNone

No program, use component properties,

events and methods to suit your own needs.

cmShow

Puts MR in Normal-mode.

Track every R/O and R/W transponder,

fire an event only if received correct,

event parameters are in TMRData record.

Can be used to open doors.

cmTrack

Keep checking if transponder is still there.

fire event OnTrackLost if transponder has

dissapeared (timed out) or if another transponder

is detected.

Can be used in a fueling system.

cmProgram

Try writing data to R/W transponder,

fires an event if succesful or after

a time-out if not succesful

>>>>> not implemented yet  <<<<<<

Can be used for private types creditcards.

cmSingle

Sends SingleShot command to MR every interval-time mS.

Fires event OnRX on reception of data.

cmLine

Puts MR in Line-mode: continually firing and reading.

OnRX fires with data in TMRData-type record.

cmNormal

Puts MR in Normal mode: data is sent only if a new

transponder is detected OR if the current transponder

is re-detected. Data in OnRX event.

cmVersion

Version info is sent in OnRX event.

TMRData record definition:

==========================

TMRData = record

TransponderType   : TTranspondertype;

StartByteDetected : Boolean;

DBCCOK            : Boolean;

FBCCOK            : Boolean;

Version           : Boolean;

Data              : TTransponderdata;

RawData           : string;

end;

Rawdata always holds the raw received data, complete from

SOH (first byte) to CRC (last byte).

TTransponderdata = string[21];

TTransponderType = (ttReadOnly,ttReadWrite,ttMPT,ttOther);

If Version = True, Data holds MR Firmware version number, e.g. 1.2.

All other fields to be ignored.

Transpondertype: 0 = R/O

1 = R/W

2 = MPT (not supported yet)

3 = Other or Unknown

If transpondertype=3 and DBCCOK = False then Nothing is Read.

If transpondertype=3 and Startbytedetected = True then something

is read, Maybe two transponders on top of each other?

In case of Type 0 and 1, Data holds the 0000-0000000000000000

format transponder data.

====================================================

In the final version ofcourse a Help-file will be

available.

====================================================

Please let me know if you find bugs or if you have

ideas about new methods, properties, events or

other features.

please write to:

John vd Waeter pc applicaties

s-mail:

Sottegemstraat 44

4382 en  Vlissingen

the Netherlands

e-mail:

pa3erp@xs4all.nl

c-mail:

100332.3264@compuserve.com

The latest versions of this component can be found on

HTTP://WWW.xs4all.nl/~pa3erp

and

on the Delphi Super Page DSP.

This component is intended to be distributed as shareware:

The evaluation version has a nag-screen.

$  50 w/o source

$ 100 with source

Registration can be done via the above adress or via Compuserve

GO SWREG, search for Keyword TIRIS, TIRISMR.ZIP

-----------------------

TIRIS is a trademark of Texas Instruments.