Slick is a Copy(c)right 1985,1994 by Gary Raymond, New Orleans, La.

Slick Modem Terminal Program V6.5a

Tech Help 504-288-6550

To pre test your RS232 serial ports for activity, type "SLICK test"

at the dos prompt. You will a message similar to the following:

Port 1 (COM1 address 3F8 hex) responding!

8250 UART detected!

UART set at 2400 Baud.

Port 2 (COM2 address 2F8 hex) responding!

16550 FIFO UART detected!

UART set at 1200 Baud.

You will be told what standard IBM default ports are installed,

what baud they are presently set for, and what type of UART chip

is installed in your serial card(s). Note: The presence of a UART

by itself does not mean there is a modem connected to it. I might

be for a mouse or other serial device. This test assumes the

addresses for COM1=3F8, COM2=2F8, COM3=3E8, and COM4=2E8.

Quote PCM Magazine, Nov 91,

"This is one of the best bargains to come along in some time!"

QUICK START - PC REQUIREMENTS - SETTING UP SLICK

There are several Terminal Programs around but only one Slick! Why

should anyone use Slick?  FIRST, Slick is entirely self modifying.

This means there are NO external configuration files or other data

files to manage. SECOND, Slick is still under 120K. This means you

can run it from a system with only ONE FLOPPY and limited RAM.

(and I know because I have talked to a lot of our users still

running older PC's.) Slick is functional with any IBM or Clone CPU

running PC/MS-DOS 3.3 or better. Using a standard 360k floppy with

the operating system installed you are still left with over 200k

for download space! THIRD, SLICK is the only affordable terminal

on the market that really delivers the power features that all

modem operators depend on.

SLICK is a Interrupt driven (on receive only) terminal that

supports the four standard DOS BIOS addresses of 3F8, 2F8, 3E8 and

2E8 respectively for COM1 thru 4. If you are using an IBM PS/2 or

any other system that addresses COM3 thru 8 at 3220h thru 5228,

this release of SLICK can accommodate those circumstances. (SLICK

pairs IRQ4 with COM1 & 3 and IRQ3 with COM2 & 4 in normal or

default IBM mode).

Slick usually works right out the box, but you may have to set

some new defaults. SLICK can also be called via a DOS Path

statement from ANY location in your hard drive directories. The

hot key combination of Alt-V enables complete on-line access to

read the documentation (this file).

Once you have the Baud, Parity, COM port and modem initialization

string set properly for your modem you are ready to dial up another

system. There are two ways of dialing. Manually, by typing the

required command at the flashing terminal prompt. Or, via the

dial directory (accessed via Alt-D). There are advantages to

doing it both ways but after a while you will come to depend a

great deal on the automated feature of the directory. Once you

invoke the directory you will get the menu illustrated below.

Dialing Directory Window:

���͵Home/End���PgUp/Dn���Up/Dn Arrow���Cr to Pic���Esc/Exit���ͻ

� Compuserve 2400      ATDT*70,733-2297            #2400  $7E1  �

� Compuserve  300      ATN0B1DT*70,734-8150        #300   $7E1  �

� Host  (E71)          CIS                                      �

� User ID              70613,3165                               �

� Password             xxxxxxxxxxxxxxx                          �

� New ZIF Signon ID    177000,5000                              �

� Signon Password      PC*MAGNET                                �

� New User Agremnt #   Z11D9200                                 �

� User ID - Utilities  60116,1                                  �

� Password             PCMAGUTIL                                �

���������������������������������������������������������������ͼ

It is only necessary to highlight the desired number and hit the

Enter key (carriage return!). The modem will begin to dial the

number for you. Because of the wide range of modems on the market

and considering that some can make use of special pre-dial

switches in the command line, I have elected to continue using

the complete string dialing prefix (ATDT) to precede the number.

Example: Suppose by default, your high speed modem is set to error

correction, compression, and auto CCITT handshake. This will

probably work on 95% of the systems you connect to. Now you decide

to connect with Compuserve at 300 baud to economically down load

your mail. There are two ways to do this. In either case you must

set the proper switches into the AT command string. Example:

ATN0B1DT999-9999. In this example the N0 and B1 reset the modem to

^^^^ the old bell telco standard for connects at 300 baud.

NOTE: It is very important to remember that any switches used

must follow the attention command and precede the DT command.

AT   =  Required modem attention command.

N0   =  Turn off auto speed and auto handshake.

%C0  =  Turn off MNP (error correction and compression)

B1   =  Select Bell in place of CCITT line standards.

DT   =  Dial with touch tone.

Next you have two choices. Either set your baud to 300 via the

Alt-B, hot key combination, then via Alt-F7 select 7E1 for the line

parameters. Or, do it by including them on the proper line of the

SLICK.DIR ascii text file. Always use a semicolon (;) to delimit

(or separate) the Phone description from the beginning of the AT

command. Use a pound character (#) to delimit the baud rate (use

no commas in the baud string) and a dollar sign ($) to delimit the

line parameters.

Compuserve  300;     ATN0B1DT*70,734-8150        #300   $7E1

Compuserve  300;ATN0B1DT*70,734-8150#300$7E1

^                    ^   ^

Both of these work ok, but the first looks better in the

editor. SLICK will separate out the description from the

dialing parameters in the dial directory window.

Remember, for baud:  Bad > 19,200  Good >19200 (no commas!)

Remember, for line   Bad > E71     Good >7E1   (data,parity,stop)

When using the baud and or parity fields on a dial string line in

the explanation above, the terminal will automatically reset the

line back to the original run time defaults. This auto restore

feature can be disabled, if necessary, by toggling Alt-F6. To

place the required reset modem string, go to the defaults menu

(Alt-Z) and press Alt-F8.

One of the nicest qualities of the SLICK.DIR file is it is just a

plain jane ascii text file.  This allows the use of your favorite

editor or word processor to add or change entries. SLICK comes with

SLICKED.EXE, our own editor. The menu for SLICKED (reached via F8)

is shown below.

Dialing Directory Editor Menu (SLICKED.EXE)

���������������������������������������������������������������Ŀ

�      Line 1    Col 1   Insert Indent     D:SLICK.DIR          �

�Compuserve 2400     ;ATDT*70,733-2297             #300   $7E1  �

�Compuserve  300     ;ATN0B1S37=1DT*70,734-8150                 �

�Host  (E71)         ;CIS                                       �

�User ID             ;70613,3165                                �

�Password            ;xxxxxxxxxxxxxxxxxx                        �

�ZIF Sign on ID      ;177000,5000                               �

�Signon Password     ;PC*MAGNET                                 �

�New User Agremnt #  ;Z11D9200                                  �

�User ID - Utilities ;60116,1                                   �

�Password            ;PCMAGUTIL                                 �

�EXECPC 14400 TwnCom ;ATDT*70,1-414-789-4375                    �

�EXECPC 14400 Reg    ;ATDT*70,1-414-789-4360                    �

�EXECPC 14000 USR    ;ATDT*70,1-414-789-4390                    �

�EXECPC 9600  CSP    ;ATDT*70,1-414-789-4450                    �

�EXECPC 2400         ;ATDT*70,1-414-789-4210                    �

�EXECPC 14 AT&T      ;ATDt*70,10288-1-414-789-4360              �

�EXECPC 14 AT&T      ;ATDt*70,10288-1-414-789-4375              �

�EXECPC 14 AT&T      ;ATDt*70,10288-1-414-789-4390              �

�                                                               �

�����������������������������������������������������������������

F1=TogM F2=Save F3=SvQt F4=FdRp F5=BgBk F6=EdBk F9=Prnt F10=Opt

F7=DelL F8=InsL Opt: WinSiz > UpDnLfRt, Pos w/ScrLk On, CR Exit

Although SLICKED.EXE is a rather basic editor, it has many

features that will surprise you. Read the separate SLICK-ED.DOC

file for more information on the various key commands. (which

follows fairly close to the old Word Star word processor key

mapping scheme).

SLICK by DEFAULT:

Slick comes preset for COM2, IRQ3, N81, 1200 baud. If this port

setting is CORRECT for your PC, SLICK will initialize your modem

by sending a generic modem string. If you see the modem

initialization string followed by OK appearing on your screen,

probability is your port address is valid for the default of COM2

(2F8) IRQ3.

If the string appears with an OK, you MAY STILL need to add some

special switches (modem commands) that are necessary for your

particular modem. On the other hand, if ERROR appears after SLICK

attempts to send the string, you must determine what switch in the

generic string is not compatible with your modem. This can be done

by reading your modem manual or sending each switch separately at

the terminal prompt looking for an OK reply indicating the string

is valid. Example: Let's assume the string is "ATDTM1V4&D2S7=45"

First, type ATDT, then look for an OK. Next, type ATM1, ATV4 etc.

etc. until you hit the one or ones that reply with ERROR. ANY one

bad switch will invalidate the entire string.

After the opening screen you  should  see  the  following  responses

from the modem indicating all is OK and ready to operate.

Standby... Initializing modem.

ATM1S0=3&D2V1S7=45 (or similar, [may vary])

OK

Explanation:

AT    (Modem attention command, must precede any new string)

M1    (Speaker ON until CONNECT - some like to hear modem dial)

S0=3  (Auto Answer phone line on 3rd ring - S0=0 disables auto ans.)

&D2   (enable Data state ready - allows SOME modems to hang up properly)

V1    (echo back VOICE replies to your AT commands like "OK")

S7=45 (wait 45 seconds for a carrier after answer then hang up)

Please note that you can change the initial command string that

comes pre-installed: (ATM1&D2S0=3V1S7=45), to  meet your model

modems particular requirements. This String MUST reside only

in  the Alt-0 macro as SLICK looks for it there on boot up. You

can also place a Hayes dial command in the Alt-0 macro and SLICK

will AUTOMATICALLY dial that number  first  on BOOTUP. This might

be nice for a BBS you call OFTEN.

Example: ATM1S12=0S11=70V1X4DT283-9785

^--------^

If you own a high speed modem (144DF types), here is a suggested

string to use with SLICK! -> ATN1%C1&C1&D2&Q5S36=7S7=90S0=0S95=1&W

A breakdown of the meaning of this string is down below. For

optimum performance, your modem port should be locked (set) at

the highest possible baud speed that your software will allow. In

the case of SLICK, you can lock the port at 57,600 baud. This

allows the PC processor to communicate with the UART (serial chip)

at a much higher speed than the telco connect speed, improving the

overall throughput.

Breakdown:

AT    Attention command switch.

N1    Use CCITT standards, NOT Bell.

%C0   TURN OFF THE MNP COMPRESSION! It does NO good with ZIPPED files!

&C1   Activate modem carrier detect.

&D2   Allow modem to go on hook (hang up) if DTR is dropped.

&Q5   Try Error Correction first then fallback to Normal mode.

S36=7 fallback to Normal if MNP fails

S7=90 Allow one and half minute for handshake with old grandpa modems.

S0=0  prevent terminal from answering phone (UNLESS you want HOST!)

S95=1 Enable ALL CONNECT messages.

&W    Write string to eprom.

----------------------------------[EOF]-----------------------------------