|-----------------------------------------------------------------------|

|The following document is derived from VESA Super VGA BIOS Extension	|

|document VS891001.  I have tried to make sure that all the information	|

|presented in that document is complete and comprehensive.  If you find	|

|any omissions or errors, please report them to me on the 		|

|Everex Systems BBS at (415) 683-2984.					|

|						Gary Lorensen		|

|						Everex Systems, Inc.	|

|						48571 Milmont Dr. B3	|

|						Fremont, CA   94538	|

|-----------------------------------------------------------------------|

|-----------------------------------------------------------------------|

|VESA Super VGA Standard				VS891001 10/1/89|

|-----------------------------------------------------------------------|

|Video Electronics Standards Association				|

|1330 South Bascom Ave. Suite D						|

|San Jose, CA   95128-4502						|

|FAX:(408) 286-8988							|

|-----------------------------------------------------------------------|

|Super VGA BIOS Extension						|

|Standard #VS891001							|

|									|

|Proposal								|

|    To standardize a common software interface to Super VGA video	|

|    adapters in order to provide simplified software application 	|

|    access to advanced VGA products.					|

|									|

|Summary								|

|    The standard provides a set of functions which an application	|

|    program can use to							|

|	 a) obtain information about the capabilities and 		|

|	    characteristics of a specific Super VGA implementation.	|

|	 b) control the operation of such hardware in terms of video	|

|	    mode initialization and video memory access.		|

|    The functions are provided as an extension to the VGA BIOS video	|

|    services, accessed through interrupt 10h.				|

|-----------------------------------------------------------------------|

1. Introduction

---------------

This document contains a specification for a standardized interface to

extended VGA video modes and functions.  The specification consists of

mechanisms for supporting standard extended video modes and functions

that have been approved by the main VESA committee and nonstandard video

modes that an individual VGA supplier may choose to add, in a uniform

manner that application software can utilize without having to understand

the intricate details of the particular VGA hardware.

The primary topics of this specification are definitions of extended VGA

video modes and the functions necessary for application software to

understand the characteristics of the video mode and manipulate the

extended memory associated with the video modes.

Readers of this document should already be familiar with programming VGAs

at the hardware level and Intel iAPX real mode assembly language.

Readers who are unfamiliar with programming the VGA should first read

one of the many VGA programming tutorials before attempting to understand

these extensions to the standard VGA.

|-----------------------------------------------------------------------|

2. Goals and Objectives

-----------------------

The IBM VGA (IBM and VGA are trademarks of International Business

Machines Corporation) has become a de-facto standard in the PC graphics

world.  A multitude of different VGA offerings exist in the marketplace,

each one providing BIOS or register compatibility with the IBM VGA.

More and more of these VGA compatible products implements various

supersets of the VGA standard.  These extensions range from higher

resolutions and more colors to improved performance and even some

graphics processing capabilities.  Intense competition has dramatically

improved the price/performance ratio, to the benefit of the end user.

However, several serious problems face a software developer who intends

to take advantage of these "Super VGA" environments.  (The term

"Super VGA" is used in this document as a term for video graphics

products implementing a superset of the standard IBM VGA display

adapter.)  Because these is no standard hardware implementation, the

developer is faced with widely disparate Super VGA hardware

architectures.  Lacking a common software interface, designing

applications for these environments is costly and technically difficult.

Except for applications supported by OEM-specific display drivers, very

few software packages can take advantage of the power and capabilities

of Super VGA products.

The purpose of the VESA VGA BIOS Extension is to remedy this situation.

Being a common software interface to Super VGA graphics products, the

primary objective is to enable application and system software to adapt

to and exploit the wide range of features available in these VGA

extensions.

Specifically, the VESA BIOS Extension attempts to address the following

two main issues:

a) Return information about the video environment to the application.

b) Assists the application in initializing and programming the

hardware.

2.1 Video environment information

---------------------------------

Today, an application has no standard mechanism to determine what Super

VGA hardware is is running on.  Only by knowing OEM-specific features

can an application determine the presence of a particular video board.

This often involves reading and testing registers located in I/O

addresses unique to each OEM.  By not knowing what hardware an

application is running on, few, if any, of the extended features of the

underlying hardware can be used.

The VESA BIOS Extension provides several functions to return information

about the video environment.  These functions return system level

information as well as video mode specific details.  Function 00h returns

general system level information, including an OEM identification string.

The function also returns a pointer to the supported video modes.

Function 01h may be used by the application to obtain information about

each supported video mode.  Function 03h returns the current video mode.

2.2 Programming support

-----------------------

Due to the fact that different Super VGA products have different hardware

implementations, application software has great difficulty in adapting

to each environment.  However, since each is based on the VGA hardware

architecture, differences are most common in video mode initialization

and memory mapping.  The rest of the architecture is usually kept intact,

including I/O mapped registers, video buffer location in the CPU address

space, DAC location and function, etc.

The VESA BIOS Extension provides several functions to interface to the

different Super VGA hardware implementations.  The most important of

these us Function 02h, Set Super VGA mode.  The function isolates the

application from the tedious and complicated task of setting up a video

mode.  Function 05h provides an interface to the underlying memory

mapping hardware.  Function 04h enables an application to save and

restore a Super VGA state without knowing anything of the specific

implementation.

2.3 Compatibility

-----------------

A primary design objective of the VESA BIOS Extension is to preserve

maximum compatibility to the standard VGA environment.  In no way should

the BIOS extensions compromise compatibility or performance.  Another but

related concern is to minimize changes necessary to an existing VGA BIOS.

RAM as well as ROM-based implementations of the BIOS extension should be

possible.

2.4 Scope of standard

---------------------

The purpose of the VESA BIOS Extension is to provide support for extended

VGA environments.  Thus, the underlying hardware architecture is assumed

to be a VGA.  Graphics software that drives a Super VGA board will

perform its graphics output in generally the same way it drives a

standard VGA, i.e. writing directly to a VGA-style frame buffer,

manipulating graphics controller registers, directly programming the

palette, etc.  No significant graphics processing will be done in

hardware.  For this reason, the VESA BIOS Extension does not provide any

graphics output functions, such as BitBlt, line or circle drawing, etc.

An important constraint of the functionality that can be placed into

the VESA BIOS Extension, is that ROM-space is severely limited in certain

existing BIOS implementations.

Outside the scope of this VESA BIOS Extension is handling of different

monitors and monitor timings.  Such items are dealt with in other VESA

fora.  The purpose of the VESA BIOS Extension is to provide a

standardized software interface to super VGA graphics modes, independent

of monitor and monitor timing issues.

|-----------------------------------------------------------------------|

3. Standard VGA BIOS

--------------------

A primary design goal with the VESA BIOS Extension is to minimize the

effects on the standard VGA BIOS.  Standard VGA BIOS functions should

need to be modified as little as possible.  This is important since ROM

as well as RAM-based versions of the extension may be implemented.

However, two standard VGA BIOS functions are affected by the VESA

extension.  These are Function 00h (Set video mode) and Function 0Fh

(Read current video state.)  VESA-aware applications will not set the

video mode using VGA BIOS function 00h.  Nor will such applications use

VGA BIOS function 0Fh.  VESA BIOS function 02h (Set Super VGA mode) and

03h (Get Super VGA mode) will be used instead.

However, VESA-unaware applications (such as old Pop-Up programs and other

TSRs, or the CLS command of MS-DOS), might use VGA BIOS function 0Fh to

get the present video mode.  Later it may call VGA BIOS function 00h to

restore/reinitialize the old video mode.

To make such applications work, VESA recommends that whatever value is

returned by VGA BIOS function 0Fh (it is up to the OEM to define this

number,) it can be used to reinitialize the video mode through VGA BIOS

function 00h.  Thus, the BIOS should keep track of the last Super VGA

mode in effect.

It is recommended, but not mandatory, to support output functions (such

as TTY-output, scroll, set pixel, etc) in Super VGA modes.  If the BIOS

extension doesn't support such output functions, bit D2 (Output functions

supported) of the ModeAttributes field (returned by the VESA BIOS

function 01h) should be cleared.

|-----------------------------------------------------------------------|

4. Super VGA mode number

------------------------

Standard VGA mode numbers are 7 bits wide and presently ranges from 00h

to 13h.  OEMs have defined extended video modes in the range 14h to 7Fh.

Values in the range 80h to FFh cannot be used, since VGA BIOS function

00h (Set video mode) interprets bit 7 as a flag to clear/not clear video

memory.

Due to the limitations of 7 bit mode numbers, VESA video mode numbers are

15 bits wide.  To initialize a Super VGA mode, its number is passed in

the BX register to VESA BIOS function 02h (Set Super VGA mode.)

The format of VESA mode numbers is as follows:

D0-D8 = Mode number

if D8==0, this is not a VESA defined mode

if D8==1, this is a VESA defined mode

D9-D14= Reserved by VESA for future expansion (=0)

D15   = Reserved (=0)

Thus, VESA mode numbers begin at 100h.  This mode numbering scheme

implements standard VGA mode numbers as well as OEM-defined mode numbers

as subsets of the VESA mode number.  That means that regular VGA modes

may be initialized through VESA BIOS function 02h (Set Super VGA mode),

simply by placing the mode number in BL and clearing the upper byte (BH).

OEM-defined video modes may be initialized in the same way.

To date, VESA has defined a 7 bit video mode number, 6Ah for the 800x600,

16-color, 4-plane graphics mode.  The corresponding 15-bit mode number

for this mode is 102h.

The following VESA mode numbers have been defined:

15-bit mode	7-bit mode

number		number		Resolution	Colors

------------------------------------------------------

100h		-		640x400		256

101h		-		640x480		256

102h		6Ah		800x600		16

103h		-		800x600		256

104h		-		1024x768	16

105h		-		1024x768	256

106h		-		1280x1024	16

107h		-		1280x1024	256

|-----------------------------------------------------------------------|

5. CPU Video Memory Windows

---------------------------

A standard VGA subsystem provides 256K bytes of memory and corresponding

mechanism to address this memory.  Super VGAs and their extended modes

require more than the standard 256K bytes of memory but also require that

the address space for this memory be restricted to the standard address

space for compatibility reasons.  CPU video memory windows provide a

means of accessing this extended VGA memory within the standard CPU

address space.

This chapter describes how several hardware implementations of CPU video

memory windows operate, their impact on application software design, and

relates them to the software model presented by the VESA VGA BIOS

extensions.

The VESA CPU video memory windows functions have been designed to put the

performance insensitive, nonstandard hardware functions into the BIOS

while putting the performance sensitive, standard hardware functions into

the application.  This provides portability among VGA systems together

with the performance that comes from accessing the hardware directly.  In

particular, the VESA BIOS is responsible for mapping video memory into

the CPU address space while the application is responsible for performing

the actual memory read and write operations.

This combination software and hardware interface is accomplished by

informing the application of the parameters that control the hardware

mechanism of mapping the video memory into the CPU address space and then

letting the application control the mapping within those parameters.

5.1 Hardware design considerations

----------------------------------

5.1.1 Limited to 64K/128K of CPU address space

----------------------------------------------

The first consideration in implementing extended video memory is to give

access to the memory to application software.

The standard VGA CPU address space for 16-color graphics modes is

typically at segment A000h for 64K.  This gives access to the 256K bytes

of the standard VGA, i.e. 64K per plane.  Access to the extended video

memory is accomplished by mapping portions of the video memory into the

standard VGA CPU address space.

Every super VGA hardware implementation provides a mechanism for software

to specify the offset from the start of video memory which is to be

mapped to the start of the CPU address space.  Providing both read and

write access to the mapped memory provides a necessary level of hardware

support for an application to manipulate the extended video memory.

5.1.2 Crossing CPU video memory window boundaries

-------------------------------------------------

The organization of most software algorithms which perform video

operations consists of a pair of nested loops: an outer loop over rows or

scan lines and an inner loop across the row or scan line.  The latter is

the proverbial inner loop, which is the bottle neck to high performance

software.

If a target rectangle is large enough, or poorly located, part of the

required memory may be within the video memory mapped into the CPU

address space and part of it may not be addressable by the CPU without

changing the mapping.  It is desirable that the test for re-mapping the

video memory is located outside of the inner loop.

This is typically accomplished by selecting the mapping offset of the

start of video memory to the start of the CPU address space so that at

least one entire row or scan line can be processed without changing the

video memory mapping.  There are currently no Super VGAs that allow this

offset to be specified on a byte boundary and there is a wide range among

Super VGAs in the ability to position a desired video memory location at

the start of the CPU address space.

The number of bytes between the closest two bytes in video memory that can

be placed on any single CPU address is defined as the granularity of the

window mapping function.  Some Super VGA systems allow any 4K video

memory boundary to be mapped to the start of the CPU address space, while

other Super VGA systems allow any 64K video memory boundary to be mapped

to the start of the CPU address space.  These two example systems would

have granularities of 4K and 64K, respectively.  This concept is very

similar to the bytes that can be accessed with a 16-bit pointer in an

Intel CPU before a segment register must be changed (the granularity of

the segment register or mapping, here is 16 bytes.)

Note that if the granularity is equal to the length of the CPU address

space, i.e. the least significant address bit of the hardware mapping

function is more significant that the most significant bit of the CPU

address, then the inner loop will have to contain the test for crossing

the end of beginning of the CPU address space.  This is because if the

length of the CPU address space (which is the granularity in this case)

is not evenly divisible by the length of a scan line, then the scan line

at the end of the CPU address will be in two different video memory which

cannot be mapped into the CPU address space simultaneously.

5.1.3 Operating on data from different areas

--------------------------------------------

It is sometimes required of convenient to move or combine data from two

different areas of video memory.  One example of this is storing menus in

the video memory beyond the displayed memory because there is hardware

support in all VGAs for transferring 32 bits of video data with an 8-bit

CPU read and write.  Two separately mappable CPU video memory windows

must be used if the distance between source and destination is larger

than the size of the CPU video memory window.

5.1.4 Combining data from two different windows

-----------------------------------------------

The above example of moving data from one CPU video memory window to

another CPU video memory window only required read access to one window

and only required write access to the other window.  Sometimes it is

convenient to have read access to both windows and write access to one

window.  An example of this would be a raster operation where the

resulting destination is the source data logically combined with the

original destination data.

5.2 Different types of hardware windows

---------------------------------------

Different hardware implementations of CPU video memory windows can be

supported by the VESA BIOS Extension.  The information necessary for an

application to understand the type of hardware implementation is provided

by the BIOS to the application.  There are three basic types of hardware

windowing implementations and they are described below.

The types of windowing schemes described below do not include differences

in granularity.

Also, note that it is possible for a VGA to use a CPU address space of

128K starting at segment A000h.

5.2.1 Single window systems

---------------------------

Some hardware implementions only provide a single window.  This single

window will be readable as well as writable.  However, this causes a

significant performance degradation when moving data in video memory a

distance that is larger than the CPU address space.

5.2.2 Dual window systems

-------------------------

Many Super VGAs provide two windows to facilitate moving data within

video memory.  There are two separate methods of providing two windows.

5.2.2.1 Overlapping windows

---------------------------

Some hardware implementations distinguish window A and window B by

determining if the CPU is attempting to do a memory read of a memory

write operation.  When the two windows are distinguished by whether the

CPU is trying to read or write the can, and usually do, share the same CPU

address space.  However, one window will be read only and the other will be

write only.

5.2.2.2 Nonoverlapping windows

-------------------------------

Another mechanism used by two window systems to distinguish window A and

window B is by looking at the CPU address within the total VGA CPU

address space.  When the two windows are distinguished by the CPU address

within the VGA CPU address space the windows cannot share the same

address space, but they can each be both read and written.

|-----------------------------------------------------------------------|

6. Extended VGA BIOS

--------------------

Several new BIOS calls have been defined to support Super VGA modes.  For

maximum compatibility with the standard VGA BIOS, these calls are grouped

under one function number.  This number is passed in the AH register to

the INT 10h handler.

The designated Super VGA extended function number is 4Fh.  This function

number is presently unused in most, if not all, VGA BIOS implementations.

A standard VGA BIOS performs no action when function call 4F is made.

6.1 Status information

----------------------

Every function returns status information in the AX register.  The format

of the status word is as follows:

AL == 4Fh: Function is supported

AL != 4Fh: Function is not supported

AH == 00h: Function call successful

AH == 01h: Function call failed

Software should treat a nonzero value in the AH register as a general

failure condition.  In later versions of the VESA BIOS Extension new

error codes might be defined.

6.2 Function 00h - Return Super VGA Information

-----------------------------------------------

The purpose of this function is to provide information to the calling

program about the general capabilities of the Super VGA environment.  The

function fills an information block structure at the address specified by

the caller.  The information block size is 256 bytes.

Entry:	AH = 4Fh: Super VGA support

AL = 00h: Return Super VGA Information

ES:DI   : Pointer to buffer

Exit:	AX	:Status

The information block has the following structure:

VgaInfoBlock	struc

VESASignature	db 'VESA'	;4 signature bytes

VESAVersion	dw ?		;VESA version number

OEMStringPtr	dd ?		;Pointer to OEM string

Capabilities	db 4 dup (?)	;capabilities of the video environment

VideoModePtr	dd ?		;pointer to supported Super VGA modes

VgaInfoBlock	ends

The VESASignature field contains the characters 'VESA' if this is a valid block.

The VESAVersion field specifies which VESA standard the Super VGA BIOS

conforms to.  The higher byte would specify the major version number.

The lower byte specify the minor version number.  The initial VESA

version number is 1.0.  Applications written to use the features of the

specific version of the VESA BIOS Extension, is guaranteed to work in

later versions.  The VESA BIOS Extension will be fully upwards compatible.

The OEMStringPtr is a far pointer to a null terminated OEM-defined

string.  The string may be used to identify the video chip, video board,

memory configuration, etc., to hardware specific display drivers.  There

are no restrictions on the format of the string.

The Capabilities field describes what general features are supported in

the video environment.  The bits are defined as follows:

D0-D31 = Reserved (=0)

The VideoModePtr points to a list of supported Super VGA (VESA-defined

as well as OEM-specific) mode numbers.  Each mode number occupies one

word (16 bits).  The list of mode numbers is terminated by a -1 (FFFFh).

Please refer to chapter 2 for a description of VESA mode numbers.  The

pointer could point into either ROM or RAM, depending on the specific

implementation.  Either the list would be a static string stored in ROM,

or the list would be generated at run-time in the information block (see

above) in RAM.

6.3 Function 01h - Return Super VGA Mode Information

----------------------------------------------------

This function returns information about a specific Super VGA video mode.

The function fills a mode information block structure at the address

specified by the caller.  The mode information block is maximum 256 bytes.

Some information provided by this function is implicitly defined by the

VESA mode number.  However, some Super VGA implementations might support

other video modes than those defined by VESA.  To provide access to these

modes, this function also returns various other information about the mode.

Entry:	AH = 4Fh: Super VGA support

AL = 01h: Return Super VGA Mode Information

CX =	: Super VGA video mode

ES:DI   : Pointer to buffer

Exit:	AX	:Status

The mode information block has the following structure:

ModeInfoBlock	struc

;mandatory information

ModeAttributes		dw ?		;mode attributes

WinAAttributes		db ?		;Window A attributes

WinBAttributes		db ?		;Window B attributes

WinGranularity		dw ?		;window granularity

WinSize		dw ?		;window size

WinASegment		dw ?		;Window A start segment

WinBSegment		dw ?		;Window B start segment

WinFuncPtr		dd ?		;pointer to window function

BytesPerScanLine	dw ?		;bytes per scan line

;extended information

;optional information

XResolution		dw ?		;horizontal resolution

YResolution		dw ?		;vertical resolution

XCharSize		db ?		;character cell width

YCharSize		db ?		;character cell height

NumberOfPlanes		db ?		;number of memory planes

BitsPerPixel		db ?		;bits per pixel

NumberOfBanks		db ?		;number of banks

MemoryModel		db ?		;memory model type

BankSize		db ?		;bank size in K

ModeInfoBlock	ends

The Mode Attributes field describes certain important characteristics of

the video mode.  Bit D0 specifies whether this mode can be initialized in

the present video configuration.  This bit can be used to block access to

a video mode if it requires a certain monitor type, and that this monitor

is presently not connected.  Bit D1 specifies whether extended mode

information is available.  Video modes defined by VESA will have certain

known characteristics, like resolution, number of planes, pixel format,

etc.  Doe to the severe space constraint for ROM-based implementations of

the VESA BIOS Extension, this information need not be given for

VESA-defined video modes.  Bit D2 indicates whether the BIOS has support

for output functions like TTY output, scroll, pixel output, etc. in this

mode (it is recommended, but not mandatory, that the BIOS have support

for all output functions.)

The field is defined as follows:

D0 = Mode supported in hardware

0=Mode not supported in hardware

1=Mode supported in hardware

D1 = Extended information available

0=Extended mode information not available

1=Extended mode information available

D2 = Output functions supported by BIOS

0=Output functions not supported by BIOS

1=Output functions supported by BIOS

D3 = Monochrome/color mode (see note below)

0=Monochrome mode

1=Color mode

D4 = Mode type

0=Text mode

1=Graphics mode

D5-D15 = Reserved (=0)

Note:  Monochrome modes have their CRTC address at 3B4h.  Color modes

have their CRTC address at 3D4h.  Monochrome modes have attributes in

which only bit 3 (video) and bit 4 (intensity) of the attribute

controller output are significant.  Therefore, monochrome text modes have

attributes of off, video, high intensity, blink, etc.  Monochrome

graphics modes are two plane graphics modes and have attributes of off,

video , high intensity, and blink.  Extended two color modes that have

their CRTC address at 3D4h, are color modes with one bit per pixel and

one plane.  The standard VGA modes, 06h and 11h would be classified as

color modes, while the standard VGA modes 07h and 0Fh would be classified

as monochrome modes.

The BytesPerScanLine field specifies how many bytes each logical scanline

consists of.  The logical scanline could be equal to or larger than the

displayed scanline.

The WinAAttributes and WinBAttributes  describe the characteristics of

the CPU windowing scheme such as whether the windows exist and are

read/writable, as follows:

D0 = Window supported

0=Window is not supported

1=Window is supported

D1 = Window readable

0=Window is not readable

1=Window is readable

D2 = Window writable

0=Window is not writable

1=Window is writable

D3-D7 = Reserved (=0)

WinGranularity specifies the smallest boundary, in KB, on which the

window can be placed in the video memory.

WinSize specifies the size of the window in KB.

WinASegment and WinBSegment address specify the segment addresses where

the windows are located in the CPU address space.

WinFuncPtr specifies the address of the CPU vide memory windowing

function.  The windowing function can be invoked either through VESA BIOS

Function 05h, or by calling the function directly.  A direct call will

provide faster access to the hardware paging registers the using INT 10h,

and is intended to be used by high performance applications.

The XResolution and YResolution specify the width and height of the video

mode.  In graphics modes, this resolution is in unites of pixels,  In

text modes this resolution is in unites of characters.  Note that text

mode resolutions, in units of pixels, can be obtained by multiplying

XResolution and YResolution by the cell width and height, if the extended

information is present.

The XCharSize and YCharSize specify the size of the character cell in

pixels.

The NumberOfPlanes field specifies the number of bits that define the

color of one pixel.  16-color and 256-color graphics modes would specify

4 and 8 respectively.  Nonstandard memory organizations can be specified

using this field and the NumberOfPlanes field.  For example, 1 16-color

packed pixel mode would be described as having 1 plane and 4 bits per

pixel.

The MemoryModel field specifies the general type of memory organization

used in this mode.  The following models have been defined:

00h = Text mode

01h = CGA graphics

02h = Hercules graphics

03h = 4-plane planar

04h = Packed pixel

05h = Non-chain 4, 256 color

06h-0Fh = Reserved, to be defined by VESA

10h-FFh = To be defined by OEM

NumberOfBanks defines the number of banks in which the scan lines are

grouped.  The remainder from dividing the scan line number by the number

of banks is the bank that contains the scan line and the quotient is the

scan line number within the bank.  For example, CGA graphics modes have

two banks and Hercules graphics mode has four banks.  For modes that

don't have scanline banks (such s VGA modes 0Dh-13h), this field should

be set to 1.

The BankSize field specifies the size of a bank (group of scan lines) in

units of 1K.  For CGA and Hercules graphics modes this is 8, as each bank

is 8192 bytes in length.  For modes that don't have scanline banks (such

as VGA modes 0Dh-13h), this field should be set to 0.

6.4 Function 02h - Set Super VGA Video Mode

-------------------------------------------

This function initializes a Super VGA video mode.  The BX register

contains the Super VGA mode to set.  The format of VESA mode numbers is

described in chapter 2.  If the mode cannot be set, the BIOS should leave

the video environment unchanged and return a failure error code.

Entry:	AH = 4Fh: Super VGA support

AL = 02h: Set Super VGA Video Mode

BX =	: Video mode

D0-D14 = Video mode

D15 = Clear memory flag

0=Clear video memory

1=Don't clear video memory

Exit:	AX	:Status

6.5 Function 03h - Return Super VGA Video Mode

----------------------------------------------

This function returns the current video mode in BX.  The format of VESA

video mode numbers is described in chapter 2 of this document.

Entry:	AH = 4Fh: Super VGA support

AL = 03h: Return current video mode

Exit:	AX	:Status

BX =	:Current video mode

Note: In a standard VGA BIOS, function 0Fh (Read current video state)

returns the current video mode in the AL register.  In D7 of AL, it also

returns the status of the memory clear bit (D7 at 40:87).  This bit is

set if the mode was set without clearing memory.  In this Super VGA

function, the memory clear bit will not be returned in BX since the

purpose of the function is to return the video mode only.  If an

application wants to obtain the memory clear bit, it should call VGA BIOS

function 0Fh.

6.6 Function 04h - Save/Restore Super VGA Video State

-----------------------------------------------------

These functions provide a mechanism ti save and restore the Super VGA

video state.  The functions are a superset of the three subfunctions

under standard VGA BIOS function 1Ch (Save/Restore video state).  The

complete Super VGA video state (except video memory) should be

savable/restorable by setting the requested states mask (in the CX

register) to 000Fh.

Entry:	AH = 4Fh: Super VGA support

AL = 04h: Save/Restore Super VGA Video State

CX = Requested states

D0 = Save/Restore video hardware state

D1 = Save/Restore video BIOS data state

D2 = Save/Restore video DAC state

D3 = Save/Restore video Super VGA state

DL = Sub-function

00=Return save/restore state buffer size

01=Save Super VGA video state

02=Restore Super VGA video state

ES:BX = pointer to buffer (01,02)

Exit:	AX	:Status

BX =	:Number of 64-byte blocks to hold the state

buffer (00)

Note:  Due to the goal of complete compatibility with the VGA

environment, the standard VGA BIOS function 1Ch (Save/Restore VGA State)

has not been extended to save the Super VGA video state.  VGA BIOS

compatibility requires that function 1Ch returns a specific buffer size

with specific contents, in which there is no room for the Super VGA state.

6.7 - CPU Window Memory Window Control

--------------------------------------

This function sets or gets the position of the specific window in the

video memory.  The function allows direct access to the hardware paging

registers.  To use this function properly, the software should use VESA

BIOS Function 01h (Return Super VGA mode information) to determine the

size, location and granularity of the windows.

Entry:	AH = 4Fh: Super VGA support

AL = 05h: Super VGA Video Memory Window Control

BH = Sub-function

00=Select Super VGA Video Memory Window

01=Return Super VGA Video Memory Window

BL = Window A/B (0/1)

DX = Window position in video memory (in window

granularity units)

Exit:	AX	:Status

DX = Window position in video memory (in window

granularity units)

Note: This function is also directly accessible through a far call from

the application.  The address os the BIOS function may be obtained by

using the VESA BIOS Function 01h, Return Super VGA Mode Information.  A

field in the ModeInfoBlock contains the address of this function.  Note

that this function may be different among video modes in a particular

BIOS implementation so the function pointer should be obtained after each

set mode.

In the far call version, no status information is returned to the

application.  Also, in the far call version, the AX and DX registers will

be destroyed.  Therefore if AX and/or DX must be preserved, the

application must do so prior to making the far call.

The application must load the input arguments in BH, BL, and DX (for set

window) but does not need to load either AH or AL in order to use the far

call version of this function.

|-----------------------------------------------------------------------|

7. Application Example

----------------------

The following sequence illustrates how an application would interface to

the VESA BIOS Extension.  The hypothetical application is VESA-aware and

calls the VESA BIOS functions.  However, the application is not limited

to supporting just VESA-defined video modes.  Thus, it will  inquire what

video modes are available before setting up the video mode.

1)	The application would first allocate a 256 byte buffer.  This

buffer will be used by the VESA BIOS to return information about

the video environment.  Some applications will statically

allocate this buffer.  Others will use system calls to

temporarily obtain buffer space.

2)	The application would then call VESA BIOS function 00h (Return

Super VGA Information.)  If the AX register does not contain

004Fh on return from the function call, the application can

determine that the VESA BIOS Extension is not present and handle

such situation.

If no error code is passed in AX, the function call was

successful.  The buffer has been filled by the VESA BIOS

Extension with various information.  The application can verify

that indeed this is a valid VESA block by identifying the

characters 'VESA'  in the beginning of the block.  The

application can inspect the VESAVersion field to determine

whether the VESA BIOS Extension has sufficient functionality.

The application may use the OEMStringPtr to locate OEM-specific

information.

Finally, the application can obtain a list of the supported Super

VGA modes by using the VideoModePtr.  This field points to a list

of the video modes supported by the video environment.

3)	The application would then create a new buffer and call the VESA

BIOS function 01h (Return Super VGA Mode Information), to obtain

information about the supported video modes.  Using the

VideoModePtr, obtained in step 2 above, the application would

call this function with a new mode number until a suitable video

mode is found.  If no appropriate video mode is found, it's up

to the application to handle this situation.

The Return Super VGA Mode Information function fills a buffer

specified by the application with information describing the

features of the video mode.  The data block contains all the

information an application needs to take advantage of the video

mode.

The application would examine the ModeAttributes field.  To

verify that the mode indeed is supported, the application would

inspect bit D0.  If Do is cleared, then the mode is not supported

by the hardware.  This might happen if a specific mode requires a

certain type of monitor, but that monitor is not present.

4)	After the application has selected a video mode, the next step

is to initialize the mode.  However, the application might first

want to save the present video mode.  When the application exits,

this mode would be restored.  To obtain the present video mode,

the VESA BIOS function 03h (Get Super VGA Mode), would be used.

If a non-VESA (standard VGA or OEM-specific) mode is in effect,

only the lower byte in the mode number is filled.  The upper byte

is cleared.

5)	To initialize the video mode, the application would use VESA BIOS

function 02h (Set Super VGA mode).  The application has from this

point on full access to the VGA hardware and video memory.

6)	When the application is about to terminate, it would restore the

prior video mode.  The prior video mode, obtained in step 4 above

could be either a standard VGA mode, OEM-specific mode, or

VESA-supported mode.  It would reinitialize the video mode by

calling VESA BIOS function 02h (Set Super VGA mode).  The

application would then exit.

|-----------------------------------------------------------------------|