NOVELL DOS ODI DRIVER FOR

"PLUS" FAMILY LAN ADAPTER

INSTALLING DOS ODI DRIVER

INSTALLATION PROCEDURE:

1) Refer to Novell DOS ODI installation menu.

2) Copy NET.CFG to the directory where ODI driver

resides. If NET.CFG does exist, append the

contents of the new NET.CFG to the end of the

existing one.  This file is used to configure the

entire system, not just the driver.

USING NET.CFG

1) There is no need to use the NET.CFG file if the

Western Digital PLUS adapter is an AT adapter

configured at Base I/O Address = 0x280,

Interrupt IRQ = 3 and RAM Base Address = 0xD000.

2) If the adapter being used is software configurable

then only the Base I/O Address (if it is not 0x280)

must be specified.

3) To change the Base I/O Address, use the line in

NET.CFG which says

Port #1 280 20

and change 280 to the appropriate Base I/O Address

4) To change the Interrupt IRQ, use the line in

NET.CFG which says

Int #1 3

and change '3' to the appropriate Interrupt IRQ

5) To change the RAM Base Address, use the line in

NET.CFG which says

Mem #1 000D0000 2000/10

and change '000D0000' to the appropriate RAM Base

Address.

e.g.  Mem #1 000C8000 2000/10

This example will cause the driver to use 0xC8000

as the RAM Base Address

6) For use with Micro Channel machines, the PS/2 Slot

entry in the NET.CFG file must be used.  Use the

line which says

PS/2 Slot ?

and change the '?' to the slot number desired.

Note: The '#' in the first column of the NET.CFG file

denotes an ignored line.  This must be removed from

all used lines and inserted into all unused lines.

(Or unused lines may be deleted altogether.)

7) To change protocol name, use the line in NET.CFG which

says

Protocol <name> h <frame type>

e.g. Protocol IPX 8137 Ethernet_II

8)  To configure link-level frame type, use the line in

NET.CFG which says

Frame <name>   or

Envelope Type <name>

e.g. Frame ETHERNET_802.2

Frame ETHERNET_802.3

Frame ETHERNET_II

Frame ETHERNET_SNAP

This is the list of error messages that the driver displays:

1. "Unknown Board ID"

means the driver does not recognize the type of adapter installed.

2. "Selected IRQ value is invalid for this adapter"

means the IRQ value either read from software-configurable

adapter or specified in NET.CFG file is not one of the IRQ

supported by the adapter.

3. "Selected I/O base address is out of range"

means the I/O base address specified in the NET.CFG file for

AT-bus adapter is out of range.

4. "Memory base address is invalid for this adapter"

means the memory base address is not usable for the configuration

or the adapter installed.

5. "Can not find Micro Channel adapter"

means could not find a micro-channel adapter in the machine.

6. "Can not find an adapter at the specified configuration"

means could not find the adapter at the I/O base address

specified in the NET.CFG for AT-bus adapter.

6. "Incorrect setting in the NET.CFG file"

means either could not find a micro-channel adapter, or

could not find AT-bus adapter in the default configuration

where there is no configuration specified in the NET.CFG file.

7. "No PS/2 Slot specified, search for the first one"

means searching for a micro-channel adapter since none specified

in the NET.CFG file.

8. "Hardware failure during card initialize"

means the adapter does not pass through the initialization process.