AutoWx.DOC

(C) 1994 N5KOA

VER 4.x   7/94

THE AUTOWX PROGRAM WAS WRITTEN BY Les Blalock, N5KOA, AND IS AVAILABLE FROM

HIM AT THE BELOW ADDRESS.  DO NOT SEND DISKS, BUT ABOUT $5.00 to COVER DISK

AND POSTAGE:    1711 Pagewood

Odessa, TX  79761

(915) 332-7112 day

332-1942 fax

367-0408 eve

THERE IS NO LONGER ROOM TO PROVIDE THIS PROGRAM AS PART OF APRS DISTRIBUTION.

APRS (Bob Bruninga) has nothing to do with this program and is providing this

DOC file for your information only.

AutoWX (Auto-Weather) is an IBM compatible packet program that accepts

weather data from the Ultimeter II weather station and makes the data

available on packet by direct connect requests and/or via automatic

beacons (in APRS beacon format).  The program assumes you are using two

serial ports (COM1 and COM2) and that the U-II is connected to one, the

TNC to the other.  Use ALT-H for online help.  The program works very

well on an XT with 256K RAM and a single floppy disk drive.  XT's are

often GIVEN away now days and one could be dedicated full time to this

application.

Compatibility with earlier versions:

The WX.CFG file contains a logger filename as of version 3.x  Note the

contents of your old WX.CFG for reference if you want, and rename or

delete the old file.  Run AutoWx to create a new configuration file.

Read PROGRAM UPDATES section of this documentation for new features.

STARTUP / CONFIGURATION Parameters

---------------------------------------------------------------

= THE ULTIITSR PROGRAM SHOULD NOT BE USED WHILE AUTOWX IS RUNNING =

---------------------------------------------------------------

(The TNC ECHO should be ON, MONITOR OFF, AUTOLF OFF and MAXUSERS 1.)

The first time AutoWx is executed, or if the WX.CFG file is not found,

the user is prompted for configuration information.  The COM port of the

TNC must be identified as (COM) "1" or "2".  The other COM port will

automatically be assigned to the Ultimeter II.

Supported baud rates for the TNC port are: 1200,2400,4800 & 9600

Your station's latitude and longitude are necessary to beacon in a format

recognized by the Automatic Packet Routing System (APRS by WB4APR).

There is very little error validation in the entry of the coordinates so

be very careful that they are entered in the format shown at the config

prompt.  Null entries at the Latitude and Longitude prompts will result

in "0000.00N" and "00000.00W".

A temperature adjustment value is included because the Ultimeter II

temperature sensor can be affected if additional lengths of cable are

attached to it.  The default value for the temperature adjustment is 56

which will be correct if the display on the U-II is correct.

Weather data can be tracked on the AutoWx screen in three different

formats.  If you enter zero (0), the status line at the bottom of the

screen will not show weather data but will show the last time someone

connected and requested weather data and how many times weather has been

requested since the program was last started.   A value of one (1) will

cause weather data to be printed on the screen every time an update is

performed.  (Usually every 5 minutes - see more information in this text

when the update is discussed.)  A value of two (2) will cause the status

line to be updated every time the program updates weather data.  (If your

computer has more than 640K RAM you can use option 1 and scroll back

through the data with the PC Magazine utility PERUSE.COM once in

awhile.)  The default value is zero and entry of any value over two will

revert to the value of two.

Note that when prompted for the Log File Name, your entry should NOT

include an extension.  The extension .LOG is automatically added.

After the configuration information is entered, it is written to the

ASCII file WX.CFG  The configuration can be changed either by deleting

the file and running AutoWx or by editing the file.  NOTE - If you edit

the WX.CFG file, remember the values for Temp Adjustment and Status

Display Format are not in quotes - the others are.  Each element is

separated by a comma.  Also, the WX.CFG file must be in the same

directory as AutoWx.EXE.

WX.DAT file

The WX.DAT file is an ASCII file that tracks the high and low

temperatures and high wind speed for the day.  If the file is not

present when AutoWx is executed it will be created.  The file is updated

whenever the Hi's and Lo's are edited (ALT-E) and when the program is

terminated with ALT-Q.  Actually you must answer the "write data" prompt

with "Y" or "Yes" - the default (No) will not write the file.

The Hi's & Lo's can be edited by typing ALT-E during the program.  At

that point you are prompted to enter  1,2 or 3 depending upon which item

you want to change.  If you enter no value the edit is aborted and the

program is back in the main loop.  If you change a value, enter the new

information in the same format as displayed.

The Hi's & Lo's displayed with connected requests are in memory only and

will be lost unless you write them to the data file when terminating the

program or editing the file.

If you stop the program and restart it on the same date, the data in the

WX.DAT file will be read and used by the program.  If the date in WX.DAT

is different than the system date on your computer, the data will be

ignored and new Hi's and Lo's will be tracked beginning with the first

data read on the wx port.  At midnite new Hi's and Lo's are begun.

The first line of the WX.DAT file is a title included only to help you

read the data when looking at the file with an editor or list program.

The contents inside the quotes on the first line are totally irrelavent

to AutoWx.

UPDATE

As of version 4.x the U-II data is read continuously and logged to disk

every 5 minutes.  Whenever the data is logged, it is also printed on the

screen (if you selected option 1 or 2 in the configuration of the status

line).  This feature can be toggled off with ALT-U but this will also

disable the logging of data to the log file.  If you are going to

disconnect or otherwise disable the weather input, toggle the updates

off with ALT-U to keep the program from hanging up during the update

sequence.  While the updates are toggled OFF, connected requests for

weather (//wx) will receive a message stating that "Weather data is

temporarily unavailable...".

When UPDATE is toggled, the timer is not reset so when UPDATE is

toggled back ON, if the programmed time (5 min.) has passed while

UPDATE was OFF, the update will be performed immediately.

If AutoWx is started with the command line parameter "/SHORT", the

weather node response will NOT include hi/lo temps/wind.  Only the

current weather data will be sent.  The startup syntax from DOS would

be:             AUTOWX /SHORT

WEATHER NODE (for connected requests)

If another packet station connects and sends "//wx", (without the

quotes - upper or lower case), AutoWx will update and send the current

weather data and Hi's and Lo's for the day.

I include instructions in my Connect Text (CTEXT) showing:

"Send //wx for weather data..."

As of version 4 it is not really necessary for a user to request the

weather info more than once because the wind speed will show the highest

windspeed recorded since data was last logged to disk.  The report is

displayed on screen if you enter the //wx directly from the keyboard but

the TNC cmd: 's clutter the display.

(The program does not indicate wind chill but I will gladly add that data

if someone will furnish me with an algorithm to calculate it from wind

speed and temperature or from a table if the table is efficient.)

While a station is connected, the APRS Beacon is suspended.  Also, if

UPDATES are OFF a connected user will see the message, "Weather

data temporarily not available...".  The last ten stations that

connected can be viewed by pressing the PgDn key.

Users may disconnect by sending //bye.

BEACONS

Automatic beacons can be toggled ON/OFF with ALT-B.  The beacon will be

digi'd through any valid path specified in the UNPROTO tnc parameter.

When Beacon is toggled ON, the user is prompted for the interval (in

minutes;  default = 30 minutes) and for any text to be tagged on the end

of the weather data.  The Beacon time will be in the nearest 5-minute

increment regardless of the value entered;  for example, if you enter 9

minutes, the actual beacon time will be 10 minutes.  Beacons are

transmitted only when the log file is updated.

The beacon will automatically include the Longitude and Latitude from

the config file.   If you enter text to be appended to the beacon

(Btext), it will be limited to 30 characters.

When the Beacon is ON, it can be forced to xmit immediately by typing

ALT-T.

The Beacon can be turned off/on remotely with //BEACON.  Make sure that

you have the correct path specified in UNPROTO.

LOG File

Every 5 minutes the program updates the binary logfile named in the

WX.CFG file.  This file can be read with the Ultimeter II Data Display

program (Vers 1 & 2).   (Log files can be appended to each other by

using the COPY /B command.  Example: copy /b file1.log+file2.log bigfile.log

See your DOS manual for details on COPYing binary files.)

NOTE:  Logging is suspended while UPDATE is toggled OFF.

The U-II Data Display program assumes that all log entries are at five

minute intervals.  Therefore, if you suspend logging for more than a

couple of minutes, the Data Display program will be reading data that

was logged after the time displayed in the Data Display program.  For

example:  If you suspended the logger with ALT-U at 10:28 and did not

resume logging until 10:58, the Display program will show the data

logged at 11:00 as if it were logged at 10:30 and there will be thirty

minutes of no data at the end of the day.  See Log File Notes for more

information.

TERMINAL Mode

If you use the program as a packet terminal to connect to other

stations temporarily, turn the Beacon OFF.  If UPDATEs are left ON, the

logger will continue to log data but your screen will also continue to

receive updated data every five minutes and the program will seem to

halt while the U-II data is read.  The program will ignore the //wx

request if you disable it with the ALT-X toggle.  If you plan an

extended QSO, use another program.  AutoWx makes a lousy terminal

program anyway.

NOTES:

Log File Notes

The binary log file is much like the one created by the Ultimeter-II

Data Logger and is very efficient.  You can probably get a whole year's

data on a 360K floppy.  The AutoWx program must run for at least ten

minutes before any useable data is logged.  Also, starting and stopping

the AutoWx program before any useable data is logged can cause errors in

the display.  The U-II Data Display program reads a file header in the

log file to determine the starting date and time but all data entries

after the header are assumed to be in five minute intervals.  The AutoWx

program writes a new date/time header each time it starts but if logging

is suspended by the ALT-U toggle the data may miss the 5 minute interval

and therefore be incorrectly displayed by the U-II Data Display program.

If the logger is suspended for more than 10 minutes, quit the AutoWx

program and restart it to avoid the problem.  Although data may be

missing for the suspended period, the data logged will be displayed with

the correct time if you follow this procedure.   If the program fails to

see a response from the U-II it will hang up and you will have to

re-boot to recover.

The AutoWx program reads the Ultimeter II port continuously and, when

logging, logs the highest windspeed recorded during the last 5 minutes.

However, the U-II data sends serial data every 4 seconds (on average)

but reads sensors about every 1.5 seconds and there may be differences

in the high windspeed recorded on the U-II display and that recorded by

AutoWx.

--------------------------

AutoWx is released for the use by other licensed amateur radio operators

and no payment or registration fees are necessary.  The program is

copyright protected and I choose to retain all rights to the code.  The

source code (QuickBasic 4.5) will not be made available unless somebody

convinces me they can substantially contribute to the project.

Comments and questions can be directed to N5KOA at N5RKN.#WTX (Odessa,

TX).  Please drop me a line if you try the program.  Disk copies are

available only when accompanied with enough green stuff to pay for the

postage, disk, etc. DO NOT SEND DISKETTES!

Ultimeter II weather station sources:

PEET BROS. as advertised in QST

Radio Com Sales  (Larry, N5VFW)

Box 118814

Carrollton, TX  75011

(214) 492-3824

APRS is a map-based packet program by WB4APR, Bob Bruniga, Annapolis, MD

WXSTAT.EXE - a program to monitor Ultimeter II weather data in realtime.

PROGRAM UPATES

Ver. 1.x - allows only connected weather requests.

Ver. 2.x - adds Updates to screen and Beacons

Ver. 3.x - adds data logger

Ver. 3.3f - allows list of last 10 connected stations with PgDn

CONNECT.LOG remains - see HELP screen

Ver. 3.3g - tweaked timer routines

Ver. 3.3h - Fixed config write routine

Ver. 3.3i - sorts connected stations (listed with PgDn) by Date/Time

Ver. 4.x  - now reads and logs highest windspeed during the past five

minutes (reads wx data approx.  every 4 seconds)

quicker //wx response

4.2  - logging will synchronize to computer time (XX:X5 or XX:X0)

4.3  - Beacon interval will occur only with logging (5 minute

intervals)

4.3c - will abort if no U-II found on indicated port

4.3e - Hi's & Lo's written to ASCII file WX.HST

Ver. 4.4a - fixes status line display for display option #2

4.5x - Tuned up log entries for rain amounts

Flush write buffer every hour

Removed ASCII file CONNECT.LOG

Allow null responses to Lat/Long entries in config

4.5d   Allow serial data starting with #  and allow /KPH startup

to correct windspeed  (see Errors/Bugs text)

4.5e   Fixed bug in 4.5d that caused Re-init errors

Errors/Bugs

All versions after Ver 3.0 log data for the binary log file every five

minutes but data is not physically written to disk until the write

buffer is full (one disk sector).  If power to the computer is

interrupted, any data in the buffer is lost.  As the program writes only

4 bytes (on average) at a time, 10 hours or more could be lost.  This

problem is characteristic of DOS I/O and a suitable solution has not

been determined yet.  To write the data every five minutes would result

in, what I feel is, excessive drive wear.  I will continue to look for

alternatives.  In the meantime, the buffer will flush every hour at 50

minutes after the hour.

It has been reported that there are two versions of the U-II in regard

to serial data output.  My U-II sends an asterisk (*) at the beginning

of each data stream and there is apparently a version that sends the

pound sign (#).  The (*) is supposed to indicate wind speed in MPH

while the (#) indicates KPH.  If your U-II sends in KPH (#), start the

AutoWx program with the switch /KPH.  Example:  AUTOWX /KPH

If you find a bug in the program, check for a later version on the ARRL

BBS (201) 666-0578.  If the bug exists in the newest version please let

me know so I can fix it.  I also welcome any suggestions or comments so

please let me know if you like or dislike the program or if you would

like to see other features added.  I will also consider customizing your

version if you will pay for the disk and postage.

Les Blalock, N5KOA

1711 Pagewood

Odessa, TX  79761

(915) 332-7112 day

332-1942 fax

367-0408 eve

TIPS

If you are using a hard drive, create a batch file that copies the

WX.HST file and the xxxxx.LOG file and then EXITs.  Shell to DOS from

AutoWx by typing Alt-D, type the name of your batch file, and the files

can be copied to a floppy disk and the program resumed very quickly.

Find a HEX-to-BIN/BIN-to-HEX program  (I'll send one if you can't find

one) and use the program to edit log files.  I create seperate log files

for each month and, using the COPY /B command, I can make one for the year

or any other particular time period.  Also, by working with smaller log

files, you don't have to wait forever when running the Data Display

Viewer program to graph your data.

Import WX.HST file entries to a database program and create custom

statistical reports and graphs.  The WX.HST file is easy to import as a

"comma-delimited ASCII file" into most popular database programs and

many spreadsheet programs.

If a BBS in your town supports modem connections, consider leaving your

weather node on the BBS frequency (or a gateway port) so the node can be

accessed by telephone, too.