\IR{-o} \c{-o} option

\IR{-f} \c{-f} option

\IR{-l} \c{-l} option

\IR{-s} \c{-s} option

\IR{-i} \c{-i} option

\IR{-p} \c{-p} option

\IR{-d} \c{-d} option

\IR{-e} \c{-e} option

\IR{-a} \c{-a} option

\IR{-w} \c{-w} option

\IR{!=} \c{!=} operator

\IR{$ here} \c{$} Here token

\IR{$$} \c{$$} token

\IR{%} \c{%} operator

\IR{%%} \c{%%} operator

\IR{%+1} \c{%+1} and \c{%-1} syntax

\IA{%-1}{%+1}

\IR{%0} \c{%0} parameter count

\IR{&} \c{&} operator

\IR{&&} \c{&&} operator

\IR{*} \c{*} operator

\IR{..@} \c{..@} symbol prefix

\IR{/} \c{/} operator

\IR{//} \c{//} operator

\IR{<} \c{<} operator

\IR{<<} \c{<<} operator

\IR{<=} \c{<=} operator

\IR{<>} \c{<>} operator

\IR{=} \c{=} operator

\IR{==} \c{==} operator

\IR{>} \c{>} operator

\IR{>=} \c{>=} operator

\IR{>>} \c{>>} operator

\IR{?} \c{?} MASM syntax

\IR{^} \c{^} operator

\IR{^^} \c{^^} operator

\IR{|} \c{|} operator

\IR{||} \c{||} operator

\IR{~} \c{~} operator

\IR{%$} \c{%$} and \c{%$$} prefixes

\IA{%$$}{%$}

\IR{+ opaddition} \c{+} operator, binary

\IR{+ opunary} \c{+} operator, unary

\IR{+ modifier} \c{+} modifier

\IR{- opsubtraction} \c{-} operator, binary

\IR{- opunary} \c{-} operator, unary

\IR{alignment, in bin sections} alignment, in \c{bin} sections

\IR{alignment, in elf sections} alignment, in \c{elf} sections

\IR{alignment, in win32 sections} alignment, in \c{win32} sections

\IR{alignment, of elf common variables} alignment, of \c{elf} common

variables

\IR{alignment, in obj sections} alignment, in \c{obj} sections

\IR{a.out, bsd version} \c{a.out}, BSD version

\IR{a.out, linux version} \c{a.out}, Linux version

\IR{autoconf} Autoconf

\IR{bitwise and} bitwise AND

\IR{bitwise or} bitwise OR

\IR{bitwise xor} bitwise XOR

\IR{block ifs} block IFs

\IR{borland pascal} Borland, Pascal

\IR{borland's win32 compilers} Borland, Win32 compilers

\IR{braces, after % sign} braces, after \c{%} sign

\IR{bsd} BSD

\IR{c calling convention} C calling convention

\IR{c symbol names} C symbol names

\IA{critical expressions}{critical expression}

\IA{command line}{command-line}

\IA{case sensitivity}{case sensitive}

\IA{case-sensitive}{case sensitive}

\IA{case-insensitive}{case sensitive}

\IA{character constants}{character constant}

\IR{common object file format} Common Object File Format

\IR{common variables, alignment in elf} common variables, alignment

in \c{elf}

\IR{common, elf extensions to} \c{COMMON}, \c{elf} extensions to

\IR{common, obj extensions to} \c{COMMON}, \c{obj} extensions to

\IR{declaring structure} declaring structures

\IR{default-wrt mechanism} default-\c{WRT} mechanism

\IR{devpac} DevPac

\IR{djgpp} DJGPP

\IR{dll symbols, exporting} DLL symbols, exporting

\IR{dll symbols, importing} DLL symbols, importing

\IR{dos} DOS

\IR{dos archive} DOS archive

\IR{dos source archive} DOS source archive

\IA{effective address}{effective addresses}

\IA{effective-address}{effective addresses}

\IR{elf shared libraries} \c{elf} shared libraries

\IR{freebsd} FreeBSD

\IR{freelink} FreeLink

\IR{functions, c calling convention} functions, C calling convention

\IR{functions, pascal calling convention} functions, Pascal calling

convention

\IR{global, aoutb extensions to} \c{GLOBAL}, \c{aoutb} extensions to

\IR{global, elf extensions to} \c{GLOBAL}, \c{elf} extensions to

\IR{got} GOT

\IR{got relocations} \c{GOT} relocations

\IR{gotoff relocation} \c{GOTOFF} relocations

\IR{gotpc relocation} \c{GOTPC} relocations

\IR{linux elf} Linux ELF

\IR{logical and} logical AND

\IR{logical or} logical OR

\IR{logical xor} logical XOR

\IR{masm} MASM

\IA{memory reference}{memory references}

\IA{misc directory}{misc subdirectory}

\IR{misc subdirectory} \c{misc} subdirectory

\IR{microsoft omf} Microsoft OMF

\IR{mmx registers} MMX registers

\IA{modr/m}{modr/m byte}

\IR{modr/m byte} ModR/M byte

\IR{ms-dos} MS-DOS

\IR{ms-dos device drivers} MS-DOS device drivers

\IR{multipush} \c{multipush} macro

\IR{nasm version} NASM version

\IR{netbsd} NetBSD

\IR{omf} OMF

\IR{openbsd} OpenBSD

\IR{operating-system} operating system

\IR{os/2} OS/2

\IR{pascal calling convention}Pascal calling convention

\IR{passes} passes, assembly

\IR{perl} Perl

\IR{pic} PIC

\IR{pharlap} PharLap

\IR{plt} PLT

\IR{plt} \c{PLT} relocations

\IA{pre-defining macros}{pre-define}

\IR{qbasic} QBasic

\IA{rdoff subdirectory}{rdoff}

\IR{rdoff} \c{rdoff} subdirectory

\IR{relocatable dynamic object file format} Relocatable Dynamic

Object File Format

\IR{relocations, pic-specific} relocations, PIC-specific

\IA{repeating}{repeating code}

\IR{section alignment, in elf} section alignment, in \c{elf}

\IR{section alignment, in bin} section alignment, in \c{bin}

\IR{section alignment, in obj} section alignment, in \c{obj}

\IR{section alignment, in win32} section alignment, in \c{win32}

\IR{section, elf extensions to} \c{SECTION}, \c{elf} extensions to

\IR{section, win32 extensions to} \c{SECTION}, \c{win32} extensions to

\IR{segment alignment, in bin} segment alignment, in \c{bin}

\IR{segment alignment, in obj} segment alignment, in \c{obj}

\IR{segment, obj extensions to} \c{SEGMENT}, \c{elf} extensions to

\IR{segment names, borland pascal} segment names, Borland Pascal

\IR{shift commane} \c{shift} command

\IA{sib}{sib byte}

\IR{sib byte} SIB byte

\IA{standard section names}{standardised section names}

\IR{symbols, exporting from dlls} symbols, exporting from DLLs

\IR{symbols, importing from dlls} symbols, importing from DLLs

\IR{tasm} TASM

\IR{test subdirectory} \c{test} subdirectory

\IR{tlink} TLINK

\IR{underscore, in c symbols} underscore, in C symbols

\IR{unix} Unix

\IR{unix source archive} Unix source archive

\IR{val} VAL

\IR{version number of nasm} version number of NASM

\IR{visual c++} Visual C++

\IR{www page} WWW page

\IR{win32} Win32

\IR{windows} Windows

\IR{windows 95} Windows 95

\IR{windows nt} Windows NT

\# \IC{program entry point}{entry point, program}

\# \IC{program entry point}{start point, program}

\# \IC{MS-DOS device drivers}{device drivers, MS-DOS}

\# \IC{16-bit mode, versus 32-bit mode}{32-bit mode, versus 16-bit mode}

\# \IC{c symbol names}{symbol names, in C}

\C{intro} Introduction

\H{whatsnasm} What Is NASM?

The Netwide Assembler, NASM, is an 80x86 assembler designed for

portability and modularity. It supports a range of object file

formats, including Linux \c{a.out} and ELF, NetBSD/FreeBSD, COFF,

Microsoft 16-bit OBJ and Win32. It will also output plain binary

files. Its syntax is designed to be simple and easy to understand,

similar to Intel's but less complex. It supports Pentium, P6 and MMX

opcodes, and has macro capability.

\S{yaasm} Why Yet Another Assembler?

The Netwide Assembler grew out of an idea on \i\c{comp.lang.asm.x86}

(or possibly \i\c{alt.lang.asm} - I forget which), which was

essentially that there didn't seem to be a good free x86-series

assembler around, and that maybe someone ought to write one.

\b \i\c{a86} is good, but not free, and in particular you don't get any

32-bit capability until you pay. It's DOS only, too.

\b \i\c{gas} is free, and ports over DOS and Unix, but it's not very good,

since it's designed to be a back end to \i\c{gcc}, which always feeds

it correct code. So its error checking is minimal. Also, its syntax

is horrible, from the point of view of anyone trying to actually

\e{write} anything in it. Plus you can't write 16-bit code in it

(properly).

\b \i\c{as86} is Linux-specific, and (my version at least) doesn't seem to

have much (or any) documentation.

\b \i{MASM} isn't very good, and it's expensive, and it runs only under

DOS.

\b \i{TASM} is better, but still strives for \i{MASM} compatibility, which

means millions of directives and tons of red tape. And its syntax is

essentially \i{MASM}'s, with the contradictions and quirks that entails

(although it sorts out some of those by means of Ideal mode). It's

expensive too. And it's DOS-only.

So here, for your coding pleasure, is NASM. At present it's

still in prototype stage - we don't promise that it can outperform

any of these assemblers. But please, \e{please} send us bug reports,

fixes, helpful information, and anything else you can get your hands

on (and thanks to the many people who've done this already! You all

know who you are), and we'll improve it out of all recognition.

Again.

\S{legal} Licence Conditions

Please see the file \c{Licence}, supplied as part of any NASM

distribution archive, for the \i{licence} conditions under which you

may use NASM.

\H{contact} Contact Information

NASM has a \i{WWW page} at

\W{http://www.cryogen.com/Nasm}\c{http://www.cryogen.com/Nasm}. The

authors are \i{e\-mail}able as

\W{mailto:jules@earthcorp.com}\c{jules@earthcorp.com} and

\W{mailto:anakin@pobox.com}\c{anakin@pobox.com}. If you want to

report a bug to us, please read \k{bugs} first.

\i{New releases} of NASM are uploaded to

\W{ftp://sunsite.unc.edu/pub/Linux/devel/lang/assemblers/}\i\c{sunsite.unc.edu},

\W{ftp://ftp.simtel.net/pub/simtelnet/msdos/asmutl/}\i\c{ftp.simtel.net}

and

\W{ftp://ftp.coast.net/coast/msdos/asmutil/}\i\c{ftp.coast.net}.

Announcements are posted to

\W{news:comp.lang.asm.x86}\i\c{comp.lang.asm.x86},

\W{news:alt.lang.asm}\i\c{alt.lang.asm},

\W{news:comp.os.linux.announce}\i\c{comp.os.linux.announce} and

\W{news:comp.archives.msdos.announce}\i\c{comp.archives.msdos.announce}

(the last one is done automagically by uploading to

\W{ftp://ftp.simtel.net/pub/simtelnet/msdos/asmutl/}\c{ftp.simtel.net}).

If you don't have Usenet access, or would rather be informed by

\i{e\-mail} when new releases come out, e\-mail

\W{mailto:anakin@pobox.com}\c{anakin@pobox.com}

and ask.

\H{install} Installation

\S{instdos} \i{Installing} NASM under MS-\i{DOS} or Windows

Once you've obtained the \i{DOS archive} for NASM, \i\c{nasmXXX.zip}

(where \c{XXX} denotes the version number of NASM contained in the

archive), unpack it into its own directory (for example

\c{c:\\nasm}).

The archive will contain four executable files: the NASM executable

files \i\c{nasm.exe} and \i\c{nasmw.exe}, and the NDISASM executable

files \i\c{ndisasm.exe} and \i\c{ndisasmw.exe}. In each case, the

file whose name ends in \c{w} is a \i{Win32} executable, designed to

run under \i{Windows 95} or \i{Windows NT} Intel, and the other one

is a 16-bit \i{DOS} executable.

The only file NASM needs to run is its own executable, so copy

(at least) one of \c{nasm.exe} and \c{nasmw.exe} to a directory on

your PATH, or alternatively edit \i\c{autoexec.bat} to add the

\c{nasm} directory to your \i\c{PATH}. (If you're only installing the

Win32 version, you may wish to rename it to \c{nasm.exe}.)

That's it - NASM is installed. You don't need the \c{nasm} directory

to be present to run NASM (unless you've added it to your \c{PATH}),

so you can delete it if you need to save space; however, you may

want to keep the documentation or test programs.

If you've downloaded the \i{DOS source archive}, \i\c{nasmXXXs.zip},

the \c{nasm} directory will also contain the full NASM \i{source

code}, and a selection of \i{Makefiles} you can (hopefully) use to

rebuild your copy of NASM from scratch. The file \c{Readme} lists

the various Makefiles and which compilers they work with. Note that

the source files \c{insnsa.c} and \c{insnsd.c} are automatically

generated from the master instruction table \c{insns.dat} by a Perl

script; a \i{QBasic} version of the program is provided, but it is

recommended that you use the Perl version. A DOS port of \i{Perl} is

available from

\W{http://www.perl.org/CPAN/ports/msdos/}\i{www.perl.org}.

\S{instdos} Installing NASM under \i{Unix}

Once you've obtained the \i{Unix source archive} for NASM,

\i\c{nasm-X.XX.tar.gz} (where \c{X.XX} denotes the version number of

NASM contained in the archive), unpack it into a directory such

as \c{/usr/local/src}. The archive, when unpacked, will create its

own subdirectory \c{nasm-X.XX}.

NASM is an \I{Autoconf}\I\c{configure}auto-configuring package: once

you've unpacked it, \c{cd} to the directory it's been unpacked into

and type \c{./configure}. This shell script will find the best C

compiler to use for building NASM and set up \i{Makefiles}

accordingly.

Once NASM has auto-configured, you can type \i\c{make} to build the

\c{nasm} and \c{ndisasm} binaries, and then \c{make install} to

install them in \c{/usr/local/bin} and install the \i{man pages}

\i\c{nasm.1} and \i\c{ndisasm.1} in \c{/usr/local/man/man1}.

Alternatively, you can give options such as \c{--prefix} to the

\c{configure} script (see the file \i\c{INSTALL} for more details), or

install the programs yourself.

NASM also comes with a set of utilities for handling the RDOFF

custom object-file format, which are in the \i\c{rdoff} subdirectory

of the NASM archive. You can build these with \c{make rdf} and

install them with \c{make rdf_install}, if you want them.

If NASM fails to auto-configure, you may still be able to make it

compile by using the fall-back Unix makefile \i\c{Makefile.unx}.

Copy or rename that file to \c{Makefile} and try typing \c{make}.

There is also a \c{Makefile.unx} file in the \c{rdoff} subdirectory.

\C{running} Running NASM

\H{syntax} NASM \i{Command-Line} Syntax

To assemble a file, you issue a command of the form

\c nasm -f <format> <filename> [-o <output>]

For example,

\c nasm -f elf myfile.asm

will assemble \c{myfile.asm} into an ELF object file \c{myfile.o}. And

\c nasm -f bin myfile.asm -o myfile.com

will assemble \c{myfile.asm} into a raw binary file \c{myfile.com}.

To produce a listing file, with the hex codes output from NASM

displayed on the left of the original sources, use the \c{-l} option

to give a listing file name, for example:

\c nasm -f coff myfile.asm -l myfile.lst

To get further usage instructions from NASM, try typing

\c nasm -h

This will also list the available output file formats, and what they

are.

If you use Linux but aren't sure whether your system is \c{a.out} or

ELF, type

\c file nasm

(in the directory in which you put the NASM binary when you

installed it). If it says something like

\c nasm: ELF 32-bit LSB executable i386 (386 and up) Version 1

then your system is ELF, and you should use the option \c{-f elf}

when you want NASM to produce Linux object files. If it says

\c nasm: Linux/i386 demand-paged executable (QMAGIC)

or something similar, your system is \c{a.out}, and you should use

\c{-f aout} instead.

Like Unix compilers and assemblers, NASM is silent unless it

goes wrong: you won't see any output at all, unless it gives error

messages.

\S{opt-o} The \i\c{-o} Option: Specifying the Output File Name

NASM will normally choose the name of your output file for you;

precisely how it does this is dependent on the object file format.

For Microsoft object file formats (\i\c{obj} and \i\c{win32}), it

will remove the \c{.asm} \i{extension} (or whatever extension you

like to use - NASM doesn't care) from your source file name and

substitute \c{.obj}. For Unix object file formats (\i\c{aout},

\i\c{coff}, \i\c{elf} and \i\c{as86}) it will substitute \c{.o}. For

\i\c{rdf}, it will use \c{.rdf}, and for the \i\c{bin} format it

will simply remove the extension, so that \c{myfile.asm} produces

the output file \c{myfile}.

If the output file already exists, NASM will overwrite it, unless it

has the same name as the input file, in which case it will give a

warning and use \i\c{nasm.out} as the output file name instead.

For situations in which this behaviour is unacceptable, NASM

provides the \c{-o} command-line option, which allows you to specify

your desired output file name. You invoke \c{-o} by following it

with the name you wish for the output file, either with or without

an intervening space. For example:

\c nasm -f bin program.asm -o program.com

\c nasm -f bin driver.asm -odriver.sys

\S{opt-f} The \i\c{-f} Option: Specifying the \i{Output File Format}

If you do not supply the \c{-f} option to NASM, it will choose an

output file format for you itself. In the distribution versions of

NASM, the default is always \i\c{bin}; if you've compiled your own

copy of NASM, you can redefine \i\c{OF_DEFAULT} at compile time and

choose what you want the default to be.

Like \c{-o}, the intervening space between \c{-f} and the output

file format is optional; so \c{-f elf} and \c{-felf} are both valid.

A complete list of the available output file formats can be given by

issuing the command \i\c{nasm -h}.

\S{opt-l} The \i\c{-l} Option: Generating a \i{Listing File}

If you supply the \c{-l} option to NASM, followed (with the usual

optional space) by a file name, NASM will generate a

\i{source-listing file} for you, in which addresses and generated

code are listed on the left, and the actual source code, with

expansions of multi-line macros (except those which specifically

request no expansion in source listings: see \k{nolist}) on the

right. For example:

\c nasm -f elf myfile.asm -l myfile.lst

\S{opt-s} The \i\c{-s} Option: Send Errors to \i\c{stdout}

Under MS-\i{DOS} it can be difficult (though there are ways) to

redirect the standard-error output of a program to a file. Since

NASM usually produces its warning and \i{error messages} on

\i\c{stderr}, this can make it hard to capture the errors if (for

example) you want to load them into an editor.

NASM therefore provides the \c{-s} option, requiring no argument,

which causes errors to be sent to standard output rather than

standard error. Therefore you can \I{redirecting errors}redirect

the errors into a file by typing

\c nasm -s -f obj myfile.asm > myfile.err

\S{opt-i} The \i\c{-i} Option: Include File Search Directories

When NASM sees the \i\c{%include} directive in a source file (see

\k{include}), it will search for the given file not only in the

current directory, but also in any directories specified on the

command line by the use of the \c{-i} option. Therefore you can

include files from a \i{macro library}, for example, by typing

\c nasm -ic:\\macrolib\\ -f obj myfile.asm

(As usual, a space between \c{-i} and the path name is allowed, and

optional).

NASM, in the interests of complete source-code portability, does not

understand the file naming conventions of the OS it is running on;

the string you provide as an argument to the \c{-i} option will be

prepended exactly as written to the name of the include file.

Therefore the trailing backslash in the above example is necessary.

Under Unix, a trailing forward slash is similarly necessary.

(You can use this to your advantage, if you're really \i{perverse},

by noting that the option \c{-ifoo} will cause \c{%include "bar.i"}

to search for the file \c{foobar.i}...)

If you want to define a \e{standard} \i{include search path},

similar to \c{/usr/include} on Unix systems, you should place one or

more \c{-i} directives in the \c{NASM} environment variable (see

\k{nasmenv}).

\S{opt-p} The \i\c{-p} Option: \I{pre-including files}Pre-Include a File

\I\c{%include}NASM allows you to specify files to be

\e{pre-included} into your source file, by the use of the \c{-p}

option. So running

\c nasm myfile.asm -p myinc.inc

is equivalent to running \c{nasm myfile.asm} and placing the

directive \c{%include "myinc.inc"} at the start of the file.

\S{opt-d} The \i\c{-d} Option: \I{pre-defining macros} Pre-Define a Macro

\I\c{%define}Just as the \c{-p} option gives an alternative to placing

\c{%include} directives at the start of a source file, the \c{-d}

option gives an alternative to placing a \c{%define} directive. You

could code

\c nasm myfile.asm -dFOO=100

as an alternative to placing the directive

\c %define FOO 100

at the start of the file. You can miss off the macro value, as well:

the option \c{-dFOO} is equivalent to coding \c{%define FOO}. This

form of the directive may be useful for selecting \i{assembly-time

options} which are then tested using \c{%ifdef}, for example

\c{-dDEBUG}.

\S{opt-e} The \i\c{-e} Option: Preprocess Only

NASM allows the \i{preprocessor} to be run on its own, up to a

point. Using the \c{-e} option (which requires no arguments) will

cause NASM to preprocess its input file, expand all the macro

references, remove all the comments and preprocessor directives, and

print the resulting file on standard output (or save it to a file,

if the \c{-o} option is also used).

This option cannot be applied to programs which require the

preprocessor to evaluate \I{preprocessor expressions}\i{expressions}

which depend on the values of symbols: so code such as

\c %assign tablesize ($-tablestart)

will cause an error in \i{preprocess-only mode}.

\S{opt-a} The \i\c{-a} Option: Don't Preprocess At All

If NASM is being used as the back end to a compiler, it might be

desirable to \I{suppressing preprocessing}suppress preprocessing

completely and assume the compiler has already done it, to save time

and increase compilation speeds. The \c{-a} option, requiring no

argument, instructs NASM to replace its powerful \i{preprocessor}

with a \i{stub preprocessor} which does nothing.

\S{opt-w} The \i\c{-w} Option: Enable or Disable Assembly \i{Warnings}

NASM can observe many conditions during the course of assembly which

are worth mentioning to the user, but not a sufficiently severe

error to justify NASM refusing to generate an output file. These

conditions are reported like errors, but come up with the word

`warning' before the message. Warnings do not prevent NASM from

generating an output file and returning a success status to the

operating system.

Some conditions are even less severe than that: they are only

sometimes worth mentioning to the user. Therefore NASM supports the

\c{-w} command-line option, which enables or disables certain

classes of assembly warning. Such warning classes are described by a

name, for example \c{orphan-labels}; you can enable warnings of

this class by the command-line option \c{-w+orphan-labels} and

disable it by \c{-w-orphan-labels}.

The \i{suppressible warning} classes are:

\b \i\c{macro-params} covers warnings about \i{multi-line macros}

being invoked with the wrong number of parameters. This warning

class is enabled by default; see \k{mlmacover} for an example of why

you might want to disable it.

\b \i\c{orphan-labels} covers warnings about source lines which

contain no instruction but define a label without a trailing colon.

NASM does not warn about this somewhat obscure condition by default;

see \k{syntax} for an example of why you might want it to.

\b \i\c{number-overflow} covers warnings about numeric constants which

don't fit in 32 bits (for example, it's easy to type one too many Fs

and produce \c{0x7ffffffff} by mistake). This warning class is

enabled by default.

\S{nasmenv} The \c{NASM} \i{Environment} Variable

If you define an environment variable called \c{NASM}, the program

will interpret it as a list of extra command-line options, which are

processed before the real command line. You can use this to define

standard search directories for include files, by putting \c{-i}

options in the \c{NASM} variable.

The value of the variable is split up at white space, so that the

value \c{-s -ic:\\nasmlib} will be treated as two separate options.

However, that means that the value \c{-dNAME="my name"} won't do

what you might want, because it will be split at the space and the

NASM command-line processing will get confused by the two

nonsensical words \c{-dNAME="my} and \c{name"}.

To get round this, NASM provides a feature whereby, if you begin the

\c{NASM} environment variable with some character that isn't a minus

sign, then NASM will treat this character as the \i{separator

character} for options. So setting the \c{NASM} variable to the

value \c{!-s!-ic:\\nasmlib} is equivalent to setting it to \c{-s

-ic:\\nasmlib}, but \c{!-dNAME="my name"} will work.

\H{qstart} \i{Quick Start} for \i{MASM} Users

If you're used to writing programs with MASM, or with \i{TASM} in

MASM-compatible (non-Ideal) mode, or with \i\c{a86}, this section

attempts to outline the major differences between MASM's syntax and

NASM's. If you're not already used to MASM, it's probably worth

skipping this section.

\S{qscs} NASM Is \I{case sensitivity}Case-Sensitive

One simple difference is that NASM is case-sensitive. It makes a

difference whether you call your label \c{foo}, \c{Foo} or \c{FOO}.

If you're assembling to DOS or OS/2 \c{.OBJ} files, you can invoke

the \i\c{UPPERCASE} directive (documented in \k{objfmt}) to ensure

that all symbols exported to other code modules are forced to be

upper case; but even then, \e{within} a single module, NASM will

distinguish between labels differing only in case.

\S{qsbrackets} NASM Requires \i{Square Brackets} For \i{Memory References}

NASM was designed with simplicity of syntax in mind. One of the

\i{design goals} of NASM is that it should be possible, as far as is

practical, for the user to look at a single line of NASM code

and tell what opcode is generated by it. You can't do this in MASM:

if you declare, for example,

\c foo       equ 1

\c bar       dw 2

then the two lines of code

\c           mov ax,foo

\c           mov ax,bar

generate completely different opcodes, despite having

identical-looking syntaxes.

NASM avoids this undesirable situation by having a much simpler

syntax for memory references. The rule is simply that any access to

the \e{contents} of a memory location requires square brackets

around the address, and any access to the \e{address} of a variable

doesn't. So an instruction of the form \c{mov ax,foo} will

\e{always} refer to a compile-time constant, whether it's an \c{EQU}

or the address of a variable; and to access the \e{contents} of the

variable \c{bar}, you must code \c{mov ax,[bar]}.

This also means that NASM has no need for MASM's \i\c{OFFSET}

keyword, since the MASM code \c{mov ax,offset bar} means exactly the

same thing as NASM's \c{mov ax,bar}. If you're trying to get

large amounts of MASM code to assemble sensibly under NASM, you

can always code \c{%idefine offset} to make the preprocessor treat

the \c{OFFSET} keyword as a no-op.

This issue is even more confusing in \i\c{a86}, where declaring a

label with a trailing colon defines it to be a `label' as opposed to

a `variable' and causes \c{a86} to adopt NASM-style semantics; so in

\c{a86}, \c{mov ax,var} has different behaviour depending on whether

\c{var} was declared as \c{var: dw 0} (a label) or \c{var dw 0} (a

word-size variable). NASM is very simple by comparison:

\e{everything} is a label.

NASM, in the interests of simplicity, also does not support the

\i{hybrid syntaxes} supported by MASM and its clones, such as

\c{mov ax,table[bx]}, where a memory reference is denoted by one

portion outside square brackets and another portion inside. The

correct syntax for the above is \c{mov ax,[table+bx]}. Likewise,

\c{mov ax,es:[di]} is wrong and \c{mov ax,[es:di]} is right.

\S{qstypes} NASM Doesn't Store \i{Variable Types}

NASM, by design, chooses not to remember the types of variables you

declare. Whereas MASM will remember, on seeing \c{var dw 0}, that

you declared \c{var} as a word-size variable, and will then be able

to fill in the \i{ambiguity} in the size of the instruction \c{mov

var,2}, NASM will deliberately remember nothing about the symbol

\c{var} except where it begins, and so you must explicitly code

\c{mov word [var],2}.

For this reason, NASM doesn't support the \c{LODS}, \c{MOVS},

\c{STOS}, \c{SCAS}, \c{CMPS}, \c{INS}, or \c{OUTS} instructions,

but only supports the forms such as \c{LODSB}, \c{MOVSW}, and

\c{SCASD}, which explicitly specify the size of the components of

the strings being manipulated.

\S{qsassume} NASM Doesn't \i\c{ASSUME}

As part of NASM's drive for simplicity, it also does not support the

\c{ASSUME} directive. NASM will not keep track of what values you

choose to put in your segment registers, and will never

\e{automatically} generate a \i{segment override} prefix.

\S{qsmodel} NASM Doesn't Support \i{Memory Models}

NASM also does not have any directives to support different 16-bit

memory models. The programmer has to keep track of which functions

are supposed to be called with a \i{far call} and which with a

\i{near call}, and is responsible for putting the correct form of

\c{RET} instruction (\c{RETN} or \c{RETF}; NASM accepts \c{RET}

itself as an alternate form for \c{RETN}); in addition, the

programmer is responsible for coding CALL FAR instructions where

necessary when calling \e{external} functions, and must also keep

track of which external variable definitions are far and which are

near.

\S{qsfpu} \i{Floating-Point} Differences

NASM uses different names to refer to floating-point registers from

MASM: where MASM would call them \c{ST(0)}, \c{ST(1)} and so on, and

\i\c{a86} would call them simply \c{0}, \c{1} and so on, NASM

chooses to call them \c{st0}, \c{st1} etc.

As of version 0.96, NASM now treats the instructions with

\i{`nowait'} forms in the same way as MASM-compatible assemblers.

The idiosyncratic treatment employed by 0.95 and earlier was based

on a misunderstanding by the authors.

\S{qsother} Other Differences

For historical reasons, NASM uses the keyword \i\c{TWORD} where MASM

and compatible assemblers use \i\c{TBYTE}.

NASM does not declare \i{uninitialised storage} in the same way as

MASM: where a MASM programmer might use \c{stack db 64 dup (?)},

NASM requires \c{stack resb 64}, intended to be read as `reserve 64

bytes'. For a limited amount of compatibility, since NASM treats

\c{?} as a valid character in symbol names, you can code \c{? equ 0}

and then writing \c{dw ?} will at least do something vaguely useful.

\I\c{RESB}\i\c{DUP} is still not a supported syntax, however.

In addition to all of this, macros and directives work completely

differently to MASM. See \k{preproc} and \k{directive} for further

details.

\C{lang} The NASM Language

\H{syntax} Layout of a NASM Source Line

Like most assemblers, each NASM source line contains (unless it

is a macro, a preprocessor directive or an assembler directive: see

\k{preproc} and \k{directive}) some combination of the four fields

\c label:    instruction operands        ; comment

As usual, most of these fields are optional; the presence or absence

of any combination of a label, an instruction and a comment is allowed.

Of course, the operand field is either required or forbidden by the

presence and nature of the instruction field.

NASM places no restrictions on white space within a line: labels may

have white space before them, or instructions may have no space

before them, or anything. The \i{colon} after a label is also

optional. (Note that this means that if you intend to code \c{lodsb}

alone on a line, and type \c{lodab} by accident, then that's still a

valid source line which does nothing but define a label. Running

NASM with the command-line option

\I{orphan-labels}\c{-w+orphan-labels} will cause it to warn you if

you define a label alone on a line without a \i{trailing colon}.)

\i{Valid characters} in labels are letters, numbers, \c{_}, \c{$},

\c{#}, \c{@}, \c{~}, \c{.}, and \c{?}. The only characters which may

be used as the \e{first} character of an identifier are letters,

\c{.} (with special meaning: see \k{locallab}), \c{_} and \c{?}.

An identifier may also be prefixed with a \I{$prefix}\c{$} to

indicate that it is intended to be read as an identifier and not a

reserved word; thus, if some other module you are linking with

defines a symbol called \c{eax}, you can refer to \c{$eax} in NASM

code to distinguish the symbol from the register.

The instruction field may contain any machine instruction: Pentium

and P6 instructions, FPU instructions, MMX instructions and even

undocumented instructions are all supported. The instruction may be

prefixed by \c{LOCK}, \c{REP}, \c{REPE}/\c{REPZ} or

\c{REPNE}/\c{REPNZ}, in the usual way. Explicit \I{address-size

prefixes}address-size and \i{operand-size prefixes} \c{A16},

\c{A32}, \c{O16} and \c{O32} are provided - one example of their use

is given in \k{mixsize}. You can also use the name of a \I{segment

override}segment register as an instruction prefix: coding

\c{es mov [bx],ax} is equivalent to coding \c{mov [es:bx],ax}. We

recommend the latter syntax, since it is consistent with other

syntactic features of the language, but for instructions such as

\c{LODSB}, which has no operands and yet can require a segment

override, there is no clean syntactic way to proceed apart from

\c{es lodsb}.

An instruction is not required to use a prefix: prefixes such as

\c{CS}, \c{A32}, \c{LOCK} or \c{REPE} can appear on a line by

themselves, and NASM will just generate the prefix bytes.

In addition to actual machine instructions, NASM also supports a

number of pseudo-instructions, described in \k{pseudop}.

Instruction \i{operands} may take a number of forms: they can be

registers, described simply by the register name (e.g. \c{ax},

\c{bp}, \c{ebx}, \c{cr0}: NASM does not use the \c{gas}-style

syntax in which register names must be prefixed by a \c{%} sign), or

they can be \i{effective addresses} (see \k{effaddr}), constants

(\k{const}) or expressions (\k{expr}).

For \i{floating-point} instructions, NASM accepts a wide range of

syntaxes: you can use two-operand forms like MASM supports, or you

can use NASM's native single-operand forms in most cases. Details of

all forms of each supported instruction are given in

\k{iref}. For example, you can code:

\c           fadd st1               ; this sets st0 := st0 + st1

\c           fadd st0,st1           ; so does this

\c

\c           fadd st1,st0           ; this sets st1 := st1 + st0

\c           fadd to st1            ; so does this

Almost any floating-point instruction that references memory must

use one of the prefixes \i\c{DWORD}, \i\c{QWORD} or \i\c{TWORD} to

indicate what size of \i{memory operand} it refers to.

\H{pseudop} \i{Pseudo-Instructions}

Pseudo-instructions are things which, though not real x86 machine

instructions, are used in the instruction field anyway because

that's the most convenient place to put them. The current

pseudo-instructions are \i\c{DB}, \i\c{DW}, \i\c{DD}, \i\c{DQ} and

\i\c{DT}, their \i{uninitialised} counterparts \i\c{RESB},

\i\c{RESW}, \i\c{RESD}, \i\c{RESQ} and \i\c{REST}, the \i\c{INCBIN}

command, the \i\c{EQU} command, and the \i\c{TIMES} prefix.

\S{db} \c{DB} and friends: Declaring Initialised Data

\i\c{DB}, \i\c{DW}, \i\c{DD}, \i\c{DQ} and \i\c{DT} are used, much

as in MASM, to declare initialised data in the output file. They can

be invoked in a wide range of ways:

\I{floating-point}\I{character constant}\I{string constant}

\c           db 0x55                ; just the byte 0x55

\c           db 0x55,0x56,0x57      ; three bytes in succession

\c           db 'a',0x55            ; character constants are OK

\c           db 'hello',13,10,'$'   ; so are string constants

\c           dw 0x1234              ; 0x34 0x12

\c           dw 'a'                 ; 0x41 0x00 (it's just a number)

\c           dw 'ab'                ; 0x41 0x42 (character constant)

\c           dw 'abc'               ; 0x41 0x42 0x43 0x00 (string)

\c           dd 0x12345678          ; 0x78 0x56 0x34 0x12

\c           dd 1.234567e20         ; floating-point constant

\c           dq 1.234567e20         ; double-precision float

\c           dt 1.234567e20         ; extended-precision float

\c{DQ} and \c{DT} do not accept \i{numeric constants} or string

constants as operands.

\S{resb} \c{RESB} and friends: Declaring \i{Uninitialised} Data

\i\c{RESB}, \i\c{RESW}, \i\c{RESD}, \i\c{RESQ} and \i\c{REST} are

designed to be used in the BSS section of a module: they declare

\e{uninitialised} storage space. Each takes a single operand, which

is the number of bytes, words, doublewords or whatever to reserve.

As stated in \k{qsother}, NASM does not support the MASM/TASM syntax

of reserving uninitialised space by writing \I\c{?}\c{DW ?} or

similar things: this is what it does instead. The operand to a

\c{RESB}-type pseudo-instruction is a \i\e{critical expression}: see

\k{crit}.

For example:

\c buffer:   resb 64                ; reserve 64 bytes

\c wordvar:  resw 1                 ; reserve a word

\c realarray resq 10                ; array of ten reals

\S{incbin} \i\c{INCBIN}: Including External \i{Binary Files}

\c{INCBIN} is borrowed from the old Amiga assembler \i{DevPac}: it

includes a binary file verbatim into the output file. This can be

handy for (for example) including \i{graphics} and \i{sound} data

directly into a game executable file. It can be called in one of

these three ways:

\c           incbin "file.dat"      ; include the whole file

\c           incbin "file.dat",1024 ; skip the first 1024 bytes

\c           incbin "file.dat",1024,512 ; skip the first 1024, and

\c                                  ; actually include at most 512

\S{equ} \i\c{EQU}: Defining Constants

\c{EQU} defines a symbol to a given constant value: when \c{EQU} is

used, the source line must contain a label. The action of \c{EQU} is

to define the given label name to the value of its (only) operand.

This definition is absolute, and cannot change later. So, for

example,

\c message   db 'hello, world'

\c msglen    equ $-message

defines \c{msglen} to be the constant 12. \c{msglen} may not then be

redefined later. This is not a \i{preprocessor} definition either:

the value of \c{msglen} is evaluated \e{once}, using the value of

\c{$} (see \k{expr} for an explanation of \c{$}) at the point of

definition, rather than being evaluated wherever it is referenced

and using the value of \c{$} at the point of reference. Note that

the operand to an \c{EQU} is also a \i{critical expression}

(\k{crit}).

\S{times} \i\c{TIMES}: \i{Repeating} Instructions or Data

The \c{TIMES} prefix causes the instruction to be assembled multiple

times. This is partly present as NASM's equivalent of the \i\c{DUP}

syntax supported by \i{MASM}-compatible assemblers, in that you can

code

\c zerobuf:  times 64 db 0

or similar things; but \c{TIMES} is more versatile than that. The

argument to \c{TIMES} is not just a numeric constant, but a numeric

\e{expression}, so you can do things like

\c buffer:   db 'hello, world'

\c           times 64-$+buffer db ' '

which will store exactly enough spaces to make the total length of

\c{buffer} up to 64. Finally, \c{TIMES} can be applied to ordinary

instructions, so you can code trivial \i{unrolled loops} in it:

\c           times 100 movsb

Note that there is no effective difference between \c{times 100 resb

1} and \c{resb 100}, except that the latter will be assembled about

100 times faster due to the internal structure of the assembler.

The operand to \c{TIMES}, like that of \c{EQU} and those of \c{RESB}

and friends, is a critical expression (\k{crit}).

Note also that \c{TIMES} can't be applied to \i{macros}: the reason

for this is that \c{TIMES} is processed after the macro phase, which

allows the argument to \c{TIMES} to contain expressions such as

\c{64-$+buffer} as above. To repeat more than one line of code, or a

complex macro, use the preprocessor \i\c{%rep} directive.

\H{effaddr} Effective Addresses

An \i{effective address} is any operand to an instruction which

\I{memory reference}references memory. Effective addresses, in NASM,

have a very simple syntax: they consist of an expression evaluating

to the desired address, enclosed in \i{square brackets}. For

example:

\c wordvar   dw 123

\c           mov ax,[wordvar]

\c           mov ax,[wordvar+1]

\c           mov ax,[es:wordvar+bx]

Anything not conforming to this simple system is not a valid memory

reference in NASM, for example \c{es:wordvar[bx]}.

More complicated effective addresses, such as those involving more

than one register, work in exactly the same way:

\c           mov eax,[ebx*2+ecx+offset]

\c           mov ax,[bp+di+8]

NASM is capable of doing \i{algebra} on these effective addresses,

so that things which don't necessarily \e{look} legal are perfectly

all right:

\c           mov eax,[ebx*5]        ; assembles as [ebx*4+ebx]

\c           mov eax,[label1*2-label2] ; ie [label1+(label1-label2)]

Some forms of effective address have more than one assembled form;

in most such cases NASM will generate the smallest form it can. For

example, there are distinct assembled forms for the 32-bit effective

addresses \c{[eax*2+0]} and \c{[eax+eax]}, and NASM will generally

generate the latter on the grounds that the former requires four

bytes to store a zero offset.

NASM has a hinting mechanism which will cause \c{[eax+ebx]} and

\c{[ebx+eax]} to generate different opcodes; this is occasionally

useful because \c{[esi+ebp]} and \c{[ebp+esi]} have different

default segment registers.

However, you can force NASM to generate an effective address in a

particular form by the use of the keywords \c{BYTE}, \c{WORD},

\c{DWORD} and \c{NOSPLIT}. If you need \c{[eax+3]} to be assembled

using a double-word offset field instead of the one byte NASM will

normally generate, you can code \c{[dword eax+3]}. Similarly, you

can force NASM to use a byte offset for a small value which it

hasn't seen on the first pass (see \k{crit} for an example of such a

code fragment) by using \c{[byte eax+offset]}. As special cases,

\c{[byte eax]} will code \c{[eax+0]} with a byte offset of zero, and

\c{[dword eax]} will code it with a double-word offset of zero. The

normal form, \c{[eax]}, will be coded with no offset field.

Similarly, NASM will split \c{[eax*2]} into \c{[eax+eax]} because

that allows the offset field to be absent and space to be saved; in

fact, it will also split \c{[eax*2+offset]} into

\c{[eax+eax+offset]}. You can combat this behaviour by the use of

the \c{NOSPLIT} keyword: \c{[nosplit eax*2]} will force

\c{[eax*2+0]} to be generated literally.

\H{const} \i{Constants}

NASM understands four different types of constant: numeric,

character, string and floating-point.

\S{numconst} \i{Numeric Constants}

A numeric constant is simply a number. NASM allows you to specify

numbers in a variety of number bases, in a variety of ways: you can

suffix \c{H}, \c{Q} and \c{B} for \i{hex}, \i{octal} and \i{binary},

or you can prefix \c{0x} for hex in the style of C, or you can

prefix \c{$} for hex in the style of Borland Pascal. Note, though,

that the \I{$prefix}\c{$} prefix does double duty as a prefix on

identifiers (see \k{syntax}), so a hex number prefixed with a \c{$}

sign must have a digit after the \c{$} rather than a letter.

Some examples:

\c           mov ax,100             ; decimal

\c           mov ax,0a2h            ; hex

\c           mov ax,$0a2            ; hex again: the 0 is required

\c           mov ax,0xa2            ; hex yet again

\c           mov ax,777q            ; octal

\c           mov ax,10010011b       ; binary

\S{chrconst} \i{Character Constants}

A character constant consists of up to four characters enclosed in

either single or double quotes. The type of quote makes no

difference to NASM, except of course that surrounding the constant

with single quotes allows double quotes to appear within it and vice

versa.

A character constant with more than one character will be arranged

with \i{little-endian} order in mind: if you code

\c           mov eax,'abcd'

then the constant generated is not \c{0x61626364}, but

\c{0x64636261}, so that if you were then to store the value into

memory, it would read \c{abcd} rather than \c{dcba}. This is also

the sense of character constants understood by the Pentium's

\i\c{CPUID} instruction (see \k{insCPUID}).

\S{strconst} String Constants

String constants are only acceptable to some pseudo-instructions,

namely the \I\c{DW}\I\c{DD}\I\c{DQ}\I\c{DT}\i\c{DB} family and

\i\c{INCBIN}.

A string constant looks like a character constant, only longer. It

is treated as a concatenation of maximum-size character constants

for the conditions. So the following are equivalent:

\c           db 'hello'             ; string constant

\c           db 'h','e','l','l','o' ; equivalent character constants

And the following are also equivalent:

\c           dd 'ninechars'         ; doubleword string constant

\c           dd 'nine','char','s'   ; becomes three doublewords

\c           db 'ninechars',0,0,0   ; and really looks like this

Note that when used as an operand to \c{db}, a constant like

\c{'ab'} is treated as a string constant despite being short enough

to be a character constant, because otherwise \c{db 'ab'} would have

the same effect as \c{db 'a'}, which would be silly. Similarly,

three-character or four-character constants are treated as strings

when they are operands to \c{dw}.

\S{fltconst} \I{floating-point, constants}Floating-Point Constants

\i{Floating-point} constants are acceptable only as arguments to

\i\c{DD}, \i\c{DQ} and \i\c{DT}. They are expressed in the

traditional form: digits, then a period, then optionally more

digits, then optionally an \c{E} followed by an exponent. The period

is mandatory, so that NASM can distinguish between \c{dd 1}, which

declares an integer constant, and \c{dd 1.0} which declares a

floating-point constant.

Some examples:

\c           dd 1.2                 ; an easy one

\c           dq 1.e10               ; 10,000,000,000

\c           dq 1.e+10              ; synonymous with 1.e10

\c           dq 1.e-10              ; 0.000 000 000 1

\c           dt 3.141592653589793238462 ; pi

NASM cannot do compile-time arithmetic on floating-point constants.

This is because NASM is designed to be portable - although it always

generates code to run on x86 processors, the assembler itself can

run on any system with an ANSI C compiler. Therefore, the assembler

cannot guarantee the presence of a floating-point unit capable of

handling the \i{Intel number formats}, and so for NASM to be able to

do floating arithmetic it would have to include its own complete set

of floating-point routines, which would significantly increase the

size of the assembler for very little benefit.

\H{expr} \i{Expressions}

Expressions in NASM are similar in syntax to those in C.

NASM does not guarantee the size of the integers used to evaluate

expressions at compile time: since NASM can compile and run on

64-bit systems quite happily, don't assume that expressions are

evaluated in 32-bit registers and so try to make deliberate use of

\i{integer overflow}. It might not always work. The only thing NASM

will guarantee is what's guaranteed by ANSI C: you always have \e{at

least} 32 bits to work in.

NASM supports two special tokens in expressions, allowing

calculations to involve the current assembly position: the

\I{$ here}\c{$} and \i\c{$$} tokens. \c{$} evaluates to the assembly

position at the beginning of the line containing the expression; so

you can code an \i{infinite loop} using \c{JMP $}. \c{$$} evaluates

to the beginning of the current section; so you can tell how far

into the section you are by using \c{($-$$)}.

The arithmetic \i{operators} provided by NASM are listed here, in

increasing order of \i{precedence}.

\S{expor} \i\c{|}: \i{Bitwise OR} Operator

The \c{|} operator gives a bitwise OR, exactly as performed by the

\c{OR} machine instruction. Bitwise OR is the lowest-priority

arithmetic operator supported by NASM.

\S{expxor} \i\c{^}: \i{Bitwise XOR} Operator

\c{^} provides the bitwise XOR operation.

\S{expand} \i\c{&}: \i{Bitwise AND} Operator

\c{&} provides the bitwise AND operation.

\S{expshift} \i\c{<<} and \i\c{>>}: \i{Bit Shift} Operators

\c{<<} gives a bit-shift to the left, just as it does in C. So \c{5<<3}

evaluates to 5 times 8, or 40. \c{>>} gives a bit-shift to the

right; in NASM, such a shift is \e{always} unsigned, so that

the bits shifted in from the left-hand end are filled with zero

rather than a sign-extension of the previous highest bit.

\S{expplmi} \I{+ opaddition}\c{+} and \I{- opsubtraction}\c{-}:

\i{Addition} and \i{Subtraction} Operators

The \c{+} and \c{-} operators do perfectly ordinary addition and

subtraction.

\S{expmul} \i\c{*}, \i\c{/}, \i\c{//}, \i\c{%} and \i\c{%%}:

\i{Multiplication} and \i{Division}

\c{*} is the multiplication operator. \c{/} and \c{//} are both

division operators: \c{/} is \i{unsigned division} and \c{//} is

\i{signed division}. Similarly, \c{%} and \c{%%} provide \I{unsigned

modulo}\I{modulo operators}unsigned and

\i{signed modulo} operators respectively.

NASM, like ANSI C, provides no guarantees about the sensible

operation of the signed modulo operator.

Since the \c{%} character is used extensively by the macro

\i{preprocessor}, you should ensure that both the signed and unsigned

modulo operators are followed by white space wherever they appear.

\S{expmul} \i{Unary Operators}: \I{+ opunary}\c{+}, \I{- opunary}\c{-},

\i\c{~} and \i\c{SEG}

The highest-priority operators in NASM's expression grammar are

those which only apply to one argument. \c{-} negates its operand,

\c{+} does nothing (it's provided for symmetry with \c{-}), \c{~}

computes the \i{one's complement} of its operand, and \c{SEG}

provides the \i{segment address} of its operand (explained in more

detail in \k{segwrt}).

\H{segwrt} \i\c{SEG} and \i\c{WRT}

When writing large 16-bit programs, which must be split into

multiple \i{segments}, it is often necessary to be able to refer to

the \I{segment address}segment part of the address of a symbol. NASM

supports the \c{SEG} operator to perform this function.

The \c{SEG} operator returns the \i\e{preferred} segment base of a

symbol, defined as the segment base relative to which the offset of

the symbol makes sense. So the code

\c           mov ax,seg symbol

\c           mov es,ax

\c           mov bx,symbol

will load \c{ES:BX} with a valid pointer to the symbol \c{symbol}.

Things can be more complex than this: since 16-bit segments and

\i{groups} may \I{overlapping segments}overlap, you might occasionally

want to refer to some symbol using a different segment base from the

preferred one. NASM lets you do this, by the use of the \c{WRT}

(With Reference To) keyword. So you can do things like

\c           mov ax,weird_seg       ; weird_seg is a segment base

\c           mov es,ax

\c           mov bx,symbol wrt weird_seg

to load \c{ES:BX} with a different, but functionally equivalent,

pointer to the symbol \c{symbol}.

NASM supports far (inter-segment) calls and jumps by means of the

syntax \c{call segment:offset}, where \c{segment} and \c{offset}

both represent immediate values. So to call a far procedure, you

could code either of

\c           call (seg procedure):procedure

\c           call weird_seg:(procedure wrt weird_seg)

(The parentheses are included for clarity, to show the intended

parsing of the above instructions. They are not necessary in

practice.)

NASM supports the syntax \I\c{CALL FAR}\c{call far procedure} as a

synonym for the first of the above usages. \c{JMP} works identically

to \c{CALL} in these examples.

To declare a \i{far pointer} to a data item in a data segment, you

must code

\c           dw symbol, seg symbol

NASM supports no convenient synonym for this, though you can always

invent one using the macro processor.

\H{crit} \i{Critical Expressions}

A limitation of NASM is that it is a \i{two-pass assembler}; unlike

TASM and others, it will always do exactly two \I{passes}\i{assembly

passes}. Therefore it is unable to cope with source files that are

complex enough to require three or more passes.

The first pass is used to determine the size of all the assembled

code and data, so that the second pass, when generating all the

code, knows all the symbol addresses the code refers to. So one

thing NASM can't handle is code whose size depends on the value of a

symbol declared after the code in question. For example,

\c           times (label-$) db 0

\c label:    db 'Where am I?'

The argument to \i\c{TIMES} in this case could equally legally

evaluate to anything at all; NASM will reject this example because

it cannot tell the size of the \c{TIMES} line when it first sees it.

It will just as firmly reject the slightly \I{paradox}paradoxical

code

\c           times (label-$+1) db 0

\c label:    db 'NOW where am I?'

in which \e{any} value for the \c{TIMES} argument is by definition

wrong!

NASM rejects these examples by means of a concept called a

\e{critical expression}, which is defined to be an expression whose

value is required to be computable in the first pass, and which must

therefore depend only on symbols defined before it. The argument to

the \c{TIMES} prefix is a critical expression; for the same reason,

the arguments to the \i\c{RESB} family of pseudo-instructions are

also critical expressions.

Critical expressions can crop up in other contexts as well: consider

the following code.

\c           mov ax,symbol1

\c symbol1   equ symbol2

\c symbol2:

On the first pass, NASM cannot determine the value of \c{symbol1},

because \c{symbol1} is defined to be equal to \c{symbol2} which NASM

hasn't seen yet. On the second pass, therefore, when it encounters

the line \c{mov ax,symbol1}, it is unable to generate the code for

it because it still doesn't know the value of \c{symbol1}. On the

next line, it would see the \i\c{EQU} again and be able to determine

the value of \c{symbol1}, but by then it would be too late.

NASM avoids this problem by defining the right-hand side of an

\c{EQU} statement to be a critical expression, so the definition of

\c{symbol1} would be rejected in the first pass.

There is a related issue involving \i{forward references}: consider

this code fragment.

\c           mov eax,[ebx+offset]

\c offset    equ 10

NASM, on pass one, must calculate the size of the instruction \c{mov

eax,[ebx+offset]} without knowing the value of \c{offset}. It has no

way of knowing that \c{offset} is small enough to fit into a

one-byte offset field and that it could therefore get away with

generating a shorter form of the \i{effective-address} encoding; for

all it knows, in pass one, \c{offset} could be a symbol in the code

segment, and it might need the full four-byte form. So it is forced

to compute the size of the instruction to accommodate a four-byte

address part. In pass two, having made this decision, it is now

forced to honour it and keep the instruction large, so the code

generated in this case is not as small as it could have been. This

problem can be solved by defining \c{offset} before using it, or by

forcing byte size in the effective address by coding \c{[byte

ebx+offset]}.

\H{locallab} \i{Local Labels}

NASM gives special treatment to symbols beginning with a \i{period}.

A label beginning with a single period is treated as a \e{local}

label, which means that it is associated with the previous non-local

label. So, for example:

\c label1    ; some code

\c .loop     ; some more code

\c           jne .loop

\c           ret

\c label2    ; some code

\c .loop     ; some more code

\c           jne .loop

\c           ret

In the above code fragment, each \c{JNE} instruction jumps to the

line immediately before it, because the two definitions of \c{.loop}

are kept separate by virtue of each being associated with the

previous non-local label.

This form of local label handling is borrowed from the old Amiga

assembler \i{DevPac}; however, NASM goes one step further, in

allowing access to local labels from other parts of the code. This

is achieved by means of \e{defining} a local label in terms of the

previous non-local label: the first definition of \c{.loop} above is

really defining a symbol called \c{label1.loop}, and the second

defines a symbol called \c{label2.loop}. So, if you really needed

to, you could write

\c label3    ; some more code

\c           ; and some more

\c           jmp label1.loop

Sometimes it is useful - in a macro, for instance - to be able to

define a label which can be referenced from anywhere but which

doesn't interfere with the normal local-label mechanism. Such a

label can't be non-local because it would interfere with subsequent

definitions of, and references to, local labels; and it can't be

local because the macro that defined it wouldn't know the label's

full name. NASM therefore introduces a third type of label, which is

probably only useful in macro definitions: if a label begins with

the \I{label prefix}special prefix \i\c{..@}, then it does nothing

to the local label mechanism. So you could code

\c label1:   ; a non-local label

\c .local:   ; this is really label1.local

\c ..@foo:   ; this is a special symbol

\c label2:   ; another non-local label

\c .local:   ; this is really label2.local

\c           jmp ..@foo             ; this will jump three lines up

NASM has the capacity to define other special symbols beginning with

a double period: for example, \c{..start} is used to specify the

entry point in the \c{obj} output format (see \k{dotdotstart}).

\C{preproc} The NASM \i{Preprocessor}

NASM contains a powerful \i{macro processor}, which supports

conditional assembly, multi-level file inclusion, two forms of macro

(single-line and multi-line), and a `context stack' mechanism for

extra macro power. Preprocessor directives all begin with a \c{%}

sign.

\H{slmacro} \i{Single-Line Macros}

\S{define} The Normal Way: \I\c{%idefine}\i\c{%define}

Single-line macros are defined using the \c{%define} preprocessor

directive. The definitions work in a similar way to C; so you can do

things like

\c %define ctrl 0x1F &

\c %define param(a,b) ((a)+(a)*(b))

\c           mov byte [param(2,ebx)], ctrl 'D'

which will expand to

\c           mov byte [(2)+(2)*(ebx)], 0x1F & 'D'

When the expansion of a single-line macro contains tokens which

invoke another macro, the expansion is performed at invocation time,

not at definition time. Thus the code

\c %define a(x) 1+b(x)

\c %define b(x) 2*x

\c           mov ax,a(8)

will evaluate in the expected way to \c{mov ax,1+2*8}, even though

the macro \c{b} wasn't defined at the time of definition of \c{a}.

Macros defined with \c{%define} are \i{case sensitive}: after

\c{%define foo bar}, only \c{foo} will expand to \c{bar}: \c{Foo} or

\c{FOO} will not. By using \c{%idefine} instead of \c{%define} (the

`i' stands for `insensitive') you can define all the case variants

of a macro at once, so that \c{%idefine foo bar} would cause

\c{foo}, \c{Foo}, \c{FOO}, \c{fOO} and so on all to expand to

\c{bar}.

There is a mechanism which detects when a macro call has occurred as

a result of a previous expansion of the same macro, to guard against

\i{circular references} and infinite loops. If this happens, the

preprocessor will only expand the first occurrence of the macro.

Hence, if you code

\c %define a(x) 1+a(x)

\c           mov ax,a(3)

the macro \c{a(3)} will expand once, becoming \c{1+a(3)}, and will

then expand no further. This behaviour can be useful: see \k{32c}

for an example of its use.

You can \I{overloading, single-line macros}overload single-line

macros: if you write

\c %define foo(x) 1+x

\c %define foo(x,y) 1+x*y

the preprocessor will be able to handle both types of macro call,

by counting the parameters you pass; so \c{foo(3)} will become

\c{1+3} whereas \c{foo(ebx,2)} will become \c{1+ebx*2}. However, if

you define

\c %define foo bar

then no other definition of \c{foo} will be accepted: a macro with

no parameters prohibits the definition of the same name as a macro

\e{with} parameters, and vice versa.

This doesn't prevent single-line macros being \e{redefined}: you can

perfectly well define a macro with

\c %define foo bar

and then re-define it later in the same source file with

\c %define foo baz

Then everywhere the macro \c{foo} is invoked, it will be expanded

according to the most recent definition. This is particularly useful

when defining single-line macros with \c{%assign} (see \k{assign}).

You can \i{pre-define} single-line macros using the `-d' option on

the NASM command line: see \k{opt-d}.

\S{assign} \i{Preprocessor Variables}: \i\c{%assign}

An alternative way to define single-line macros is by means of the

\c{%assign} command (and its \i{case sensitive}case-insensitive

counterpart \i\c{%iassign}, which differs from \c{%assign} in

exactly the same way that \c{%idefine} differs from \c{%define}).

\c{%assign} is used to define single-line macros which take no

parameters and have a numeric value. This value can be specified in

the form of an expression, and it will be evaluated once, when the

\c{%assign} directive is processed.

Like \c{%define}, macros defined using \c{%assign} can be re-defined

later, so you can do things like

\c %assign i i+1

to increment the numeric value of a macro.

\c{%assign} is useful for controlling the termination of \c{%rep}

preprocessor loops: see \k{rep} for an example of this. Another

use for \c{%assign} is given in \k{16c} and \k{32c}.

The expression passed to \c{%assign} is a \i{critical expression}

(see \k{crit}), and must also evaluate to a pure number (rather than

a relocatable reference such as a code or data address, or anything

involving a register).

\H{mlmacro} \i{Multi-Line Macros}: \I\c{%imacro}\i\c{%macro}

Multi-line macros are much more like the type of macro seen in MASM

and TASM: a multi-line macro definition in NASM looks something like

this.

\c %macro prologue 1

\c           push ebp

\c           mov ebp,esp

\c           sub esp,%1

\c %endmacro

This defines a C-like function prologue as a macro: so you would

invoke the macro with a call such as

\c myfunc:   prologue 12

which would expand to the three lines of code

\c myfunc:   push ebp

\c           mov ebp,esp

\c           sub esp,12

The number \c{1} after the macro name in the \c{%macro} line defines

the number of parameters the macro \c{prologue} expects to receive.

The use of \c{%1} inside the macro definition refers to the first

parameter to the macro call. With a macro taking more than one

parameter, subsequent parameters would be referred to as \c{%2},

\c{%3} and so on.

Multi-line macros, like single-line macros, are \i{case-sensitive},

unless you define them using the alternative directive \c{%imacro}.

If you need to pass a comma as \e{part} of a parameter to a

multi-line macro, you can do that by enclosing the entire parameter

in \I{braces, around macro parameters}braces. So you could code

things like

\c %macro silly 2

\c %2:       db %1

\c %endmacro

\c           silly 'a', letter_a    ; letter_a:  db 'a'

\c           silly 'ab', string_ab  ; string_ab: db 'ab'

\c           silly {13,10}, crlf    ; crlf:      db 13,10

\S{mlmacover} \i{Overloading Multi-Line Macros}

As with single-line macros, multi-line macros can be overloaded by

defining the same macro name several times with different numbers of

parameters. This time, no exception is made for macros with no

parameters at all. So you could define

\c %macro prologue 0

\c           push ebp

\c           mov ebp,esp

\c %endmacro

to define an alternative form of the function prologue which

allocates no local stack space.

Sometimes, however, you might want to `overload' a machine

instruction; for example, you might want to define

\c %macro push 2

\c           push %1

\c           push %2

\c %endmacro

so that you could code

\c           push ebx               ; this line is not a macro call

\c           push eax,ecx           ; but this one is

Ordinarily, NASM will give a warning for the first of the above two

lines, since \c{push} is now defined to be a macro, and is being

invoked with a number of parameters for which no definition has been

given. The correct code will still be generated, but the assembler

will give a warning. This warning can be disabled by the use of the

\c{-w-macro-params} command-line option (see \k{opt-w}).

\S{maclocal} \i{Macro-Local Labels}

NASM allows you to define labels within a multi-line macro

definition in such a way as to make them local to the macro call: so

calling the same macro multiple times will use a different label

each time. You do this by prefixing \i\c{%%} to the label name. So

you can invent an instruction which executes a \c{RET} if the \c{Z}

flag is set by doing this:

\c %macro retz 0

\c           jnz %%skip

\c           ret

\c %%skip:

\c %endmacro

You can call this macro as many times as you want, and every time

you call it NASM will make up a different `real' name to substitute

for the label \c{%%skip}. The names NASM invents are of the form

\c{..@2345.skip}, where the number 2345 changes with every macro

call. The \i\c{..@} prefix prevents macro-local labels from

interfering with the local label mechanism, as described in

\k{locallab}. You should avoid defining your own labels in this form

(the \c{..@} prefix, then a number, then another period) in case

they interfere with macro-local labels.

\S{mlmacgre} \i{Greedy Macro Parameters}

Occasionally it is useful to define a macro which lumps its entire

command line into one parameter definition, possibly after

extracting one or two smaller parameters from the front. An example

might be a macro to write a text string to a file in MS-DOS, where

you might want to be able to write

\c           writefile [filehandle],"hello, world",13,10

NASM allows you to define the last parameter of a macro to be

\e{greedy}, meaning that if you invoke the macro with more

parameters than it expects, all the spare parameters get lumped into

the last defined one along with the separating commas. So if you

code:

\c %macro writefile 2+

\c           jmp %%endstr

\c %%str:    db %2

\c %%endstr: mov dx,%%str

\c           mov cx,%%endstr-%%str

\c           mov bx,%1

\c           mov ah,0x40

\c           int 0x21

\c %endmacro

then the example call to \c{writefile} above will work as expected:

the text before the first comma, \c{[filehandle]}, is used as the

first macro parameter and expanded when \c{%1} is referred to, and

all the subsequent text is lumped into \c{%2} and placed after the

\c{db}.

The greedy nature of the macro is indicated to NASM by the use of

the \I{+ modifier}\c{+} sign after the parameter count on the

\c{%macro} line.

If you define a greedy macro, you are effectively telling NASM how

it should expand the macro given \e{any} number of parameters from

the actual number specified up to infinity; in this case, for

example, NASM now knows what to do when it sees a call to

\c{writefile} with 2, 3, 4 or more parameters. NASM will take this

into account when overloading macros, and will not allow you to

define another form of \c{writefile} taking 4 parameters (for

example).

Of course, the above macro could have been implemented as a

non-greedy macro, in which case the call to it would have had to

look like

\c           writefile [filehandle], {"hello, world",13,10}

NASM provides both mechanisms for putting \i{commas in macro

parameters}, and you choose which one you prefer for each macro

definition.

See \k{sectmac} for a better way to write the above macro.

\S{mlmacdef} \i{Default Macro Parameters}

NASM also allows you to define a multi-line macro with a \e{range}

of allowable parameter counts. If you do this, you can specify

defaults for \i{omitted parameters}. So, for example:

\c %macro die 0-1 "Painful program death has occurred."

\c           writefile 2,%1

\c           mov ax,0x4c01

\c           int 0x21

\c %endmacro

This macro (which makes use of the \c{writefile} macro defined in

\k{mlmacgre}) can be called with an explicit error message, which it

will display on the error output stream before exiting, or it can be

called with no parameters, in which case it will use the default

error message supplied in the macro definition.

In general, you supply a minimum and maximum number of parameters

for a macro of this type; the minimum number of parameters are then

required in the macro call, and then you provide defaults for the

optional ones. So if a macro definition began with the line

\c %macro foobar 1-3 eax,[ebx+2]

then it could be called with between one and three parameters, and

\c{%1} would always be taken from the macro call. \c{%2}, if not

specified by the macro call, would default to \c{eax}, and \c{%3} if

not specified would default to \c{[ebx+2]}.

You may omit parameter defaults from the macro definition, in which

case the parameter default is taken to be blank. This can be useful

for macros which can take a variable number of parameters, since the

\i\c{%0} token (see \k{percent0}) allows you to determine how many

parameters were really passed to the macro call.

This defaulting mechanism can be combined with the greedy-parameter

mechanism; so the \c{die} macro above could be made more powerful,

and more useful, by changing the first line of the definition to

\c %macro die 0-1+ "Painful program death has occurred.",13,10

The maximum parameter count can be infinite, denoted by \c{*}. In

this case, of course, it is impossible to provide a \e{full} set of

default parameters. Examples of this usage are shown in \k{rotate}.

\S{percent0} \i\c{%0}: \I{counting macro parameters}Macro Parameter Counter

For a macro which can take a variable number of parameters, the

parameter reference \c{%0} will return a numeric constant giving the

number of parameters passed to the macro. This can be used as an

argument to \c{%rep} (see \k{rep}) in order to iterate through all

the parameters of a macro. Examples are given in \k{rotate}.

\S{rotate} \i\c{%rotate}: \i{Rotating Macro Parameters}

Unix shell programmers will be familiar with the \I{shift

command}\c{shift} shell command, which allows the arguments passed

to a shell script (referenced as \c{$1}, \c{$2} and so on) to be

moved left by one place, so that the argument previously referenced

as \c{$2} becomes available as \c{$1}, and the argument previously

referenced as \c{$1} is no longer available at all.

NASM provides a similar mechanism, in the form of \c{%rotate}. As

its name suggests, it differs from the Unix \c{shift} in that no

parameters are lost: parameters rotated off the left end of the

argument list reappear on the right, and vice versa.

\c{%rotate} is invoked with a single numeric argument (which may be

an expression). The macro parameters are rotated to the left by that

many places. If the argument to \c{%rotate} is negative, the macro

parameters are rotated to the right.

\I{iterating over macro parameters}So a pair of macros to save and

restore a set of registers might work as follows:

\c %macro multipush 1-*

\c %rep %0

\c           push %1

\c %rotate 1

\c %endrep

\c %endmacro

This macro invokes the \c{PUSH} instruction on each of its arguments

in turn, from left to right. It begins by pushing its first

argument, \c{%1}, then invokes \c{%rotate} to move all the arguments

one place to the left, so that the original second argument is now

available as \c{%1}. Repeating this procedure as many times as there

were arguments (achieved by supplying \c{%0} as the argument to

\c{%rep}) causes each argument in turn to be pushed.

Note also the use of \c{*} as the maximum parameter count,

indicating that there is no upper limit on the number of parameters

you may supply to the \i\c{multipush} macro.

It would be convenient, when using this macro, to have a \c{POP}

equivalent, which \e{didn't} require the arguments to be given in

reverse order. Ideally, you would write the \c{multipush} macro

call, then cut-and-paste the line to where the pop needed to be

done, and change the name of the called macro to \c{multipop}, and

the macro would take care of popping the registers in the opposite

order from the one in which they were pushed.

This can be done by the following definition:

\c %macro multipop 1-*

\c %rep %0

\c %rotate -1

\c           pop %1

\c %endrep

\c %endmacro

This macro begins by rotating its arguments one place to the

\e{right}, so that the original \e{last} argument appears as \c{%1}.

This is then popped, and the arguments are rotated right again, so

the second-to-last argument becomes \c{%1}. Thus the arguments are

iterated through in reverse order.

\S{concat} \i{Concatenating Macro Parameters}

NASM can concatenate macro parameters on to other text surrounding

them. This allows you to declare a family of symbols, for example,

in a macro definition. If, for example, you wanted to generate a

table of key codes along with offsets into the table, you could code

something like

\c %macro keytab_entry 2

\c keypos%1 equ $-keytab

\c           db %2

\c %endmacro

\c keytab:

\c           keytab_entry F1,128+1

\c           keytab_entry F2,128+2

\c           keytab_entry Return,13

which would expand to

\c keytab:

\c keyposF1 equ $-keytab

\c           db 128+1

\c keyposF2 equ $-keytab

\c           db 128+2

\c keyposReturn equ $-keytab

\c           db 13

You can just as easily concatenate text on to the other end of a

macro parameter, by writing \c{%1foo}.

If you need to append a \e{digit} to a macro parameter, for example

defining labels \c{foo1} and \c{foo2} when passed the parameter

\c{foo}, you can't code \c{%11} because that would be taken as the

eleventh macro parameter. Instead, you must code

\I{braces, after % sign}\c{%\{1\}1}, which will separate the first

\c{1} (giving the number of the macro parameter) from the second

(literal text to be concatenated to the parameter).

This concatenation can also be applied to other preprocessor in-line

objects, such as macro-local labels (\k{maclocal}) and context-local

labels (\k{ctxlocal}). In all cases, ambiguities in syntax can be

resolved by enclosing everything after the \c{%} sign and before the

literal text in braces: so \c{%\{%foo\}bar} concatenates the text

\c{bar} to the end of the real name of the macro-local label

\c{%%foo}. (This is unnecessary, since the form NASM uses for the

real names of macro-local labels means that the two usages

\c{%\{%foo\}bar} and \c{%%foobar} would both expand to the same

thing anyway; nevertheless, the capability is there.)

\S{mlmaccc} \i{Condition Codes as Macro Parameters}

NASM can give special treatment to a macro parameter which contains

a condition code. For a start, you can refer to the macro parameter

\c{%1} by means of the alternative syntax \i\c{%+1}, which informs

NASM that this macro parameter is supposed to contain a condition

code, and will cause the preprocessor to report an error message if

the macro is called with a parameter which is \e{not} a valid

condition code.

Far more usefully, though, you can refer to the macro parameter by

means of \i\c{%-1}, which NASM will expand as the \e{inverse}

condition code. So the \c{retz} macro defined in \k{maclocal} can be

replaced by a general \i{conditional-return macro} like this:

\c %macro retc 1

\c           j%-1 %%skip

\c           ret

\c %%skip:

\c %endmacro

This macro can now be invoked using calls like \c{retc ne}, which

will cause the conditional-jump instruction in the macro expansion

to come out as \c{JE}, or \c{retc po} which will make the jump a

\c{JPE}.

The \c{%+1} macro-parameter reference is quite happy to interpret

the arguments \c{CXZ} and \c{ECXZ} as valid condition codes;

however, \c{%-1} will report an error if passed either of these,

because no inverse condition code exists.

\S{nolist} \i{Disabling Listing Expansion}\I\c{.nolist}

When NASM is generating a listing file from your program, it will

generally expand multi-line macros by means of writing the macro

call and then listing each line of the expansion. This allows you to

see which instructions in the macro expansion are generating what

code; however, for some macros this clutters the listing up

unnecessarily.

NASM therefore provides the \c{.nolist} qualifier, which you can

include in a macro definition to inhibit the expansion of the macro

in the listing file. The \c{.nolist} qualifier comes directly after

the number of parameters, like this:

\c %macro foo 1.nolist

Or like this:

\c %macro bar 1-5+.nolist a,b,c,d,e,f,g,h

\H{condasm} \i{Conditional Assembly}\I\c{%if}

Similarly to the C preprocessor, NASM allows sections of a source

file to be assembled only if certain conditions are met. The general

syntax of this feature looks like this:

\c %if<condition>

\c ; some code which only appears if <condition> is met

\c %elif<condition2>

\c ; only appears if <condition> is not met but <condition2> is

\c %else

\c ; this appears if neither <condition> nor <condition2> was met

\c %endif

The \i\c{%else} clause is optional, as is the \i\c{%elif} clause.

You can have more than one \c{%elif} clause as well.

\S{ifdef} \i\c{%ifdef}: \i{Testing Single-Line Macro Existence}

Beginning a conditional-assembly block with the line \c{%ifdef

MACRO} will assemble the subsequent code if, and only if, a

single-line macro called \c{MACRO} is defined. If not, then the

\c{%elif} and \c{%else} blocks (if any) will be processed instead.

For example, when debugging a program, you might want to write code

such as

\c           ; perform some function

\c %ifdef DEBUG

\c           writefile 2,"Function performed successfully",13,10

\c %endif

\c           ; go and do something else

Then you could use the command-line option \c{-dDEBUG} to create a

version of the program which produced debugging messages, and remove

the option to generate the final release version of the program.

You can test for a macro \e{not} being defined by using

\i\c{%ifndef} instead of \c{%ifdef}. You can also test for macro

definitions in \c{%elif} blocks by using \i\c{%elifdef} and

\i\c{%elifndef}.

\S{ifctx} \i\c{%ifctx}: \i{Testing the Context Stack}

The conditional-assembly construct \c{%ifctx ctxname} will cause the

subsequent code to be assembled if and only if the top context on

the preprocessor's context stack has the name \c{ctxname}. As with

\c{%ifdef}, the inverse and \c{%elif} forms \i\c{%ifnctx},

\i\c{%elifctx} and \i\c{%elifnctx} are also supported.

For more details of the context stack, see \k{ctxstack}. For a

sample use of \c{%ifctx}, see \k{blockif}.

\S{if} \i\c{%if}: \i{Testing Arbitrary Numeric Expressions}

The conditional-assembly construct \c{%if expr} will cause the

subsequent code to be assembled if and only if the value of the

numeric expression \c{expr} is non-zero. An example of the use of

this feature is in deciding when to break out of a \c{%rep}

preprocessor loop: see \k{rep} for a detailed example.

The expression given to \c{%if}, and its counterpart \i\c{%elif}, is

a critical expression (see \k{crit}).

\c{%if} extends the normal NASM expression syntax, by providing a

set of \i{relational operators} which are not normally available in

expressions. The operators \i\c{=}, \i\c{<}, \i\c{>}, \i\c{<=},

\i\c{>=} and \i\c{<>} test equality, less-than, greater-than,

less-or-equal, greater-or-equal and not-equal respectively. The

C-like forms \i\c{==} and \i\c{!=} are supported as alternative

forms of \c{=} and \c{<>}. In addition, low-priority logical

operators \i\c{&&}, \i\c{^^} and \i\c{||} are provided, supplying

\i{logical AND}, \i{logical XOR} and \i{logical OR}. These work like

the C logical operators (although C has no logical XOR), in that

they always return either 0 or 1, and treat any non-zero input as 1

(so that \c{^^}, for example, returns 1 if exactly one of its inputs

is zero, and 0 otherwise). The relational operators also return 1

for true and 0 for false.

\S{ifidn} \i\c{%ifidn} and \i\c{%ifidni}: \i{Testing Exact Text

Identity}

The construct \c{%ifidn text1,text2} will cause the subsequent code

to be assembled if and only if \c{text1} and \c{text2}, after

expanding single-line macros, are identical pieces of text.

Differences in white space are not counted.

\c{%ifidni} is similar to \c{%ifidn}, but is \i{case-insensitive}.

For example, the following macro pushes a register or number on the

stack, and allows you to treat \c{IP} as a real register:

\c %macro pushparam 1

\c %ifidni %1,ip

\c           call %%label

\c %%label:

\c %else

\c           push %1

\c %endif

\c %endmacro

Like most other \c{%if} constructs, \c{%ifidn} has a counterpart

\i\c{%elifidn}, and negative forms \i\c{%ifnidn} and \i\c{%elifnidn}.

Similarly, \c{%ifidni} has counterparts \i\c{%elifidni},

\i\c{%ifnidni} and \i\c{%elifnidni}.

\S{iftyp} \i\c{%ifid}, \i\c{%ifnum}, \i\c{%ifstr}: \i{Testing Token

Types}

Some macros will want to perform different tasks depending on

whether they are passed a number, a string, or an identifier. For

example, a string output macro might want to be able to cope with

being passed either a string constant or a pointer to an existing

string.

The conditional assembly construct \c{%ifid}, taking one parameter

(which may be blank), assembles the subsequent code if and only if

the first token in the parameter exists and is an identifier.

\c{%ifnum} works similarly, but tests for the token being a numeric

constant; \c{%ifstr} tests for it being a string.

For example, the \c{writefile} macro defined in \k{mlmacgre} can be

extended to take advantage of \c{%ifstr} in the following fashion:

\c %macro writefile 2-3+

\c %ifstr %2

\c           jmp %%endstr

\c %if %0 = 3

\c %%str:	  db %2,%3

\c %else

\c %%str:	  db %2

\c %endif

\c %%endstr: mov dx,%%str

\c           mov cx,%%endstr-%%str

\c %else

\c 	  mov dx,%2

\c 	  mov cx,%3

\c %endif

\c           mov bx,%1

\c           mov ah,0x40

\c           int 0x21

\c %endmacro

Then the \c{writefile} macro can cope with being called in either of

the following two ways:

\c           writefile [file], strpointer, length

\c           writefile [file], "hello", 13, 10

In the first, \c{strpointer} is used as the address of an

already-declared string, and \c{length} is used as its length; in

the second, a string is given to the macro, which therefore declares

it itself and works out the address and length for itself.

Note the use of \c{%if} inside the \c{%ifstr}: this is to detect

whether the macro was passed two arguments (so the string would be a

single string constant, and \c{db %2} would be adequate) or more (in

which case, all but the first two would be lumped together into

\c{%3}, and \c{db %2,%3} would be required).

\I\c{%ifnid}\I\c{%elifid}\I\c{%elifnid}\I\c{%ifnnum}\I\c{%elifnum}\I\c{%elifnnum}\I\c{%ifnstr}\I\c{%elifstr}\I\c{%elifnstr}

The usual \c{%elifXXX}, \c{%ifnXXX} and \c{%elifnXXX} versions exist

for each of \c{%ifid}, \c{%ifnum} and \c{%ifstr}.

\S{pperror} \i\c{%error}: Reporting \i{User-Defined Errors}

The preprocessor directive \c{%error} will cause NASM to report an

error if it occurs in assembled code. So if other users are going to

try to assemble your source files, you can ensure that they define

the right macros by means of code like this:

\c %ifdef SOME_MACRO

\c ; do some setup

\c %elifdef SOME_OTHER_MACRO

\c ; do some different setup

\c %else

\c %error Neither SOME_MACRO nor SOME_OTHER_MACRO was defined.

\c %endif

Then any user who fails to understand the way your code is supposed

to be assembled will be quickly warned of their mistake, rather than

having to wait until the program crashes on being run and then not

knowing what went wrong.

\H{rep} \i{Preprocessor Loops}\I{repeating code}: \i\c{%rep}

NASM's \c{TIMES} prefix, though useful, cannot be used to invoke a

multi-line macro multiple times, because it is processed by NASM

after macros have already been expanded. Therefore NASM provides

another form of loop, this time at the preprocessor level: \c{%rep}.

The directives \c{%rep} and \i\c{%endrep} (\c{%rep} takes a numeric

argument, which can be an expression; \c{%endrep} takes no

arguments) can be used to enclose a chunk of code, which is then

replicated as many times as specified by the preprocessor:

\c %assign i 0

\c %rep 64

\c           inc word [table+2*i]

\c %assign i i+1

\c %endrep

This will generate a sequence of 64 \c{INC} instructions,

incrementing every word of memory from \c{[table]} to

\c{[table+126]}.

For more complex termination conditions, or to break out of a repeat

loop part way along, you can use the \i\c{%exitrep} directive to

terminate the loop, like this:

\c fibonacci:

\c %assign i 0

\c %assign j 1

\c %rep 100

\c %if j > 65535

\c %exitrep

\c %endif

\c           dw j

\c %assign k j+i

\c %assign i j

\c %assign j k

\c %endrep

\c fib_number equ ($-fibonacci)/2

This produces a list of all the Fibonacci numbers that will fit in

16 bits. Note that a maximum repeat count must still be given to

\c{%rep}. This is to prevent the possibility of NASM getting into an

infinite loop in the preprocessor, which (on multitasking or

multi-user systems) would typically cause all the system memory to

be gradually used up and other applications to start crashing.

\H{include} \i{Including Other Files}

Using, once again, a very similar syntax to the C preprocessor,

NASM's preprocessor lets you include other source files into your

code. This is done by the use of the \i\c{%include} directive:

\c %include "macros.mac"

will include the contents of the file \c{macros.mac} into the source

file containing the \c{%include} directive.

Include files are \I{searching for include files}searched for in the

current directory (the directory you're in when you run NASM, as

opposed to the location of the NASM executable or the location of

the source file), plus any directories specified on the NASM command

line using the \c{-i} option.

The standard C idiom for preventing a file being included more than

once is just as applicable in NASM: if the file \c{macros.mac} has

the form

\c %ifndef MACROS_MAC

\c %define MACROS_MAC

\c ; now define some macros

\c %endif

then including the file more than once will not cause errors,

because the second time the file is included nothing will happen

because the macro \c{MACROS_MAC} will already be defined.

You can force a file to be included even if there is no \c{%include}

directive that explicitly includes it, by using the \i\c{-p} option

on the NASM command line (see \k{opt-p}).

\H{ctxstack} The \i{Context Stack}

Having labels that are local to a macro definition is sometimes not

quite powerful enough: sometimes you want to be able to share labels

between several macro calls. An example might be a \c{REPEAT} ...

\c{UNTIL} loop, in which the expansion of the \c{REPEAT} macro

would need to be able to refer to a label which the \c{UNTIL} macro

had defined. However, for such a macro you would also want to be

able to nest these loops.

NASM provides this level of power by means of a \e{context stack}.

The preprocessor maintains a stack of \e{contexts}, each of which is

characterised by a name. You add a new context to the stack using

the \i\c{%push} directive, and remove one using \i\c{%pop}. You can

define labels that are local to a particular context on the stack.

\S{pushpop} \i\c{%push} and \i\c{%pop}: \I{creating

contexts}\I{removing contexts}Creating and Removing Contexts

The \c{%push} directive is used to create a new context and place it

on the top of the context stack. \c{%push} requires one argument,

which is the name of the context. For example:

\c %push foobar

This pushes a new context called \c{foobar} on the stack. You can

have several contexts on the stack with the same name: they can

still be distinguished.

The directive \c{%pop}, requiring no arguments, removes the top

context from the context stack and destroys it, along with any

labels associated with it.

\S{ctxlocal} \i{Context-Local Labels}

Just as the usage \c{%%foo} defines a label which is local to the

particular macro call in which it is used, the usage \I{%$}\c{%$foo}

is used to define a label which is local to the context on the top

of the context stack. So the \c{REPEAT} and \c{UNTIL} example given

above could be implemented by means of:

\c %macro repeat 0

\c %push repeat

\c %$begin:

\c %endmacro

\c %macro until 1

\c           j%-1 %$begin

\c %pop

\c %endmacro

and invoked by means of, for example,

\c           mov cx,string

\c           repeat

\c           add cx,3

\c           scasb

\c           until e

which would scan every fourth byte of a string in search of the byte

in \c{AL}.

If you need to define, or access, labels local to the context

\e{below} the top one on the stack, you can use \I{%$$}\c{%$$foo}, or

\c{%$$$foo} for the context below that, and so on.

\S{ctxdefine} \i{Context-Local Single-Line Macros}

NASM also allows you to define single-line macros which are local to

a particular context, in just the same way:

\c %define %$localmac 3

will define the single-line macro \c{%$localmac} to be local to the

top context on the stack. Of course, after a subsequent \c{%push},

it can then still be accessed by the name \c{%$$localmac}.

\S{ctxrepl} \i\c{%repl}: \I{renaming contexts}Renaming a Context

If you need to change the name of the top context on the stack (in

order, for example, to have it respond differently to \c{%ifctx}),

you can execute a \c{%pop} followed by a \c{%push}; but this will

have the side effect of destroying all context-local labels and

macros associated with the context that was just popped.

NASM provides the directive \c{%repl}, which \e{replaces} a context

with a different name, without touching the associated macros and

labels. So you could replace the destructive code

\c %pop

\c %push newname

with the non-destructive version \c{%repl newname}.

\S{blockif} Example Use of the \i{Context Stack}: \i{Block IFs}

This example makes use of almost all the context-stack features,

including the conditional-assembly construct \i\c{%ifctx}, to

implement a block IF statement as a set of macros.

\c %macro if 1

\c     %push if

\c     j%-1 %$ifnot

\c %endmacro

\c %macro else 0

\c     %ifctx if

\c         %repl else

\c         jmp %$ifend

\c         %$ifnot:

\c     %else

\c         %error "expected `if' before `else'"

\c     %endif

\c %endmacro

\c %macro endif 0

\c     %ifctx if

\c         %$ifnot:

\c         %pop

\c     %elifctx else

\c         %$ifend:

\c         %pop

\c     %else

\c         %error "expected `if' or `else' before `endif'"

\c     %endif

\c %endmacro

This code is more robust than the \c{REPEAT} and \c{UNTIL} macros

given in \k{ctxlocal}, because it uses conditional assembly to check

that the macros are issued in the right order (for example, not

calling \c{endif} before \c{if}) and issues a \c{%error} if they're

not.

In addition, the \c{endif} macro has to be able to cope with the two

distinct cases of either directly following an \c{if}, or following

an \c{else}. It achieves this, again, by using conditional assembly

to do different things depending on whether the context on top of

the stack is \c{if} or \c{else}.

The \c{else} macro has to preserve the context on the stack, in

order to have the \c{%$ifnot} referred to by the \c{if} macro be the

same as the one defined by the \c{endif} macro, but has to change

the context's name so that \c{endif} will know there was an

intervening \c{else}. It does this by the use of \c{%repl}.

A sample usage of these macros might look like:

\c           cmp ax,bx

\c           if ae

\c             cmp bx,cx

\c             if ae

\c               mov ax,cx

\c             else

\c               mov ax,bx

\c             endif

\c           else

\c             cmp ax,cx

\c             if ae

\c               mov ax,cx

\c             endif

\c           endif

The block-\c{IF} macros handle nesting quite happily, by means of

pushing another context, describing the inner \c{if}, on top of the

one describing the outer \c{if}; thus \c{else} and \c{endif} always

refer to the last unmatched \c{if} or \c{else}.

\H{stdmac} \i{Standard Macros}

NASM defines a set of standard macros, which are already defined

when it starts to process any source file. If you really need a

program to be assembled with no pre-defined macros, you can use the

\i\c{%clear} directive to empty the preprocessor of everything.

Most \i{user-level assembler directives} (see \k{directive}) are

implemented as macros which invoke primitive directives; these are

described in \k{directive}. The rest of the standard macro set is

described here.

\S{stdmacver} \i\c{__NASM_MAJOR__} and \i\c{__NASM_MINOR__}: \i{NASM

Version}

The single-line macros \c{__NASM_MAJOR__} and \c{__NASM_MINOR__}

expand to the major and minor parts of the \i{version number of

NASM} being used. So, under NASM 0.96 for example,

\c{__NASM_MAJOR__} would be defined to be 0 and \c{__NASM_MINOR__}

would be defined as 96.

\S{fileline} \i\c{__FILE__} and \i\c{__LINE__}: File Name and Line Number

Like the C preprocessor, NASM allows the user to find out the file

name and line number containing the current instruction. The macro

\c{__FILE__} expands to a string constant giving the name of the

current input file (which may change through the course of assembly

if \c{%include} directives are used), and \c{__LINE__} expands to a

numeric constant giving the current line number in the input file.

These macros could be used, for example, to communicate debugging

information to a macro, since invoking \c{__LINE__} inside a macro

definition (either single-line or multi-line) will return the line

number of the macro \e{call}, rather than \e{definition}. So to

determine where in a piece of code a crash is occurring, for

example, one could write a routine \c{stillhere}, which is passed a

line number in \c{EAX} and outputs something like `line 155: still

here'. You could then write a macro

\c %macro notdeadyet 0

\c           push eax

\c           mov eax,__LINE__

\c           call stillhere

\c           pop eax

\c %endmacro

and then pepper your code with calls to \c{notdeadyet} until you

find the crash point.

\S{struc} \i\c{STRUC} and \i\c{ENDSTRUC}: \i{Declaring Structure} Data Types

The core of NASM contains no intrinsic means of defining data

structures; instead, the preprocessor is sufficiently powerful that

data structures can be implemented as a set of macros. The macros

\c{STRUC} and \c{ENDSTRUC} are used to define a structure data type.

\c{STRUC} takes one parameter, which is the name of the data type.

This name is defined as a symbol with the value zero, and also has

the suffix \c{_size} appended to it and is then defined as an

\c{EQU} giving the size of the structure. Once \c{STRUC} has been

issued, you are defining the structure, and should define fields

using the \c{RESB} family of pseudo-instructions, and then invoke

\c{ENDSTRUC} to finish the definition.

For example, to define a structure called \c{mytype} containing a

longword, a word, a byte and a string of bytes, you might code

\c           struc mytype

\c mt_long:  resd 1

\c mt_word:  resw 1

\c mt_byte:  resb 1

\c mt_str:   resb 32

\c           endstruc

The above code defines six symbols: \c{mt_long} as 0 (the offset

from the beginning of a \c{mytype} structure to the longword field),

\c{mt_word} as 4, \c{mt_byte} as 6, \c{mt_str} as 7, \c{mytype_size}

as 39, and \c{mytype} itself as zero.

The reason why the structure type name is defined at zero is a side

effect of allowing structures to work with the local label

mechanism: if your structure members tend to have the same names in

more than one structure, you can define the above structure like this:

\c           struc mytype

\c .long:    resd 1

\c .word:    resw 1

\c .byte:    resb 1

\c .str:     resb 32

\c           endstruc

This defines the offsets to the structure fields as \c{mytype.long},

\c{mytype.word}, \c{mytype.byte} and \c{mytype.str}.

NASM, since it has no \e{intrinsic} structure support, does not

support any form of period notation to refer to the elements of a

structure once you have one (except the above local-label notation),

so code such as \c{mov ax,[mystruc.mt_word]} is not valid.

\c{mt_word} is a constant just like any other constant, so the

correct syntax is \c{mov ax,[mystruc+mt_word]} or \c{mov

ax,[mystruc+mytype.word]}.

\S{istruc} \i\c{ISTRUC}, \i\c{AT} and \i\c{IEND}: Declaring

\i{Instances of Structures}

Having defined a structure type, the next thing you typically want

to do is to declare instances of that structure in your data

segment. NASM provides an easy way to do this in the \c{ISTRUC}

mechanism. To declare a structure of type \c{mytype} in a program,

you code something like this:

\c mystruc:  istruc mytype

\c           at mt_long, dd 123456

\c           at mt_word, dw 1024

\c           at mt_byte, db 'x'

\c           at mt_str, db 'hello, world', 13, 10, 0

\c           iend

The function of the \c{AT} macro is to make use of the \c{TIMES}

prefix to advance the assembly position to the correct point for the

specified structure field, and then to declare the specified data.

Therefore the structure fields must be declared in the same order as

they were specified in the structure definition.

If the data to go in a structure field requires more than one source

line to specify, the remaining source lines can easily come after

the \c{AT} line. For example:

\c           at mt_str, db 123,134,145,156,167,178,189

\c           db 190,100,0

Depending on personal taste, you can also omit the code part of the

\c{AT} line completely, and start the structure field on the next

line:

\c           at mt_str

\c           db 'hello, world'

\c           db 13,10,0

\S{align} \i\c{ALIGN} and \i\c{ALIGNB}: Data Alignment

The \c{ALIGN} and \c{ALIGNB} macros provides a convenient way to

align code or data on a word, longword, paragraph or other boundary.

(Some assemblers call this directive \i\c{EVEN}.) The syntax of the

\c{ALIGN} and \c{ALIGNB} macros is

\c           align 4                ; align on 4-byte boundary

\c           align 16               ; align on 16-byte boundary

\c           align 8,db 0           ; pad with 0s rather than NOPs

\c           align 4,resb 1         ; align to 4 in the BSS

\c           alignb 4               ; equivalent to previous line

Both macros require their first argument to be a power of two; they

both compute the number of additional bytes required to bring the

length of the current section up to a multiple of that power of two,

and then apply the \c{TIMES} prefix to their second argument to

perform the alignment.

If the second argument is not specified, the default for \c{ALIGN}

is \c{NOP}, and the default for \c{ALIGNB} is \c{RESB 1}. So if the

second argument is specified, the two macros are equivalent.

Normally, you can just use \c{ALIGN} in code and data sections and

\c{ALIGNB} in BSS sections, and never need the second argument

except for special purposes.

\c{ALIGN} and \c{ALIGNB}, being simple macros, perform no error

checking: they cannot warn you if their first argument fails to be a

power of two, or if their second argument generates more than one

byte of code. In each of these cases they will silently do the wrong

thing.

\c{ALIGNB} (or \c{ALIGN} with a second argument of \c{RESB 1}) can

be used within structure definitions:

\c           struc mytype2

\c mt_byte:  resb 1

\c           alignb 2

\c mt_word:  resw 1

\c           alignb 4

\c mt_long:  resd 1

\c mt_str:   resb 32

\c           endstruc

This will ensure that the structure members are sensibly aligned

relative to the base of the structure.

A final caveat: \c{ALIGN} and \c{ALIGNB} work relative to the

beginning of the \e{section}, not the beginning of the address space

in the final executable. Aligning to a 16-byte boundary when the

section you're in is only guaranteed to be aligned to a 4-byte

boundary, for example, is a waste of effort. Again, NASM does not

check that the section's alignment characteristics are sensible for

the use of \c{ALIGN} or \c{ALIGNB}.

\C{directive} \i{Assembler Directives}

NASM, though it attempts to avoid the bureaucracy of assemblers like

MASM and TASM, is nevertheless forced to support a \e{few}

directives. These are described in this chapter.

NASM's directives come in two types: \i{user-level

directives}\e{user-level} directives and \i{primitive

directives}\e{primitive} directives. Typically, each directive has a

user-level form and a primitive form. In almost all cases, we

recommend that users use the user-level forms of the directives,

which are implemented as macros which call the primitive forms.

Primitive directives are enclosed in square brackets; user-level

directives are not.

In addition to the universal directives described in this chapter,

each object file format can optionally supply extra directives in

order to control particular features of that file format. These

\i{format-specific directives}\e{format-specific} directives are

documented along with the formats that implement them, in \k{outfmt}.

\H{bits} \i\c{BITS}: Specifying Target \i{Processor Mode}

The \c{BITS} directive specifies whether NASM should generate code

\I{16-bit mode, versus 32-bit mode}designed to run on a processor

operating in 16-bit mode, or code designed to run on a processor

operating in 32-bit mode. The syntax is \c{BITS 16} or \c{BITS 32}.

In most cases, you should not need to use \c{BITS} explicitly. The

\c{aout}, \c{coff}, \c{elf} and \c{win32} object formats, which are

designed for use in 32-bit operating systems, all cause NASM to

select 32-bit mode by default. The \c{obj} object format allows you

to specify each segment you define as either \c{USE16} or \c{USE32},

and NASM will set its operating mode accordingly, so the use of the

\c{BITS} directive is once again unnecessary.

The most likely reason for using the \c{BITS} directive is to write

32-bit code in a flat binary file; this is because the \c{bin}

output format defaults to 16-bit mode in anticipation of it being

used most frequently to write DOS \c{.COM} programs, DOS \c{.SYS}

device drivers and boot loader software.

You do \e{not} need to specify \c{BITS 32} merely in order to use

32-bit instructions in a 16-bit DOS program; if you do, the

assembler will generate incorrect code because it will be writing

code targeted at a 32-bit platform, to be run on a 16-bit one.

When NASM is in \c{BITS 16} state, instructions which use 32-bit

data are prefixed with an 0x66 byte, and those referring to 32-bit

addresses have an 0x67 prefix. In \c{BITS 32} state, the reverse is

true: 32-bit instructions require no prefixes, whereas instructions

using 16-bit data need an 0x66 and those working in 16-bit addresses

need an 0x67.

The \c{BITS} directive has an exactly equivalent primitive form,

\c{[BITS 16]} and \c{[BITS 32]}. The user-level form is a macro

which has no function other than to call the primitive form.

\H{section} \i\c{SECTION} or \i\c{SEGMENT}: Changing and \i{Defining

Sections}

\I{changing sections}\I{switching between sections}The \c{SECTION}

directive (\c{SEGMENT} is an exactly equivalent synonym) changes

which section of the output file the code you write will be

assembled into. In some object file formats, the number and names of

sections are fixed; in others, the user may make up as many as they

wish. Hence \c{SECTION} may sometimes give an error message, or may

define a new section, if you try to switch to a section that does

not (yet) exist.

The Unix object formats, and the \c{bin} object format, all support

the \i{standardised section names} \c{.text}, \c{.data} and \c{.bss}

for the code, data and uninitialised-data sections. The \c{obj}

format, by contrast, does not recognise these section names as being

special, and indeed will strip off the leading period of any section

name that has one.

\S{sectmac} The \i\c{__SECT__} Macro

The \c{SECTION} directive is unusual in that its user-level form

functions differently from its primitive form. The primitive form,

\c{[SECTION xyz]}, simply switches the current target section to the

one given. The user-level form, \c{SECTION xyz}, however, first

defines the single-line macro \c{__SECT__} to be the primitive

\c{[SECTION]} directive which it is about to issue, and then issues

it. So the user-level directive

\c           SECTION .text

expands to the two lines

\c %define __SECT__ [SECTION .text]

\c           [SECTION .text]

Users may find it useful to make use of this in their own macros.

For example, the \c{writefile} macro defined in \k{mlmacgre} can be

usefully rewritten in the following more sophisticated form:

\c %macro writefile 2+

\c           [section .data]

\c %%str:    db %2

\c %%endstr:

\c           __SECT__

\c           mov dx,%%str

\c           mov cx,%%endstr-%%str

\c           mov bx,%1

\c           mov ah,0x40

\c           int 0x21

\c %endmacro

This form of the macro, once passed a string to output, first

switches temporarily to the data section of the file, using the

primitive form of the \c{SECTION} directive so as not to modify

\c{__SECT__}. It then declares its string in the data section, and

then invokes \c{__SECT__} to switch back to \e{whichever} section

the user was previously working in. It thus avoids the need, in the

previous version of the macro, to include a \c{JMP} instruction to

jump over the data, and also does not fail if, in a complicated

\c{OBJ} format module, the user could potentially be assembling the

code in any of several separate code sections.

\H{absolute} \i\c{ABSOLUTE}: Defining Absolute Labels

The \c{ABSOLUTE} directive can be thought of as an alternative form

of \c{SECTION}: it causes the subsequent code to be directed at no

physical section, but at the hypothetical section starting at the

given absolute address. The only instructions you can use in this

mode are the \c{RESB} family.

\c{ABSOLUTE} is used as follows:

\c           absolute 0x1A

\c kbuf_chr  resw 1

\c kbuf_free resw 1

\c kbuf      resw 16

This example describes a section of the PC BIOS data area, at

segment address 0x40: the above code defines \c{kbuf_chr} to be

0x1A, \c{kbuf_free} to be 0x1C, and \c{kbuf} to be 0x1E.

The user-level form of \c{ABSOLUTE}, like that of \c{SECTION},

redefines the \i\c{__SECT__} macro when it is invoked.

\i\c{STRUC} and \i\c{ENDSTRUC} are defined as macros which use

\c{ABSOLUTE} (and also \c{__SECT__}).

\c{ABSOLUTE} doesn't have to take an absolute constant as an

argument: it can take an expression (actually, a \i{critical

expression}: see \k{crit}) and it can be a value in a segment. For

example, a TSR can re-use its setup code as run-time BSS like this:

\c           org 100h               ; it's a .COM program

\c           jmp setup              ; setup code comes last

\c           ; the resident part of the TSR goes here

\c setup:    ; now write the code that installs the TSR here

\c           absolute setup

\c runtimevar1 resw 1

\c runtimevar2 resd 20

\c tsr_end:

This defines some variables `on top of' the setup code, so that

after the setup has finished running, the space it took up can be

re-used as data storage for the running TSR. The symbol `tsr_end'

can be used to calculate the total size of the part of the TSR that

needs to be made resident.

\H{extern} \i\c{EXTERN}: \i{Importing Symbols} from Other Modules

\c{EXTERN} is similar to the MASM directive \c{EXTRN} and the C

keyword \c{extern}: it is used to declare a symbol which is not

defined anywhere in the module being assembled, but is assumed to be

defined in some other module and needs to be referred to by this

one. Not every object-file format can support external variables:

the \c{bin} format cannot.

The \c{EXTERN} directive takes as many arguments as you like. Each

argument is the name of a symbol:

\c           extern _printf

\c           extern _sscanf,_fscanf

Some object-file formats provide extra features to the \c{EXTERN}

directive. In all cases, the extra features are used by suffixing a

colon to the symbol name followed by object-format specific text.

For example, the \c{obj} format allows you to declare that the

default segment base of an external should be the group \c{dgroup}

by means of the directive

\c           extern _variable:wrt dgroup

The primitive form of \c{EXTERN} differs from the user-level form

only in that it can take only one argument at a time: the support

for multiple arguments is implemented at the preprocessor level.

You can declare the same variable as \c{EXTERN} more than once: NASM

will quietly ignore the second and later redeclarations. You can't

declare a variable as \c{EXTERN} as well as something else, though.

\H{global} \i\c{GLOBAL}: \i{Exporting Symbols} to Other Modules

\c{GLOBAL} is the other end of \c{EXTERN}: if one module declares a

symbol as \c{EXTERN} and refers to it, then in order to prevent

linker errors, some other module must actually \e{define} the

symbol and declare it as \c{GLOBAL}. Some assemblers use the name

\i\c{PUBLIC} for this purpose.

The \c{GLOBAL} directive applying to a symbol must appear \e{before}

the definition of the symbol.

\c{GLOBAL} uses the same syntax as \c{EXTERN}, except that it must

refer to symbols which \e{are} defined in the same module as the

\c{GLOBAL} directive. For example:

\c           global _main

\c _main:    ; some code

\c{GLOBAL}, like \c{EXTERN}, allows object formats to define private

extensions by means of a colon. The \c{elf} object format, for

example, lets you specify whether global data items are functions or

data:

\c           global hashlookup:function, hashtable:data

Like \c{EXTERN}, the primitive form of \c{GLOBAL} differs from the

user-level form only in that it can take only one argument at a

time.

\H{common} \i\c{COMMON}: Defining Common Data Areas

The \c{COMMON} directive is used to declare \i\e{common variables}.

A common variable is much like a global variable declared in the

uninitialised data section, so that

\c           common intvar 4

is similar in function to

\c           global intvar

\c           section .bss

\c intvar    resd 1

The difference is that if more than one module defines the same

common variable, then at link time those variables will be

\e{merged}, and references to \c{intvar} in all modules will point

at the same piece of memory.

Like \c{GLOBAL} and \c{EXTERN}, \c{COMMON} supports object-format

specific extensions. For example, the \c{obj} format allows common

variables to be NEAR or FAR, and the \c{elf} format allows you to

specify the alignment requirements of a common variable:

\c           common commvar 4:near  ; works in OBJ

\c           common intarray 100:4  ; works in ELF: 4 byte aligned

Once again, like \c{EXTERN} and \c{GLOBAL}, the primitive form of

\c{COMMON} differs from the user-level form only in that it can take

only one argument at a time.

\C{outfmt} \i{Output Formats}

NASM is a portable assembler, designed to be able to compile on any

ANSI C-supporting platform and produce output to run on a variety of

Intel x86 operating systems. For this reason, it has a large number

of available output formats, selected using the \i\c{-f} option on

the NASM \i{command line}. Each of these formats, along with its

extensions to the base NASM syntax, is detailed in this chapter.

As stated in \k{opt-o}, NASM chooses a \i{default name} for your

output file based on the input file name and the chosen output

format. This will be generated by removing the \i{extension}

(\c{.asm}, \c{.s}, or whatever you like to use) from the input file

name, and substituting an extension defined by the output format.

The extensions are given with each format below.

\H{binfmt} \i\c{bin}: \i{Flat-Form Binary}\I{pure binary} Output

The \c{bin} format does not produce object files: it generates

nothing in the output file except the code you wrote. Such `pure

binary' files are used by \i{MS-DOS}: \i\c{.COM} executables and

\i\c{.SYS} device drivers are pure binary files. Pure binary output

is also useful for \i{operating-system} and \i{boot loader}

development.

\c{bin} supports the three \i{standardised section names} \i\c{.text},

\i\c{.data} and \i\c{.bss} only. The file NASM outputs will contain the

contents of the \c{.text} section first, followed by the contents of

the \c{.data} section, aligned on a four-byte boundary. The \c{.bss}

section is not stored in the output file at all, but is assumed to

appear directly after the end of the \c{.data} section, again

aligned on a four-byte boundary.

If you specify no explicit \c{SECTION} directive, the code you write

will be directed by default into the \c{.text} section.

Using the \c{bin} format puts NASM by default into 16-bit mode (see

\k{bits}). In order to use \c{bin} to write 32-bit code such as an

OS kernel, you need to explicitly issue the \I\c{BITS}\c{BITS 32}

directive.

\c{bin} has no default output file name extension: instead, it

leaves your file name as it is once the original extension has been

removed. Thus, the default is for NASM to assemble \c{binprog.asm}

into a binary file called \c{binprog}.

\S{org} \i\c{ORG}: Binary File \i{Program Origin}

The \c{bin} format provides an additional directive to the list

given in \k{directive}: \c{ORG}. The function of the \c{ORG}

directive is to specify the origin address which NASM will assume

the program begins at when it is loaded into memory.

For example, the following code will generate the longword

\c{0x00000104}:

\c           org 0x100

\c           dd label

\c label:

Unlike the \c{ORG} directive provided by MASM-compatible assemblers,

which allows you to jump around in the object file and overwrite

code you have already generated, NASM's \c{ORG} does exactly what

the directive says: \e{origin}. Its sole function is to specify one

offset which is added to all internal address references within the

file; it does not permit any of the trickery that MASM's version

does. See \k{proborg} for further comments.

\S{binseg} \c{bin} Extensions to the \c{SECTION}

Directive\I{SECTION, bin extensions to}

The \c{bin} output format extends the \c{SECTION} (or \c{SEGMENT})

directive to allow you to specify the alignment requirements of

segments. This is done by appending the \i\c{ALIGN} qualifier to the

end of the section-definition line. For example,

\c           section .data align=16

switches to the section \c{.data} and also specifies that it must be

aligned on a 16-byte boundary.

The parameter to \c{ALIGN} specifies how many low bits of the

section start address must be forced to zero. The alignment value

given may be any power of two.\I{section alignment, in

bin}\I{segment alignment, in bin}\I{alignment, in bin sections}

\H{objfmt} \i\c{obj}: \i{Microsoft OMF}\I{OMF} Object Files

The \c{obj} file format (NASM calls it \c{obj} rather than \c{omf}

for historical reasons) is the one produced by \i{MASM} and

\i{TASM}, which is typically fed to 16-bit DOS linkers to produce

\i\c{.EXE} files. It is also the format used by \i{OS/2}.

\c{obj} provides a default output file-name extension of \c{.obj}.

\c{obj} is not exclusively a 16-bit format, though: NASM has full

support for the 32-bit extensions to the format. In particular,

32-bit \c{obj} format files are used by \i{Borland's Win32

compilers}, instead of using Microsoft's newer \i\c{win32} object

file format.

The \c{obj} format does not define any special segment names: you

can call your segments anything you like. Typical names for segments

in \c{obj} format files are \c{CODE}, \c{DATA} and \c{BSS}.

If your source file contains code before specifying an explicit

\c{SEGMENT} directive, then NASM will invent its own segment called

\i\c{__NASMDEFSEG} for you.

When you define a segment in an \c{obj} file, NASM defines the

segment name as a symbol as well, so that you can access the segment

address of the segment. So, for example:

\c           segment data

\c dvar:     dw 1234

\c           segment code

\c function: mov ax,data            ; get segment address of data

\c           mov ds,ax              ; and move it into DS

\c           inc word [dvar]        ; now this reference will work

\c           ret

The \c{obj} format also enables the use of the \i\c{SEG} and

\i\c{WRT} operators, so that you can write code which does things

like

\c           extern foo

\c           mov ax,seg foo         ; get preferred segment of foo

\c           mov ds,ax

\c           mov ax,data            ; a different segment

\c           mov es,ax

\c           mov ax,[ds:foo]        ; this accesses `foo'

\c           mov [es:foo wrt data],bx  ; so does this

\S{objseg} \c{obj} Extensions to the \c{SEGMENT}

Directive\I{SEGMENT, obj extensions to}

The \c{obj} output format extends the \c{SEGMENT} (or \c{SECTION})

directive to allow you to specify various properties of the segment

you are defining. This is done by appending extra qualifiers to the

end of the segment-definition line. For example,

\c           segment code private align=16

defines the segment \c{code}, but also declares it to be a private

segment, and requires that the portion of it described in this code

module must be aligned on a 16-byte boundary.

The available qualifiers are:

\b \i\c{PRIVATE}, \i\c{PUBLIC}, \i\c{COMMON} and \i\c{STACK} specify

the combination characteristics of the segment. \c{PRIVATE} segments

do not get combined with any others by the linker; \c{PUBLIC} and

\c{STACK} segments get concatenated together at link time; and

\c{COMMON} segments all get overlaid on top of each other rather

than stuck end-to-end.

\b \i\c{ALIGN} is used, as shown above, to specify how many low bits

of the segment start address must be forced to zero. The alignment

value given may be any power of two from 1 to 4096; in reality, the

only values supported are 1, 2, 4, 16, 256 and 4096, so if 8 is

specified it will be rounded up to 16, and 32, 64 and 128 will all

be rounded up to 256, and so on. Note that alignment to 4096-byte

boundaries is a \i{PharLap} extension to the format and may not be

supported by all linkers.\I{section alignment, in OBJ}\I{segment

alignment, in OBJ}\I{alignment, in OBJ sections}

\b \i\c{CLASS} can be used to specify the segment class; this feature

indicates to the linker that segments of the same class should be

placed near each other in the output file. The class name can be any

word, e.g. \c{CLASS=CODE}.

\b \i\c{OVERLAY}, like \c{CLASS}, is specified with an arbitrary word

as an argument, and provides overlay information to an

overlay-capable linker.

\b Segments can be declared as \i\c{USE16} or \i\c{USE32}, which has

the effect of recording the choice in the object file and also

ensuring that NASM's default assembly mode when assembling in that

segment is 16-bit or 32-bit respectively.

\b When writing \i{OS/2} object files, you should declare 32-bit

segments as \i\c{FLAT}, which causes the default segment base for

anything in the segment to be the special group \c{FLAT}, and also

defines the group if it is not already defined.

\b The \c{obj} file format also allows segments to be declared as

having a pre-defined absolute segment address, although no linkers

are currently known to make sensible use of this feature;

nevertheless, NASM allows you to declare a segment such as

\c{SEGMENT SCREEN ABSOLUTE=0xB800} if you need to. The \i\c{ABSOLUTE}

and \c{ALIGN} keywords are mutually exclusive.

NASM's default segment attributes are \c{PUBLIC}, \c{ALIGN=1}, no

class, no overlay, and \c{USE16}.

\S{group} \i\c{GROUP}: Defining Groups of Segments\I{segments, groups of}

The \c{obj} format also allows segments to be grouped, so that a

single segment register can be used to refer to all the segments in

a group. NASM therefore supplies the \c{GROUP} directive, whereby

you can code

\c           segment data

\c           ; some data

\c           segment bss

\c           ; some uninitialised data

\c           group dgroup data bss

which will define a group called \c{dgroup} to contain the segments

\c{data} and \c{bss}. Like \c{SEGMENT}, \c{GROUP} causes the group

name to be defined as a symbol, so that you can refer to a variable

\c{var} in the \c{data} segment as \c{var wrt data} or as \c{var wrt

dgroup}, depending on which segment value is currently in your

segment register.

If you just refer to \c{var}, however, and \c{var} is declared in a

segment which is part of a group, then NASM will default to giving

you the offset of \c{var} from the beginning of the \e{group}, not

the \e{segment}. Therefore \c{SEG var}, also, will return the group

base rather than the segment base.

NASM will allow a segment to be part of more than one group, but

will generate a warning if you do this. Variables declared in a

segment which is part of more than one group will default to being

relative to the first group that was defined to contain the segment.

A group does not have to contain any segments; you can still make

\c{WRT} references to a group which does not contain the variable

you are referring to. OS/2, for example, defines the special group

\c{FLAT} with no segments in it.

\S{uppercase} \i\c{UPPERCASE}: Disabling Case Sensitivity in Output

Although NASM itself is \i{case sensitive}, some OMF linkers are

not; therefore it can be useful for NASM to output single-case

object files. The \c{UPPERCASE} format-specific directive causes all

segment, group and symbol names that are written to the object file

to be forced to upper case just before being written. Within a

source file, NASM is still case-sensitive; but the object file can

be written entirely in upper case if desired.

\c{UPPERCASE} is used alone on a line; it requires no parameters.

\S{import} \i\c{IMPORT}: Importing DLL Symbols\I{DLL symbols,

importing}\I{symbols, importing from DLLs}

The \c{IMPORT} format-specific directive defines a symbol to be

imported from a DLL, for use if you are writing a DLL's \i{import

library} in NASM. You still need to declare the symbol as \c{EXTERN}

as well as using the \c{IMPORT} directive.

The \c{IMPORT} directive takes two required parameters, separated by

white space, which are (respectively) the name of the symbol you

wish to import and the name of the library you wish to import it

from. For example:

\c           import WSAStartup wsock32.dll

A third optional parameter gives the name by which the symbol is

known in the library you are importing it from, in case this is not

the same as the name you wish the symbol to be known by to your code

once you have imported it. For example:

\c           import asyncsel wsock32.dll WSAAsyncSelect

\S{export} \i\c{EXPORT}: Exporting DLL Symbols\I{DLL symbols,

exporting}\I{symbols, exporting from DLLs}

The \c{EXPORT} format-specific directive defines a global symbol to

be exported as a DLL symbol, for use if you are writing a DLL in

NASM. You still need to declare the symbol as \c{GLOBAL} as well as

using the \c{EXPORT} directive.

\c{EXPORT} takes one required parameter, which is the name of the

symbol you wish to export, as it was defined in your source file. An

optional second parameter (separated by white space from the first)

gives the \e{external} name of the symbol: the name by which you

wish the symbol to be known to programs using the DLL. If this name

is the same as the internal name, you may leave the second parameter

off.

Further parameters can be given to define attributes of the exported

symbol. These parameters, like the second, are separated by white

space. If further parameters are given, the external name must also

be specified, even if it is the same as the internal name. The

available attributes are:

\b \c{resident} indicates that the exported name is to be kept

resident by the system loader. This is an optimisation for

frequently used symbols imported by name.

\b \c{nodata} indicates that the exported symbol is a function which

does not make use of any initialised data.

\b \c{parm=NNN}, where \c{NNN} is an integer, sets the number of

parameter words for the case in which the symbol is a call gate

between 32-bit and 16-bit segments.

\b An attribute which is just a number indicates that the symbol

should be exported with an identifying number (ordinal), and gives

the desired number.

For example:

\c           export myfunc

\c           export myfunc TheRealMoreFormalLookingFunctionName

\c           export myfunc myfunc 1234  ; export by ordinal

\c           export myfunc myfunc resident parm=23 nodata

\S{dotdotstart} \i\c{..start}: Defining the \i{Program Entry

Point}

OMF linkers require exactly one of the object files being linked to

define the program entry point, where execution will begin when the

program is run. If the object file that defines the entry point is

assembled using NASM, you specify the entry point by declaring the

special symbol \c{..start} at the point where you wish execution to

begin.

\S{objextern} \c{obj} Extensions to the \c{EXTERN}

Directive\I{EXTERN, obj extensions to}

If you declare an external symbol with the directive

\c           extern foo

then references such as \c{mov ax,foo} will give you the offset of

\c{foo} from its preferred segment base (as specified in whichever

module \c{foo} is actually defined in). So to access the contents of

\c{foo} you will usually need to do something like

\c           mov ax,seg foo         ; get preferred segment base

\c           mov es,ax              ; move it into ES

\c           mov ax,[es:foo]        ; and use offset `foo' from it

This is a little unwieldy, particularly if you know that an external

is going to be accessible from a given segment or group, say

\c{dgroup}. So if \c{DS} already contained \c{dgroup}, you could

simply code

\c           mov ax,[foo wrt dgroup]

However, having to type this every time you want to access \c{foo}

can be a pain; so NASM allows you to declare \c{foo} in the

alternative form

\c           extern foo:wrt dgroup

This form causes NASM to pretend that the preferred segment base of

\c{foo} is in fact \c{dgroup}; so the expression \c{seg foo} will

now return \c{dgroup}, and the expression \c{foo} is equivalent to

\c{foo wrt dgroup}.

This \I{default-WRT mechanism}default-\c{WRT} mechanism can be used

to make externals appear to be relative to any group or segment in

your program. It can also be applied to common variables: see

\k{objcommon}.

\S{objcommon} \c{obj} Extensions to the \c{COMMON}

Directive\I{COMMON, obj extensions to}

The \c{obj} format allows common variables to be either near\I{near

common variables} or far\I{far common variables}; NASM allows you to

specify which your variables should be by the use of the syntax

\c           common nearvar 2:near  ; `nearvar' is a near common

\c           common farvar 10:far   ; and `farvar' is far

Far common variables may be greater in size than 64Kb, and so the

OMF specification says that they are declared as a number of

\e{elements} of a given size. So a 10-byte far common variable could

be declared as ten one-byte elements, five two-byte elements, two

five-byte elements or one ten-byte element.

Some OMF linkers require the \I{element size, in common

variables}\I{common variables, element size}element size, as well as

the variable size, to match when resolving common variables declared

in more than one module. Therefore NASM must allow you to specify

the element size on your far common variables. This is done by the

following syntax:

\c           common c_5by2 10:far 5 ; two five-byte elements

\c           common c_2by5 10:far 2 ; five two-byte elements

If no element size is specified, the default is 1. Also, the \c{FAR}

keyword is not required when an element size is specified, since

only far commons may have element sizes at all. So the above

declarations could equivalently be

\c           common c_5by2 10:5     ; two five-byte elements

\c           common c_2by5 10:2     ; five two-byte elements

In addition to these extensions, the \c{COMMON} directive in \c{obj}

also supports default-\c{WRT} specification like \c{EXTERN} does

(explained in \k{objextern}). So you can also declare things like

\c           common foo 10:wrt dgroup

\c           common bar 16:far 2:wrt data

\c           common baz 24:wrt data:6

\H{win32fmt} \i\c{win32}: Microsoft Win32 Object Files

The \c{win32} output format generates Microsoft Win32 object files,

suitable for passing to Microsoft linkers such as \i{Visual C++}.

Note that Borland Win32 compilers do not use this format, but use

\c{obj} instead (see \k{objfmt}).

\c{win32} provides a default output file-name extension of \c{.obj}.

Note that although Microsoft say that Win32 object files follow the

COFF (Common Object File Format) standard, the object files produced

by Microsoft Win32 compilers are not compatible with COFF linkers

such as DJGPP's, and vice versa. This is due to a difference of

opinion over the precise semantics of PC-relative relocations. To

produce COFF files suitable for DJGPP, use NASM's \c{coff} output

format; conversely, the \c{coff} format does not produce object

files that Win32 linkers can generate correct output from.

\S{win32sect} \c{win32} Extensions to the \c{SECTION}

Directive\I{SECTION, win32 extensions to}

Like the \c{obj} format, \c{win32} allows you to specify additional

information on the \c{SECTION} directive line, to control the type

and properties of sections you declare. Section types and properties

are generated automatically by NASM for the \i{standard section names}

\c{.text}, \c{.data} and \c{.bss}, but may still be overridden by

these qualifiers.

The available qualifiers are:

\b \c{code}, or equivalently \c{text}, defines the section to be a

code section. This marks the section as readable and executable, but

not writable, and also indicates to the linker that the type of the

section is code.

\b \c{data} and \c{bss} define the section to be a data section,

analogously to \c{code}. Data sections are marked as readable and

writable, but not executable. \c{data} declares an initialised data

section, whereas \c{bss} declares an uninitialised data section.

\b \c{info} defines the section to be an \i{informational section},

which is not included in the executable file by the linker, but may

(for example) pass information \e{to} the linker. For example,

declaring an \c{info}-type section called \i\c{.drectve} causes the

linker to interpret the contents of the section as command-line

options.

\b \c{align=}, used with a trailing number as in \c{obj}, gives the

\I{section alignment, in win32}\I{alignment, in win32

sections}alignment requirements of the section. The maximum you may

specify is 64: the Win32 object file format contains no means to

request a greater section alignment than this. If alignment is not

explicitly specified, the defaults are 16-byte alignment for code

sections, and 4-byte alignment for data (and BSS) sections.

Informational sections get a default alignment of 1 byte (no

alignment), though the value does not matter.

The defaults assumed by NASM if you do not specify the above

qualifiers are:

\c           section .text code align=16

\c           section .data data align=4

\c           section .bss bss align=4

Any other section name is treated by default like \c{.text}.

\H{cofffmt} \i\c{coff}: \i{Common Object File Format}

The \c{coff} output type produces COFF object files suitable for

linking with the \i{DJGPP} linker.

\c{coff} provides a default output file-name extension of \c{.o}.

The \c{coff} format supports the same extensions to the \c{SECTION}

directive as \c{win32} does, except that the \c{align} qualifier and

the \c{info} section type are not supported.

\H{elffmt} \i\c{elf}: \i{Linux ELF}\I{Executable and Linkable

Format}Object Files

The \c{elf} output format generates ELF32 (Executable and Linkable

Format) object files, as used by Linux. \c{elf} provides a default

output file-name extension of \c{.o}.

\S{elfsect} \c{elf} Extensions to the \c{SECTION}

Directive\I{SECTION, elf extensions to}

Like the \c{obj} format, \c{elf} allows you to specify additional

information on the \c{SECTION} directive line, to control the type

and properties of sections you declare. Section types and properties

are generated automatically by NASM for the \i{standard section

names} \i\c{.text}, \i\c{.data} and \i\c{.bss}, but may still be

overridden by these qualifiers.

The available qualifiers are:

\b \i\c{alloc} defines the section to be one which is loaded into

memory when the program is run. \i\c{noalloc} defines it to be one

which is not, such as an informational or comment section.

\b \i\c{exec} defines the section to be one which should have execute

permission when the program is run. \i\c{noexec} defines it as one

which should not.

\b \i\c{write} defines the section to be one which should be writable

when the program is run. \i\c{nowrite} defines it as one which should

not.

\b \i\c{progbits} defines the section to be one with explicit contents

stored in the object file: an ordinary code or data section, for

example, \i\c{nobits} defines the section to be one with no explicit

contents given, such as a BSS section.

\b \c{align=}, used with a trailing number as in \c{obj}, gives the

\I{section alignment, in elf}\I{alignment, in elf sections}alignment

requirements of the section.

The defaults assumed by NASM if you do not specify the above

qualifiers are:

\c           section .text progbits alloc   exec nowrite align=16

\c           section .data progbits alloc noexec   write align=4

\c           section .bss    nobits alloc noexec   write align=4

\c           section other progbits alloc noexec nowrite align=1

(Any section name other than \c{.text}, \c{.data} and \c{.bss} is

treated by default like \c{other} in the above code.)

\S{elfwrt} \i{Position-Independent Code}\I{PIC}: \c{elf} Special

Symbols and \i\c{WRT}

The ELF specification contains enough features to allow

position-independent code (PIC) to be written, which makes \i{ELF

shared libraries} very flexible. However, it also means NASM has to

be able to generate a variety of strange relocation types in ELF

object files, if it is to be an assembler which can write PIC.

Since ELF does not support segment-base references, the \c{WRT}

operator is not used for its normal purpose; therefore NASM's

\c{elf} output format makes use of \c{WRT} for a different purpose,

namely the PIC-specific \I{relocations, PIC-specific}relocation

types.

\c{elf} defines five special symbols which you can use as the

right-hand side of the \c{WRT} operator to obtain PIC relocation

types. They are \i\c{..gotpc}, \i\c{..gotoff}, \i\c{..got},

\i\c{..plt} and \i\c{..sym}. Their functions are summarised here:

\b Referring to the symbol marking the global offset table base

using \c{wrt ..gotpc} will end up giving the distance from the

beginning of the current section to the global offset table.

(\i\c{_GLOBAL_OFFSET_TABLE_} is the standard symbol name used to

refer to the \i{GOT}.) So you would then need to add \i\c{$$} to the

result to get the real address of the GOT.

\b Referring to a location in one of your own sections using \c{wrt

..gotoff} will give the distance from the beginning of the GOT to

the specified location, so that adding on the address of the GOT

would give the real address of the location you wanted.

\b Referring to an external or global symbol using \c{wrt ..got}

causes the linker to build an entry \e{in} the GOT containing the

address of the symbol, and the reference gives the distance from the

beginning of the GOT to the entry; so you can add on the address of

the GOT, load from the resulting address, and end up with the

address of the symbol.

\b Referring to a procedure name using \c{wrt ..plt} causes the

linker to build a \i{procedure linkage table} entry for the symbol,

and the reference gives the address of the \i{PLT} entry. You can

only use this in contexts which would generate a PC-relative

relocation normally (i.e. as the destination for \c{CALL} or

\c{JMP}), since ELF contains no relocation type to refer to PLT

entries absolutely.

\b Referring to a symbol name using \c{wrt ..sym} causes NASM to

write an ordinary relocation, but instead of making the relocation

relative to the start of the section and then adding on the offset

to the symbol, it will write a relocation record aimed directly at

the symbol in question. The distinction is a necessary one due to a

peculiarity of the dynamic linker.

A fuller explanation of how to use these relocation types to write

shared libraries entirely in NASM is given in \k{picdll}.

\S{elfglob} \c{elf} Extensions to the \c{GLOBAL} Directive\I{GLOBAL,

elf extensions to}\I{GLOBAL, aoutb extensions to}

ELF object files can contain more information about a global symbol

than just its address: they can contain the \I{symbol sizes,

specifying}\I{size, of symbols}size of the symbol and its \I{symbol

types, specifying}\I{type, of symbols}type as well. These are not

merely debugger conveniences, but are actually necessary when the

program being written is a \i{shared library}. NASM therefore

supports some extensions to the \c{GLOBAL} directive, allowing you

to specify these features.

You can specify whether a global variable is a function or a data

object by suffixing the name with a colon and the word

\i\c{function} or \i\c{data}. (\i\c{object} is a synonym for

\c{data}.) For example:

\c           global hashlookup:function, hashtable:data

exports the global symbol \c{hashlookup} as a function and

\c{hashtable} as a data object.

You can also specify the size of the data associated with the

symbol, as a numeric expression (which may involve labels, and even

forward references) after the type specifier. Like this:

\c           global hashtable:data (hashtable.end - hashtable)

\c hashtable:

\c           db this,that,theother  ; some data here

\c .end:

This makes NASM automatically calculate the length of the table and

place that information into the ELF symbol table.

Declaring the type and size of global symbols is necessary when

writing shared library code. For more information, see

\k{picglobal}.

\S{elfcomm} \c{elf} Extensions to the \c{COMMON} Directive\I{COMMON,

elf extensions to}

ELF also allows you to specify alignment requirements \I{common

variables, alignment in elf}\I{alignment, of elf common variables}on

common variables. This is done by putting a number (which must be a

power of two) after the name and size of the common variable,

separated (as usual) by a colon. For example, an array of

doublewords would benefit from 4-byte alignment:

\c           common dwordarray 128:4

This declares the total size of the array to be 128 bytes, and

requires that it be aligned on a 4-byte boundary.

\H{aoutfmt} \i\c{aout}: Linux \I{a.out, Linux version}\c{a.out} Object Files

The \c{aout} format generates \c{a.out} object files, in the form

used by early Linux systems. (These differ from other \c{a.out}

object files in that the magic number in the first four bytes of the

file is different. Also, some implementations of \c{a.out}, for

example NetBSD's, support position-independent code, which Linux's

implementation doesn't.)

\c{a.out} provides a default output file-name extension of \c{.o}.

\c{a.out} is a very simple object format. It supports no special

directives, no special symbols, no use of \c{SEG} or \c{WRT}, and no

extensions to any standard directives. It supports only the three

\i{standard section names} \i\c{.text}, \i\c{.data} and \i\c{.bss}.

\H{aoutfmt} \i\c{aoutb}: \i{NetBSD}/\i{FreeBSD}/\i{OpenBSD}

\I{a.out, BSD version}\c{a.out} Object Files

The \c{aoutb} format generates \c{a.out} object files, in the form

used by the various free BSD Unix clones, NetBSD, FreeBSD and

OpenBSD. For simple object files, this object format is exactly the

same as \c{aout} except for the magic number in the first four bytes

of the file. However, the \c{aoutb} format supports

\I{PIC}\i{position-independent code} in the same way as the \c{elf}

format, so you can use it to write BSD \i{shared libraries}.

\c{aoutb} provides a default output file-name extension of \c{.o}.

\c{aoutb} supports no special directives, no special symbols, and

only the three \i{standard section names} \i\c{.text}, \i\c{.data}

and \i\c{.bss}. However, it also supports the same use of \i\c{WRT} as

\c{elf} does, to provide position-independent code relocation types.

See \k{elfwrt} for full documentation of this feature.

\c{aoutb} also supports the same extensions to the \c{GLOBAL}

directive as \c{elf} does: see \k{elfglob} for documentation of

this.

\H{as86fmt} \c{as86}: Linux \i\c{as86} Object Files

The Linux 16-bit assembler \c{as86} has its own non-standard object

file format. Although its companion linker \i\c{ld86} produces

something close to ordinary \c{a.out} binaries as output, the object

file format used to communicate between \c{as86} and \c{ld86} is not

itself \c{a.out}.

NASM supports this format, just in case it is useful, as \c{as86}.

\c{as86} provides a default output file-name extension of \c{.o}.

\c{as86} is a very simple object format (from the NASM user's point

of view). It supports no special directives, no special symbols, no

use of \c{SEG} or \c{WRT}, and no extensions to any standard

directives. It supports only the three \i{standard section names}

\i\c{.text}, \i\c{.data} and \i\c{.bss}.

\H{rdffmt} \I{RDOFF}\i\c{rdf}: \i{Relocatable Dynamic Object File

Format}

The \c{rdf} output format produces RDOFF object files. RDOFF

(Relocatable Dynamic Object File Format) is a home-grown object-file

format, designed alongside NASM itself and reflecting in its file

format the internal structure of the assembler.

RDOFF is not used by any well-known operating systems. Those writing

their own systems, however, may well wish to use RDOFF as their

object format, on the grounds that it is designed primarily for

simplicity and contains very little file-header bureaucracy.

The Unix NASM archive, and the DOS archive which includes sources,

both contain an \I{rdoff subdirectory}\c{rdoff} subdirectory holding

a set of RDOFF utilities: an RDF linker, an RDF static-library

manager, an RDF file dump utility, and a program which will load and

execute an RDF executable under Linux.

\c{rdf} supports only the \i{standard section names} \i\c{.text},

\i\c{.data} and \i\c{.bss}.

\S{rdflib} Requiring a Library: The \i\c{LIBRARY} Directive

RDOFF contains a mechanism for an object file to demand a given

library to be linked to the module, either at load time or run time.

This is done by the \c{LIBRARY} directive, which takes one argument

which is the name of the module:

\c           library mylib.rdl

\H{dbgfmt} \i\c{dbg}: Debugging Format

The \c{dbg} output format is not built into NASM in the default

configuration. If you are building your own NASM executable from the

sources, you can define \i\c{OF_DBG} in \c{outform.h} or on the

compiler command line, and obtain the \c{dbg} output format.

The \c{dbg} format does not output an object file as such; instead,

it outputs a text file which contains a complete list of all the

transactions between the main body of NASM and the output-format

back end module. It is primarily intended to aid people who want to

write their own output drivers, so that they can get a clearer idea

of the various requests the main program makes of the output driver,

and in what order they happen.

For simple files, one can easily use the \c{dbg} format like this:

\c nasm -f dbg filename.asm

which will generate a diagnostic file called \c{filename.dbg}.

However, this will not work well on files which were designed for a

different object format, because each object format defines its own

macros (usually user-level forms of directives), and those macros

will not be defined in the \c{dbg} format. Therefore it can be

useful to run NASM twice, in order to do the preprocessing with the

native object format selected:

\c nasm -e -f rdf -o rdfprog.i rdfprog.asm

\c nasm -a -f dbg rdfprog.i

This preprocesses \c{rdfprog.asm} into \c{rdfprog.i}, keeping the

\c{rdf} object format selected in order to make sure RDF special

directives are converted into primitive form correctly. Then the

preprocessed source is fed through the \c{dbg} format to generate

the final diagnostic output.

This workaround will still typically not work for programs intended

for \c{obj} format, because the \c{obj} \c{SEGMENT} and \c{GROUP}

directives have side effects of defining the segment and group names

as symbols; \c{dbg} will not do this, so the program will not

assemble. You will have to work around that by defining the symbols

yourself (using \c{EXTERN}, for example) if you really need to get a

\c{dbg} trace of an \c{obj}-specific source file.

\c{dbg} accepts any section name and any directives at all, and logs

them all to its output file.

\C{16bit} Writing 16-bit Code (DOS, Windows 3/3.1)

This chapter attempts to cover some of the common issues encountered

when writing 16-bit code to run under MS-DOS or Windows 3.x. It

covers how to link programs to produce \c{.EXE} or \c{.COM} files,

how to write \c{.SYS} device drivers, and how to interface assembly

language code with 16-bit C compilers and with Borland Pascal.

\H{exefiles} Producing \i\c{.EXE} Files

Any large program written under DOS needs to be built as a \c{.EXE}

file: only \c{.EXE} files have the necessary internal structure

required to span more than one 64K segment. \i{Windows} programs,

also, have to be built as \c{.EXE} files, since Windows does not

support the \c{.COM} format.

In general, you generate \c{.EXE} files by using the \c{obj} output

format to produce one or more \i\c{.OBJ} files, and then linking

them together using a linker. However, NASM also supports the direct

generation of simple DOS \c{.EXE} files using the \c{bin} output

format (by using \c{DB} and \c{DW} to construct the \c{.EXE} file

header), and a macro package is supplied to do this. Thanks to

Yann Guidon for contributing the code for this.

NASM may also support \c{.EXE} natively as another output format in

future releases.

\S{objexe} Using the \c{obj} Format To Generate \c{.EXE} Files

This section describes the usual method of generating \c{.EXE} files

by linking \c{.OBJ} files together.

Most 16-bit programming language packages come with a suitable

linker; if you have none of these, there is a free linker called

\i{VAL}\I{linker, free}, available in \c{LZH} archive format from

\W{ftp://x2ftp.oulu.fi/pub/msdos/programming/lang/}\i\c{x2ftp.oulu.fi}.

An LZH archiver can be found at

\W{ftp://ftp.simtel.net/pub/simtelnet/msdos/arcers}\i\c{ftp.simtel.net}.

There is another `free' linker (though this one doesn't come with

sources) called \i{FREELINK}, available from

\W{http://www.pcorner.com/tpc/old/3-101.html}\i\c{www.pcorner.com}.

A third, \i\c{djlink}, written by DJ Delorie, is available at

\W{http://www.delorie.com/djgpp/16bit/djlink/}\i\c{www.delorie.com}.

When linking several \c{.OBJ} files into a \c{.EXE} file, you should

ensure that exactly one of them has a start point defined (using the

\I{program entry point}\i\c{..start} special symbol defined by the

\c{obj} format: see \k{dotdotstart}). If no module defines a start

point, the linker will not know what value to give the entry-point

field in the output file header; if more than one defines a start

point, the linker will not know \e{which} value to use.

An example of a NASM source file which can be assembled to a

\c{.OBJ} file and linked on its own to a \c{.EXE} is given here. It

demonstrates the basic principles of defining a stack, initialising

the segment registers, and declaring a start point. This file is

also provided in the \I{test subdirectory}\c{test} subdirectory of

the NASM archives, under the name \c{objexe.asm}.

\c           segment code

\c

\c ..start:  mov ax,data

\c           mov ds,ax

\c           mov ax,stack

\c           mov ss,ax

\c           mov sp,stacktop

This initial piece of code sets up \c{DS} to point to the data

segment, and initialises \c{SS} and \c{SP} to point to the top of

the provided stack. Notice that interrupts are implicitly disabled

for one instruction after a move into \c{SS}, precisely for this

situation, so that there's no chance of an interrupt occurring

between the loads of \c{SS} and \c{SP} and not having a stack to

execute on.

Note also that the special symbol \c{..start} is defined at the

beginning of this code, which means that will be the entry point

into the resulting executable file.

\c           mov dx,hello

\c           mov ah,9

\c           int 0x21

The above is the main program: load \c{DS:DX} with a pointer to the

greeting message (\c{hello} is implicitly relative to the segment

\c{data}, which was loaded into \c{DS} in the setup code, so the

full pointer is valid), and call the DOS print-string function.

\c           mov ax,0x4c00

\c           int 0x21

This terminates the program using another DOS system call.

\c           segment data

\c hello:    db 'hello, world', 13, 10, '$'

The data segment contains the string we want to display.

\c           segment stack stack

\c           resb 64

\c stacktop:

The above code declares a stack segment containing 64 bytes of

uninitialised stack space, and points \c{stacktop} at the top of it.

The directive \c{segment stack stack} defines a segment \e{called}

\c{stack}, and also of \e{type} \c{STACK}. The latter is not

necessary to the correct running of the program, but linkers are

likely to issue warnings or errors if your program has no segment of

type \c{STACK}.

The above file, when assembled into a \c{.OBJ} file, will link on

its own to a valid \c{.EXE} file, which when run will print `hello,

world' and then exit.

\S{binexe} Using the \c{bin} Format To Generate \c{.EXE} Files

The \c{.EXE} file format is simple enough that it's possible to

build a \c{.EXE} file by writing a pure-binary program and sticking

a 32-byte header on the front. This header is simple enough that it

can be generated using \c{DB} and \c{DW} commands by NASM itself, so

that you can use the \c{bin} output format to directly generate

\c{.EXE} files.

Included in the NASM archives, in the \I{misc subdirectory}\c{misc}

subdirectory, is a file \i\c{exebin.mac} of macros. It defines three

macros: \i\c{EXE_begin}, \i\c{EXE_stack} and \i\c{EXE_end}.

To produce a \c{.EXE} file using this method, you should start by

using \c{%include} to load the \c{exebin.mac} macro package into

your source file. You should then issue the \c{EXE_begin} macro call

(which takes no arguments) to generate the file header data. Then

write code as normal for the \c{bin} format - you can use all three

standard sections \c{.text}, \c{.data} and \c{.bss}. At the end of

the file you should call the \c{EXE_end} macro (again, no arguments),

which defines some symbols to mark section sizes, and these symbols

are referred to in the header code generated by \c{EXE_begin}.

In this model, the code you end up writing starts at \c{0x100}, just

like a \c{.COM} file - in fact, if you strip off the 32-byte header

from the resulting \c{.EXE} file, you will have a valid \c{.COM}

program. All the segment bases are the same, so you are limited to a

64K program, again just like a \c{.COM} file. Note that an \c{ORG}

directive is issued by the \c{EXE_begin} macro, so you should not

explicitly issue one of your own.

You can't directly refer to your segment base value, unfortunately,

since this would require a relocation in the header, and things

would get a lot more complicated. So you should get your segment

base by copying it out of \c{CS} instead.

On entry to your \c{.EXE} file, \c{SS:SP} are already set up to

point to the top of a 2Kb stack. You can adjust the default stack

size of 2Kb by calling the \c{EXE_stack} macro. For example, to

change the stack size of your program to 64 bytes, you would call

\c{EXE_stack 64}.

A sample program which generates a \c{.EXE} file in this way is

given in the \c{test} subdirectory of the NASM archive, as

\c{binexe.asm}.

\H{comfiles} Producing \i\c{.COM} Files

While large DOS programs must be written as \c{.EXE} files, small

ones are often better written as \c{.COM} files. \c{.COM} files are

pure binary, and therefore most easily produced using the \c{bin}

output format.

\S{combinfmt} Using the \c{bin} Format To Generate \c{.COM} Files

\c{.COM} files expect to be loaded at offset \c{100h} into their

segment (though the segment may change). Execution then begins at

\I\c{ORG}\c{100h}, i.e. right at the start of the program. So to

write a \c{.COM} program, you would create a source file looking

like

\c           org 100h

\c           section .text

\c start:    ; put your code here

\c           section .data

\c           ; put data items here

\c           section .bss

\c           ; put uninitialised data here

The \c{bin} format puts the \c{.text} section first in the file, so

you can declare data or BSS items before beginning to write code if

you want to and the code will still end up at the front of the file

where it belongs.

The BSS (uninitialised data) section does not take up space in the

\c{.COM} file itself: instead, addresses of BSS items are resolved

to point at space beyond the end of the file, on the grounds that

this will be free memory when the program is run. Therefore you

should not rely on your BSS being initialised to all zeros when you

run.

To assemble the above program, you should use a command line like

\c nasm myprog.asm -fbin -o myprog.com

The \c{bin} format would produce a file called \c{myprog} if no

explicit output file name were specified, so you have to override it

and give the desired file name.

\S{comobjfmt} Using the \c{obj} Format To Generate \c{.COM} Files

If you are writing a \c{.COM} program as more than one module, you

may wish to assemble several \c{.OBJ} files and link them together

into a \c{.COM} program. You can do this, provided you have a linker

capable of outputting \c{.COM} files directly (\i{TLINK} does this),

or alternatively a converter program such as \i\c{EXE2BIN} to

transform the \c{.EXE} file output from the linker into a \c{.COM}

file.

If you do this, you need to take care of several things:

\b The first object file containing code should start its code

segment with a line like \c{RESB 100h}. This is to ensure that the

code begins at offset \c{100h} relative to the beginning of the code

segment, so that the linker or converter program does not have to

adjust address references within the file when generating the

\c{.COM} file. Other assemblers use an \i\c{ORG} directive for this

purpose, but \c{ORG} in NASM is a format-specific directive to the

\c{bin} output format, and does not mean the same thing as it does

in MASM-compatible assemblers.

\b You don't need to define a stack segment.

\b All your segments should be in the same group, so that every time

your code or data references a symbol offset, all offsets are

relative to the same segment base. This is because, when a \c{.COM}

file is loaded, all the segment registers contain the same value.

\H{sysfiles} Producing \i\c{.SYS} Files

\i{MS-DOS device drivers} - \c{.SYS} files - are pure binary files,

similar to \c{.COM} files, except that they start at origin zero

rather than \c{100h}. Therefore, if you are writing a device driver

using the \c{bin} format, you do not need the \c{ORG} directive,

since the default origin for \c{bin} is zero. Similarly, if you are

using \c{obj}, you do not need the \c{RESB 100h} at the start of

your code segment.

\c{.SYS} files start with a header structure, containing pointers to

the various routines inside the driver which do the work. This

structure should be defined at the start of the code segment, even

though it is not actually code.

For more information on the format of \c{.SYS} files, and the data

which has to go in the header structure, a list of books is given in

the Frequently Asked Questions list for the newsgroup

\W{news:comp.os.msdos.programmer}\i\c{comp.os.msdos.programmer}.

\H{16c} Interfacing to 16-bit C Programs

This section covers the basics of writing assembly routines that

call, or are called from, C programs. To do this, you would

typically write an assembly module as a \c{.OBJ} file, and link it

with your C modules to produce a \i{mixed-language program}.

\S{16cunder} External Symbol Names

\I{C symbol names}\I{underscore, in C symbols}C compilers have the

convention that the names of all global symbols (functions or data)

they define are formed by prefixing an underscore to the name as it

appears in the C program. So, for example, the function a C

programmer thinks of as \c{printf} appears to an assembly language

programmer as \c{_printf}. This means that in your assembly

programs, you can define symbols without a leading underscore, and

not have to worry about name clashes with C symbols.

If you find the underscores inconvenient, you can define macros to

replace the \c{GLOBAL} and \c{EXTERN} directives as follows:

\c %macro cglobal 1

\c           global _%1

\c %define %1 _%1

\c %endmacro

\c %macro cextern 1

\c           extern _%1

\c %define %1 _%1

\c %endmacro

(These forms of the macros only take one argument at a time; a

\c{%rep} construct could solve this.)

If you then declare an external like this:

\c           cextern printf

then the macro will expand it as

\c           extern _printf

\c %define printf _printf

Thereafter, you can reference \c{printf} as if it was a symbol, and

the preprocessor will put the leading underscore on where necessary.

The \c{cglobal} macro works similarly. You must use \c{cglobal}

before defining the symbol in question, but you would have had to do

that anyway if you used \c{GLOBAL}.

\S{16cmodels} \i{Memory Models}

NASM contains no mechanism to support the various C memory models

directly; you have to keep track yourself of which one you are

writing for. This means you have to keep track of the following

things:

\b In models using a single code segment (tiny, small and compact),

functions are near. This means that function pointers, when stored

in data segments or pushed on the stack as function arguments, are

16 bits long and contain only an offset field (the \c{CS} register

never changes its value, and always gives the segment part of the

full function address), and that functions are called using ordinary

near \c{CALL} instructions and return using \c{RETN} (which, in

NASM, is synonymous with \c{RET} anyway). This means both that you

should write your own routines to return with \c{RETN}, and that you

should call external C routines with near \c{CALL} instructions.

\b In models using more than one code segment (medium, large and

huge), functions are far. This means that function pointers are 32

bits long (consisting of a 16-bit offset followed by a 16-bit

segment), and that functions are called using \c{CALL FAR} (or

\c{CALL seg:offset}) and return using \c{RETF}. Again, you should

therefore write your own routines to return with \c{RETF} and use

\c{CALL FAR} to call external routines.

\b In models using a single data segment (tiny, small and medium),

data pointers are 16 bits long, containing only an offset field (the

\c{DS} register doesn't change its value, and always gives the

segment part of the full data item address).

\b In models using more than one data segment (compact, large and

huge), data pointers are 32 bits long, consisting of a 16-bit offset

followed by a 16-bit segment. You should still be careful not to

modify \c{DS} in your routines without restoring it afterwards, but

\c{ES} is free for you to use to access the contents of 32-bit data

pointers you are passed.

\b The huge memory model allows single data items to exceed 64K in

size. In all other memory models, you can access the whole of a data

item just by doing arithmetic on the offset field of the pointer you

are given, whether a segment field is present or not; in huge model,

you have to be more careful of your pointer arithmetic.

\b In most memory models, there is a \e{default} data segment, whose

segment address is kept in \c{DS} throughout the program. This data

segment is typically the same segment as the stack, kept in \c{SS},

so that functions' local variables (which are stored on the stack)

and global data items can both be accessed easily without changing

\c{DS}. Particularly large data items are typically stored in other

segments. However, some memory models (though not the standard

ones, usually) allow the assumption that \c{SS} and \c{DS} hold the

same value to be removed. Be careful about functions' local

variables in this latter case.

In models with a single code segment, the segment is called

\i\c{_TEXT}, so your code segment must also go by this name in order

to be linked into the same place as the main code segment. In models

with a single data segment, or with a default data segment, it is

called \i\c{_DATA}.

\S{16cfunc} Function Definitions and Function Calls

\I{functions, C calling convention}The \i{C calling convention} in

16-bit programs is as follows. In the following description, the

words \e{caller} and \e{callee} are used to denote the function

doing the calling and the function which gets called.

\b The caller pushes the function's parameters on the stack, one

after another, in reverse order (right to left, so that the first

argument specified to the function is pushed last).

\b The caller then executes a \c{CALL} instruction to pass control

to the callee. This \c{CALL} is either near or far depending on the

memory model.

\b The callee receives control, and typically (although this is not

actually necessary, in functions which do not need to access their

parameters) starts by saving the value of \c{SP} in \c{BP} so as to

be able to use \c{BP} as a base pointer to find its parameters on

the stack. However, the caller was probably doing this too, so part

of the calling convention states that \c{BP} must be preserved by

any C function. Hence the callee, if it is going to set up \c{BP} as

a \i\e{frame pointer}, must push the previous value first.

\b The callee may then access its parameters relative to \c{BP}.

The word at \c{[BP]} holds the previous value of \c{BP} as it was

pushed; the next word, at \c{[BP+2]}, holds the offset part of the

return address, pushed implicitly by \c{CALL}. In a small-model

(near) function, the parameters start after that, at \c{[BP+4]}; in

a large-model (far) function, the segment part of the return address

lives at \c{[BP+4]}, and the parameters begin at \c{[BP+6]}. The

leftmost parameter of the function, since it was pushed last, is

accessible at this offset from \c{BP}; the others follow, at

successively greater offsets. Thus, in a function such as \c{printf}

which takes a variable number of parameters, the pushing of the

parameters in reverse order means that the function knows where to

find its first parameter, which tells it the number and type of the

remaining ones.

\b The callee may also wish to decrease \c{SP} further, so as to

allocate space on the stack for local variables, which will then be

accessible at negative offsets from \c{BP}.

\b The callee, if it wishes to return a value to the caller, should

leave the value in \c{AL}, \c{AX} or \c{DX:AX} depending on the size

of the value. Floating-point results are sometimes (depending on the

compiler) returned in \c{ST0}.

\b Once the callee has finished processing, it restores \c{SP} from

\c{BP} if it had allocated local stack space, then pops the previous

value of \c{BP}, and returns via \c{RETN} or \c{RETF} depending on

memory model.

\b When the caller regains control from the callee, the function

parameters are still on the stack, so it typically adds an immediate

constant to \c{SP} to remove them (instead of executing a number of

slow \c{POP} instructions). Thus, if a function is accidentally

called with the wrong number of parameters due to a prototype

mismatch, the stack will still be returned to a sensible state since

the caller, which \e{knows} how many parameters it pushed, does the

removing.

It is instructive to compare this calling convention with that for

Pascal programs (described in \k{16bpfunc}). Pascal has a simpler

convention, since no functions have variable numbers of parameters.

Therefore the callee knows how many parameters it should have been

passed, and is able to deallocate them from the stack itself by

passing an immediate argument to the \c{RET} or \c{RETF}

instruction, so the caller does not have to do it. Also, the

parameters are pushed in left-to-right order, not right-to-left,

which means that a compiler can give better guarantees about

sequence points without performance suffering.

Thus, you would define a function in C style in the following way.

The following example is for small model:

\c           global _myfunc

\c _myfunc:  push bp

\c           mov bp,sp

\c           sub sp,0x40            ; 64 bytes of local stack space

\c           mov bx,[bp+4]          ; first parameter to function

\c           ; some more code

\c           mov sp,bp              ; undo "sub sp,0x40" above

\c           pop bp

\c           ret

For a large-model function, you would replace \c{RET} by \c{RETF},

and look for the first parameter at \c{[BP+6]} instead of

\c{[BP+4]}. Of course, if one of the parameters is a pointer, then

the offsets of \e{subsequent} parameters will change depending on

the memory model as well: far pointers take up four bytes on the

stack when passed as a parameter, whereas near pointers take up two.

At the other end of the process, to call a C function from your

assembly code, you would do something like this:

\c           extern _printf

\c           ; and then, further down...

\c           push word [myint]      ; one of my integer variables

\c           push word mystring     ; pointer into my data segment

\c           call _printf

\c           add sp,byte 4          ; `byte' saves space

\c           ; then those data items...

\c           segment _DATA

\c myint     dw 1234

\c mystring  db 'This number -> %d <- should be 1234',10,0

This piece of code is the small-model assembly equivalent of the C

code

\c     int myint = 1234;

\c     printf("This number -> %d <- should be 1234\n", myint);

In large model, the function-call code might look more like this. In

this example, it is assumed that \c{DS} already holds the segment

base of the segment \c{_DATA}. If not, you would have to initialise

it first.

\c           push word [myint]

\c           push word seg mystring ; Now push the segment, and...

\c           push word mystring     ; ... offset of "mystring"

\c           call far _printf

\c           add sp,byte 6

The integer value still takes up one word on the stack, since large

model does not affect the size of the \c{int} data type. The first

argument (pushed last) to \c{printf}, however, is a data pointer,

and therefore has to contain a segment and offset part. The segment

should be stored second in memory, and therefore must be pushed

first. (Of course, \c{PUSH DS} would have been a shorter instruction

than \c{PUSH WORD SEG mystring}, if \c{DS} was set up as the above

example assumed.) Then the actual call becomes a far call, since

functions expect far calls in large model; and \c{SP} has to be

increased by 6 rather than 4 afterwards to make up for the extra

word of parameters.

\S{16cdata} Accessing Data Items

To get at the contents of C variables, or to declare variables which

C can access, you need only declare the names as \c{GLOBAL} or

\c{EXTERN}. (Again, the names require leading underscores, as stated

in \k{16cunder}.) Thus, a C variable declared as \c{int i} can be

accessed from assembler as

\c           extern _i

\c           mov ax,[_i]

And to declare your own integer variable which C programs can access

as \c{extern int j}, you do this (making sure you are assembling in

the \c{_DATA} segment, if necessary):

\c           global _j

\c _j        dw 0

To access a C array, you need to know the size of the components of

the array. For example, \c{int} variables are two bytes long, so if

a C program declares an array as \c{int a[10]}, you can access

\c{a[3]} by coding \c{mov ax,[_a+6]}. (The byte offset 6 is obtained

by multiplying the desired array index, 3, by the size of the array

element, 2.) The sizes of the C base types in 16-bit compilers are:

1 for \c{char}, 2 for \c{short} and \c{int}, 4 for \c{long} and

\c{float}, and 8 for \c{double}.

To access a C \i{data structure}, you need to know the offset from

the base of the structure to the field you are interested in. You

can either do this by converting the C structure definition into a

NASM structure definition (using \i\c{STRUC}), or by calculating the

one offset and using just that.

To do either of these, you should read your C compiler's manual to

find out how it organises data structures. NASM gives no special

alignment to structure members in its own \c{STRUC} macro, so you

have to specify alignment yourself if the C compiler generates it.

Typically, you might find that a structure like

\c struct {

\c     char c;

\c     int i;

\c } foo;

might be four bytes long rather than three, since the \c{int} field

would be aligned to a two-byte boundary. However, this sort of

feature tends to be a configurable option in the C compiler, either

using command-line options or \c{#pragma} lines, so you have to find

out how your own compiler does it.

\S{16cmacro} \i\c{c16.mac}: Helper Macros for the 16-bit C Interface

Included in the NASM archives, in the \I{misc subdirectory}\c{misc}

directory, is a file \c{c16.mac} of macros. It defines three macros:

\i\c{proc}, \i\c{arg} and \i\c{endproc}. These are intended to be

used for C-style procedure definitions, and they automate a lot of

the work involved in keeping track of the calling convention.

An example of an assembly function using the macro set is given

here:

\c           proc _nearproc

\c %$i       arg

\c %$j       arg

\c           mov ax,[bp + %$i]

\c           mov bx,[bp + %$j]

\c           add ax,[bx]

\c           endproc

This defines \c{_nearproc} to be a procedure taking two arguments,

the first (\c{i}) an integer and the second (\c{j}) a pointer to an

integer. It returns \c{i + *j}.

Note that the \c{arg} macro has an \c{EQU} as the first line of its

expansion, and since the label before the macro call gets prepended

to the first line of the expanded macro, the \c{EQU} works, defining

\c{%$i} to be an offset from \c{BP}. A context-local variable is

used, local to the context pushed by the \c{proc} macro and popped

by the \c{endproc} macro, so that the same argument name can be used

in later procedures. Of course, you don't \e{have} to do that.

The macro set produces code for near functions (tiny, small and

compact-model code) by default. You can have it generate far

functions (medium, large and huge-model code) by means of coding

\I\c{FARCODE}\c{%define FARCODE}. This changes the kind of return

instruction generated by \c{endproc}, and also changes the starting

point for the argument offsets. The macro set contains no intrinsic

dependency on whether data pointers are far or not.

\c{arg} can take an optional parameter, giving the size of the

argument. If no size is given, 2 is assumed, since it is likely that

many function parameters will be of type \c{int}.

The large-model equivalent of the above function would look like this:

\c %define FARCODE

\c           proc _farproc

\c %$i       arg

\c %$j       arg 4

\c           mov ax,[bp + %$i]

\c           mov bx,[bp + %$j]

\c           mov es,[bp + %$j + 2]

\c           add ax,[bx]

\c           endproc

This makes use of the argument to the \c{arg} macro to define a

parameter of size 4, because \c{j} is now a far pointer. When we

load from \c{j}, we must load a segment and an offset.

\H{16bp} Interfacing to \i{Borland Pascal} Programs

Interfacing to Borland Pascal programs is similar in concept to

interfacing to 16-bit C programs. The differences are:

\b The leading underscore required for interfacing to C programs is

not required for Pascal.

\b The memory model is always large: functions are far, data

pointers are far, and no data item can be more than 64K long.

(Actually, some functions are near, but only those functions that

are local to a Pascal unit and never called from outside it. All

assembly functions that Pascal calls, and all Pascal functions that

assembly routines are able to call, are far.) However, all static

data declared in a Pascal program goes into the default data

segment, which is the one whose segment address will be in \c{DS}

when control is passed to your assembly code. The only things that

do not live in the default data segment are local variables (they

live in the stack segment) and dynamically allocated variables. All

data \e{pointers}, however, are far.

\b The function calling convention is different - described below.

\b Some data types, such as strings, are stored differently.

\b There are restrictions on the segment names you are allowed to

use - Borland Pascal will ignore code or data declared in a segment

it doesn't like the name of. The restrictions are described below.

\S{16bpfunc} The Pascal Calling Convention

\I{functions, Pascal calling convention}\I{Pascal calling

convention}The 16-bit Pascal calling convention is as follows. In

the following description, the words \e{caller} and \e{callee} are

used to denote the function doing the calling and the function which

gets called.

\b The caller pushes the function's parameters on the stack, one

after another, in normal order (left to right, so that the first

argument specified to the function is pushed first).

\b The caller then executes a far \c{CALL} instruction to pass

control to the callee.

\b The callee receives control, and typically (although this is not

actually necessary, in functions which do not need to access their

parameters) starts by saving the value of \c{SP} in \c{BP} so as to

be able to use \c{BP} as a base pointer to find its parameters on

the stack. However, the caller was probably doing this too, so part

of the calling convention states that \c{BP} must be preserved by

any function. Hence the callee, if it is going to set up \c{BP} as a

\i{frame pointer}, must push the previous value first.

\b The callee may then access its parameters relative to \c{BP}.

The word at \c{[BP]} holds the previous value of \c{BP} as it was

pushed. The next word, at \c{[BP+2]}, holds the offset part of the

return address, and the next one at \c{[BP+4]} the segment part. The

parameters begin at \c{[BP+6]}. The rightmost parameter of the

function, since it was pushed last, is accessible at this offset

from \c{BP}; the others follow, at successively greater offsets.

\b The callee may also wish to decrease \c{SP} further, so as to

allocate space on the stack for local variables, which will then be

accessible at negative offsets from \c{BP}.

\b The callee, if it wishes to return a value to the caller, should

leave the value in \c{AL}, \c{AX} or \c{DX:AX} depending on the size

of the value. Floating-point results are returned in \c{ST0}.

Results of type \c{Real} (Borland's own custom floating-point data

type, not handled directly by the FPU) are returned in \c{DX:BX:AX}.

To return a result of type \c{String}, the caller pushes a pointer

to a temporary string before pushing the parameters, and the callee

places the returned string value at that location. The pointer is

not a parameter, and should not be removed from the stack by the

\c{RETF} instruction.

\b Once the callee has finished processing, it restores \c{SP} from

\c{BP} if it had allocated local stack space, then pops the previous

value of \c{BP}, and returns via \c{RETF}. It uses the form of

\c{RETF} with an immediate parameter, giving the number of bytes

taken up by the parameters on the stack. This causes the parameters

to be removed from the stack as a side effect of the return

instruction.

\b When the caller regains control from the callee, the function

parameters have already been removed from the stack, so it needs to

do nothing further.

Thus, you would define a function in Pascal style, taking two

\c{Integer}-type parameters, in the following way:

\c           global myfunc

\c myfunc:   push bp

\c           mov bp,sp

\c           sub sp,0x40            ; 64 bytes of local stack space

\c           mov bx,[bp+8]          ; first parameter to function

\c           mov bx,[bp+6]          ; second parameter to function

\c           ; some more code

\c           mov sp,bp              ; undo "sub sp,0x40" above

\c           pop bp

\c           retf 4                 ; total size of params is 4

At the other end of the process, to call a Pascal function from your

assembly code, you would do something like this:

\c           extern SomeFunc

\c           ; and then, further down...

\c           push word seg mystring ; Now push the segment, and...

\c           push word mystring     ; ... offset of "mystring"

\c           push word [myint]      ; one of my variables

\c           call far SomeFunc

This is equivalent to the Pascal code

\c procedure SomeFunc(String: PChar; Int: Integer);

\c     SomeFunc(@mystring, myint);

\S{16bpseg} Borland Pascal \I{segment names, Borland Pascal}Segment

Name Restrictions

Since Borland Pascal's internal unit file format is completely

different from \c{OBJ}, it only makes a very sketchy job of actually

reading and understanding the various information contained in a

real \c{OBJ} file when it links that in. Therefore an object file

intended to be linked to a Pascal program must obey a number of

restrictions:

\b Procedures and functions must be in a segment whose name is

either \c{CODE}, \c{CSEG}, or something ending in \c{_TEXT}.

\b Initialised data must be in a segment whose name is either

\c{CONST} or something ending in \c{_DATA}.

\b Uninitialised data must be in a segment whose name is either

\c{DATA}, \c{DSEG}, or something ending in \c{_BSS}.

\b Any other segments in the object file are completely ignored.

\c{GROUP} directives and segment attributes are also ignored.

\S{16bpmacro} Using \i\c{c16.mac} With Pascal Programs

The \c{c16.mac} macro package, described in \k{16cmacro}, can also

be used to simplify writing functions to be called from Pascal

programs, if you code \I\c{PASCAL}\c{%define PASCAL}. This

definition ensures that functions are far (it implies

\i\c{FARCODE}), and also causes procedure return instructions to be

generated with an operand.

Defining \c{PASCAL} does not change the code which calculates the

argument offsets; you must declare your function's arguments in

reverse order. For example:

\c %define PASCAL

\c           proc _pascalproc

\c %$j       arg 4

\c %$i       arg

\c           mov ax,[bp + %$i]

\c           mov bx,[bp + %$j]

\c           mov es,[bp + %$j + 2]

\c           add ax,[bx]

\c           endproc

This defines the same routine, conceptually, as the example in

\k{16cmacro}: it defines a function taking two arguments, an integer

and a pointer to an integer, which returns the sum of the integer

and the contents of the pointer. The only difference between this

code and the large-model C version is that \c{PASCAL} is defined

instead of \c{FARCODE}, and that the arguments are declared in

reverse order.

\C{32bit} Writing 32-bit Code (Unix, Win32, DJGPP)

This chapter attempts to cover some of the common issues involved

when writing 32-bit code, to run under \i{Win32} or Unix, or to be

linked with C code generated by a Unix-style C compiler such as

\i{DJGPP}. It covers how to write assembly code to interface with

32-bit C routines, and how to write position-independent code for

shared libraries.

Almost all 32-bit code, and in particular all code running under

Win32, DJGPP or any of the PC Unix variants, runs in \I{flat memory

model}\e{flat} memory model. This means that the segment registers

and paging have already been set up to give you the same 32-bit 4Gb

address space no matter what segment you work relative to, and that

you should ignore all segment registers completely. When writing

flat-model application code, you never need to use a segment

override or modify any segment register, and the code-section

addresses you pass to \c{CALL} and \c{JMP} live in the same address

space as the data-section addresses you access your variables by and

the stack-section addresses you access local variables and procedure

parameters by. Every address is 32 bits long and contains only an

offset part.

\H{32c} Interfacing to 32-bit C Programs

A lot of the discussion in \k{16c}, about interfacing to 16-bit C

programs, still applies when working in 32 bits. The absence of

memory models or segmentation worries simplifies things a lot.

\S{32cunder} External Symbol Names

Most 32-bit C compilers share the convention used by 16-bit

compilers, that the names of all global symbols (functions or data)

they define are formed by prefixing an underscore to the name as it

appears in the C program. However, not all of them do: the ELF

specification states that C symbols do \e{not} have a leading

underscore on their assembly-language names.

The older Linux \c{a.out} C compiler, all Win32 compilers, DJGPP,

and NetBSD and FreeBSD, all use the leading underscore; for these

compilers, the macros \c{cextern} and \c{cglobal}, as given in

\k{16cunder}, will still work. For ELF, though, the leading

underscore should not be used.

\S{32cfunc} Function Definitions and Function Calls

\I{functions, C calling convention}The \i{C calling convention}The C

calling convention in 32-bit programs is as follows. In the

following description, the words \e{caller} and \e{callee} are used

to denote the function doing the calling and the function which gets

called.

\b The caller pushes the function's parameters on the stack, one

after another, in reverse order (right to left, so that the first

argument specified to the function is pushed last).

\b The caller then executes a near \c{CALL} instruction to pass

control to the callee.

\b The callee receives control, and typically (although this is not

actually necessary, in functions which do not need to access their

parameters) starts by saving the value of \c{ESP} in \c{EBP} so as

to be able to use \c{EBP} as a base pointer to find its parameters

on the stack. However, the caller was probably doing this too, so

part of the calling convention states that \c{EBP} must be preserved

by any C function. Hence the callee, if it is going to set up

\c{EBP} as a \i{frame pointer}, must push the previous value first.

\b The callee may then access its parameters relative to \c{EBP}.

The doubleword at \c{[EBP]} holds the previous value of \c{EBP} as

it was pushed; the next doubleword, at \c{[EBP+4]}, holds the return

address, pushed implicitly by \c{CALL}. The parameters start after

that, at \c{[EBP+8]}. The leftmost parameter of the function, since

it was pushed last, is accessible at this offset from \c{EBP}; the

others follow, at successively greater offsets. Thus, in a function

such as \c{printf} which takes a variable number of parameters, the

pushing of the parameters in reverse order means that the function

knows where to find its first parameter, which tells it the number

and type of the remaining ones.

\b The callee may also wish to decrease \c{ESP} further, so as to

allocate space on the stack for local variables, which will then be

accessible at negative offsets from \c{EBP}.

\b The callee, if it wishes to return a value to the caller, should

leave the value in \c{AL}, \c{AX} or \c{EAX} depending on the size

of the value. Floating-point results are typically returned in

\c{ST0}.

\b Once the callee has finished processing, it restores \c{ESP} from

\c{EBP} if it had allocated local stack space, then pops the previous

value of \c{EBP}, and returns via \c{RET} (equivalently, \c{RETN}).

\b When the caller regains control from the callee, the function

parameters are still on the stack, so it typically adds an immediate

constant to \c{ESP} to remove them (instead of executing a number of

slow \c{POP} instructions). Thus, if a function is accidentally

called with the wrong number of parameters due to a prototype

mismatch, the stack will still be returned to a sensible state since

the caller, which \e{knows} how many parameters it pushed, does the

removing.

There is an alternative calling convention used by Win32 programs

for Windows API calls, and also for functions called \e{by} the

Windows API such as window procedures: they follow what Microsoft

calls the \c{__stdcall} convention. This is slightly closer to the

Pascal convention, in that the callee clears the stack by passing a

parameter to the \c{RET} instruction. However, the parameters are

still pushed in right-to-left order.

Thus, you would define a function in C style in the following way:

\c           global _myfunc

\c _myfunc:  push ebp

\c           mov ebp,esp

\c           sub esp,0x40           ; 64 bytes of local stack space

\c           mov ebx,[ebp+8]        ; first parameter to function

\c           ; some more code

\c           leave                  ; mov esp,ebp / pop ebp

\c           ret

At the other end of the process, to call a C function from your

assembly code, you would do something like this:

\c           extern _printf

\c           ; and then, further down...

\c           push dword [myint]     ; one of my integer variables

\c           push dword mystring    ; pointer into my data segment

\c           call _printf

\c           add esp,byte 8         ; `byte' saves space

\c           ; then those data items...

\c           segment _DATA

\c myint     dd 1234

\c mystring  db 'This number -> %d <- should be 1234',10,0

This piece of code is the assembly equivalent of the C code

\c     int myint = 1234;

\c     printf("This number -> %d <- should be 1234\n", myint);

\S{32cdata} Accessing Data Items

To get at the contents of C variables, or to declare variables which

C can access, you need only declare the names as \c{GLOBAL} or

\c{EXTERN}. (Again, the names require leading underscores, as stated

in \k{32cunder}.) Thus, a C variable declared as \c{int i} can be

accessed from assembler as

\c           extern _i

\c           mov eax,[_i]

And to declare your own integer variable which C programs can access

as \c{extern int j}, you do this (making sure you are assembling in

the \c{_DATA} segment, if necessary):

\c           global _j

\c _j        dd 0

To access a C array, you need to know the size of the components of

the array. For example, \c{int} variables are four bytes long, so if

a C program declares an array as \c{int a[10]}, you can access

\c{a[3]} by coding \c{mov ax,[_a+12]}. (The byte offset 12 is obtained

by multiplying the desired array index, 3, by the size of the array

element, 4.) The sizes of the C base types in 32-bit compilers are:

1 for \c{char}, 2 for \c{short}, 4 for \c{int}, \c{long} and

\c{float}, and 8 for \c{double}. Pointers, being 32-bit addresses,

are also 4 bytes long.

To access a C \i{data structure}, you need to know the offset from

the base of the structure to the field you are interested in. You

can either do this by converting the C structure definition into a

NASM structure definition (using \c{STRUC}), or by calculating the

one offset and using just that.

To do either of these, you should read your C compiler's manual to

find out how it organises data structures. NASM gives no special

alignment to structure members in its own \i\c{STRUC} macro, so you

have to specify alignment yourself if the C compiler generates it.

Typically, you might find that a structure like

\c struct {

\c     char c;

\c     int i;

\c } foo;

might be eight bytes long rather than five, since the \c{int} field

would be aligned to a four-byte boundary. However, this sort of

feature is sometimes a configurable option in the C compiler, either

using command-line options or \c{#pragma} lines, so you have to find

out how your own compiler does it.

\S{32cmacro} \i\c{c32.mac}: Helper Macros for the 32-bit C Interface

Included in the NASM archives, in the \I{misc directory}\c{misc}

directory, is a file \c{c32.mac} of macros. It defines three macros:

\i\c{proc}, \i\c{arg} and \i\c{endproc}. These are intended to be

used for C-style procedure definitions, and they automate a lot of

the work involved in keeping track of the calling convention.

An example of an assembly function using the macro set is given

here:

\c           proc _proc32

\c %$i       arg

\c %$j       arg

\c           mov eax,[ebp + %$i]

\c           mov ebx,[ebp + %$j]

\c           add eax,[ebx]

\c           endproc

This defines \c{_proc32} to be a procedure taking two arguments, the

first (\c{i}) an integer and the second (\c{j}) a pointer to an

integer. It returns \c{i + *j}.

Note that the \c{arg} macro has an \c{EQU} as the first line of its

expansion, and since the label before the macro call gets prepended

to the first line of the expanded macro, the \c{EQU} works, defining

\c{%$i} to be an offset from \c{BP}. A context-local variable is

used, local to the context pushed by the \c{proc} macro and popped

by the \c{endproc} macro, so that the same argument name can be used

in later procedures. Of course, you don't \e{have} to do that.

\c{arg} can take an optional parameter, giving the size of the

argument. If no size is given, 4 is assumed, since it is likely that

many function parameters will be of type \c{int} or pointers.

\H{picdll} Writing NetBSD/FreeBSD/OpenBSD and Linux/ELF \i{Shared

Libraries}

ELF replaced the older \c{a.out} object file format under Linux

because it contains support for \i{position-independent code}

(\i{PIC}), which makes writing shared libraries much easier. NASM

supports the ELF position-independent code features, so you can

write Linux ELF shared libraries in NASM.

\i{NetBSD}, and its close cousins \i{FreeBSD} and \i{OpenBSD}, take

a different approach by hacking PIC support into the \c{a.out}

format. NASM supports this as the \i\c{aoutb} output format, so you

can write \i{BSD} shared libraries in NASM too.

The operating system loads a PIC shared library by memory-mapping

the library file at an arbitrarily chosen point in the address space

of the running process. The contents of the library's code section

must therefore not depend on where it is loaded in memory.

Therefore, you cannot get at your variables by writing code like

this:

\c           mov eax,[myvar]        ; WRONG

Instead, the linker provides an area of memory called the

\i\e{global offset table}, or \i{GOT}; the GOT is situated at a

constant distance from your library's code, so if you can find out

where your library is loaded (which is typically done using a

\c{CALL} and \c{POP} combination), you can obtain the address of the

GOT, and you can then load the addresses of your variables out of

linker-generated entries in the GOT.

The \e{data} section of a PIC shared library does not have these

restrictions: since the data section is writable, it has to be

copied into memory anyway rather than just paged in from the library

file, so as long as it's being copied it can be relocated too. So

you can put ordinary types of relocation in the data section without

too much worry (but see \k{picglobal} for a caveat).

\S{picgot} Obtaining the Address of the GOT

Each code module in your shared library should define the GOT as an

external symbol:

\c           extern _GLOBAL_OFFSET_TABLE_   ; in ELF

\c           extern __GLOBAL_OFFSET_TABLE_  ; in BSD a.out

At the beginning of any function in your shared library which plans

to access your data or BSS sections, you must first calculate the

address of the GOT. This is typically done by writing the function

in this form:

\c func:     push ebp

\c           mov ebp,esp

\c           push ebx

\c           call .get_GOT

\c .get_GOT: pop ebx

\c           add ebx,_GLOBAL_OFFSET_TABLE_+$$-.get_GOT wrt ..gotpc

\c           ; the function body comes here

\c           mov ebx,[ebp-4]

\c           mov esp,ebp

\c           pop ebp

\c           ret

(For BSD, again, the symbol \c{_GLOBAL_OFFSET_TABLE} requires a

second leading underscore.)

The first two lines of this function are simply the standard C

prologue to set up a stack frame, and the last three lines are

standard C function epilogue. The third line, and the fourth to last

line, save and restore the \c{EBX} register, because PIC shared

libraries use this register to store the address of the GOT.

The interesting bit is the \c{CALL} instruction and the following

two lines. The \c{CALL} and \c{POP} combination obtains the address

of the label \c{.get_GOT}, without having to know in advance where

the program was loaded (since the \c{CALL} instruction is encoded

relative to the current position). The \c{ADD} instruction makes use

of one of the special PIC relocation types: \i{GOTPC relocation}.

With the \i\c{WRT ..gotpc} qualifier specified, the symbol

referenced (here \c{_GLOBAL_OFFSET_TABLE_}, the special symbol

assigned to the GOT) is given as an offset from the beginning of the

section. (Actually, ELF encodes it as the offset from the operand

field of the \c{ADD} instruction, but NASM simplifies this

deliberately, so you do things the same way for both ELF and BSD.)

So the instruction then \e{adds} the beginning of the section, to

get the real address of the GOT, and subtracts the value of

\c{.get_GOT} which it knows is in \c{EBX}. Therefore, by the time

that instruction has finished,

\c{EBX} contains the address of the GOT.

If you didn't follow that, don't worry: it's never necessary to

obtain the address of the GOT by any other means, so you can put

those three instructions into a macro and safely ignore them:

\c %macro get_GOT 0

\c           call %%getgot

\c %%getgot: pop ebx

\c           add ebx,_GLOBAL_OFFSET_TABLE_+$$-%%getgot wrt ..gotpc

\c %endmacro

\S{piclocal} Finding Your Local Data Items

Having got the GOT, you can then use it to obtain the addresses of

your data items. Most variables will reside in the sections you have

declared; they can be accessed using the \I{GOTOFF

relocation}\c{..gotoff} special \I\c{WRT ..gotoff}\c{WRT} type. The

way this works is like this:

\c           lea eax,[ebx+myvar wrt ..gotoff]

The expression \c{myvar wrt ..gotoff} is calculated, when the shared

library is linked, to be the offset to the local variable \c{myvar}

from the beginning of the GOT. Therefore, adding it to \c{EBX} as

above will place the real address of \c{myvar} in \c{EAX}.

If you declare variables as \c{GLOBAL} without specifying a size for

them, they are shared between code modules in the library, but do

not get exported from the library to the program that loaded it.

They will still be in your ordinary data and BSS sections, so you

can access them in the same way as local variables, using the above

\c{..gotoff} mechanism.

Note that due to a peculiarity of the way BSD \c{a.out} format

handles this relocation type, there must be at least one non-local

symbol in the same section as the address you're trying to access.

\S{picextern} Finding External and Common Data Items

If your library needs to get at an external variable (external to

the \e{library}, not just to one of the modules within it), you must

use the \I{GOT relocations}\I\c{WRT ..got}\c{..got} type to get at

it. The \c{..got} type, instead of giving you the offset from the

GOT base to the variable, gives you the offset from the GOT base to

a GOT \e{entry} containing the address of the variable. The linker

will set up this GOT entry when it builds the library, and the

dynamic linker will place the correct address in it at load time. So

to obtain the address of an external variable \c{extvar} in \c{EAX},

you would code

\c           mov eax,[ebx+extvar wrt ..got]

This loads the address of \c{extvar} out of an entry in the GOT. The

linker, when it builds the shared library, collects together every

relocation of type \c{..got}, and builds the GOT so as to ensure it

has every necessary entry present.

Common variables must also be accessed in this way.

\S{picglobal} Exporting Symbols to the Library User

If you want to export symbols to the user of the library, you have

to declare whether they are functions or data, and if they are data,

you have to give the size of the data item. This is because the

dynamic linker has to build \I{PLT}\i{procedure linkage table}

entries for any exported functions, and also moves exported data

items away from the library's data section in which they were

declared.

So to export a function to users of the library, you must use

\c           global func:function   ; declare it as a function

\c func:     push ebp

\c           ; etc.

And to export a data item such as an array, you would have to code

\c           global array:data array.end-array ; give the size too

\c array:    resd 128

\c .end:

Be careful: If you export a variable to the library user, by

declaring it as \c{GLOBAL} and supplying a size, the variable will

end up living in the data section of the main program, rather than

in your library's data section, where you declared it. So you will

have to access your own global variable with the \c{..got} mechanism

rather than \c{..gotoff}, as if it were external (which,

effectively, it has become).

Equally, if you need to store the address of an exported global in

one of your data sections, you can't do it by means of the standard

sort of code:

\c dataptr:  dd global_data_item    ; WRONG

NASM will interpret this code as an ordinary relocation, in which

\c{global_data_item} is merely an offset from the beginning of the

\c{.data} section (or whatever); so this reference will end up

pointing at your data section instead of at the exported global

which resides elsewhere.

Instead of the above code, then, you must write

\c dataptr:  dd global_data_item wrt ..sym

which makes use of the special \c{WRT} type \I\c{WRT ..sym}\c{..sym}

to instruct NASM to search the symbol table for a particular symbol

at that address, rather than just relocating by section base.

Either method will work for functions: referring to one of your

functions by means of

\c funcptr:  dd my_function

will give the user the address of the code you wrote, whereas

\c funcptr:  dd my_function wrt ..sym

will give the address of the procedure linkage table for the

function, which is where the calling program will \e{believe} the

function lives. Either address is a valid way to call the function.

\S{picproc} Calling Procedures Outside the Library

Calling procedures outside your shared library has to be done by

means of a \i\e{procedure linkage table}, or \i{PLT}. The PLT is

placed at a known offset from where the library is loaded, so the

library code can make calls to the PLT in a position-independent

way. Within the PLT there is code to jump to offsets contained in

the GOT, so function calls to other shared libraries or to routines

in the main program can be transparently passed off to their real

destinations.

To call an external routine, you must use another special PIC

relocation type, \I{PLT relocations}\i\c{WRT ..plt}. This is much

easier than the GOT-based ones: you simply replace calls such as

\c{CALL printf} with the PLT-relative version \c{CALL printf WRT

..plt}.

\S{link} Generating the Library File

Having written some code modules and assembled them to \c{.o} files,

you then generate your shared library with a command such as

\c ld -shared -o library.so module1.o module2.o       # for ELF

\c ld -Bshareable -o library.so module1.o module2.o   # for BSD

For ELF, if your shared library is going to reside in system

directories such as \c{/usr/lib} or \c{/lib}, it is usually worth

using the \i\c{-soname} flag to the linker, to store the final

library file name, with a version number, into the library:

\c ld -shared -soname library.so.1 -o library.so.1.2 *.o

You would then copy \c{library.so.1.2} into the library directory,

and create \c{library.so.1} as a symbolic link to it.

\C{mixsize} Mixing 16 and 32 Bit Code

This chapter tries to cover some of the issues, largely related to

unusual forms of addressing and jump instructions, encountered when

writing operating system code such as protected-mode initialisation

routines, which require code that operates in mixed segment sizes,

such as code in a 16-bit segment trying to modify data in a 32-bit

one, or jumps between different-size segments.

\H{mixjump} Mixed-Size Jumps\I{jumps, mixed-size}

\I{operating system, writing}\I{writing operating systems}The most

common form of \i{mixed-size instruction} is the one used when

writing a 32-bit OS: having done your setup in 16-bit mode, such as

loading the kernel, you then have to boot it by switching into

protected mode and jumping to the 32-bit kernel start address. In a

fully 32-bit OS, this tends to be the \e{only} mixed-size

instruction you need, since everything before it can be done in pure

16-bit code, and everything after it can be pure 32-bit.

This jump must specify a 48-bit far address, since the target

segment is a 32-bit one. However, it must be assembled in a 16-bit

segment, so just coding, for example,

\c           jmp 0x1234:0x56789ABC  ; wrong!

will not work, since the offset part of the address will be

truncated to \c{0x9ABC} and the jump will be an ordinary 16-bit far

one.

The Linux kernel setup code gets round the inability of \c{as86} to

generate the required instruction by coding it manually, using

\c{DB} instructions. NASM can go one better than that, by actually

generating the right instruction itself. Here's how to do it right:

\c           jmp dword 0x1234:0x56789ABC  ; right

\I\c{JMP DWORD}The \c{DWORD} prefix (strictly speaking, it should

come \e{after} the colon, since it is declaring the \e{offset} field

to be a doubleword; but NASM will accept either form, since both are

unambiguous) forces the offset part to be treated as far, in the

assumption that you are deliberately writing a jump from a 16-bit

segment to a 32-bit one.

You can do the reverse operation, jumping from a 32-bit segment to a

16-bit one, by means of the \c{WORD} prefix:

\c           jmp word 0x8765:0x4321 ; 32 to 16 bit

If the \c{WORD} prefix is specified in 16-bit mode, or the \c{DWORD}

prefix in 32-bit mode, they will be ignored, since each is

explicitly forcing NASM into a mode it was in anyway.

\H{mixaddr} Addressing Between Different-Size Segments\I{addressing,

mixed-size}\I{mixed-size addressing}

If your OS is mixed 16 and 32-bit, or if you are writing a DOS

extender, you are likely to have to deal with some 16-bit segments

and some 32-bit ones. At some point, you will probably end up

writing code in a 16-bit segment which has to access data in a

32-bit segment, or vice versa.

If the data you are trying to access in a 32-bit segment lies within

the first 64K of the segment, you may be able to get away with using

an ordinary 16-bit addressing operation for the purpose; but sooner

or later, you will want to do 32-bit addressing from 16-bit mode.

The easiest way to do this is to make sure you use a register for

the address, since any effective address containing a 32-bit

register is forced to be a 32-bit address. So you can do

\c           mov eax,offset_into_32_bit_segment_specified_by_fs

\c           mov dword [fs:eax],0x11223344

This is fine, but slightly cumbersome (since it wastes an

instruction and a register) if you already know the precise offset

you are aiming at. The x86 architecture does allow 32-bit effective

addresses to specify nothing but a 4-byte offset, so why shouldn't

NASM be able to generate the best instruction for the purpose?

It can. As in \k{mixjump}, you need only prefix the address with the

\c{DWORD} keyword, and it will be forced to be a 32-bit address:

\c           mov dword [fs:dword my_offset],0x11223344

Also as in \k{mixjump}, NASM is not fussy about whether the

\c{DWORD} prefix comes before or after the segment override, so

arguably a nicer-looking way to code the above instruction is

\c           mov dword [dword fs:my_offset],0x11223344

Don't confuse the \c{DWORD} prefix \e{outside} the square brackets,

which controls the size of the data stored at the address, with the

one \c{inside} the square brackets which controls the length of the

address itself. The two can quite easily be different:

\c           mov word [dword 0x12345678],0x9ABC

This moves 16 bits of data to an address specified by a 32-bit

offset.

You can also specify \c{WORD} or \c{DWORD} prefixes along with the

\c{FAR} prefix to indirect far jumps or calls. For example:

\c           call dword far [fs:word 0x4321]

This instruction contains an address specified by a 16-bit offset;

it loads a 48-bit far pointer from that (16-bit segment and 32-bit

offset), and calls that address.

\H{mixother} Other Mixed-Size Instructions

The other way you might want to access data might be using the

string instructions (\c{LODSx}, \c{STOSx} and so on) or the

\c{XLATB} instruction. These instructions, since they take no

parameters, might seem to have no easy way to make them perform

32-bit addressing when assembled in a 16-bit segment.

This is the purpose of NASM's \i\c{a16} and \i\c{a32} prefixes. If

you are coding \c{LODSB} in a 16-bit segment but it is supposed to

be accessing a string in a 32-bit segment, you should load the

desired address into \c{ESI} and then code

\c           a32 lodsb

The prefix forces the addressing size to 32 bits, meaning that

\c{LODSB} loads from \c{[DS:ESI]} instead of \c{[DS:SI]}. To access

a string in a 16-bit segment when coding in a 32-bit one, the

corresponding \c{a16} prefix can be used.

The \c{a16} and \c{a32} prefixes can be applied to any instruction

in NASM's instruction table, but most of them can generate all the

useful forms without them. The prefixes are necessary only for

instructions with implicit addressing: \c{CMPSx} (\k{insCMPSB}),

\c{SCASx} (\k{insSCASB}), \c{LODSx} (\k{insLODSB}), \c{STOSx}

(\k{insSTOSB}), \c{MOVSx} (\k{insMOVSB}), \c{INSx} (\k{insINSB}),

\c{OUTSx} (\k{insOUTSB}), and \c{XLATB} (\k{insXLATB}). Also, the

various push and pop instructions (\c{PUSHA} and \c{POPF} as well as

the more usual \c{PUSH} and \c{POP}) can accept \c{a16} or \c{a32}

prefixes to force a particular one of \c{SP} or \c{ESP} to be used

as a stack pointer, in case the stack segment in use is a different

size from the code segment.

\c{PUSH} and \c{POP}, when applied to segment registers in 32-bit

mode, also have the slightly odd behaviour that they push and pop 4

bytes at a time, of which the top two are ignored and the bottom two

give the value of the segment register being manipulated. To force

the 16-bit behaviour of segment-register push and pop instructions,

you can use the operand-size prefix \i\c{o16}:

\c           o16 push ss

\c           o16 push ds

This code saves a doubleword of stack space by fitting two segment

registers into the space which would normally be consumed by pushing

one.

(You can also use the \i\c{o32} prefix to force the 32-bit behaviour

when in 16-bit mode, but this seems less useful.)

\C{trouble} Troubleshooting

This chapter describes some of the common problems that users have

been known to encounter with NASM, and answers them. It also gives

instructions for reporting bugs in NASM if you find a difficulty

that isn't listed here.

\H{problems} Common Problems

\S{inefficient} NASM Generates \i{Inefficient Code}

I get a lot of `bug' reports about NASM generating inefficient, or

even `wrong', code on instructions such as \c{ADD ESP,8}. This is a

deliberate design feature, connected to predictability of output:

NASM, on seeing \c{ADD ESP,8}, will generate the form of the

instruction which leaves room for a 32-bit offset. You need to code

\I\c{BYTE}\c{ADD ESP,BYTE 8} if you want the space-efficient

form of the instruction. This isn't a bug: at worst it's a

misfeature, and that's a matter of opinion only.

\S{jmprange} My Jumps are Out of Range\I{out of range, jumps}

Similarly, people complain that when they issue \i{conditional

jumps} (which are \c{SHORT} by default) that try to jump too far,

NASM reports `short jump out of range' instead of making the jumps

longer.

This, again, is partly a predictability issue, but in fact has a

more practical reason as well. NASM has no means of being told what

type of processor the code it is generating will be run on; so it

cannot decide for itself that it should generate \i\c{Jcc NEAR} type

instructions, because it doesn't know that it's working for a 386 or

above. Alternatively, it could replace the out-of-range short

\c{JNE} instruction with a very short \c{JE} instruction that jumps

over a \c{JMP NEAR}; this is a sensible solution for processors

below a 386, but hardly efficient on processors which have good

branch prediction \e{and} could have used \c{JNE NEAR} instead. So,

once again, it's up to the user, not the assembler, to decide what

instructions should be generated.

\S{proborg} \i\c{ORG} Doesn't Work

People writing \i{boot sector} programs in the \c{bin} format often

complain that \c{ORG} doesn't work the way they'd like: in order to

place the \c{0xAA55} signature word at the end of a 512-byte boot

sector, people who are used to MASM tend to code

\c           ORG 0

\c           ; some boot sector code

\c           ORG 510

\c           DW 0xAA55

This is not the intended use of the \c{ORG} directive in NASM, and

will not work. The correct way to solve this problem in NASM is to

use the \i\c{TIMES} directive, like this:

\c           ORG 0

\c           ; some boot sector code

\c           TIMES 510-($-$$) DB 0

\c           DW 0xAA55

The \c{TIMES} directive will insert exactly enough zero bytes into

the output to move the assembly point up to 510. This method also

has the advantage that if you accidentally fill your boot sector too

full, NASM will catch the problem at assembly time and report it, so

you won't end up with a boot sector that you have to disassemble to

find out what's wrong with it.

\S{probtimes} \i\c{TIMES} Doesn't Work

The other common problem with the above code is people who write the

\c{TIMES} line as

\c           TIMES 510-$ DB 0

by reasoning that \c{$} should be a pure number, just like 510, so

the difference between them is also a pure number and can happily be

fed to \c{TIMES}.

NASM is a \e{modular} assembler: the various component parts are

designed to be easily separable for re-use, so they don't exchange

information unnecessarily. In consequence, the \c{bin} output

format, even though it has been told by the \c{ORG} directive that

the \c{.text} section should start at 0, does not pass that

information back to the expression evaluator. So from the

evaluator's point of view, \c{$} isn't a pure number: it's an offset

from a section base. Therefore the difference between \c{$} and 510

is also not a pure number, but involves a section base. Values

involving section bases cannot be passed as arguments to \c{TIMES}.

The solution, as in the previous section, is to code the \c{TIMES}

line in the form

\c           TIMES 510-($-$$) DB 0

in which \c{$} and \c{$$} are offsets from the same section base,

and so their difference is a pure number. This will solve the

problem and generate sensible code.

\H{bugs} \i{Bugs}\I{reporting bugs}

We have never yet released a version of NASM with any \e{known}

bugs. That doesn't usually stop there being plenty we didn't know

about, though. Any that you find should be reported to

\W{mailto:anakin@pobox.com}\c{anakin@pobox.com}.

Please read \k{qstart} first, and don't report the bug if it's

listed in there as a deliberate feature. (If you think the feature

is badly thought out, feel free to send us reasons why you think it

should be changed, but don't just send us mail saying `This is a

bug' if the documentation says we did it on purpose.) Then read

\k{problems}, and don't bother reporting the bug if it's listed

there.

If you do report a bug, \e{please} give us all of the following

information:

\b What operating system you're running NASM under. DOS, Linux,

NetBSD, Win16, Win32, VMS (I'd be impressed), whatever.

\b If you're running NASM under DOS or Win32, tell us whether you've

compiled your own executable from the DOS source archive, or whether

you were using the standard distribution binaries out of the

archive. If you were using a locally built executable, try to

reproduce the problem using one of the standard binaries, as this

will make it easier for us to reproduce your problem prior to fixing

it.

\b Which version of NASM you're using, and exactly how you invoked

it. Give us the precise command line, and the contents of the

\c{NASM} environment variable if any.

\b Which versions of any supplementary programs you're using, and

how you invoked them. If the problem only becomes visible at link

time, tell us what linker you're using, what version of it you've

got, and the exact linker command line. If the problem involves

linking against object files generated by a compiler, tell us what

compiler, what version, and what command line or options you used.

(If you're compiling in an IDE, please try to reproduce the problem

with the command-line version of the compiler.)

\b If at all possible, send us a NASM source file which exhibits the

problem. If this causes copyright problems (e.g. you can only

reproduce the bug in restricted-distribution code) then bear in mind

the following two points: firstly, we guarantee that any source code

sent to us for the purposes of debugging NASM will be used \e{only}

for the purposes of debugging NASM, and that we will delete all our

copies of it as soon as we have found and fixed the bug or bugs in

question; and secondly, we would prefer \e{not} to be mailed large

chunks of code anyway. The smaller the file, the better. A

three-line sample file that does nothing useful \e{except}

demonstrate the problem is much easier to work with than a

fully fledged ten-thousand-line program. (Of course, some errors

\e{do} only crop up in large files, so this may not be possible.)

\b A description of what the problem actually \e{is}. `It doesn't

work' is \e{not} a helpful description! Please describe exactly what

is happening that shouldn't be, or what isn't happening that should.

Examples might be: `NASM generates an error message saying Line 3

for an error that's actually on Line 5'; `NASM generates an error

message that I believe it shouldn't be generating at all'; `NASM

fails to generate an error message that I believe it \e{should} be

generating'; `the object file produced from this source code crashes

my linker'; `the ninth byte of the output file is 66 and I think it

should be 77 instead'.

\b If you believe the output file from NASM to be faulty, send it to

us. That allows us to determine whether our own copy of NASM

generates the same file, or whether the problem is related to

portability issues between our development platforms and yours. We

can handle binary files mailed to us as MIME attachments, uuencoded,

and even BinHex. Alternatively, we may be able to provide an FTP

site you can upload the suspect files to; but mailing them is easier

for us.

\b Any other information or data files that might be helpful. If,

for example, the problem involves NASM failing to generate an object

file while TASM can generate an equivalent file without trouble,

then send us \e{both} object files, so we can see what TASM is doing

differently from us.

\A{iref} Intel x86 Instruction Reference

This appendix provides a complete list of the machine instructions

which NASM will assemble, and a short description of the function of

each one.

It is not intended to be exhaustive documentation on the fine

details of the instructions' function, such as which exceptions they

can trigger: for such documentation, you should go to Intel's Web

site, \W{http://www.intel.com}\c{http://www.intel.com}.

Instead, this appendix is intended primarily to provide

documentation on the way the instructions may be used within NASM.

For example, looking up \c{LOOP} will tell you that NASM allows

\c{CX} or \c{ECX} to be specified as an optional second argument to

the \c{LOOP} instruction, to enforce which of the two possible

counter registers should be used if the default is not the one

desired.

The instructions are not quite listed in alphabetical order, since

groups of instructions with similar functions are lumped together in

the same entry. Most of them don't move very far from their

alphabetic position because of this.

\H{iref-opr} Key to Operand Specifications

The instruction descriptions in this appendix specify their operands

using the following notation:

\b Registers: \c{reg8} denotes an 8-bit \i{general purpose

register}, \c{reg16} denotes a 16-bit general purpose register, and

\c{reg32} a 32-bit one. \c{fpureg} denotes one of the eight FPU

stack registers, \c{mmxreg} denotes one of the eight 64-bit MMX

registers, and \c{segreg} denotes a segment register. In addition,

some registers (such as \c{AL}, \c{DX} or

\c{ECX}) may be specified explicitly.

\b Immediate operands: \c{imm} denotes a generic \i{immediate operand}.

\c{imm8}, \c{imm16} and \c{imm32} are used when the operand is

intended to be a specific size. For some of these instructions, NASM

needs an explicit specifier: for example, \c{ADD ESP,16} could be

interpreted as either \c{ADD r/m32,imm32} or \c{ADD r/m32,imm8}.

NASM chooses the former by default, and so you must specify \c{ADD

ESP,BYTE 16} for the latter.

\b Memory references: \c{mem} denotes a generic \i{memory reference};

\c{mem8}, \c{mem16}, \c{mem32}, \c{mem64} and \c{mem80} are used

when the operand needs to be a specific size. Again, a specifier is

needed in some cases: \c{DEC [address]} is ambiguous and will be

rejected by NASM. You must specify \c{DEC BYTE [address]}, \c{DEC

WORD [address]} or \c{DEC DWORD [address]} instead.

\b \i{Restricted memory references}: one form of the \c{MOV}

instruction allows a memory address to be specified \e{without}

allowing the normal range of register combinations and effective

address processing. This is denoted by \c{memoffs8}, \c{memoffs16}

and \c{memoffs32}.

\b Register or memory choices: many instructions can accept either a

register \e{or} a memory reference as an operand. \c{r/m8} is a

shorthand for \c{reg8/mem8}; similarly \c{r/m16} and \c{r/m32}.

\c{r/m64} is MMX-related, and is a shorthand for \c{mmxreg/mem64}.

\H{iref-opc} Key to Opcode Descriptions

This appendix also provides the opcodes which NASM will generate for

each form of each instruction. The opcodes are listed in the

following way:

\b A hex number, such as \c{3F}, indicates a fixed byte containing

that number.

\b A hex number followed by \c{+r}, such as \c{C8+r}, indicates that

one of the operands to the instruction is a register, and the

`register value' of that register should be added to the hex number

to produce the generated byte. For example, EDX has register value

2, so the code \c{C8+r}, when the register operand is EDX, generates

the hex byte \c{CA}. Register values for specific registers are

given in \k{iref-rv}.

\b A hex number followed by \c{+cc}, such as \c{40+cc}, indicates

that the instruction name has a condition code suffix, and the

numeric representation of the condition code should be added to the

hex number to produce the generated byte. For example, the code

\c{40+cc}, when the instruction contains the \c{NE} condition,

generates the hex byte \c{45}. Condition codes and their numeric

representations are given in \k{iref-cc}.

\b A slash followed by a digit, such as \c{/2}, indicates that one

of the operands to the instruction is a memory address or register

(denoted \c{mem} or \c{r/m}, with an optional size). This is to be

encoded as an effective address, with a \i{ModR/M byte}, an optional

\i{SIB byte}, and an optional displacement, and the spare (register)

field of the ModR/M byte should be the digit given (which will be

from 0 to 7, so it fits in three bits). The encoding of effective

addresses is given in \k{iref-ea}.

\b The code \c{/r} combines the above two: it indicates that one of

the operands is a memory address or \c{r/m}, and another is a

register, and that an effective address should be generated with the

spare (register) field in the ModR/M byte being equal to the

`register value' of the register operand. The encoding of effective

addresses is given in \k{iref-ea}; register values are given in

\k{iref-rv}.

\b The codes \c{ib}, \c{iw} and \c{id} indicate that one of the

operands to the instruction is an immediate value, and that this is

to be encoded as a byte, little-endian word or little-endian

doubleword respectively.

\b The codes \c{rb}, \c{rw} and \c{rd} indicate that one of the

operands to the instruction is an immediate value, and that the

\e{difference} between this value and the address of the end of the

instruction is to be encoded as a byte, word or doubleword

respectively. Where the form \c{rw/rd} appears, it indicates that

either \c{rw} or \c{rd} should be used according to whether assembly

is being performed in \c{BITS 16} or \c{BITS 32} state respectively.

\b The codes \c{ow} and \c{od} indicate that one of the operands to

the instruction is a reference to the contents of a memory address

specified as an immediate value: this encoding is used in some forms

of the \c{MOV} instruction in place of the standard

effective-address mechanism. The displacement is encoded as a word

or doubleword. Again, \c{ow/od} denotes that \c{ow} or \c{od} should

be chosen according to the \c{BITS} setting.

\b The codes \c{o16} and \c{o32} indicate that the given form of the

instruction should be assembled with operand size 16 or 32 bits. In

other words, \c{o16} indicates a \c{66} prefix in \c{BITS 32} state,

but generates no code in \c{BITS 16} state; and \c{o32} indicates a

\c{66} prefix in \c{BITS 16} state but generates nothing in \c{BITS

32}.

\b The codes \c{a16} and \c{a32}, similarly to \c{o16} and \c{o32},

indicate the address size of the given form of the instruction.

Where this does not match the \c{BITS} setting, a \c{67} prefix is

required.

\S{iref-rv} Register Values

Where an instruction requires a register value, it is already

implicit in the encoding of the rest of the instruction what type of

register is intended: an 8-bit general-purpose register, a segment

register, a debug register, an MMX register, or whatever. Therefore

there is no problem with registers of different types sharing an

encoding value.

The encodings for the various classes of register are:

\b 8-bit general registers: \c{AL} is 0, \c{CL} is 1, \c{DL} is 2,

\c{BL} is 3, \c{AH} is 4, \c{CH} is 5, \c{DH} is 6, and \c{BH} is

7.

\b 16-bit general registers: \c{AX} is 0, \c{CX} is 1, \c{DX} is 2,

\c{BX} is 3, \c{SP} is 4, \c{BP} is 5, \c{SI} is 6, and \c{DI} is 7.

\b 32-bit general registers: \c{EAX} is 0, \c{ECX} is 1, \c{EDX} is

2, \c{EBX} is 3, \c{ESP} is 4, \c{EBP} is 5, \c{ESI} is 6, and

\c{EDI} is 7.

\b \i{Segment registers}: \c{ES} is 0, \c{CS} is 1, \c{SS} is 2, \c{DS}

is 3, \c{FS} is 4, and \c{GS} is 5.

\b \I{floating-point, registers}{Floating-point registers}: \c{ST0}

is 0, \c{ST1} is 1, \c{ST2} is 2, \c{ST3} is 3, \c{ST4} is 4,

\c{ST5} is 5, \c{ST6} is 6, and \c{ST7} is 7.

\b 64-bit \i{MMX registers}: \c{MM0} is 0, \c{MM1} is 1, \c{MM2} is 2,

\c{MM3} is 3, \c{MM4} is 4, \c{MM5} is 5, \c{MM6} is 6, and \c{MM7}

is 7.

\b \i{Control registers}: \c{CR0} is 0, \c{CR2} is 2, \c{CR3} is 3,

and \c{CR4} is 4.

\b \i{Debug registers}: \c{DR0} is 0, \c{DR1} is 1, \c{DR2} is 2,

\c{DR3} is 3, \c{DR6} is 6, and \c{DR7} is 7.

\b \i{Test registers}: \c{TR3} is 3, \c{TR4} is 4, \c{TR5} is 5,

\c{TR6} is 6, and \c{TR7} is 7.

(Note that wherever a register name contains a number, that number

is also the register value for that register.)

\S{iref-cc} \i{Condition Codes}

The available condition codes are given here, along with their

numeric representations as part of opcodes. Many of these condition

codes have synonyms, so several will be listed at a time.

In the following descriptions, the word `either', when applied to two

possible trigger conditions, is used to mean `either or both'. If

`either but not both' is meant, the phrase `exactly one of' is used.

\b \c{O} is 0 (trigger if the overflow flag is set); \c{NO} is 1.

\b \c{B}, \c{C} and \c{NAE} are 2 (trigger if the carry flag is

set); \c{AE}, \c{NB} and \c{NC} are 3.

\b \c{E} and \c{Z} are 4 (trigger if the zero flag is set); \c{NE}

and \c{NZ} are 5.

\b \c{BE} and \c{NA} are 6 (trigger if either of the carry or zero

flags is set); \c{A} and \c{NBE} are 7.

\b \c{S} is 8 (trigger if the sign flag is set); \c{NS} is 9.

\b \c{P} and \c{PE} are 10 (trigger if the parity flag is set);

\c{NP} and \c{PO} are 11.

\b \c{L} and \c{NGE} are 12 (trigger if exactly one of the sign and

overflow flags is set); \c{GE} and \c{NL} are 13.

\b \c{LE} and \c{NG} are 14 (trigger if either the zero flag is set,

or exactly one of the sign and overflow flags is set); \c{G} and

\c{NLE} are 15.

Note that in all cases, the sense of a condition code may be

reversed by changing the low bit of the numeric representation.

\S{iref-ea} Effective Address Encoding: \i{ModR/M} and \i{SIB}

An \i{effective address} is encoded in up to three parts: a ModR/M

byte, an optional SIB byte, and an optional byte, word or doubleword

displacement field.

The ModR/M byte consists of three fields: the \c{mod} field, ranging

from 0 to 3, in the upper two bits of the byte, the \c{r/m} field,

ranging from 0 to 7, in the lower three bits, and the spare

(register) field in the middle (bit 3 to bit 5). The spare field is

not relevant to the effective address being encoded, and either

contains an extension to the instruction opcode or the register

value of another operand.

The ModR/M system can be used to encode a direct register reference

rather than a memory access. This is always done by setting the

\c{mod} field to 3 and the \c{r/m} field to the register value of

the register in question (it must be a general-purpose register, and

the size of the register must already be implicit in the encoding of

the rest of the instruction). In this case, the SIB byte and

displacement field are both absent.

In 16-bit addressing mode (either \c{BITS 16} with no \c{67} prefix,

or \c{BITS 32} with a \c{67} prefix), the SIB byte is never used.

The general rules for \c{mod} and \c{r/m} (there is an exception,

given below) are:

\b The \c{mod} field gives the length of the displacement field: 0

means no displacement, 1 means one byte, and 2 means two bytes.

\b The \c{r/m} field encodes the combination of registers to be

added to the displacement to give the accessed address: 0 means

\c{BX+SI}, 1 means \c{BX+DI}, 2 means \c{BP+SI}, 3 means \c{BP+DI},

4 means \c{SI} only, 5 means \c{DI} only, 6 means \c{BP} only, and 7

means \c{BX} only.

However, there is a special case:

\b If \c{mod} is 0 and \c{r/m} is 6, the effective address encoded

is not \c{[BP]} as the above rules would suggest, but instead

\c{[disp16]}: the displacement field is present and is two bytes

long, and no registers are added to the displacement.

Therefore the effective address \c{[BP]} cannot be encoded as

efficiently as \c{[BX]}; so if you code \c{[BP]} in a program, NASM

adds a notional 8-bit zero displacement, and sets \c{mod} to 1,

\c{r/m} to 6, and the one-byte displacement field to 0.

In 32-bit addressing mode (either \c{BITS 16} with a \c{67} prefix,

or \c{BITS 32} with no \c{67} prefix) the general rules (again,

there are exceptions) for \c{mod} and \c{r/m} are:

\b The \c{mod} field gives the length of the displacement field: 0

means no displacement, 1 means one byte, and 2 means four bytes.

\b If only one register is to be added to the displacement, and it

is not \c{ESP}, the \c{r/m} field gives its register value, and the

SIB byte is absent. If the \c{r/m} field is 4 (which would encode

\c{ESP}), the SIB byte is present and gives the combination and

scaling of registers to be added to the displacement.

If the SIB byte is present, it describes the combination of

registers (an optional base register, and an optional index register

scaled by multiplication by 1, 2, 4 or 8) to be added to the

displacement. The SIB byte is divided into the \c{scale} field, in

the top two bits, the \c{index} field in the next three, and the

\c{base} field in the bottom three. The general rules are:

\b The \c{base} field encodes the register value of the base

register.

\b The \c{index} field encodes the register value of the index

register, unless it is 4, in which case no index register is used

(so \c{ESP} cannot be used as an index register).

\b The \c{scale} field encodes the multiplier by which the index

register is scaled before adding it to the base and displacement: 0

encodes a multiplier of 1, 1 encodes 2, 2 encodes 4 and 3 encodes 8.

The exceptions to the 32-bit encoding rules are:

\b If \c{mod} is 0 and \c{r/m} is 5, the effective address encoded

is not \c{[EBP]} as the above rules would suggest, but instead

\c{[disp32]}: the displacement field is present and is four bytes

long, and no registers are added to the displacement.

\b If \c{mod} is 0, \c{r/m} is 4 (meaning the SIB byte is present)

and \c{base} is 4, the effective address encoded is not

\c{[EBP+index]} as the above rules would suggest, but instead

\c{[disp32+index]}: the displacement field is present and is four

bytes long, and there is no base register (but the index register is

still processed in the normal way).

\H{iref-flg} Key to Instruction Flags

Given along with each instruction in this appendix is a set of

flags, denoting the type of the instruction. The types are as follows:

\b \c{8086}, \c{186}, \c{286}, \c{386}, \c{486}, \c{PENT} and \c{P6}

denote the lowest processor type that supports the instruction. Most

instructions run on all processors above the given type; those that

do not are documented. The Pentium II contains no additional

instructions beyond the P6 (Pentium Pro); from the point of view of

its instruction set, it can be thought of as a P6 with MMX

capability.

\b \c{CYRIX} indicates that the instruction is specific to Cyrix

processors, for example the extra MMX instructions in the Cyrix

extended MMX instruction set.

\b \c{FPU} indicates that the instruction is a floating-point one,

and will only run on machines with a coprocessor (automatically

including 486DX, Pentium and above).

\b \c{MMX} indicates that the instruction is an MMX one, and will

run on MMX-capable Pentium processors and the Pentium II.

\b \c{PRIV} indicates that the instruction is a protected-mode

management instruction. Many of these may only be used in protected

mode, or only at privilege level zero.

\b \c{UNDOC} indicates that the instruction is an undocumented one,

and not part of the official Intel Architecture; it may or may not

be supported on any given machine.

\H{insAAA} \i\c{AAA}, \i\c{AAS}, \i\c{AAM}, \i\c{AAD}: ASCII

Adjustments

\c AAA                           ; 37                   [8086]

\c AAS                           ; 3F                   [8086]

\c AAD                           ; D5 0A                [8086]

\c AAD imm                       ; D5 ib                [8086]

\c AAM                           ; D4 0A                [8086]

\c AAM imm                       ; D4 ib                [8086]

These instructions are used in conjunction with the add, subtract,

multiply and divide instructions to perform binary-coded decimal

arithmetic in \e{unpacked} (one BCD digit per byte - easy to

translate to and from ASCII, hence the instruction names) form.

There are also packed BCD instructions \c{DAA} and \c{DAS}: see

\k{insDAA}.

\c{AAA} should be used after a one-byte \c{ADD} instruction whose

destination was the \c{AL} register: by means of examining the value

in the low nibble of \c{AL} and also the auxiliary carry flag

\c{AF}, it determines whether the addition has overflowed, and

adjusts it (and sets the carry flag) if so. You can add long BCD

strings together by doing \c{ADD}/\c{AAA} on the low digits, then

doing \c{ADC}/\c{AAA} on each subsequent digit.

\c{AAS} works similarly to \c{AAA}, but is for use after \c{SUB}

instructions rather than \c{ADD}.

\c{AAM} is for use after you have multiplied two decimal digits

together and left the result in \c{AL}: it divides \c{AL} by ten and

stores the quotient in \c{AH}, leaving the remainder in \c{AL}. The

divisor 10 can be changed by specifying an operand to the

instruction: a particularly handy use of this is \c{AAM 16}, causing

the two nibbles in \c{AL} to be separated into \c{AH} and \c{AL}.

\c{AAD} performs the inverse operation to \c{AAM}: it multiplies

\c{AH} by ten, adds it to \c{AL}, and sets \c{AH} to zero. Again,

the multiplier 10 can be changed.

\H{insADC} \i\c{ADC}: Add with Carry

\c ADC r/m8,reg8                 ; 10 /r                [8086]

\c ADC r/m16,reg16               ; o16 11 /r            [8086]

\c ADC r/m32,reg32               ; o32 11 /r            [386]

\c ADC reg8,r/m8                 ; 12 /r                [8086]

\c ADC reg16,r/m16               ; o16 13 /r            [8086]

\c ADC reg32,r/m32               ; o32 13 /r            [386]

\c ADC r/m8,imm8                 ; 80 /2 ib             [8086]

\c ADC r/m16,imm16               ; o16 81 /2 iw         [8086]

\c ADC r/m32,imm32               ; o32 81 /2 id         [386]

\c ADC r/m16,imm8                ; o16 83 /2 ib         [8086]

\c ADC r/m32,imm8                ; o32 83 /2 ib         [386]

\c ADC AL,imm8                   ; 14 ib                [8086]

\c ADC AX,imm16                  ; o16 15 iw            [8086]

\c ADC EAX,imm32                 ; o32 15 id            [386]

\c{ADC} performs integer addition: it adds its two operands

together, plus the value of the carry flag, and leaves the result in

its destination (first) operand. The flags are set according to the

result of the operation: in particular, the carry flag is affected

and can be used by a subsequent \c{ADC} instruction.

In the forms with an 8-bit immediate second operand and a longer

first operand, the second operand is considered to be signed, and is

sign-extended to the length of the first operand. In these cases,

the \c{BYTE} qualifier is necessary to force NASM to generate this

form of the instruction.

To add two numbers without also adding the contents of the carry

flag, use \c{ADD} (\k{insADD}).

\H{insADD} \i\c{ADD}: Add Integers

\c ADD r/m8,reg8                 ; 00 /r                [8086]

\c ADD r/m16,reg16               ; o16 01 /r            [8086]

\c ADD r/m32,reg32               ; o32 01 /r            [386]

\c ADD reg8,r/m8                 ; 02 /r                [8086]

\c ADD reg16,r/m16               ; o16 03 /r            [8086]

\c ADD reg32,r/m32               ; o32 03 /r            [386]

\c ADD r/m8,imm8                 ; 80 /0 ib             [8086]

\c ADD r/m16,imm16               ; o16 81 /0 iw         [8086]

\c ADD r/m32,imm32               ; o32 81 /0 id         [386]

\c ADD r/m16,imm8                ; o16 83 /0 ib         [8086]

\c ADD r/m32,imm8                ; o32 83 /0 ib         [386]

\c ADD AL,imm8                   ; 04 ib                [8086]

\c ADD AX,imm16                  ; o16 05 iw            [8086]

\c ADD EAX,imm32                 ; o32 05 id            [386]

\c{ADD} performs integer addition: it adds its two operands

together, and leaves the result in its destination (first) operand.

The flags are set according to the result of the operation: in

particular, the carry flag is affected and can be used by a

subsequent \c{ADC} instruction (\k{insADC}).

In the forms with an 8-bit immediate second operand and a longer

first operand, the second operand is considered to be signed, and is

sign-extended to the length of the first operand. In these cases,

the \c{BYTE} qualifier is necessary to force NASM to generate this

form of the instruction.

\H{insAND} \i\c{AND}: Bitwise AND

\c AND r/m8,reg8                 ; 20 /r                [8086]

\c AND r/m16,reg16               ; o16 21 /r            [8086]

\c AND r/m32,reg32               ; o32 21 /r            [386]

\c AND reg8,r/m8                 ; 22 /r                [8086]

\c AND reg16,r/m16               ; o16 23 /r            [8086]

\c AND reg32,r/m32               ; o32 23 /r            [386]

\c AND r/m8,imm8                 ; 80 /4 ib             [8086]

\c AND r/m16,imm16               ; o16 81 /4 iw         [8086]

\c AND r/m32,imm32               ; o32 81 /4 id         [386]

\c AND r/m16,imm8                ; o16 83 /4 ib         [8086]

\c AND r/m32,imm8                ; o32 83 /4 ib         [386]

\c AND AL,imm8                   ; 24 ib                [8086]

\c AND AX,imm16                  ; o16 25 iw            [8086]

\c AND EAX,imm32                 ; o32 25 id            [386]

\c{AND} performs a bitwise AND operation between its two operands

(i.e. each bit of the result is 1 if and only if the corresponding

bits of the two inputs were both 1), and stores the result in the

destination (first) operand.

In the forms with an 8-bit immediate second operand and a longer

first operand, the second operand is considered to be signed, and is

sign-extended to the length of the first operand. In these cases,

the \c{BYTE} qualifier is necessary to force NASM to generate this

form of the instruction.

The MMX instruction \c{PAND} (see \k{insPAND}) performs the same

operation on the 64-bit MMX registers.

\H{insARPL} \i\c{ARPL}: Adjust RPL Field of Selector

\c ARPL r/m16,reg16              ; 63 /r                [286,PRIV]

\c{ARPL} expects its two word operands to be segment selectors. It

adjusts the RPL (requested privilege level - stored in the bottom

two bits of the selector) field of the destination (first) operand

to ensure that it is no less (i.e. no more privileged than) the RPL

field of the source operand. The zero flag is set if and only if a

change had to be made.

\H{insBOUND} \i\c{BOUND}: Check Array Index against Bounds

\c BOUND reg16,mem               ; o16 62 /r            [186]

\c BOUND reg32,mem               ; o32 62 /r            [386]

\c{BOUND} expects its second operand to point to an area of memory

containing two signed values of the same size as its first operand

(i.e. two words for the 16-bit form; two doublewords for the 32-bit

form). It performs two signed comparisons: if the value in the

register passed as its first operand is less than the first of the

in-memory values, or is greater than or equal to the second, it

throws a BR exception. Otherwise, it does nothing.

\H{insBSF} \i\c{BSF}, \i\c{BSR}: Bit Scan

\c BSF reg16,r/m16               ; o16 0F BC /r         [386]

\c BSF reg32,r/m32               ; o32 0F BC /r         [386]

\c BSR reg16,r/m16               ; o16 0F BD /r         [386]

\c BSR reg32,r/m32               ; o32 0F BD /r         [386]

\c{BSF} searches for a set bit in its source (second) operand,

starting from the bottom, and if it finds one, stores the index in

its destination (first) operand. If no set bit is found, the

contents of the destination operand are undefined.

\c{BSR} performs the same function, but searches from the top

instead, so it finds the most significant set bit.

Bit indices are from 0 (least significant) to 15 or 31 (most

significant).

\H{insBSWAP} \i\c{BSWAP}: Byte Swap

\c BSWAP reg32                   ; o32 0F C8+r          [486]

\c{BSWAP} swaps the order of the four bytes of a 32-bit register:

bits 0-7 exchange places with bits 24-31, and bits 8-15 swap with

bits 16-23. There is no explicit 16-bit equivalent: to byte-swap

\c{AX}, \c{BX}, \c{CX} or \c{DX}, \c{XCHG} can be used.

\H{insBT} \i\c{BT}, \i\c{BTC}, \i\c{BTR}, \i\c{BTS}: Bit Test

\c BT r/m16,reg16                ; o16 0F A3 /r         [386]

\c BT r/m32,reg32                ; o32 0F A3 /r         [386]

\c BT r/m16,imm8                 ; o16 0F BA /4 ib      [386]

\c BT r/m32,imm8                 ; o32 0F BA /4 ib      [386]

\c BTC r/m16,reg16               ; o16 0F BB /r         [386]

\c BTC r/m32,reg32               ; o32 0F BB /r         [386]

\c BTC r/m16,imm8                ; o16 0F BA /7 ib      [386]

\c BTC r/m32,imm8                ; o32 0F BA /7 ib      [386]

\c BTR r/m16,reg16               ; o16 0F B3 /r         [386]

\c BTR r/m32,reg32               ; o32 0F B3 /r         [386]

\c BTR r/m16,imm8                ; o16 0F BA /6 ib      [386]

\c BTR r/m32,imm8                ; o32 0F BA /6 ib      [386]

\c BTS r/m16,reg16               ; o16 0F AB /r         [386]

\c BTS r/m32,reg32               ; o32 0F AB /r         [386]

\c BTS r/m16,imm                 ; o16 0F BA /5 ib      [386]

\c BTS r/m32,imm                 ; o32 0F BA /5 ib      [386]

These instructions all test one bit of their first operand, whose

index is given by the second operand, and store the value of that

bit into the carry flag. Bit indices are from 0 (least significant)

to 15 or 31 (most significant).

In addition to storing the original value of the bit into the carry

flag, \c{BTR} also resets (clears) the bit in the operand itself.

\c{BTS} sets the bit, and \c{BTC} complements the bit. \c{BT} does

not modify its operands.

The bit offset should be no greater than the size of the operand.

\H{insCALL} \i\c{CALL}: Call Subroutine

\c CALL imm                      ; E8 rw/rd             [8086]

\c CALL imm:imm16                ; o16 9A iw iw         [8086]

\c CALL imm:imm32                ; o32 9A id iw         [386]

\c CALL FAR mem16                ; o16 FF /3            [8086]

\c CALL FAR mem32                ; o32 FF /3            [386]

\c CALL r/m16                    ; o16 FF /2            [8086]

\c CALL r/m32                    ; o32 FF /2            [386]

\c{CALL} calls a subroutine, by means of pushing the current

instruction pointer (\c{IP}) and optionally \c{CS} as well on the

stack, and then jumping to a given address.

\c{CS} is pushed as well as \c{IP} if and only if the call is a far

call, i.e. a destination segment address is specified in the

instruction. The forms involving two colon-separated arguments are

far calls; so are the \c{CALL FAR mem} forms.

You can choose between the two immediate \i{far call} forms (\c{CALL

imm:imm}) by the use of the \c{WORD} and \c{DWORD} keywords: \c{CALL

WORD 0x1234:0x5678}) or \c{CALL DWORD 0x1234:0x56789abc}.

The \c{CALL FAR mem} forms execute a far call by loading the

destination address out of memory. The address loaded consists of 16

or 32 bits of offset (depending on the operand size), and 16 bits of

segment. The operand size may be overridden using \c{CALL WORD FAR

mem} or \c{CALL DWORD FAR mem}.

The \c{CALL r/m} forms execute a \i{near call} (within the same

segment), loading the destination address out of memory or out of a

register. The keyword \c{NEAR} may be specified, for clarity, in

these forms, but is not necessary. Again, operand size can be

overridden using \c{CALL WORD mem} or \c{CALL DWORD mem}.

As a convenience, NASM does not require you to call a far procedure

symbol by coding the cumbersome \c{CALL SEG routine:routine}, but

instead allows the easier synonym \c{CALL FAR routine}.

The \c{CALL r/m} forms given above are near calls; NASM will accept

the \c{NEAR} keyword (e.g. \c{CALL NEAR [address]}), even though it

is not strictly necessary.

\H{insCBW} \i\c{CBW}, \i\c{CWD}, \i\c{CDQ}, \i\c{CWDE}: Sign Extensions

\c CBW                           ; o16 98               [8086]

\c CWD                           ; o16 99               [8086]

\c CDQ                           ; o32 99               [386]

\c CWDE                          ; o32 98               [386]

All these instructions sign-extend a short value into a longer one,

by replicating the top bit of the original value to fill the

extended one.

\c{CBW} extends \c{AL} into \c{AX} by repeating the top bit of

\c{AL} in every bit of \c{AH}. \c{CWD} extends \c{AX} into \c{DX:AX}

by repeating the top bit of \c{AX} throughout \c{DX}. \c{CWDE}

extends \c{AX} into \c{EAX}, and \c{CDQ} extends \c{EAX} into

\c{EDX:EAX}.

\H{insCLC} \i\c{CLC}, \i\c{CLD}, \i\c{CLI}, \i\c{CLTS}: Clear Flags

\c CLC                           ; F8                   [8086]

\c CLD                           ; FC                   [8086]

\c CLI                           ; FA                   [8086]

\c CLTS                          ; 0F 06                [286,PRIV]

These instructions clear various flags. \c{CLC} clears the carry

flag; \c{CLD} clears the direction flag; \c{CLI} clears the

interrupt flag (thus disabling interrupts); and \c{CLTS} clears the

task-switched (\c{TS}) flag in \c{CR0}.

To set the carry, direction, or interrupt flags, use the \c{STC},

\c{STD} and \c{STI} instructions (\k{insSTC}). To invert the carry

flag, use \c{CMC} (\k{insCMC}).

\H{insCMC} \i\c{CMC}: Complement Carry Flag

\c CMC                           ; F5                   [8086]

\c{CMC} changes the value of the carry flag: if it was 0, it sets it

to 1, and vice versa.

\H{insCMOVcc} \i\c{CMOVcc}: Conditional Move

\c CMOVcc reg16,r/m16            ; o16 0F 40+cc /r      [P6]

\c CMOVcc reg32,r/m32            ; o32 0F 40+cc /r      [P6]

\c{CMOV} moves its source (second) operand into its destination

(first) operand if the given condition code is satisfied; otherwise

it does nothing.

For a list of condition codes, see \k{iref-cc}.

Although the \c{CMOV} instructions are flagged \c{P6} above, they

may not be supported by all Pentium Pro processors; the \c{CPUID}

instruction (\k{insCPUID}) will return a bit which indicates whether

conditional moves are supported.

\H{insCMP} \i\c{CMP}: Compare Integers

\c CMP r/m8,reg8                 ; 38 /r                [8086]

\c CMP r/m16,reg16               ; o16 39 /r            [8086]

\c CMP r/m32,reg32               ; o32 39 /r            [386]

\c CMP reg8,r/m8                 ; 3A /r                [8086]

\c CMP reg16,r/m16               ; o16 3B /r            [8086]

\c CMP reg32,r/m32               ; o32 3B /r            [386]

\c CMP r/m8,imm8                 ; 80 /0 ib             [8086]

\c CMP r/m16,imm16               ; o16 81 /0 iw         [8086]

\c CMP r/m32,imm32               ; o32 81 /0 id         [386]

\c CMP r/m16,imm8                ; o16 83 /0 ib         [8086]

\c CMP r/m32,imm8                ; o32 83 /0 ib         [386]

\c CMP AL,imm8                   ; 3C ib                [8086]

\c CMP AX,imm16                  ; o16 3D iw            [8086]

\c CMP EAX,imm32                 ; o32 3D id            [386]

\c{CMP} performs a `mental' subtraction of its second operand from

its first operand, and affects the flags as if the subtraction had

taken place, but does not store the result of the subtraction

anywhere.

In the forms with an 8-bit immediate second operand and a longer

first operand, the second operand is considered to be signed, and is

sign-extended to the length of the first operand. In these cases,

the \c{BYTE} qualifier is necessary to force NASM to generate this

form of the instruction.

\H{insCMPSB} \i\c{CMPSB}, \i\c{CMPSW}, \i\c{CMPSD}: Compare Strings

\c CMPSB                         ; A6                   [8086]

\c CMPSW                         ; o16 A7               [8086]

\c CMPSD                         ; o32 A7               [386]

\c{CMPSB} compares the byte at \c{[DS:SI]} or \c{[DS:ESI]} with the

byte at \c{[ES:DI]} or \c{[ES:EDI]}, and sets the flags accordingly.

It then increments or decrements (depending on the direction flag:

increments if the flag is clear, decrements if it is set) \c{SI} and

\c{DI} (or \c{ESI} and \c{EDI}).

The registers used are \c{SI} and \c{DI} if the address size is 16

bits, and \c{ESI} and \c{EDI} if it is 32 bits. If you need to use

an address size not equal to the current \c{BITS} setting, you can

use an explicit \i\c{a16} or \i\c{a32} prefix.

The segment register used to load from \c{[SI]} or \c{[ESI]} can be

overridden by using a segment register name as a prefix (for

example, \c{es cmpsb}). The use of \c{ES} for the load from \c{[DI]}

or \c{[EDI]} cannot be overridden.

\c{CMPSW} and \c{CMPSD} work in the same way, but they compare a

word or a doubleword instead of a byte, and increment or decrement

the addressing registers by 2 or 4 instead of 1.

The \c{REPE} and \c{REPNE} prefixes (equivalently, \c{REPZ} and

\c{REPNZ}) may be used to repeat the instruction up to \c{CX} (or

\c{ECX} - again, the address size chooses which) times until the

first unequal or equal byte is found.

\H{insCMPXCHG} \i\c{CMPXCHG}, \i\c{CMPXCHG486}: Compare and Exchange

\c CMPXCHG r/m8,reg8             ; 0F B0 /r             [PENT]

\c CMPXCHG r/m16,reg16           ; o16 0F B1 /r         [PENT]

\c CMPXCHG r/m32,reg32           ; o32 0F B1 /r         [PENT]

\c CMPXCHG486 r/m8,reg8          ; 0F A6 /r             [486,UNDOC]

\c CMPXCHG486 r/m16,reg16        ; o16 0F A7 /r         [486,UNDOC]

\c CMPXCHG486 r/m32,reg32        ; o32 0F A7 /r         [486,UNDOC]

These two instructions perform exactly the same operation; however,

apparently some (not all) 486 processors support it under a

non-standard opcode, so NASM provides the undocumented

\c{CMPXCHG486} form to generate the non-standard opcode.

\c{CMPXCHG} compares its destination (first) operand to the value in

\c{AL}, \c{AX} or \c{EAX} (depending on the size of the

instruction). If they are equal, it copies its source (second)

operand into the destination and sets the zero flag. Otherwise, it

clears the zero flag and leaves the destination alone.

\c{CMPXCHG} is intended to be used for atomic operations in

multitasking or multiprocessor environments. To safely update a

value in shared memory, for example, you might load the value into

\c{EAX}, load the updated value into \c{EBX}, and then execute the

instruction \c{lock cmpxchg [value],ebx}. If \c{value} has not

changed since being loaded, it is updated with your desired new

value, and the zero flag is set to let you know it has worked. (The

\c{LOCK} prefix prevents another processor doing anything in the

middle of this operation: it guarantees atomicity.) However, if

another processor has modified the value in between your load and

your attempted store, the store does not happen, and you are

notified of the failure by a cleared zero flag, so you can go round

and try again.

\H{insCMPXCHG8B} \i\c{CMPXCHG8B}: Compare and Exchange Eight Bytes

\c CMPXCHG8B mem                 ; 0F C7 /1             [PENT]

This is a larger and more unwieldy version of \c{CMPXCHG}: it

compares the 64-bit (eight-byte) value stored at \c{[mem]} with the

value in \c{EDX:EAX}. If they are equal, it sets the zero flag and

stores \c{ECX:EBX} into the memory area. If they are unequal, it

clears the zero flag and leaves the memory area untouched.

\H{insCPUID} \i\c{CPUID}: Get CPU Identification Code

\c CPUID                         ; 0F A2                [PENT]

\c{CPUID} returns various information about the processor it is

being executed on. It fills the four registers \c{EAX}, \c{EBX},

\c{ECX} and \c{EDX} with information, which varies depending on the

input contents of \c{EAX}.

\c{CPUID} also acts as a barrier to serialise instruction execution:

executing the \c{CPUID} instruction guarantees that all the effects

(memory modification, flag modification, register modification) of

previous instructions have been completed before the next

instruction gets fetched.

The information returned is as follows:

\b If \c{EAX} is zero on input, \c{EAX} on output holds the maximum

acceptable input value of \c{EAX}, and \c{EBX:EDX:ECX} contain the

string \c{"GenuineIntel"} (or not, if you have a clone processor).

That is to say, \c{EBX} contains \c{"Genu"} (in NASM's own sense of

character constants, described in \k{chrconst}), \c{EDX} contains

\c{"ineI"} and \c{ECX} contains \c{"ntel"}.

\b If \c{EAX} is one on input, \c{EAX} on output contains version

information about the processor, and \c{EDX} contains a set of

feature flags, showing the presence and absence of various features.

For example, bit 8 is set if the \c{CMPXCHG8B} instruction

(\k{insCMPXCHG8B}) is supported, bit 15 is set if the conditional

move instructions (\k{insCMOVcc} and \k{insFCMOVB}) are supported,

and bit 23 is set if MMX instructions are supported.

\b If \c{EAX} is two on input, \c{EAX}, \c{EBX}, \c{ECX} and \c{EDX}

all contain information about caches and TLBs (Translation Lookahead

Buffers).

For more information on the data returned from \c{CPUID}, see the

documentation on Intel's web site.

\H{insDAA} \i\c{DAA}, \i\c{DAS}: Decimal Adjustments

\c DAA                           ; 27                   [8086]

\c DAS                           ; 2F                   [8086]

These instructions are used in conjunction with the add and subtract

instructions to perform binary-coded decimal arithmetic in

\e{packed} (one BCD digit per nibble) form. For the unpacked

equivalents, see \k{insAAA}.

\c{DAA} should be used after a one-byte \c{ADD} instruction whose

destination was the \c{AL} register: by means of examining the value

in the \c{AL} and also the auxiliary carry flag \c{AF}, it

determines whether either digit of the addition has overflowed, and

adjusts it (and sets the carry and auxiliary-carry flags) if so. You

can add long BCD strings together by doing \c{ADD}/\c{DAA} on the

low two digits, then doing \c{ADC}/\c{DAA} on each subsequent pair

of digits.

\c{DAS} works similarly to \c{DAA}, but is for use after \c{SUB}

instructions rather than \c{ADD}.

\H{insDEC} \i\c{DEC}: Decrement Integer

\c DEC reg16                     ; o16 48+r             [8086]

\c DEC reg32                     ; o32 48+r             [386]

\c DEC r/m8                      ; FE /1                [8086]

\c DEC r/m16                     ; o16 FF /1            [8086]

\c DEC r/m32                     ; o32 FF /1            [386]

\c{DEC} subtracts 1 from its operand. It does \e{not} affect the

carry flag: to affect the carry flag, use \c{SUB something,1} (see

\k{insSUB}). See also \c{INC} (\k{insINC}).

\H{insDIV} \i\c{DIV}: Unsigned Integer Divide

\c DIV r/m8                      ; F6 /6                [8086]

\c DIV r/m16                     ; o16 F7 /6            [8086]

\c DIV r/m32                     ; o32 F7 /6            [386]

\c{DIV} performs unsigned integer division. The explicit operand

provided is the divisor; the dividend and destination operands are

implicit, in the following way:

\b For \c{DIV r/m8}, \c{AX} is divided by the given operand; the

quotient is stored in \c{AL} and the remainder in \c{AH}.

\b For \c{DIV r/m16}, \c{DX:AX} is divided by the given operand; the

quotient is stored in \c{AX} and the remainder in \c{DX}.

\b For \c{DIV r/m32}, \c{EDX:EAX} is divided by the given operand;

the quotient is stored in \c{EAX} and the remainder in \c{EDX}.

Signed integer division is performed by the \c{IDIV} instruction:

see \k{insIDIV}.

\H{insEMMS} \i\c{EMMS}: Empty MMX State

\c EMMS                          ; 0F 77                [PENT,MMX]

\c{EMMS} sets the FPU tag word (marking which floating-point

registers are available) to all ones, meaning all registers are

available for the FPU to use. It should be used after executing MMX

instructions and before executing any subsequent floating-point

operations.

\H{insENTER} \i\c{ENTER}: Create Stack Frame

\c ENTER imm,imm                 ; C8 iw ib             [186]

\c{ENTER} constructs a stack frame for a high-level language

procedure call. The first operand (the \c{iw} in the opcode

definition above refers to the first operand) gives the amount of

stack space to allocate for local variables; the second (the \c{ib}

above) gives the nesting level of the procedure (for languages like

Pascal, with nested procedures).

The function of \c{ENTER}, with a nesting level of zero, is

equivalent to

\c           PUSH EBP            ; or PUSH BP         in 16 bits

\c           MOV EBP,ESP         ; or MOV BP,SP       in 16 bits

\c           SUB ESP,operand1    ; or SUB SP,operand1 in 16 bits

This creates a stack frame with the procedure parameters accessible

upwards from \c{EBP}, and local variables accessible downwards from

\c{EBP}.

With a nesting level of one, the stack frame created is 4 (or 2)

bytes bigger, and the value of the final frame pointer \c{EBP} is

accessible in memory at \c{[EBP-4]}.

This allows \c{ENTER}, when called with a nesting level of two, to

look at the stack frame described by the \e{previous} value of

\c{EBP}, find the frame pointer at offset -4 from that, and push it

along with its new frame pointer, so that when a level-two procedure

is called from within a level-one procedure, \c{[EBP-4]} holds the

frame pointer of the most recent level-one procedure call and

\c{[EBP-8]} holds that of the most recent level-two call. And so on,

for nesting levels up to 31.

Stack frames created by \c{ENTER} can be destroyed by the \c{LEAVE}

instruction: see \k{insLEAVE}.

\H{insF2XM1} \i\c{F2XM1}: Calculate 2**X-1

\c F2XM1                         ; D9 F0                [8086,FPU]

\c{F2XM1} raises 2 to the power of \c{ST0}, subtracts one, and

stores the result back into \c{ST0}. The initial contents of \c{ST0}

must be a number in the range -1 to +1.

\H{insFABS} \i\c{FABS}: Floating-Point Absolute Value

\c FABS                          ; D9 E1                [8086,FPU]

\c{FABS} computes the absolute value of \c{ST0}, storing the result

back in \c{ST0}.

\H{insFADD} \i\c{FADD}, \i\c{FADDP}: Floating-Point Addition

\c FADD mem32                    ; D8 /0                [8086,FPU]

\c FADD mem64                    ; DC /0                [8086,FPU]

\c FADD fpureg                   ; D8 C0+r              [8086,FPU]

\c FADD ST0,fpureg               ; D8 C0+r              [8086,FPU]

\c FADD TO fpureg                ; DC C0+r              [8086,FPU]

\c FADD fpureg,ST0               ; DC C0+r              [8086,FPU]

\c FADDP fpureg                  ; DE C0+r              [8086,FPU]

\c FADDP fpureg,ST0              ; DE C0+r              [8086,FPU]

\c{FADD}, given one operand, adds the operand to \c{ST0} and stores

the result back in \c{ST0}. If the operand has the \c{TO} modifier,

the result is stored in the register given rather than in \c{ST0}.

\c{FADDP} performs the same function as \c{FADD TO}, but pops the

register stack after storing the result.

The given two-operand forms are synonyms for the one-operand forms.

\H{insFBLD} \i\c{FBLD}, \i\c{FBSTP}: BCD Floating-Point Load and Store

\c FBLD mem80                    ; DF /4                [8086,FPU]

\c FBSTP mem80                   ; DF /6                [8086,FPU]

\c{FBLD} loads an 80-bit (ten-byte) packed binary-coded decimal

number from the given memory address, converts it to a real, and

pushes it on the register stack. \c{FBSTP} stores the value of

\c{ST0}, in packed BCD, at the given address and then pops the

register stack.

\H{insFCHS} \i\c{FCHS}: Floating-Point Change Sign

\c FCHS                          ; D9 E0                [8086,FPU]

\c{FCHS} negates the number in \c{ST0}: negative numbers become

positive, and vice versa.

\H{insFCLEX} \i\c{FCLEX}, \{FNCLEX}: Clear Floating-Point Exceptions

\c FCLEX                         ; 9B DB E2             [8086,FPU]

\c FNCLEX                        ; DB E2                [8086,FPU]

\c{FCLEX} clears any floating-point exceptions which may be pending.

\c{FNCLEX} does the same thing but doesn't wait for previous

floating-point operations (including the \e{handling} of pending

exceptions) to finish first.

\H{insFCMOVB} \i\c{FCMOVcc}: Floating-Point Conditional Move

\c FCMOVB fpureg                 ; DA C0+r              [P6,FPU]

\c FCMOVB ST0,fpureg             ; DA C0+r              [P6,FPU]

\c FCMOVBE fpureg                ; DA D0+r              [P6,FPU]

\c FCMOVBE ST0,fpureg            ; DA D0+r              [P6,FPU]

\c FCMOVE fpureg                 ; DA C8+r              [P6,FPU]

\c FCMOVE ST0,fpureg             ; DA C8+r              [P6,FPU]

\c FCMOVNB fpureg                ; DB C0+r              [P6,FPU]

\c FCMOVNB ST0,fpureg            ; DB C0+r              [P6,FPU]

\c FCMOVNBE fpureg               ; DB D0+r              [P6,FPU]

\c FCMOVNBE ST0,fpureg           ; DB D0+r              [P6,FPU]

\c FCMOVNE fpureg                ; DB C8+r              [P6,FPU]

\c FCMOVNE ST0,fpureg            ; DB C8+r              [P6,FPU]

\c FCMOVNU fpureg                ; DB D8+r              [P6,FPU]

\c FCMOVNU ST0,fpureg            ; DB D8+r              [P6,FPU]

\c FCMOVU fpureg                 ; DA D8+r              [P6,FPU]

\c FCMOVU ST0,fpureg             ; DA D8+r              [P6,FPU]

The \c{FCMOV} instructions perform conditional move operations: each

of them moves the contents of the given register into \c{ST0} if its

condition is satisfied, and does nothing if not.

The conditions are not the same as the standard condition codes used

with conditional jump instructions. The conditions \c{B}, \c{BE},

\c{NB}, \c{NBE}, \c{E} and \c{NE} are exactly as normal, but none of

the other standard ones are supported. Instead, the condition \c{U}

and its counterpart \c{NU} are provided; the \c{U} condition is

satisfied if the last two floating-point numbers compared were

\e{unordered}, i.e. they were not equal but neither one could be

said to be greater than the other, for example if they were NaNs.

(The flag state which signals this is the setting of the parity

flag: so the \c{U} condition is notionally equivalent to \c{PE}, and

\c{NU} is equivalent to \c{PO}.)

The \c{FCMOV} conditions test the main processor's status flags, not

the FPU status flags, so using \c{FCMOV} directly after \c{FCOM}

will not work. Instead, you should either use \c{FCOMI} which writes

directly to the main CPU flags word, or use \c{FSTSW} to extract the

FPU flags.

Although the \c{FCMOV} instructions are flagged \c{P6} above, they

may not be supported by all Pentium Pro processors; the \c{CPUID}

instruction (\k{insCPUID}) will return a bit which indicates whether

conditional moves are supported.

\H{insFCOM} \i\c{FCOM}, \i\c{FCOMP}, \i\c{FCOMPP}, \i\c{FCOMI}, \i\c{FCOMIP}: Floating-Point Compare

\c FCOM mem32                    ; D8 /2                [8086,FPU]

\c FCOM mem64                    ; DC /2                [8086,FPU]

\c FCOM fpureg                   ; D8 D0+r              [8086,FPU]

\c FCOM ST0,fpureg               ; D8 D0+r              [8086,FPU]

\c FCOMP mem32                   ; D8 /3                [8086,FPU]

\c FCOMP mem64                   ; DC /3                [8086,FPU]

\c FCOMP fpureg                  ; D8 D8+r              [8086,FPU]

\c FCOMP ST0,fpureg              ; D8 D8+r              [8086,FPU]

\c FCOMPP                        ; DE D9                [8086,FPU]

\c FCOMI fpureg                  ; DB F0+r              [P6,FPU]

\c FCOMI ST0,fpureg              ; DB F0+r              [P6,FPU]

\c FCOMIP fpureg                 ; DF F0+r              [P6,FPU]

\c FCOMIP ST0,fpureg             ; DF F0+r              [P6,FPU]

\c{FCOM} compares \c{ST0} with the given operand, and sets the FPU

flags accordingly. \c{ST0} is treated as the left-hand side of the

comparison, so that the carry flag is set (for a `less-than' result)

if \c{ST0} is less than the given operand.

\c{FCOMP} does the same as \c{FCOM}, but pops the register stack

afterwards. \c{FCOMPP} compares \c{ST0} with \c{ST1} and then pops

the register stack twice.

\c{FCOMI} and \c{FCOMIP} work like the corresponding forms of

\c{FCOM} and \c{FCOMP}, but write their results directly to the CPU

flags register rather than the FPU status word, so they can be

immediately followed by conditional jump or conditional move

instructions.

The \c{FCOM} instructions differ from the \c{FUCOM} instructions

(\k{insFUCOM}) only in the way they handle quiet NaNs: \c{FUCOM}

will handle them silently and set the condition code flags to an

`unordered' result, whereas \c{FCOM} will generate an exception.

\H{insFCOS} \i\c{FCOS}: Cosine

\c FCOS                          ; D9 FF                [386,FPU]

\c{FCOS} computes the cosine of \c{ST0} (in radians), and stores the

result in \c{ST0}. See also \c{FSINCOS} (\k{insFSIN}).

\H{insFDECSTP} \i\c{FDECSTP}: Decrement Floating-Point Stack Pointer

\c FDECSTP                       ; D9 F6                [8086,FPU]

\c{FDECSTP} decrements the `top' field in the floating-point status

word. This has the effect of rotating the FPU register stack by one,

as if the contents of \c{ST7} had been pushed on the stack. See also

\c{FINCSTP} (\k{insFINCSTP}).

\H{insFDISI} \i\c{FxDISI}, \i\c{FxENI}: Disable and Enable Floating-Point Interrupts

\c FDISI                         ; 9B DB E1             [8086,FPU]

\c FNDISI                        ; DB E1                [8086,FPU]

\c FENI                          ; 9B DB E0             [8086,FPU]

\c FNENI                         ; DB E0                [8086,FPU]

\c{FDISI} and \c{FENI} disable and enable floating-point interrupts.

These instructions are only meaningful on original 8087 processors:

the 287 and above treat them as no-operation instructions.

\c{FNDISI} and \c{FNENI} do the same thing as \c{FDISI} and \c{FENI}

respectively, but without waiting for the floating-point processor

to finish what it was doing first.

\H{insFDIV} \i\c{FDIV}, \i\c{FDIVP}, \i\c{FDIVR}, \i\c{FDIVRP}: Floating-Point Division

\c FDIV mem32                    ; D8 /6                [8086,FPU]

\c FDIV mem64                    ; DC /6                [8086,FPU]

\c FDIV fpureg                   ; D8 F0+r              [8086,FPU]

\c FDIV ST0,fpureg               ; D8 F0+r              [8086,FPU]

\c FDIV TO fpureg                ; DC F8+r              [8086,FPU]

\c FDIV fpureg,ST0               ; DC F8+r              [8086,FPU]

\c FDIVR mem32                   ; D8 /0                [8086,FPU]

\c FDIVR mem64                   ; DC /0                [8086,FPU]

\c FDIVR fpureg                  ; D8 F8+r              [8086,FPU]

\c FDIVR ST0,fpureg              ; D8 F8+r              [8086,FPU]

\c FDIVR TO fpureg               ; DC F0+r              [8086,FPU]

\c FDIVR fpureg,ST0              ; DC F0+r              [8086,FPU]

\c FDIVP fpureg                  ; DE F8+r              [8086,FPU]

\c FDIVP fpureg,ST0              ; DE F8+r              [8086,FPU]

\c FDIVRP fpureg                 ; DE F0+r              [8086,FPU]

\c FDIVRP fpureg,ST0             ; DE F0+r              [8086,FPU]

\c{FDIV} divides \c{ST0} by the given operand and stores the result

back in \c{ST0}, unless the \c{TO} qualifier is given, in which case

it divides the given operand by \c{ST0} and stores the result in the

operand.

\c{FDIVR} does the same thing, but does the division the other way

up: so if \c{TO} is not given, it divides the given operand by

\c{ST0} and stores the result in \c{ST0}, whereas if \c{TO} is given

it divides \c{ST0} by its operand and stores the result in the

operand.

\c{FDIVP} operates like \c{FDIV TO}, but pops the register stack

once it has finished. \c{FDIVRP} operates like \c{FDIVR TO}, but

pops the register stack once it has finished.

\H{insFFREE} \i\c{FFREE}: Flag Floating-Point Register as Unused

\c FFREE fpureg                  ; DD C0+r              [8086,FPU]

\c{FFREE} marks the given register as being empty.

\H{insFIADD} \i\c{FIADD}: Floating-Point/Integer Addition

\c FIADD mem16                   ; DE /0                [8086,FPU]

\c FIADD mem32                   ; DA /0                [8086,FPU]

\c{FIADD} adds the 16-bit or 32-bit integer stored in the given

memory location to \c{ST0}, storing the result in \c{ST0}.

\H{insFICOM} \i\c{FICOM}, \i\c{FICOMP}: Floating-Point/Integer Compare

\c FICOM mem16                   ; DE /2                [8086,FPU]

\c FICOM mem32                   ; DA /2                [8086,FPU]

\c FICOMP mem16                  ; DE /3                [8086,FPU]

\c FICOMP mem32                  ; DA /3                [8086,FPU]

\c{FICOM} compares \c{ST0} with the 16-bit or 32-bit integer stored

in the given memory location, and sets the FPU flags accordingly.

\c{FICOMP} does the same, but pops the register stack afterwards.

\H{insFIDIV} \i\c{FIDIV}, \i\c{FIDIVR}: Floating-Point/Integer Division

\c FIDIV mem16                   ; DE /6                [8086,FPU]

\c FIDIV mem32                   ; DA /6                [8086,FPU]

\c FIDIVR mem16                  ; DE /0                [8086,FPU]

\c FIDIVR mem32                  ; DA /0                [8086,FPU]

\c{FIDIV} divides \c{ST0} by the 16-bit or 32-bit integer stored in

the given memory location, and stores the result in \c{ST0}.

\c{FIDIVR} does the division the other way up: it divides the

integer by \c{ST0}, but still stores the result in \c{ST0}.

\H{insFILD} \i\c{FILD}, \i\c{FIST}, \i\c{FISTP}: Floating-Point/Integer Conversion

\c FILD mem16                    ; DF /0                [8086,FPU]

\c FILD mem32                    ; DB /0                [8086,FPU]

\c FILD mem64                    ; DF /5                [8086,FPU]

\c FIST mem16                    ; DF /2                [8086,FPU]

\c FIST mem32                    ; DB /2                [8086,FPU]

\c FISTP mem16                   ; DF /3                [8086,FPU]

\c FISTP mem32                   ; DB /3                [8086,FPU]

\c FISTP mem64                   ; DF /0                [8086,FPU]

\c{FILD} loads an integer out of a memory location, converts it to a

real, and pushes it on the FPU register stack. \c{FIST} converts

\c{ST0} to an integer and stores that in memory; \c{FISTP} does the

same as \c{FIST}, but pops the register stack afterwards.

\H{insFIMUL} \i\c{FIMUL}: Floating-Point/Integer Multiplication

\c FIMUL mem16                   ; DE /1                [8086,FPU]

\c FIMUL mem32                   ; DA /1                [8086,FPU]

\c{FIMUL} multiplies \c{ST0} by the 16-bit or 32-bit integer stored

in the given memory location, and stores the result in \c{ST0}.

\H{insFINCSTP} \i\c{FINCSTP}: Increment Floating-Point Stack Pointer

\c FINCSTP                       ; D9 F7                [8086,FPU]

\c{FINCSTP} increments the `top' field in the floating-point status

word. This has the effect of rotating the FPU register stack by one,

as if the register stack had been popped; however, unlike the

popping of the stack performed by many FPU instructions, it does not

flag the new \c{ST7} (previously \c{ST0}) as empty. See also

\c{FDECSTP} (\k{insFDECSTP}).

\H{insFINIT} \i\c{FINIT}, \i\c{FNINIT}: Initialise Floating-Point Unit

\c FINIT                         ; 9B DB E3             [8086,FPU]

\c FNINIT                        ; DB E3                [8086,FPU]

\c{FINIT} initialises the FPU to its default state. It flags all

registers as empty, though it does not actually change their values.

\c{FNINIT} does the same, without first waiting for pending

exceptions to clear.

\H{insFISUB} \i\c{FISUB}: Floating-Point/Integer Subtraction

\c FISUB mem16                   ; DE /4                [8086,FPU]

\c FISUB mem32                   ; DA /4                [8086,FPU]

\c FISUBR mem16                  ; DE /5                [8086,FPU]

\c FISUBR mem32                  ; DA /5                [8086,FPU]

\c{FISUB} subtracts the 16-bit or 32-bit integer stored in the given

memory location from \c{ST0}, and stores the result in \c{ST0}.

\c{FISUBR} does the subtraction the other way round, i.e. it

subtracts \c{ST0} from the given integer, but still stores the

result in \c{ST0}.

\H{insFLD} \i\c{FLD}: Floating-Point Load

\c FLD mem32                     ; D9 /0                [8086,FPU]

\c FLD mem64                     ; DD /0                [8086,FPU]

\c FLD mem80                     ; DB /5                [8086,FPU]

\c FLD fpureg                    ; D9 C0+r              [8086,FPU]

\c{FLD} loads a floating-point value out of the given register or

memory location, and pushes it on the FPU register stack.

\H{insFLD1} \i\c{FLDxx}: Floating-Point Load Constants

\c FLD1                          ; D9 E8                [8086,FPU]

\c FLDL2E                        ; D9 EA                [8086,FPU]

\c FLDL2T                        ; D9 E9                [8086,FPU]

\c FLDLG2                        ; D9 EC                [8086,FPU]

\c FLDLN2                        ; D9 ED                [8086,FPU]

\c FLDPI                         ; D9 EB                [8086,FPU]

\c FLDZ                          ; D9 EE                [8086,FPU]

These instructions push specific standard constants on the FPU

register stack. \c{FLD1} pushes the value 1; \c{FLDL2E} pushes the

base-2 logarithm of e; \c{FLDL2T} pushes the base-2 log of 10;

\c{FLDLG2} pushes the base-10 log of 2; \c{FLDLN2} pushes the base-e

log of 2; \c{FLDPI} pushes pi; and \c{FLDZ} pushes zero.

\H{insFLDCW} \i\c{FLDCW}: Load Floating-Point Control Word

\c FLDCW mem16                   ; D9 /5                [8086,FPU]

\c{FLDCW} loads a 16-bit value out of memory and stores it into the

FPU control word (governing things like the rounding mode, the

precision, and the exception masks). See also \c{FSTCW}

(\k{insFSTCW}).

\H{insFLDENV} \i\c{FLDENV}: Load Floating-Point Environment

\c FLDENV mem                    ; D9 /4                [8086,FPU]

\c{FLDENV} loads the FPU operating environment (control word, status

word, tag word, instruction pointer, data pointer and last opcode)

from memory. The memory area is 14 or 28 bytes long, depending on

the CPU mode at the time. See also \c{FSTENV} (\k{insFSTENV}).

\H{insFMUL} \i\c{FMUL}, \i\c{FMULP}: Floating-Point Multiply

\c FMUL mem32                    ; D8 /1                [8086,FPU]

\c FMUL mem64                    ; DC /1                [8086,FPU]

\c FMUL fpureg                   ; D8 C8+r              [8086,FPU]

\c FMUL ST0,fpureg               ; D8 C8+r              [8086,FPU]

\c FMUL TO fpureg                ; DC C8+r              [8086,FPU]

\c FMUL fpureg,ST0               ; DC C8+r              [8086,FPU]

\c FMULP fpureg                  ; DE C8+r              [8086,FPU]

\c FMULP fpureg,ST0              ; DE C8+r              [8086,FPU]

\c{FMUL} multiplies \c{ST0} by the given operand, and stores the

result in \c{ST0}, unless the \c{TO} qualifier is used in which case

it stores the result in the operand. \c{FMULP} performs the same

operation as \c{FMUL TO}, and then pops the register stack.

\H{insFNOP} \i\c{FNOP}: Floating-Point No Operation

\c FNOP                          ; D9 D0                [8086,FPU]

\c{FNOP} does nothing.

\H{insFPATAN} \i\c{FPATAN}, \i\c{FPTAN}: Arctangent and Tangent

\c FPATAN                        ; D9 F3                [8086,FPU]

\c FPTAN                         ; D9 F2                [8086,FPU]

\c{FPATAN} computes the arctangent, in radians, of the result of

dividing \c{ST1} by \c{ST0}, stores the result in \c{ST1}, and pops

the register stack. It works like the C \c{atan2} function, in that

changing the sign of both \c{ST0} and \c{ST1} changes the output

value by pi (so it performs true rectangular-to-polar coordinate

conversion, with \c{ST1} being the Y coordinate and \c{ST0} being

the X coordinate, not merely an arctangent).

\c{FPTAN} computes the tangent of the value in \c{ST0} (in radians),

and stores the result back into \c{ST0}.

\H{insFPREM} \i\c{FPREM}, \i\c{FPREM1}: Floating-Point Partial Remainder

\c FPREM                         ; D9 F8                [8086,FPU]

\c FPREM1                        ; D9 F5                [386,FPU]

These instructions both produce the remainder obtained by dividing

\c{ST0} by \c{ST1}. This is calculated, notionally, by dividing

\c{ST0} by \c{ST1}, rounding the result to an integer, multiplying

by \c{ST1} again, and computing the value which would need to be

added back on to the result to get back to the original value in

\c{ST0}.

The two instructions differ in the way the notional round-to-integer

operation is performed. \c{FPREM} does it by rounding towards zero,

so that the remainder it returns always has the same sign as the

original value in \c{ST0}; \c{FPREM1} does it by rounding to the

nearest integer, so that the remainder always has at most half the

magnitude of \c{ST1}.

Both instructions calculate \e{partial} remainders, meaning that

they may not manage to provide the final result, but might leave

intermediate results in \c{ST0} instead. If this happens, they will

set the C2 flag in the FPU status word; therefore, to calculate a

remainder, you should repeatedly execute \c{FPREM} or \c{FPREM1}

until C2 becomes clear.

\H{insFRNDINT} \i\c{FRNDINT}: Floating-Point Round to Integer

\c FRNDINT                       ; D9 FC                [8086,FPU]

\c{FRNDINT} rounds the contents of \c{ST0} to an integer, according

to the current rounding mode set in the FPU control word, and stores

the result back in \c{ST0}.

\H{insFRSTOR} \i\c{FSAVE}, \i\c{FRSTOR}: Save/Restore Floating-Point State

\c FSAVE mem                     ; 9B DD /6             [8086,FPU]

\c FNSAVE mem                    ; DD /6                [8086,FPU]

\c FRSTOR mem                    ; DD /4                [8086,FPU]

\c{FSAVE} saves the entire floating-point unit state, including all

the information saved by \c{FSTENV} (\k{insFSTENV}) plus the

contents of all the registers, to a 94 or 108 byte area of memory

(depending on the CPU mode). \c{FRSTOR} restores the floating-point

state from the same area of memory.

\c{FNSAVE} does the same as \c{FSAVE}, without first waiting for

pending floating-point exceptions to clear.

\H{insFSCALE} \i\c{FSCALE}: Scale Floating-Point Value by Power of Two

\c FSCALE                        ; D9 FD                [8086,FPU]

\c{FSCALE} scales a number by a power of two: it rounds \c{ST1}

towards zero to obtain an integer, then multiplies \c{ST0} by two to

the power of that integer, and stores the result in \c{ST0}.

\H{insFSETPM} \i\c{FSETPM}: Set Protected Mode

\c FSETPM                        ; DB E4                [286,FPU]

This instruction initalises protected mode on the 287 floating-point

coprocessor. It is only meaningful on that processor: the 387 and

above treat the instruction as a no-operation.

\H{insFSIN} \i\c{FSIN}, \i\c{FSINCOS}: Sine and Cosine

\c FSIN                          ; D9 FE                [386,FPU]

\c FSINCOS                       ; D9 FB                [386,FPU]

\c{FSIN} calculates the sine of \c{ST0} (in radians) and stores the

result in \c{ST0}. \c{FSINCOS} does the same, but then pushes the

cosine of the same value on the register stack, so that the sine

ends up in \c{ST1} and the cosine in \c{ST0}. \c{FSINCOS} is faster

than executing \c{FSIN} and \c{FCOS} (see \k{insFCOS}) in

succession.

\H{insFSQRT} \i\c{FSQRT}: Floating-Point Square Root

\c FSQRT                         ; D9 FA                [8086,FPU]

\c{FSQRT} calculates the square root of \c{ST0} and stores the

result in \c{ST0}.

\H{insFST} \i\c{FST}, \i\c{FSTP}: Floating-Point Store

\c FST mem32                     ; D9 /2                [8086,FPU]

\c FST mem64                     ; DD /2                [8086,FPU]

\c FST fpureg                    ; DD D0+r              [8086,FPU]

\c FSTP mem32                    ; D9 /3                [8086,FPU]

\c FSTP mem64                    ; DD /3                [8086,FPU]

\c FSTP mem80                    ; DB /0                [8086,FPU]

\c FSTP fpureg                   ; DD D8+r              [8086,FPU]

\c{FST} stores the value in \c{ST0} into the given memory location

or other FPU register. \c{FSTP} does the same, but then pops the

register stack.

\H{insFSTCW} \i\c{FSTCW}: Store Floating-Point Control Word

\c FSTCW mem16                   ; 9B D9 /0             [8086,FPU]

\c FNSTCW mem16                  ; D9 /0                [8086,FPU]

\c{FSTCW} stores the FPU control word (governing things like the

rounding mode, the precision, and the exception masks) into a 2-byte

memory area. See also \c{FLDCW} (\k{insFLDCW}).

\c{FNSTCW} does the same thing as \c{FSTCW}, without first waiting

for pending floating-point exceptions to clear.

\H{insFSTENV} \i\c{FSTENV}: Store Floating-Point Environment

\c FSTENV mem                    ; 9B D9 /6             [8086,FPU]

\c FNSTENV mem                   ; D9 /6                [8086,FPU]

\c{FSTENV} stores the FPU operating environment (control word,

status word, tag word, instruction pointer, data pointer and last

opcode) into memory. The memory area is 14 or 28 bytes long,

depending on the CPU mode at the time. See also \c{FLDENV}

(\k{insFLDENV}).

\c{FNSTENV} does the same thing as \c{FSTENV}, without first waiting

for pending floating-point exceptions to clear.

\H{insFSTSW} \i\c{FSTSW}: Store Floating-Point Status Word

\c FSTSW mem16                   ; 9B DD /0             [8086,FPU]

\c FSTSW AX                      ; 9B DF E0             [286,FPU]

\c FNSTSW mem16                  ; DD /0                [8086,FPU]

\c FNSTSW AX                     ; DF E0                [286,FPU]

\c{FSTSW} stores the FPU status word into \c{AX} or into a 2-byte

memory area.

\c{FNSTSW} does the same thing as \c{FSTSW}, without first waiting

for pending floating-point exceptions to clear.

\H{insFSUB} \i\c{FSUB}, \i\c{FSUBP}, \i\c{FSUBR}, \i\c{FSUBRP}: Floating-Point Subtract

\c FSUB mem32                    ; D8 /4                [8086,FPU]

\c FSUB mem64                    ; DC /4                [8086,FPU]

\c FSUB fpureg                   ; D8 E0+r              [8086,FPU]

\c FSUB ST0,fpureg               ; D8 E0+r              [8086,FPU]

\c FSUB TO fpureg                ; DC E8+r              [8086,FPU]

\c FSUB fpureg,ST0               ; DC E8+r              [8086,FPU]

\c FSUBR mem32                   ; D8 /5                [8086,FPU]

\c FSUBR mem64                   ; DC /5                [8086,FPU]

\c FSUBR fpureg                  ; D8 E8+r              [8086,FPU]

\c FSUBR ST0,fpureg              ; D8 E8+r              [8086,FPU]

\c FSUBR TO fpureg               ; DC E0+r              [8086,FPU]

\c FSUBR fpureg,ST0              ; DC E0+r              [8086,FPU]

\c FSUBP fpureg                  ; DE E8+r              [8086,FPU]

\c FSUBP fpureg,ST0              ; DE E8+r              [8086,FPU]

\c FSUBRP fpureg                 ; DE E0+r              [8086,FPU]

\c FSUBRP fpureg,ST0             ; DE E0+r              [8086,FPU]

\c{FSUB} subtracts the given operand from \c{ST0} and stores the

result back in \c{ST0}, unless the \c{TO} qualifier is given, in

which case it subtracts \c{ST0} from the given operand and stores

the result in the operand.

\c{FSUBR} does the same thing, but does the subtraction the other way

up: so if \c{TO} is not given, it subtracts \c{ST0} from the given

operand and stores the result in \c{ST0}, whereas if \c{TO} is given

it subtracts its operand from \c{ST0} and stores the result in the

operand.

\c{FSUBP} operates like \c{FSUB TO}, but pops the register stack

once it has finished. \c{FSUBRP} operates like \c{FSUBR TO}, but

pops the register stack once it has finished.

\H{insFTST} \i\c{FTST}: Test \c{ST0} Against Zero

\c FTST                          ; D9 E4                [8086,FPU]

\c{FTST} compares \c{ST0} with zero and sets the FPU flags

accordingly. \c{ST0} is treated as the left-hand side of the

comparison, so that a `less-than' result is generated if \c{ST0} is

negative.

\H{insFUCOM} \i\c{FUCOMxx}: Floating-Point Unordered Compare

\c FUCOM fpureg                  ; DD E0+r              [386,FPU]

\c FUCOM ST0,fpureg              ; DD E0+r              [386,FPU]

\c FUCOMP fpureg                 ; DD E8+r              [386,FPU]

\c FUCOMP ST0,fpureg             ; DD E8+r              [386,FPU]

\c FUCOMPP                       ; DA E9                [386,FPU]

\c FUCOMI fpureg                 ; DB E8+r              [P6,FPU]

\c FUCOMI ST0,fpureg             ; DB E8+r              [P6,FPU]

\c FUCOMIP fpureg                ; DF E8+r              [P6,FPU]

\c FUCOMIP ST0,fpureg            ; DF E8+r              [P6,FPU]

\c{FUCOM} compares \c{ST0} with the given operand, and sets the FPU

flags accordingly. \c{ST0} is treated as the left-hand side of the

comparison, so that the carry flag is set (for a `less-than' result)

if \c{ST0} is less than the given operand.

\c{FUCOMP} does the same as \c{FUCOM}, but pops the register stack

afterwards. \c{FUCOMPP} compares \c{ST0} with \c{ST1} and then pops

the register stack twice.

\c{FUCOMI} and \c{FUCOMIP} work like the corresponding forms of

\c{FUCOM} and \c{FUCOMP}, but write their results directly to the CPU

flags register rather than the FPU status word, so they can be

immediately followed by conditional jump or conditional move

instructions.

The \c{FUCOM} instructions differ from the \c{FCOM} instructions

(\k{insFCOM}) only in the way they handle quiet NaNs: \c{FUCOM} will

handle them silently and set the condition code flags to an

`unordered' result, whereas \c{FCOM} will generate an exception.

\H{insFXAM} \i\c{FXAM}: Examine Class of Value in \c{ST0}

\c FXAM                          ; D9 E5                [8086,FPU]

\c{FXAM} sets the FPU flags C3, C2 and C0 depending on the type of

value stored in \c{ST0}: 000 (respectively) for an unsupported

format, 001 for a NaN, 010 for a normal finite number, 011 for an

infinity, 100 for a zero, 101 for an empty register, and 110 for a

denormal. It also sets the C1 flag to the sign of the number.

\H{insFXCH} \i\c{FXCH}: Floating-Point Exchange

\c FXCH                          ; D9 C9                [8086,FPU]

\c FXCH fpureg                   ; D9 C8+r              [8086,FPU]

\c FXCH fpureg,ST0               ; D9 C8+r              [8086,FPU]

\c FXCH ST0,fpureg               ; D9 C8+r              [8086,FPU]

\c{FXCH} exchanges \c{ST0} with a given FPU register. The no-operand

form exchanges \c{ST0} with \c{ST1}.

\H{insFXTRACT} \i\c{FXTRACT}: Extract Exponent and Significand

\c FXTRACT                       ; D9 F4                [8086,FPU]

\c{FXTRACT} separates the number in \c{ST0} into its exponent and

significand (mantissa), stores the exponent back into \c{ST0}, and

then pushes the significand on the register stack (so that the

significand ends up in \c{ST0}, and the exponent in \c{ST1}).

\H{insFYL2X} \i\c{FYL2X}, \i\c{FYL2XP1}: Compute Y times Log2(X) or Log2(X+1)

\c FYL2X                         ; D9 F1                [8086,FPU]

\c FYL2XP1                       ; D9 F9                [8086,FPU]

\c{FYL2X} multiplies \c{ST1} by the base-2 logarithm of \c{ST0},

stores the result in \c{ST1}, and pops the register stack (so that

the result ends up in \c{ST0}). \c{ST0} must be non-zero and

positive.

\c{FYL2XP1} works the same way, but replacing the base-2 log of

\c{ST0} with that of \c{ST0} plus one. This time, \c{ST0} must have

magnitude no greater than 1 minus half the square root of two.

\H{insHLT} \i\c{HLT}: Halt Processor

\c HLT                           ; F4                   [8086]

\c{HLT} puts the processor into a halted state, where it will

perform no more operations until restarted by an interrupt or a

reset.

\H{insIBTS} \i\c{IBTS}: Insert Bit String

\c IBTS r/m16,reg16              ; o16 0F A7 /r         [386,UNDOC]

\c IBTS r/m32,reg32              ; o32 0F A7 /r         [386,UNDOC]

No clear documentation seems to be available for this instruction:

the best I've been able to find reads `Takes a string of bits from

the second operand and puts them in the first operand'. It is

present only in early 386 processors, and conflicts with the opcodes

for \c{CMPXCHG486}. NASM supports it only for completeness. Its

counterpart is \c{XBTS} (see \k{insXBTS}).

\H{insIDIV} \i\c{IDIV}: Signed Integer Divide

\c IDIV r/m8                     ; F6 /7                [8086]

\c IDIV r/m16                    ; o16 F7 /7            [8086]

\c IDIV r/m32                    ; o32 F7 /7            [386]

\c{IDIV} performs signed integer division. The explicit operand

provided is the divisor; the dividend and destination operands are

implicit, in the following way:

\b For \c{IDIV r/m8}, \c{AX} is divided by the given operand; the

quotient is stored in \c{AL} and the remainder in \c{AH}.

\b For \c{IDIV r/m16}, \c{DX:AX} is divided by the given operand; the

quotient is stored in \c{AX} and the remainder in \c{DX}.

\b For \c{IDIV r/m32}, \c{EDX:EAX} is divided by the given operand;

the quotient is stored in \c{EAX} and the remainder in \c{EDX}.

Unsigned integer division is performed by the \c{DIV} instruction:

see \k{insDIV}.

\H{insIMUL} \i\c{IMUL}: Signed Integer Multiply

\c IMUL r/m8                     ; F6 /5                [8086]

\c IMUL r/m16                    ; o16 F7 /5            [8086]

\c IMUL r/m32                    ; o32 F7 /5            [386]

\c IMUL reg16,r/m16              ; o16 0F AF /r         [386]

\c IMUL reg32,r/m32              ; o32 0F AF /r         [386]

\c IMUL reg16,imm8               ; o16 6B /r ib         [286]

\c IMUL reg16,imm16              ; o16 69 /r iw         [286]

\c IMUL reg32,imm8               ; o32 6B /r ib         [386]

\c IMUL reg32,imm32              ; o32 69 /r id         [386]

\c IMUL reg16,r/m16,imm8         ; o16 6B /r ib         [286]

\c IMUL reg16,r/m16,imm16        ; o16 69 /r iw         [286]

\c IMUL reg32,r/m32,imm8         ; o32 6B /r ib         [386]

\c IMUL reg32,r/m32,imm32        ; o32 69 /r id         [386]

\c{IMUL} performs signed integer multiplication. For the

single-operand form, the other operand and destination are implicit,

in the following way:

\b For \c{IMUL r/m8}, \c{AL} is multiplied by the given operand; the

product is stored in \c{AX}.

\b For \c{IMUL r/m16}, \c{AX} is multiplied by the given operand;

the product is stored in \c{DX:AX}.

\b For \c{IMUL r/m32}, \c{EAX} is multiplied by the given operand;

the product is stored in \c{EDX:EAX}.

The two-operand form multiplies its two operands and stores the

result in the destination (first) operand. The three-operand form

multiplies its last two operands and stores the result in the first

operand.

The two-operand form is in fact a shorthand for the three-operand

form, as can be seen by examining the opcode descriptions: in the

two-operand form, the code \c{/r} takes both its register and

\c{r/m} parts from the same operand (the first one).

In the forms with an 8-bit immediate operand and another longer

source operand, the immediate operand is considered to be signed,

and is sign-extended to the length of the other source operand. In

these cases, the \c{BYTE} qualifier is necessary to force NASM to

generate this form of the instruction.

Unsigned integer multiplication is performed by the \c{MUL}

instruction: see \k{insMUL}.

\H{insIN} \i\c{IN}: Input from I/O Port

\c IN AL,imm8                    ; E4 ib                [8086]

\c IN AX,imm8                    ; o16 E5 ib            [8086]

\c IN EAX,imm8                   ; o32 E5 ib            [386]

\c IN AL,DX                      ; EC                   [8086]

\c IN AX,DX                      ; o16 ED               [8086]

\c IN EAX,DX                     ; o32 ED               [386]

\c{IN} reads a byte, word or doubleword from the specified I/O port,

and stores it in the given destination register. The port number may

be specified as an immediate value if it is between 0 and 255, and

otherwise must be stored in \c{DX}. See also \c{OUT} (\k{insOUT}).

\H{insINC} \i\c{INC}: Increment Integer

\c INC reg16                     ; o16 40+r             [8086]

\c INC reg32                     ; o32 40+r             [386]

\c INC r/m8                      ; FE /0                [8086]

\c INC r/m16                     ; o16 FF /0            [8086]

\c INC r/m32                     ; o32 FF /0            [386]

\c{INC} adds 1 to its operand. It does \e{not} affect the carry

flag: to affect the carry flag, use \c{ADD something,1} (see

\k{insADD}). See also \c{DEC} (\k{insDEC}).

\H{insINSB} \i\c{INSB}, \i\c{INSW}, \i\c{INSD}: Input String from I/O Port

\c INSB                          ; 6C                   [186]

\c INSW                          ; o16 6D               [186]

\c INSD                          ; o32 6D               [386]

\c{INSB} inputs a byte from the I/O port specified in \c{DX} and

stores it at \c{[ES:DI]} or \c{[ES:EDI]}. It then increments or

decrements (depending on the direction flag: increments if the flag

is clear, decrements if it is set) \c{DI} or \c{EDI}.

The register used is \c{DI} if the address size is 16 bits, and

\c{EDI} if it is 32 bits. If you need to use an address size not

equal to the current \c{BITS} setting, you can use an explicit

\i\c{a16} or \i\c{a32} prefix.

Segment override prefixes have no effect for this instruction: the

use of \c{ES} for the load from \c{[DI]} or \c{[EDI]} cannot be

overridden.

\c{INSW} and \c{INSD} work in the same way, but they input a word or

a doubleword instead of a byte, and increment or decrement the

addressing register by 2 or 4 instead of 1.

The \c{REP} prefix may be used to repeat the instruction \c{CX} (or

\c{ECX} - again, the address size chooses which) times.

See also \c{OUTSB}, \c{OUTSW} and \c{OUTSD} (\k{insOUTSB}).

\H{insINT} \i\c{INT}: Software Interrupt

\c INT imm8                      ; CD ib                [8086]

\c{INT} causes a software interrupt through a specified vector

number from 0 to 255.

The code generated by the \c{INT} instruction is always two bytes

long: although there are short forms for some \c{INT} instructions,

NASM does not generate them when it sees the \c{INT} mnemonic. In

order to generate single-byte breakpoint instructions, use the

\c{INT3} or \c{INT1} instructions (see \k{insINT1}) instead.

\H{insINT1} \i\c{INT3}, \i\c{INT1}, \i\c{ICEBP}, \i\c{INT01}: Breakpoints

\c INT1                          ; F1                   [P6]

\c ICEBP                         ; F1                   [P6]

\c INT01                         ; F1                   [P6]

\c INT3                          ; CC                   [8086]

\c{INT1} and \c{INT3} are short one-byte forms of the instructions

\c{INT 1} and \c{INT 3} (see \k{insINT}). They perform a similar

function to their longer counterparts, but take up less code space.

They are used as breakpoints by debuggers.

\c{INT1}, and its alternative synonyms \c{INT01} and \c{ICEBP}, is

an instruction used by in-circuit emulators (ICEs). It is present,

though not documented, on some processors down to the 286, but is

only documented for the Pentium Pro. \c{INT3} is the instruction

normally used as a breakpoint by debuggers.

\c{INT3} is not precisely equivalent to \c{INT 3}: the short form,

since it is designed to be used as a breakpoint, bypasses the normal

IOPL checks in virtual-8086 mode, and also does not go through

interrupt redirection.

\H{insINTO} \i\c{INTO}: Interrupt if Overflow

\c INTO                          ; CE                   [8086]

\c{INTO} performs an \c{INT 4} software interrupt (see \k{insINT})

if and only if the overflow flag is set.

\H{insINVD} \i\c{INVD}: Invalidate Internal Caches

\c INVD                          ; 0F 08                [486]

\c{INVD} invalidates and empties the processor's internal caches,

and causes the processor to instruct external caches to do the same.

It does not write the contents of the caches back to memory first:

any modified data held in the caches will be lost. To write the data

back first, use \c{WBINVD} (\k{insWBINVD}).

\H{insINVLPG} \i\c{INVLPG}: Invalidate TLB Entry

\c INVLPG mem                    ; 0F 01 /0             [486]

\c{INVLPG} invalidates the translation lookahead buffer (TLB) entry

associated with the supplied memory address.

\H{insIRET} \i\c{IRET}, \i\c{IRETW}, \i\c{IRETD}: Return from Interrupt

\c IRET                          ; CF                   [8086]

\c IRETW                         ; o16 CF               [8086]

\c IRETD                         ; o32 CF               [386]

\c{IRET} returns from an interrupt (hardware or software) by means

of popping \c{IP} (or \c{EIP}), \c{CS} and the flags off the stack

and then continuing execution from the new \c{CS:IP}.

\c{IRETW} pops \c{IP}, \c{CS} and the flags as 2 bytes each, taking

6 bytes off the stack in total. \c{IRETD} pops \c{EIP} as 4 bytes,

pops a further 4 bytes of which the top two are discarded and the

bottom two go into \c{CS}, and pops the flags as 4 bytes as well,

taking 12 bytes off the stack.

\c{IRET} is a shorthand for either \c{IRETW} or \c{IRETD}, depending

on the default \c{BITS} setting at the time.

\H{insJCXZ} \i\c{JCXZ}, \i\c{JECXZ}: Jump if CX/ECX Zero

\c JCXZ imm                      ; o16 E3 rb            [8086]

\c JECXZ imm                     ; o32 E3 rb            [386]

\c{JCXZ} performs a short jump (with maximum range 128 bytes) if and

only if the contents of the \c{CX} register is 0. \c{JECXZ} does the

same thing, but with \c{ECX}.

\H{insJMP} \i\c{JMP}: Jump

\c JMP imm                       ; E9 rw/rd             [8086]

\c JMP SHORT imm                 ; EB rb                [8086]

\c JMP imm:imm16                 ; o16 EA iw iw         [8086]

\c JMP imm:imm32                 ; o32 EA id iw         [386]

\c JMP FAR mem                   ; o16 FF /5            [8086]

\c JMP FAR mem                   ; o32 FF /5            [386]

\c JMP r/m16                     ; o16 FF /4            [8086]

\c JMP r/m32                     ; o32 FF /4            [386]

\c{JMP} jumps to a given address. The address may be specified as an

absolute segment and offset, or as a relative jump within the

current segment.

\c{JMP SHORT imm} has a maximum range of 128 bytes, since the

displacement is specified as only 8 bits, but takes up less code

space. NASM does not choose when to generate \c{JMP SHORT} for you:

you must explicitly code \c{SHORT} every time you want a short jump.

You can choose between the two immediate \i{far jump} forms (\c{JMP

imm:imm}) by the use of the \c{WORD} and \c{DWORD} keywords: \c{JMP

WORD 0x1234:0x5678}) or \c{JMP DWORD 0x1234:0x56789abc}.

The \c{JMP FAR mem} forms execute a far jump by loading the

destination address out of memory. The address loaded consists of 16

or 32 bits of offset (depending on the operand size), and 16 bits of

segment. The operand size may be overridden using \c{JMP WORD FAR

mem} or \c{JMP DWORD FAR mem}.

The \c{JMP r/m} forms execute a \i{near jump} (within the same

segment), loading the destination address out of memory or out of a

register. The keyword \c{NEAR} may be specified, for clarity, in

these forms, but is not necessary. Again, operand size can be

overridden using \c{JMP WORD mem} or \c{JMP DWORD mem}.

As a convenience, NASM does not require you to jump to a far symbol

by coding the cumbersome \c{JMP SEG routine:routine}, but instead

allows the easier synonym \c{JMP FAR routine}.

The \c{CALL r/m} forms given above are near calls; NASM will accept

the \c{NEAR} keyword (e.g. \c{CALL NEAR [address]}), even though it

is not strictly necessary.

\H{insJcc} \i\c{Jcc}: Conditional Branch

\c Jcc imm                       ; 70+cc rb             [8086]

\c Jcc NEAR imm                  ; 0F 80+cc rw/rd       [386]

The \i{conditional jump} instructions execute a near (same segment)

jump if and only if their conditions are satisfied. For example,

\c{JNZ} jumps only if the zero flag is not set.

The ordinary form of the instructions has only a 128-byte range; the

\c{NEAR} form is a 386 extension to the instruction set, and can

span the full size of a segment. NASM will not override your choice

of jump instruction: if you want \c{Jcc NEAR}, you have to use the

\c{NEAR} keyword.

The \c{SHORT} keyword is allowed on the first form of the

instruction, for clarity, but is not necessary.

\H{insLAHF} \i\c{LAHF}: Load AH from Flags

\c LAHF                          ; 9F                   [8086]

\c{LAHF} sets the \c{AH} register according to the contents of the

low byte of the flags word. See also \c{SAHF} (\k{insSAHF}).

\H{insLAR} \i\c{LAR}: Load Access Rights

\c LAR reg16,r/m16               ; o16 0F 02 /r         [286,PRIV]

\c LAR reg32,r/m32               ; o32 0F 02 /r         [286,PRIV]

\c{LAR} takes the segment selector specified by its source (second)

operand, finds the corresponding segment descriptor in the GDT or

LDT, and loads the access-rights byte of the descriptor into its

destination (first) operand.

\H{insLDS} \i\c{LDS}, \i\c{LES}, \i\c{LFS}, \i\c{LGS}, \i\c{LSS}: Load Far Pointer

\c LDS reg16,mem                 ; o16 C5 /r            [8086]

\c LDS reg32,mem                 ; o32 C5 /r            [8086]

\c LES reg16,mem                 ; o16 C4 /r            [8086]

\c LES reg32,mem                 ; o32 C4 /r            [8086]

\c LFS reg16,mem                 ; o16 0F B4 /r         [386]

\c LFS reg32,mem                 ; o32 0F B4 /r         [386]

\c LGS reg16,mem                 ; o16 0F B5 /r         [386]

\c LGS reg32,mem                 ; o32 0F B5 /r         [386]

\c LSS reg16,mem                 ; o16 0F B2 /r         [386]

\c LSS reg32,mem                 ; o32 0F B2 /r         [386]

These instructions load an entire far pointer (16 or 32 bits of

offset, plus 16 bits of segment) out of memory in one go. \c{LDS},

for example, loads 16 or 32 bits from the given memory address into

the given register (depending on the size of the register), then

loads the \e{next} 16 bits from memory into \c{DS}. \c{LES},

\c{LFS}, \c{LGS} and \c{LSS} work in the same way but use the other

segment registers.

\H{insLEA} \i\c{LEA}: Load Effective Address

\c LEA reg16,mem                 ; o16 8D /r            [8086]

\c LEA reg32,mem                 ; o32 8D /r            [8086]

\c{LEA}, despite its syntax, does not access memory. It calculates

the effective address specified by its second operand as if it were

going to load or store data from it, but instead it stores the

calculated address into the register specified by its first operand.

This can be used to perform quite complex calculations (e.g. \c{LEA

EAX,[EBX+ECX*4+100]}) in one instruction.

\c{LEA}, despite being a purely arithmetic instruction which

accesses no memory, still requires square brackets around its second

operand, as if it were a memory reference.

\H{insLEAVE} \i\c{LEAVE}: Destroy Stack Frame

\c LEAVE                         ; C9                   [186]

\c{LEAVE} destroys a stack frame of the form created by the

\c{ENTER} instruction (see \k{insENTER}). It is functionally

equivalent to \c{MOV ESP,EBP} followed by \c{POP EBP} (or \c{MOV

SP,BP} followed by \c{POP BP} in 16-bit mode).

\H{insLGDT} \i\c{LGDT}, \i\c{LIDT}, \i\c{LLDT}: Load Descriptor Tables

\c LGDT mem                      ; 0F 01 /2             [286,PRIV]

\c LIDT mem                      ; 0F 01 /3             [286,PRIV]

\c LLDT r/m16                    ; 0F 00 /2             [286,PRIV]

\c{LGDT} and \c{LIDT} both take a 6-byte memory area as an operand:

they load a 32-bit linear address and a 16-bit size limit from that

area (in the opposite order) into the GDTR (global descriptor table

register) or IDTR (interrupt descriptor table register). These are

the only instructions which directly use \e{linear} addresses,

rather than segment/offset pairs.

\c{LLDT} takes a segment selector as an operand. The processor looks

up that selector in the GDT and stores the limit and base address

given there into the LDTR (local descriptor table register).

See also \c{SGDT}, \c{SIDT} and \c{SLDT} (\k{insSGDT}).

\H{insLMSW} \i\c{LMSW}: Load/Store Machine Status Word

\c LMSW r/m16                    ; 0F 01 /6             [286,PRIV]

\c{LMSW} loads the bottom four bits of the source operand into the

bottom four bits of the \c{CR0} control register (or the Machine

Status Word, on 286 processors). See also \c{SMSW} (\k{insSMSW}).

\H{insLOADALL} \i\c{LOADALL}, \i\c{LOADALL286}: Load Processor State

\c LOADALL                       ; 0F 07                [386,UNDOC]

\c LOADALL286                    ; 0F 05                [286,UNDOC]

This instruction, in its two different-opcode forms, is apparently

supported on most 286 processors, some 386 and possibly some 486.

The opcode differs between the 286 and the 386.

The function of the instruction is to load all information relating

to the state of the processor out of a block of memory: on the 286,

this block is located implicitly at absolute address \c{0x800}, and

on the 386 and 486 it is at \c{[ES:EDI]}.

\H{insLODSB} \i\c{LODSB}, \i\c{LODSW}, \i\c{LODSD}: Load from String

\c LODSB                         ; AC                   [8086]

\c LODSW                         ; o16 AD               [8086]

\c LODSD                         ; o32 AD               [386]

\c{LODSB} loads a byte from \c{[DS:SI]} or \c{[DS:ESI]} into \c{AL}.

It then increments or decrements (depending on the direction flag:

increments if the flag is clear, decrements if it is set) \c{SI} or

\c{ESI}.

The register used is \c{SI} if the address size is 16 bits, and

\c{ESI} if it is 32 bits. If you need to use an address size not

equal to the current \c{BITS} setting, you can use an explicit

\i\c{a16} or \i\c{a32} prefix.

The segment register used to load from \c{[SI]} or \c{[ESI]} can be

overridden by using a segment register name as a prefix (for

example, \c{es lodsb}).

\c{LODSW} and \c{LODSD} work in the same way, but they load a

word or a doubleword instead of a byte, and increment or decrement

the addressing registers by 2 or 4 instead of 1.

\H{insLOOP} \i\c{LOOP}, \i\c{LOOPE}, \i\c{LOOPZ}, \i\c{LOOPNE}, \i\c{LOOPNZ}: Loop with Counter

\c LOOP imm                      ; E2 rb                [8086]

\c LOOP imm,CX                   ; a16 E2 rb            [8086]

\c LOOP imm,ECX                  ; a32 E2 rb            [386]

\c LOOPE imm                     ; E1 rb                [8086]

\c LOOPE imm,CX                  ; a16 E1 rb            [8086]

\c LOOPE imm,ECX                 ; a32 E1 rb            [386]

\c LOOPZ imm                     ; E1 rb                [8086]

\c LOOPZ imm,CX                  ; a16 E1 rb            [8086]

\c LOOPZ imm,ECX                 ; a32 E1 rb            [386]

\c LOOPNE imm                    ; E0 rb                [8086]

\c LOOPNE imm,CX                 ; a16 E0 rb            [8086]

\c LOOPNE imm,ECX                ; a32 E0 rb            [386]

\c LOOPNZ imm                    ; E0 rb                [8086]

\c LOOPNZ imm,CX                 ; a16 E0 rb            [8086]

\c LOOPNZ imm,ECX                ; a32 E0 rb            [386]

\c{LOOP} decrements its counter register (either \c{CX} or \c{ECX} -

if one is not specified explicitly, the \c{BITS} setting dictates

which is used) by one, and if the counter does not become zero as a

result of this operation, it jumps to the given label. The jump has

a range of 128 bytes.

\c{LOOPE} (or its synonym \c{LOOPZ}) adds the additional condition

that it only jumps if the counter is nonzero \e{and} the zero flag

is set. Similarly, \c{LOOPNE} (and \c{LOOPNZ}) jumps only if the

counter is nonzero and the zero flag is clear.

\H{insLSL} \i\c{LSL}: Load Segment Limit

\c LSL reg16,r/m16               ; o16 0F 03 /r         [286,PRIV]

\c LSL reg32,r/m32               ; o32 0F 03 /r         [286,PRIV]

\c{LSL} is given a segment selector in its source (second) operand;

it computes the segment limit value by loading the segment limit

field from the associated segment descriptor in the GDT or LDT.

(This involves shifting left by 12 bits if the segment limit is

page-granular, and not if it is byte-granular; so you end up with a

byte limit in either case.) The segment limit obtained is then

loaded into the destination (first) operand.

\H{insLTR} \i\c{LTR}: Load Task Register

\c LTR r/m16                     ; 0F 00 /3             [286,PRIV]

\c{LTR} looks up the segment base and limit in the GDT or LDT

descriptor specified by the segment selector given as its operand,

and loads them into the Task Register.

\H{insMOV} \i\c{MOV}: Move Data

\c MOV r/m8,reg8                 ; 88 /r                [8086]

\c MOV r/m16,reg16               ; o16 89 /r            [8086]

\c MOV r/m32,reg32               ; o32 89 /r            [386]

\c MOV reg8,r/m8                 ; 8A /r                [8086]

\c MOV reg16,r/m16               ; o16 8B /r            [8086]

\c MOV reg32,r/m32               ; o32 8B /r            [386]

\c MOV reg8,imm8                 ; B0+r ib              [8086]

\c MOV reg16,imm16               ; o16 B8+r iw          [8086]

\c MOV reg32,imm32               ; o32 B8+r id          [386]

\c MOV r/m8,imm8                 ; C6 /0 ib             [8086]

\c MOV r/m16,imm16               ; o16 C7 /0 iw         [8086]

\c MOV r/m32,imm32               ; o32 C7 /0 id         [386]

\c MOV AL,memoffs8               ; A0 ow/od             [8086]

\c MOV AX,memoffs16              ; o16 A1 ow/od         [8086]

\c MOV EAX,memoffs32             ; o32 A1 ow/od         [386]

\c MOV memoffs8,AL               ; A2 ow/od             [8086]

\c MOV memoffs16,AX              ; o16 A3 ow/od         [8086]

\c MOV memoffs32,EAX             ; o32 A3 ow/od         [386]

\c MOV r/m16,segreg              ; o16 8C /r            [8086]

\c MOV r/m32,segreg              ; o32 8C /r            [386]

\c MOV segreg,r/m16              ; o16 8E /r            [8086]

\c MOV segreg,r/m32              ; o32 8E /r            [386]

\c MOV reg32,CR0/2/3/4           ; 0F 20 /r             [386]

\c MOV reg32,DR0/1/2/3/6/7       ; 0F 21 /r             [386]

\c MOV reg32,TR3/4/5/6/7         ; 0F 24 /r             [386]

\c MOV CR0/2/3/4,reg32           ; 0F 22 /r             [386]

\c MOV DR0/1/2/3/6/7,reg32       ; 0F 23 /r             [386]

\c MOV TR3/4/5/6/7,reg32         ; 0F 26 /r             [386]

\c{MOV} copies the contents of its source (second) operand into its

destination (first) operand.

In all forms of the \c{MOV} instruction, the two operands are the

same size, except for moving between a segment register and an

\c{r/m32} operand. These instructions are treated exactly like the

corresponding 16-bit equivalent (so that, for example, \c{MOV

DS,EAX} functions identically to \c{MOV DS,AX} but saves a prefix

when in 32-bit mode), except that when a segment register is moved

into a 32-bit destination, the top two bytes of the result are

undefined.

\c{MOV} may not use \c{CS} as a destination.

\c{CR4} is only a supported register on the Pentium and above.

\H{insMOVD} \i\c{MOVD}: Move Doubleword to/from MMX Register

\c MOVD mmxreg,r/m32             ; 0F 6E /r             [PENT,MMX]

\c MOVD r/m32,mmxreg             ; 0F 7E /r             [PENT,MMX]

\c{MOVD} copies 32 bits from its source (second) operand into its

destination (first) operand. When the destination is a 64-bit MMX

register, the top 32 bits are set to zero.

\H{insMOVQ} \i\c{MOVQ}: Move Quadword to/from MMX Register

\c MOVQ mmxreg,r/m64             ; 0F 6F /r             [PENT,MMX]

\c MOVQ r/m64,mmxreg             ; 0F 7F /r             [PENT,MMX]

\c{MOVQ} copies 64 bits from its source (second) operand into its

destination (first) operand.

\H{insMOVSB} \i\c{MOVSB}, \i\c{MOVSW}, \i\c{MOVSD}: Move String

\c MOVSB                         ; A4                   [8086]

\c MOVSW                         ; o16 A5               [8086]

\c MOVSD                         ; o32 A5               [386]

\c{MOVSB} copies the byte at \c{[ES:DI]} or \c{[ES:EDI]} to

\c{[DS:SI]} or \c{[DS:ESI]}. It then increments or decrements

(depending on the direction flag: increments if the flag is clear,

decrements if it is set) \c{SI} and \c{DI} (or \c{ESI} and \c{EDI}).

The registers used are \c{SI} and \c{DI} if the address size is 16

bits, and \c{ESI} and \c{EDI} if it is 32 bits. If you need to use

an address size not equal to the current \c{BITS} setting, you can

use an explicit \i\c{a16} or \i\c{a32} prefix.

The segment register used to load from \c{[SI]} or \c{[ESI]} can be

overridden by using a segment register name as a prefix (for

example, \c{es movsb}). The use of \c{ES} for the store to \c{[DI]}

or \c{[EDI]} cannot be overridden.

\c{MOVSW} and \c{MOVSD} work in the same way, but they copy a word

or a doubleword instead of a byte, and increment or decrement the

addressing registers by 2 or 4 instead of 1.

The \c{REP} prefix may be used to repeat the instruction \c{CX} (or

\c{ECX} - again, the address size chooses which) times.

\H{insMOVSX} \i\c{MOVSX}, \i\c{MOVZX}: Move Data with Sign or Zero Extend

\c MOVSX reg16,r/m8              ; o16 0F BE /r         [386]

\c MOVSX reg32,r/m8              ; o32 0F BE /r         [386]

\c MOVSX reg32,r/m16             ; o32 0F BF /r         [386]

\c MOVZX reg16,r/m8              ; o16 0F B6 /r         [386]

\c MOVZX reg32,r/m8              ; o32 0F B6 /r         [386]

\c MOVZX reg32,r/m16             ; o32 0F B7 /r         [386]

\c{MOVSX} sign-extends its source (second) operand to the length of

its destination (first) operand, and copies the result into the

destination operand. \c{MOVZX} does the same, but zero-extends

rather than sign-extending.

\H{insMUL} \i\c{MUL}: Unsigned Integer Multiply

\c MUL r/m8                      ; F6 /4                [8086]

\c MUL r/m16                     ; o16 F7 /4            [8086]

\c MUL r/m32                     ; o32 F7 /4            [386]

\c{MUL} performs unsigned integer multiplication. The other operand

to the multiplication, and the destination operand, are implicit, in

the following way:

\b For \c{MUL r/m8}, \c{AL} is multiplied by the given operand; the

product is stored in \c{AX}.

\b For \c{MUL r/m16}, \c{AX} is multiplied by the given operand;

the product is stored in \c{DX:AX}.

\b For \c{MUL r/m32}, \c{EAX} is multiplied by the given operand;

the product is stored in \c{EDX:EAX}.

Signed integer multiplication is performed by the \c{IMUL}

instruction: see \k{insIMUL}.

\H{insNEG} \i\c{NEG}, \i\c{NOT}: Two's and One's Complement

\c NEG r/m8                      ; F6 /3                [8086]

\c NEG r/m16                     ; o16 F7 /3            [8086]

\c NEG r/m32                     ; o32 F7 /3            [386]

\c NOT r/m8                      ; F6 /2                [8086]

\c NOT r/m16                     ; o16 F7 /2            [8086]

\c NOT r/m32                     ; o32 F7 /2            [386]

\c{NEG} replaces the contents of its operand by the two's complement

negation (invert all the bits and then add one) of the original

value. \c{NOT}, similarly, performs one's complement (inverts all

the bits).

\H{insNOP} \i\c{NOP}: No Operation

\c NOP                           ; 90                   [8086]

\c{NOP} performs no operation. Its opcode is the same as that

generated by \c{XCHG AX,AX} or \c{XCHG EAX,EAX} (depending on the

processor mode; see \k{insXCHG}).

\H{insOR} \i\c{OR}: Bitwise OR

\c OR r/m8,reg8                  ; 08 /r                [8086]

\c OR r/m16,reg16                ; o16 09 /r            [8086]

\c OR r/m32,reg32                ; o32 09 /r            [386]

\c OR reg8,r/m8                  ; 0A /r                [8086]

\c OR reg16,r/m16                ; o16 0B /r            [8086]

\c OR reg32,r/m32                ; o32 0B /r            [386]

\c OR r/m8,imm8                  ; 80 /1 ib             [8086]

\c OR r/m16,imm16                ; o16 81 /1 iw         [8086]

\c OR r/m32,imm32                ; o32 81 /1 id         [386]

\c OR r/m16,imm8                 ; o16 83 /1 ib         [8086]

\c OR r/m32,imm8                 ; o32 83 /1 ib         [386]

\c OR AL,imm8                    ; 0C ib                [8086]

\c OR AX,imm16                   ; o16 0D iw            [8086]

\c OR EAX,imm32                  ; o32 0D id            [386]

\c{OR} performs a bitwise OR operation between its two operands

(i.e. each bit of the result is 1 if and only if at least one of the

corresponding bits of the two inputs was 1), and stores the result

in the destination (first) operand.

In the forms with an 8-bit immediate second operand and a longer

first operand, the second operand is considered to be signed, and is

sign-extended to the length of the first operand. In these cases,

the \c{BYTE} qualifier is necessary to force NASM to generate this

form of the instruction.

The MMX instruction \c{POR} (see \k{insPOR}) performs the same

operation on the 64-bit MMX registers.

\H{insOUT} \i\c{OUT}: Output Data to I/O Port

\c OUT imm8,AL                   ; E6 ib                [8086]

\c OUT imm8,AX                   ; o16 E7 ib            [8086]

\c OUT imm8,EAX                  ; o32 E7 ib            [386]

\c OUT DX,AL                     ; EE                   [8086]

\c OUT DX,AX                     ; o16 EF               [8086]

\c OUT DX,EAX                    ; o32 EF               [386]

\c{IN} writes the contents of the given source register to the

specified I/O port. The port number may be specified as an immediate

value if it is between 0 and 255, and otherwise must be stored in

\c{DX}. See also \c{IN} (\k{insIN}).

\H{insOUTSB} \i\c{OUTSB}, \i\c{OUTSW}, \i\c{OUTSD}: Output String to I/O Port

\c OUTSB                         ; 6E                   [186]

\c OUTSW                         ; o16 6F               [186]

\c OUTSD                         ; o32 6F               [386]

\c{OUTSB} loads a byte from \c{[DS:SI]} or \c{[DS:ESI]} and writes

it to the I/O port specified in \c{DX}. It then increments or

decrements (depending on the direction flag: increments if the flag

is clear, decrements if it is set) \c{SI} or \c{ESI}.

The register used is \c{SI} if the address size is 16 bits, and

\c{ESI} if it is 32 bits. If you need to use an address size not

equal to the current \c{BITS} setting, you can use an explicit

\i\c{a16} or \i\c{a32} prefix.

The segment register used to load from \c{[SI]} or \c{[ESI]} can be

overridden by using a segment register name as a prefix (for

example, \c{es outsb}).

\c{OUTSW} and \c{OUTSD} work in the same way, but they output a

word or a doubleword instead of a byte, and increment or decrement

the addressing registers by 2 or 4 instead of 1.

The \c{REP} prefix may be used to repeat the instruction \c{CX} (or

\c{ECX} - again, the address size chooses which) times.

\H{insPACKSSDW} \i\c{PACKSSDW}, \i\c{PACKSSWB}, \i\c{PACKUSWB}: Pack Data

\c PACKSSDW mmxreg,r/m64         ; 0F 6B /r             [PENT,MMX]

\c PACKSSWB mmxreg,r/m64         ; 0F 63 /r             [PENT,MMX]

\c PACKUSWB mmxreg,r/m64         ; 0F 67 /r             [PENT,MMX]

All these instructions start by forming a notional 128-bit word by

placing the source (second) operand on the left of the destination

(first) operand. \c{PACKSSDW} then splits this 128-bit word into

four doublewords, converts each to a word, and loads them side by

side into the destination register; \c{PACKSSWB} and \c{PACKUSWB}

both split the 128-bit word into eight words, converts each to a

byte, and loads \e{those} side by side into the destination

register.

\c{PACKSSDW} and \c{PACKSSWB} perform signed saturation when

reducing the length of numbers: if the number is too large to fit

into the reduced space, they replace it by the largest signed number

(\c{7FFFh} or \c{7Fh}) that \e{will} fit, and if it is too small

then they replace it by the smallest signed number (\c{8000h} or

\c{80h}) that will fit. \c{PACKUSWB} performs unsigned saturation:

it treats its input as unsigned, and replaces it by the largest

unsigned number that will fit.

\H{insPADDB} \i\c{PADDxx}: MMX Packed Addition

\c PADDB mmxreg,r/m64            ; 0F FC /r             [PENT,MMX]

\c PADDW mmxreg,r/m64            ; 0F FD /r             [PENT,MMX]

\c PADDD mmxreg,r/m64            ; 0F FE /r             [PENT,MMX]

\c PADDSB mmxreg,r/m64           ; 0F EC /r             [PENT,MMX]

\c PADDSW mmxreg,r/m64           ; 0F ED /r             [PENT,MMX]

\c PADDUSB mmxreg,r/m64          ; 0F DC /r             [PENT,MMX]

\c PADDUSW mmxreg,r/m64          ; 0F DD /r             [PENT,MMX]

\c{PADDxx} all perform packed addition between their two 64-bit

operands, storing the result in the destination (first) operand. The

\c{PADDxB} forms treat the 64-bit operands as vectors of eight

bytes, and add each byte individually; \c{PADDxW} treat the operands

as vectors of four words; and \c{PADDD} treats its operands as

vectors of two doublewords.

\c{PADDSB} and \c{PADDSW} perform signed saturation on the sum of

each pair of bytes or words: if the result of an addition is too

large or too small to fit into a signed byte or word result, it is

clipped (saturated) to the largest or smallest value which \e{will}

fit. \c{PADDUSB} and \c{PADDUSW} similarly perform unsigned

saturation, clipping to \c{0FFh} or \c{0FFFFh} if the result is

larger than that.

\H{insPADDSIW} \i\c{PADDSIW}: MMX Packed Addition to Implicit

Destination

\c PADDSIW mmxreg,r/m64          ; 0F 51 /r             [CYRIX,MMX]

\c{PADDSIW}, specific to the Cyrix extensions to the MMX instruction

set, performs the same function as \c{PADDSW}, except that the

result is not placed in the register specified by the first operand,

but instead in the register whose number differs from the first

operand only in the last bit. So \c{PADDSIW MM0,MM2} would put the

result in \c{MM1}, but \c{PADDSIW MM1,MM2} would put the result in

\c{MM0}.

\H{insPAND} \i\c{PAND}, \i\c{PANDN}: MMX Bitwise AND and AND-NOT

\c PAND mmxreg,r/m64             ; 0F DB /r             [PENT,MMX]

\c PANDN mmxreg,r/m64            ; 0F DF /r             [PENT,MMX]

\c{PAND} performs a bitwise AND operation between its two operands

(i.e. each bit of the result is 1 if and only if the corresponding

bits of the two inputs were both 1), and stores the result in the

destination (first) operand.

\c{PANDN} performs the same operation, but performs a one's

complement operation on the destination (first) operand first.

\H{insPAVEB} \i\c{PAVEB}: MMX Packed Average

\c PAVEB mmxreg,r/m64            ; 0F 50 /r             [CYRIX,MMX]

\c{PAVEB}, specific to the Cyrix MMX extensions, treats its two

operands as vectors of eight unsigned bytes, and calculates the

average of the corresponding bytes in the operands. The resulting

vector of eight averages is stored in the first operand.

\H{insPCMPEQB} \i\c{PCMPxx}: MMX Packed Comparison

\c PCMPEQB mmxreg,r/m64          ; 0F 74 /r             [PENT,MMX]

\c PCMPEQW mmxreg,r/m64          ; 0F 75 /r             [PENT,MMX]

\c PCMPEQD mmxreg,r/m64          ; 0F 76 /r             [PENT,MMX]

\c PCMPGTB mmxreg,r/m64          ; 0F 64 /r             [PENT,MMX]

\c PCMPGTW mmxreg,r/m64          ; 0F 65 /r             [PENT,MMX]

\c PCMPGTD mmxreg,r/m64          ; 0F 66 /r             [PENT,MMX]

The \c{PCMPxx} instructions all treat their operands as vectors of

bytes, words, or doublewords; corresponding elements of the source

and destination are compared, and the corresponding element of the

destination (first) operand is set to all zeros or all ones

depending on the result of the comparison.

\c{PCMPxxB} treats the operands as vectors of eight bytes,

\c{PCMPxxW} treats them as vectors of four words, and \c{PCMPxxD} as

two doublewords.

\c{PCMPEQx} sets the corresponding element of the destination

operand to all ones if the two elements compared are equal;

\c{PCMPGTx} sets the destination element to all ones if the element

of the first (destination) operand is greater (treated as a signed

integer) than that of the second (source) operand.

\H{insPDISTIB} \i\c{PDISTIB}: MMX Packed Distance and Accumulate

with Implied Register

\c PDISTIB mmxreg,mem64          ; 0F 54 /r             [CYRIX,MMX]

\c{PDISTIB}, specific to the Cyrix MMX extensions, treats its two

input operands as vectors of eight unsigned bytes. For each byte

position, it finds the absolute difference between the bytes in that

position in the two input operands, and adds that value to the byte

in the same position in the implied output register. The addition is

saturated to an unsigned byte in the same way as \c{PADDUSB}.

The implied output register is found in the same way as \c{PADDSIW}

(\k{insPADDSIW}).

Note that \c{PDISTIB} cannot take a register as its second source

operand.

\H{insPMACHRIW} \i\c{PMACHRIW}: MMX Packed Multiply and Accumulate

with Rounding

\c PMACHRIW mmxreg,mem64         ; 0F 5E /r             [CYRIX,MMX]

\c{PMACHRIW} acts almost identically to \c{PMULHRIW}

(\k{insPMULHRW}), but instead of \e{storing} its result in the

implied destination register, it \e{adds} its result, as four packed

words, to the implied destination register. No saturation is done:

the addition can wrap around.

Note that \c{PMACHRIW} cannot take a register as its second source

operand.

\H{insPMADDWD} \i\c{PMADDWD}: MMX Packed Multiply and Add

\c PMADDWD mmxreg,r/m64          ; 0F F5 /r             [PENT,MMX]

\c{PMADDWD} treats its two inputs as vectors of four signed words.

It multiplies corresponding elements of the two operands, giving

four signed doubleword results. The top two of these are added and

placed in the top 32 bits of the destination (first) operand; the

bottom two are added and placed in the bottom 32 bits.

\H{insPMAGW} \i\c{PMAGW}: MMX Packed Magnitude

\c PMAGW mmxreg,r/m64            ; 0F 52 /r             [CYRIX,MMX]

\c{PMAGW}, specific to the Cyrix MMX extensions, treats both its

operands as vectors of four signed words. It compares the absolute

values of the words in corresponding positions, and sets each word

of the destination (first) operand to whichever of the two words in

that position had the larger absolute value.

\H{insPMULHRW} \i\c{PMULHRW}, \i\c{PMULHRIW}: MMX Packed Multiply

High with Rounding

\c PMULHRW mmxreg,r/m64          ; 0F 59 /r             [CYRIX,MMX]

\c PMULHRIW mmxreg,r/m64         ; 0F 5D /r             [CYRIX,MMX]

These instructions, specific to the Cyrix MMX extensions, treat

their operands as vectors of four signed words. Words in

corresponding positions are multiplied, to give a 32-bit value in

which bits 30 and 31 are guaranteed equal. Bits 30 to 15 of this

value (bit mask \c{0x7FFF8000}) are taken and stored in the

corresponding position of the destination operand, after first

rounding the low bit (equivalent to adding \c{0x4000} before

extracting bits 30 to 15).

For \c{PMULHRW}, the destination operand is the first operand; for

\c{PMULHRIW} the destination operand is implied by the first operand

in the manner of \c{PADDSIW} (\k{insPADDSIW}).

\H{insPMULHW} \i\c{PMULHW}, \i\c{PMULLW}: MMX Packed Multiply

\c PMULHW mmxreg,r/m64           ; 0F E5 /r             [PENT,MMX]

\c PMULLW mmxreg,r/m64           ; 0F D5 /r             [PENT,MMX]

\c{PMULxW} treats its two inputs as vectors of four signed words. It

multiplies corresponding elements of the two operands, giving four

signed doubleword results.

\c{PMULHW} then stores the top 16 bits of each doubleword in the

destination (first) operand; \c{PMULLW} stores the bottom 16 bits of

each doubleword in the destination operand.

\H{insPMVccZB} \i\c{PMVccZB}: MMX Packed Conditional Move

\c PMVZB mmxreg,mem64            ; 0F 58 /r             [CYRIX,MMX]

\c PMVNZB mmxreg,mem64           ; 0F 5A /r             [CYRIX,MMX]

\c PMVLZB mmxreg,mem64           ; 0F 5B /r             [CYRIX,MMX]

\c PMVGEZB mmxreg,mem64          ; 0F 5C /r             [CYRIX,MMX]

These instructions, specific to the Cyrix MMX extensions, perform

parallel conditional moves. The two input operands are treated as

vectors of eight bytes. Each byte of the destination (first) operand

is either written from the corresponding byte of the source (second)

operand, or left alone, depending on the value of the byte in the

\e{implied} operand (specified in the same way as \c{PADDSIW}, in

\k{insPADDSIW}).

\c{PMVZB} performs each move if the corresponding byte in the

implied operand is zero. \c{PMVNZB} moves if the byte is non-zero.

\c{PMVLZB} moves if the byte is less than zero, and \c{PMVGEZB}

moves if the byte is greater than or equal to zero.

Note that these instructions cannot take a register as their second

source operand.

\H{insPOP} \i\c{POP}: Pop Data from Stack

\c POP reg16                     ; o16 58+r             [8086]

\c POP reg32                     ; o32 58+r             [386]

\c POP r/m16                     ; o16 8F /0            [8086]

\c POP r/m32                     ; o32 8F /0            [386]

\c POP CS                        ; 0F                   [8086,UNDOC]

\c POP DS                        ; 1F                   [8086]

\c POP ES                        ; 07                   [8086]

\c POP SS                        ; 17                   [8086]

\c POP FS                        ; 0F A1                [386]

\c POP GS                        ; 0F A9                [386]

\c{POP} loads a value from the stack (from \c{[SS:SP]} or

\c{[SS:ESP]}) and then increments the stack pointer.

The address-size attribute of the instruction determines whether

\c{SP} or \c{ESP} is used as the stack pointer: to deliberately

override the default given by the \c{BITS} setting, you can use an

\i\c{a16} or \i\c{a32} prefix.

The operand-size attribute of the instruction determines whether the

stack pointer is incremented by 2 or 4: this means that segment

register pops in \c{BITS 32} mode will pop 4 bytes off the stack and

discard the upper two of them. If you need to override that, you can

use an \i\c{o16} or \i\c{o32} prefix.

The above opcode listings give two forms for general-purpose

register pop instructions: for example, \c{POP BX} has the two forms

\c{5B} and \c{8F C3}. NASM will always generate the shorter form

when given \c{POP BX}. NDISASM will disassemble both.

\c{POP CS} is not a documented instruction, and is not supported on

any processor above the 8086 (since they use \c{0Fh} as an opcode

prefix for instruction set extensions). However, at least some 8086

processors do support it, and so NASM generates it for completeness.

\H{insPOPA} \i\c{POPAx}: Pop All General-Purpose Registers

\c POPA                          ; 61                   [186]

\c POPAW                         ; o16 61               [186]

\c POPAD                         ; o32 61               [386]

\c{POPAW} pops a word from the stack into each of, successively,

\c{DI}, \c{SI}, \c{BP}, nothing (it discards a word from the stack

which was a placeholder for \c{SP}), \c{BX}, \c{DX}, \c{CX} and

\c{AX}. It is intended to reverse the operation of \c{PUSHAW} (see

\k{insPUSHA}), but it ignores the value for \c{SP} that was pushed

on the stack by \c{PUSHAW}.

\c{POPAD} pops twice as much data, and places the results in

\c{EDI}, \c{ESI}, \c{EBP}, nothing (placeholder for \c{ESP}),

\c{EBX}, \c{EDX}, \c{ECX} and \c{EAX}. It reverses the operation of

\c{PUSHAD}.

\c{POPA} is an alias mnemonic for either \c{POPAW} or \c{POPAD},

depending on the current \c{BITS} setting.

Note that the registers are popped in reverse order of their numeric

values in opcodes (see \k{iref-rv}).

\H{insPOPF} \i\c{POPFx}: Pop Flags Register

\c POPF                          ; 9D                   [186]

\c POPFW                         ; o16 9D               [186]

\c POPFD                         ; o32 9D               [386]

\c{POPFW} pops a word from the stack and stores it in the bottom 16

bits of the flags register (or the whole flags register, on

processors below a 386). \c{POPFD} pops a doubleword and stores it

in the entire flags register.

\c{POPF} is an alias mnemonic for either \c{POPFW} or \c{POPFD},

depending on the current \c{BITS} setting.

See also \c{PUSHF} (\k{insPUSHF}).

\H{insPOR} \i\c{POR}: MMX Bitwise OR

\c POR mmxreg,r/m64              ; 0F EB /r             [PENT,MMX]

\c{POR} performs a bitwise OR operation between its two operands

(i.e. each bit of the result is 1 if and only if at least one of the

corresponding bits of the two inputs was 1), and stores the result

in the destination (first) operand.

\H{insPSLLD} \i\c{PSLLx}, \i\c{PSRLx}, \i\c{PSRAx}: MMX Bit Shifts

\c PSLLW mmxreg,r/m64            ; 0F F1 /r             [PENT,MMX]

\c PSLLW mmxreg,imm8             ; 0F 71 /6 ib          [PENT,MMX]

\c PSLLD mmxreg,r/m64            ; 0F F2 /r             [PENT,MMX]

\c PSLLD mmxreg,imm8             ; 0F 72 /6 ib          [PENT,MMX]

\c PSLLQ mmxreg,r/m64            ; 0F F3 /r             [PENT,MMX]

\c PSLLQ mmxreg,imm8             ; 0F 73 /6 ib          [PENT,MMX]

\c PSRAW mmxreg,r/m64            ; 0F E1 /r             [PENT,MMX]

\c PSRAW mmxreg,imm8             ; 0F 71 /4 ib          [PENT,MMX]

\c PSRAD mmxreg,r/m64            ; 0F E2 /r             [PENT,MMX]

\c PSRAD mmxreg,imm8             ; 0F 72 /4 ib          [PENT,MMX]

\c PSRLW mmxreg,r/m64            ; 0F D1 /r             [PENT,MMX]

\c PSRLW mmxreg,imm8             ; 0F 71 /2 ib          [PENT,MMX]

\c PSRLD mmxreg,r/m64            ; 0F D2 /r             [PENT,MMX]

\c PSRLD mmxreg,imm8             ; 0F 72 /2 ib          [PENT,MMX]

\c PSRLQ mmxreg,r/m64            ; 0F D3 /r             [PENT,MMX]

\c PSRLQ mmxreg,imm8             ; 0F 73 /2 ib          [PENT,MMX]

\c{PSxxQ} perform simple bit shifts on the 64-bit MMX registers: the

destination (first) operand is shifted left or right by the number of

bits given in the source (second) operand, and the vacated bits are

filled in with zeros (for a logical shift) or copies of the original

sign bit (for an arithmetic right shift).

\c{PSxxW} and \c{PSxxD} perform packed bit shifts: the destination

operand is treated as a vector of four words or two doublewords, and

each element is shifted individually, so bits shifted out of one

element do not interfere with empty bits coming into the next.

\c{PSLLx} and \c{PSRLx} perform logical shifts: the vacated bits at

one end of the shifted number are filled with zeros. \c{PSRAx}

performs an arithmetic right shift: the vacated bits at the top of

the shifted number are filled with copies of the original top (sign)

bit.

\H{insPSUBB} \i\c{PSUBxx}: MMX Packed Subtraction

\c PSUBB mmxreg,r/m64            ; 0F F8 /r             [PENT,MMX]

\c PSUBW mmxreg,r/m64            ; 0F F9 /r             [PENT,MMX]

\c PSUBD mmxreg,r/m64            ; 0F FA /r             [PENT,MMX]

\c PSUBSB mmxreg,r/m64           ; 0F E8 /r             [PENT,MMX]

\c PSUBSW mmxreg,r/m64           ; 0F E9 /r             [PENT,MMX]

\c PSUBUSB mmxreg,r/m64          ; 0F D8 /r             [PENT,MMX]

\c PSUBUSW mmxreg,r/m64          ; 0F D9 /r             [PENT,MMX]

\c{PSUBxx} all perform packed subtraction between their two 64-bit

operands, storing the result in the destination (first) operand. The

\c{PSUBxB} forms treat the 64-bit operands as vectors of eight

bytes, and subtract each byte individually; \c{PSUBxW} treat the operands

as vectors of four words; and \c{PSUBD} treats its operands as

vectors of two doublewords.

In all cases, the elements of the operand on the right are

subtracted from the corresponding elements of the operand on the

left, not the other way round.

\c{PSUBSB} and \c{PSUBSW} perform signed saturation on the sum of

each pair of bytes or words: if the result of a subtraction is too

large or too small to fit into a signed byte or word result, it is

clipped (saturated) to the largest or smallest value which \e{will}

fit. \c{PSUBUSB} and \c{PSUBUSW} similarly perform unsigned

saturation, clipping to \c{0FFh} or \c{0FFFFh} if the result is

larger than that.

\H{insPSUBSIW} \i\c{PSUBSIW}: MMX Packed Subtract with Saturation to

Implied Destination

\c PSUBSIW mmxreg,r/m64          ; 0F 55 /r             [CYRIX,MMX]

\c{PSUBSIW}, specific to the Cyrix extensions to the MMX instruction

set, performs the same function as \c{PSUBSW}, except that the

result is not placed in the register specified by the first operand,

but instead in the implied destination register, specified as for

\c{PADDSIW} (\k{insPADDSIW}).

\H{insPUNPCKHBW} \i\c{PUNPCKxxx}: Unpack Data

\c PUNPCKHBW mmxreg,r/m64        ; 0F 68 /r             [PENT,MMX]

\c PUNPCKHWD mmxreg,r/m64        ; 0F 69 /r             [PENT,MMX]

\c PUNPCKHDQ mmxreg,r/m64        ; 0F 6A /r             [PENT,MMX]

\c PUNPCKLBW mmxreg,r/m64        ; 0F 60 /r             [PENT,MMX]

\c PUNPCKLWD mmxreg,r/m64        ; 0F 61 /r             [PENT,MMX]

\c PUNPCKLDQ mmxreg,r/m64        ; 0F 62 /r             [PENT,MMX]

\c{PUNPCKxx} all treat their operands as vectors, and produce a new

vector generated by interleaving elements from the two inputs. The

\c{PUNPCKHxx} instructions start by throwing away the bottom half of

each input operand, and the \c{PUNPCKLxx} instructions throw away

the top half.

The remaining elements, totalling 64 bits, are then interleaved into

the destination, alternating elements from the second (source)

operand and the first (destination) operand: so the leftmost element

in the result always comes from the second operand, and the

rightmost from the destination.

\c{PUNPCKxBW} works a byte at a time, \c{PUNPCKxWD} a word at a

time, and \c{PUNPCKxDQ} a doubleword at a time.

So, for example, if the first operand held \c{0x7A6A5A4A3A2A1A0A}

and the second held \c{0x7B6B5B4B3B2B1B0B}, then:

\b \c{PUNPCKHBW} would return \c{0x7B7A6B6A5B5A4B4A}.

\b \c{PUNPCKHWD} would return \c{0x7B6B7A6A5B4B5A4A}.

\b \c{PUNPCKHDQ} would return \c{0x7B6B5B4B7A6A5A4A}.

\b \c{PUNPCKLBW} would return \c{0x3B3A2B2A1B1A0B0A}.

\b \c{PUNPCKLWD} would return \c{0x3B2B3A2A1B0B1A0A}.

\b \c{PUNPCKLDQ} would return \c{0x3B2B1B0B3A2A1A0A}.

\H{insPUSH} \i\c{PUSH}: Push Data on Stack

\c PUSH reg16                    ; o16 50+r             [8086]

\c PUSH reg32                    ; o32 50+r             [386]

\c PUSH r/m16                    ; o16 FF /6            [8086]

\c PUSH r/m32                    ; o32 FF /6            [386]

\c PUSH CS                       ; 0E                   [8086]

\c PUSH DS                       ; 1E                   [8086]

\c PUSH ES                       ; 06                   [8086]

\c PUSH SS                       ; 16                   [8086]

\c PUSH FS                       ; 0F A0                [386]

\c PUSH GS                       ; 0F A8                [386]

\c PUSH imm8                     ; 6A ib                [286]

\c PUSH imm16                    ; o16 68 iw            [286]

\c PUSH imm32                    ; o32 68 id            [386]

\c{PUSH} decrements the stack pointer (\c{SP} or \c{ESP}) by 2 or 4,

and then stores the given value at \c{[SS:SP]} or \c{[SS:ESP]}.

The address-size attribute of the instruction determines whether

\c{SP} or \c{ESP} is used as the stack pointer: to deliberately

override the default given by the \c{BITS} setting, you can use an

\i\c{a16} or \i\c{a32} prefix.

The operand-size attribute of the instruction determines whether the

stack pointer is decremented by 2 or 4: this means that segment

register pushes in \c{BITS 32} mode will push 4 bytes on the stack,

of which the upper two are undefined. If you need to override that,

you can use an \i\c{o16} or \i\c{o32} prefix.

The above opcode listings give two forms for general-purpose

\i{register push} instructions: for example, \c{PUSH BX} has the two

forms \c{53} and \c{FF F3}. NASM will always generate the shorter

form when given \c{PUSH BX}. NDISASM will disassemble both.

Unlike the undocumented and barely supported \c{POP CS}, \c{PUSH CS}

is a perfectly valid and sensible instruction, supported on all

processors.

The instruction \c{PUSH SP} may be used to distinguish an 8086 from

later processors: on an 8086, the value of \c{SP} stored is the

value it has \e{after} the push instruction, whereas on later

processors it is the value \e{before} the push instruction.

\H{insPUSHA} \i\c{PUSHAx}: Push All General-Purpose Registers

\c PUSHA                         ; 60                   [186]

\c PUSHAD                        ; o32 60               [386]

\c PUSHAW                        ; o16 60               [186]

\c{PUSHAW} pushes, in succession, \c{AX}, \c{CX}, \c{DX}, \c{BX},

\c{SP}, \c{BP}, \c{SI} and \c{DI} on the stack, decrementing the

stack pointer by a total of 16.

\c{PUSHAD} pushes, in succession, \c{EAX}, \c{ECX}, \c{EDX},

\c{EBX}, \c{ESP}, \c{EBP}, \c{ESI} and \c{EDI} on the stack,

decrementing the stack pointer by a total of 32.

In both cases, the value of \c{SP} or \c{ESP} pushed is its

\e{original} value, as it had before the instruction was executed.

\c{PUSHA} is an alias mnemonic for either \c{PUSHAW} or \c{PUSHAD},

depending on the current \c{BITS} setting.

Note that the registers are pushed in order of their numeric values

in opcodes (see \k{iref-rv}).

See also \c{POPA} (\k{insPOPA}).

\H{insPUSHF} \i\c{PUSHFx}: Push Flags Register

\c PUSHF                         ; 9C                   [186]

\c PUSHFD                        ; o32 9C               [386]

\c PUSHFW                        ; o16 9C               [186]

\c{PUSHFW} pops a word from the stack and stores it in the bottom 16

bits of the flags register (or the whole flags register, on

processors below a 386). \c{PUSHFD} pops a doubleword and stores it

in the entire flags register.

\c{PUSHF} is an alias mnemonic for either \c{PUSHFW} or \c{PUSHFD},

depending on the current \c{BITS} setting.

See also \c{POPF} (\k{insPOPF}).

\H{insPXOR} \i\c{PXOR}: MMX Bitwise XOR

\c PXOR mmxreg,r/m64             ; 0F EF /r             [PENT,MMX]

\c{PXOR} performs a bitwise XOR operation between its two operands

(i.e. each bit of the result is 1 if and only if exactly one of the

corresponding bits of the two inputs was 1), and stores the result

in the destination (first) operand.

\H{insRCL} \i\c{RCL}, \i\c{RCR}: Bitwise Rotate through Carry Bit

\c RCL r/m8,1                    ; D0 /2                [8086]

\c RCL r/m8,CL                   ; D2 /2                [8086]

\c RCL r/m8,imm8                 ; C0 /2 ib             [286]

\c RCL r/m16,1                   ; o16 D1 /2            [8086]

\c RCL r/m16,CL                  ; o16 D3 /2            [8086]

\c RCL r/m16,imm8                ; o16 C1 /2 ib         [286]

\c RCL r/m32,1                   ; o32 D1 /2            [386]

\c RCL r/m32,CL                  ; o32 D3 /2            [386]

\c RCL r/m32,imm8                ; o32 C1 /2 ib         [386]

\c RCR r/m8,1                    ; D0 /3                [8086]

\c RCR r/m8,CL                   ; D2 /3                [8086]

\c RCR r/m8,imm8                 ; C0 /3 ib             [286]

\c RCR r/m16,1                   ; o16 D1 /3            [8086]

\c RCR r/m16,CL                  ; o16 D3 /3            [8086]

\c RCR r/m16,imm8                ; o16 C1 /3 ib         [286]

\c RCR r/m32,1                   ; o32 D1 /3            [386]

\c RCR r/m32,CL                  ; o32 D3 /3            [386]

\c RCR r/m32,imm8                ; o32 C1 /3 ib         [386]

\c{RCL} and \c{RCR} perform a 9-bit, 17-bit or 33-bit bitwise

rotation operation, involving the given source/destination (first)

operand and the carry bit. Thus, for example, in the operation

\c{RCR AL,1}, a 9-bit rotation is performed in which \c{AL} is

shifted left by 1, the top bit of \c{AL} moves into the carry flag,

and the original value of the carry flag is placed in the low bit of

\c{AL}.

The number of bits to rotate by is given by the second operand. Only

the bottom five bits of the rotation count are considered by

processors above the 8086.

You can force the longer (286 and upwards, beginning with a \c{C1}

byte) form of \c{RCL foo,1} by using a \c{BYTE} prefix: \c{RCL

foo,BYTE 1}. Similarly with \c{RCR}.

\H{insRDMSR} \i\c{RDMSR}: Read Model-Specific Registers

\c RDMSR                         ; 0F 32                [PENT]

\c{RDMSR} reads the processor Model-Specific Register (MSR) whose

index is stored in \c{ECX}, and stores the result in \c{EDX:EAX}.

See also \c{WRMSR} (\k{insWRMSR}).

\H{insRDPMC} \i\c{RDPMC}: Read Performance-Monitoring Counters

\c RDPMC                         ; 0F 33                [P6]

\c{RDPMC} reads the processor performance-monitoring counter whose

index is stored in \c{ECX}, and stores the result in \c{EDX:EAX}.

\H{insRDTSC} \i\c{RDTSC}: Read Time-Stamp Counter

\c RDTSC                         ; 0F 31                [PENT]

\c{RDTSC} reads the processor's time-stamp counter into \c{EDX:EAX}.

\H{insRET} \i\c{RET}, \i\c{RETF}, \i\c{RETN}: Return from Procedure Call

\c RET                           ; C3                   [8086]

\c RET imm16                     ; C2 iw                [8086]

\c RETF                          ; CB                   [8086]

\c RETF imm16                    ; CA iw                [8086]

\c RETN                          ; C3                   [8086]

\c RETN imm16                    ; C2 iw                [8086]

\c{RET}, and its exact synonym \c{RETN}, pop \c{IP} or \c{EIP} from

the stack and transfer control to the new address. Optionally, if a

numeric second operand is provided, they increment the stack pointer

by a further \c{imm16} bytes after popping the return address.

\c{RETF} executes a far return: after popping \c{IP}/\c{EIP}, it

then pops \c{CS}, and \e{then} increments the stack pointer by the

optional argument if present.

\H{insROL} \i\c{ROL}, \i\c{ROR}: Bitwise Rotate

\c ROL r/m8,1                    ; D0 /0                [8086]

\c ROL r/m8,CL                   ; D2 /0                [8086]

\c ROL r/m8,imm8                 ; C0 /0 ib             [286]

\c ROL r/m16,1                   ; o16 D1 /0            [8086]

\c ROL r/m16,CL                  ; o16 D3 /0            [8086]

\c ROL r/m16,imm8                ; o16 C1 /0 ib         [286]

\c ROL r/m32,1                   ; o32 D1 /0            [386]

\c ROL r/m32,CL                  ; o32 D3 /0            [386]

\c ROL r/m32,imm8                ; o32 C1 /0 ib         [386]

\c ROR r/m8,1                    ; D0 /1                [8086]

\c ROR r/m8,CL                   ; D2 /1                [8086]

\c ROR r/m8,imm8                 ; C0 /1 ib             [286]

\c ROR r/m16,1                   ; o16 D1 /1            [8086]

\c ROR r/m16,CL                  ; o16 D3 /1            [8086]

\c ROR r/m16,imm8                ; o16 C1 /1 ib         [286]

\c ROR r/m32,1                   ; o32 D1 /1            [386]

\c ROR r/m32,CL                  ; o32 D3 /1            [386]

\c ROR r/m32,imm8                ; o32 C1 /1 ib         [386]

\c{ROL} and \c{ROR} perform a bitwise rotation operation on the given

source/destination (first) operand. Thus, for example, in the

operation \c{ROR AL,1}, an 8-bit rotation is performed in which

\c{AL} is shifted left by 1 and the original top bit of \c{AL} moves

round into the low bit.

The number of bits to rotate by is given by the second operand. Only

the bottom 3, 4 or 5 bits (depending on the source operand size) of

the rotation count are considered by processors above the 8086.

You can force the longer (286 and upwards, beginning with a \c{C1}

byte) form of \c{ROL foo,1} by using a \c{BYTE} prefix: \c{ROL

foo,BYTE 1}. Similarly with \c{ROR}.

\H{insRSM} \i\c{RSM}: Resume from System-Management Mode

\c RSM                           ; 0F AA                [PENT]

\c{RSM} returns the processor to its normal operating mode when it

was in System-Management Mode.

\H{insSAHF} \i\c{SAHF}: Store AH to Flags

\c SAHF                          ; 9E                   [8086]

\c{SAHF} sets the low byte of the flags word according to the

contents of the \c{AH} register. See also \c{LAHF} (\k{insLAHF}).

\H{insSAL} \i\c{SAL}, \i\c{SAR}: Bitwise Arithmetic Shifts

\c SAL r/m8,1                    ; D0 /4                [8086]

\c SAL r/m8,CL                   ; D2 /4                [8086]

\c SAL r/m8,imm8                 ; C0 /4 ib             [286]

\c SAL r/m16,1                   ; o16 D1 /4            [8086]

\c SAL r/m16,CL                  ; o16 D3 /4            [8086]

\c SAL r/m16,imm8                ; o16 C1 /4 ib         [286]

\c SAL r/m32,1                   ; o32 D1 /4            [386]

\c SAL r/m32,CL                  ; o32 D3 /4            [386]

\c SAL r/m32,imm8                ; o32 C1 /4 ib         [386]

\c SAR r/m8,1                    ; D0 /0                [8086]

\c SAR r/m8,CL                   ; D2 /0                [8086]

\c SAR r/m8,imm8                 ; C0 /0 ib             [286]

\c SAR r/m16,1                   ; o16 D1 /0            [8086]

\c SAR r/m16,CL                  ; o16 D3 /0            [8086]

\c SAR r/m16,imm8                ; o16 C1 /0 ib         [286]

\c SAR r/m32,1                   ; o32 D1 /0            [386]

\c SAR r/m32,CL                  ; o32 D3 /0            [386]

\c SAR r/m32,imm8                ; o32 C1 /0 ib         [386]

\c{SAL} and \c{SAR} perform an arithmetic shift operation on the given

source/destination (first) operand. The vacated bits are filled with

zero for \c{SAL}, and with copies of the original high bit of the

source operand for \c{SAR}.

\c{SAL} is a synonym for \c{SHL} (see \k{insSHL}). NASM will

assemble either one to the same code, but NDISASM will always

disassemble that code as \c{SHL}.

The number of bits to shift by is given by the second operand. Only

the bottom 3, 4 or 5 bits (depending on the source operand size) of

the shift count are considered by processors above the 8086.

You can force the longer (286 and upwards, beginning with a \c{C1}

byte) form of \c{SAL foo,1} by using a \c{BYTE} prefix: \c{SAL

foo,BYTE 1}. Similarly with \c{SAR}.

\H{insSALC} \i\c{SALC}: Set AL from Carry Flag

\c SALC                          ; D6                   [8086,UNDOC]

\c{SALC} is an early undocumented instruction similar in concept to

\c{SETcc} (\k{insSETcc}). Its function is to set \c{AL} to zero if

the carry flag is clear, or to \c{0xFF} if it is set.

\H{insSBB} \i\c{SBB}: Subtract with Borrow

\c SBB r/m8,reg8                 ; 18 /r                [8086]

\c SBB r/m16,reg16               ; o16 19 /r            [8086]

\c SBB r/m32,reg32               ; o32 19 /r            [386]

\c SBB reg8,r/m8                 ; 1A /r                [8086]

\c SBB reg16,r/m16               ; o16 1B /r            [8086]

\c SBB reg32,r/m32               ; o32 1B /r            [386]

\c SBB r/m8,imm8                 ; 80 /3 ib             [8086]

\c SBB r/m16,imm16               ; o16 81 /3 iw         [8086]

\c SBB r/m32,imm32               ; o32 81 /3 id         [386]

\c SBB r/m16,imm8                ; o16 83 /3 ib         [8086]

\c SBB r/m32,imm8                ; o32 83 /3 ib         [8086]

\c SBB AL,imm8                   ; 1C ib                [8086]

\c SBB AX,imm16                  ; o16 1D iw            [8086]

\c SBB EAX,imm32                 ; o32 1D id            [386]

\c{SBB} performs integer subtraction: it subtracts its second

operand, plus the value of the carry flag, from its first, and

leaves the result in its destination (first) operand. The flags are

set according to the result of the operation: in particular, the

carry flag is affected and can be used by a subsequent \c{SBB}

instruction.

In the forms with an 8-bit immediate second operand and a longer

first operand, the second operand is considered to be signed, and is

sign-extended to the length of the first operand. In these cases,

the \c{BYTE} qualifier is necessary to force NASM to generate this

form of the instruction.

To subtract one number from another without also subtracting the

contents of the carry flag, use \c{SUB} (\k{insSUB}).

\H{insSCASB} \i\c{SCASB}, \i\c{SCASW}, \i\c{SCASD}: Scan String

\c SCASB                         ; AE                   [8086]

\c SCASW                         ; o16 AF               [8086]

\c SCASD                         ; o32 AF               [386]

\c{SCASB} compares the byte in \c{AL} with the byte at \c{[ES:DI]}

or \c{[ES:EDI]}, and sets the flags accordingly. It then increments

or decrements (depending on the direction flag: increments if the

flag is clear, decrements if it is set) \c{DI} (or \c{EDI}).

The register used is \c{DI} if the address size is 16 bits, and

\c{EDI} if it is 32 bits. If you need to use an address size not

equal to the current \c{BITS} setting, you can use an explicit

\i\c{a16} or \i\c{a32} prefix.

Segment override prefixes have no effect for this instruction: the

use of \c{ES} for the load from \c{[DI]} or \c{[EDI]} cannot be

overridden.

\c{SCASW} and \c{SCASD} work in the same way, but they compare a

word to \c{AX} or a doubleword to \c{EAX} instead of a byte to

\c{AL}, and increment or decrement the addressing registers by 2 or

4 instead of 1.

The \c{REPE} and \c{REPNE} prefixes (equivalently, \c{REPZ} and

\c{REPNZ}) may be used to repeat the instruction up to \c{CX} (or

\c{ECX} - again, the address size chooses which) times until the

first unequal or equal byte is found.

\H{insSETcc} \i\c{SETcc}: Set Register from Condition

\c SETcc r/m8                    ; 0F 90+cc /2          [386]

\c{SETcc} sets the given 8-bit operand to zero if its condition is

not satisfied, and to 1 if it is.

\H{insSGDT} \i\c{SGDT}, \i\c{SIDT}, \i\c{SLDT}: Store Descriptor Table Pointers

\c SGDT mem                      ; 0F 01 /0             [286,PRIV]

\c SIDT mem                      ; 0F 01 /1             [286,PRIV]

\c SLDT r/m16                    ; 0F 00 /0             [286,PRIV]

\c{SGDT} and \c{SIDT} both take a 6-byte memory area as an operand:

they store the contents of the GDTR (global descriptor table

register) or IDTR (interrupt descriptor table register) into that

area as a 32-bit linear address and a 16-bit size limit from that

area (in that order). These are the only instructions which directly

use \e{linear} addresses, rather than segment/offset pairs.

\c{SLDT} stores the segment selector corresponding to the LDT (local

descriptor table) into the given operand.

See also \c{LGDT}, \c{LIDT} and \c{LLDT} (\k{insLGDT}).

\H{insSHL} \i\c{SHL}, \i\c{SHR}: Bitwise Logical Shifts

\c SHL r/m8,1                    ; D0 /4                [8086]

\c SHL r/m8,CL                   ; D2 /4                [8086]

\c SHL r/m8,imm8                 ; C0 /4 ib             [286]

\c SHL r/m16,1                   ; o16 D1 /4            [8086]

\c SHL r/m16,CL                  ; o16 D3 /4            [8086]

\c SHL r/m16,imm8                ; o16 C1 /4 ib         [286]

\c SHL r/m32,1                   ; o32 D1 /4            [386]

\c SHL r/m32,CL                  ; o32 D3 /4            [386]

\c SHL r/m32,imm8                ; o32 C1 /4 ib         [386]

\c SHR r/m8,1                    ; D0 /5                [8086]

\c SHR r/m8,CL                   ; D2 /5                [8086]

\c SHR r/m8,imm8                 ; C0 /5 ib             [286]

\c SHR r/m16,1                   ; o16 D1 /5            [8086]

\c SHR r/m16,CL                  ; o16 D3 /5            [8086]

\c SHR r/m16,imm8                ; o16 C1 /5 ib         [286]

\c SHR r/m32,1                   ; o32 D1 /5            [386]

\c SHR r/m32,CL                  ; o32 D3 /5            [386]

\c SHR r/m32,imm8                ; o32 C1 /5 ib         [386]

\c{SHL} and \c{SHR} perform a logical shift operation on the given

source/destination (first) operand. The vacated bits are filled with

zero.

A synonym for \c{SHL} is \c{SAL} (see \k{insSAL}). NASM will

assemble either one to the same code, but NDISASM will always

disassemble that code as \c{SHL}.

The number of bits to shift by is given by the second operand. Only

the bottom 3, 4 or 5 bits (depending on the source operand size) of

the shift count are considered by processors above the 8086.

You can force the longer (286 and upwards, beginning with a \c{C1}

byte) form of \c{SHL foo,1} by using a \c{BYTE} prefix: \c{SHL

foo,BYTE 1}. Similarly with \c{SHR}.

\H{insSHLD} \i\c{SHLD}, \i\c{SHRD}: Bitwise Double-Precision Shifts

\c SHLD r/m16,reg16,imm8         ; o16 0F A4 /r ib      [386]

\c SHLD r/m16,reg32,imm8         ; o32 0F A4 /r ib      [386]

\c SHLD r/m16,reg16,CL           ; o16 0F A5 /r         [386]

\c SHLD r/m16,reg32,CL           ; o32 0F A5 /r         [386]

\c SHRD r/m16,reg16,imm8         ; o16 0F AC /r ib      [386]

\c SHRD r/m32,reg32,imm8         ; o32 0F AC /r ib      [386]

\c SHRD r/m16,reg16,CL           ; o16 0F AD /r         [386]

\c SHRD r/m32,reg32,CL           ; o32 0F AD /r         [386]

\c{SHLD} performs a double-precision left shift. It notionally places

its second operand to the right of its first, then shifts the entire

bit string thus generated to the left by a number of bits specified

in the third operand. It then updates only the \e{first} operand

according to the result of this. The second operand is not modified.

\c{SHRD} performs the corresponding right shift: it notionally

places the second operand to the \e{left} of the first, shifts the

whole bit string right, and updates only the first operand.

For example, if \c{EAX} holds \c{0x01234567} and \c{EBX} holds

\c{0x89ABCDEF}, then the instruction \c{SHLD EAX,EBX,4} would update

\c{EAX} to hold \c{0x12345678}. Under the same conditions, \c{SHRD

EAX,EBX,4} would update \c{EAX} to hold \c{0xF0123456}.

The number of bits to shift by is given by the third operand. Only

the bottom 5 bits of the shift count are considered.

\H{insSMI} \i\c{SMI}: System Management Interrupt

\c SMI                           ; F1                   [386,UNDOC]

This is an opcode apparently supported by some AMD processors (which

is why it can generate the same opcode as \c{INT1}), and places the

machine into system-management mode, a special debugging mode.

\H{insSMSW} \i\c{SMSW}: Store Machine Status Word

\c SMSW r/m16                    ; 0F 01 /4             [286,PRIV]

\c{SMSW} stores the bottom half of the \c{CR0} control register (or

the Machine Status Word, on 286 processors) into the destination

operand. See also \c{LMSW} (\k{insLMSW}).

\H{insSTC} \i\c{STC}, \i\c{STD}, \i\c{STI}: Set Flags

\c STC                           ; F9                   [8086]

\c STD                           ; FD                   [8086]

\c STI                           ; FB                   [8086]

These instructions set various flags. \c{STC} sets the carry flag;

\c{STD} sets the direction flag; and \c{STI} sets the interrupt flag

(thus enabling interrupts).

To clear the carry, direction, or interrupt flags, use the \c{CLC},

\c{CLD} and \c{CLI} instructions (\k{insCLC}). To invert the carry

flag, use \c{CMC} (\k{insCMC}).

\H{insSTOSB} \i\c{STOSB}, \i\c{STOSW}, \i\c{STOSD}: Store Byte to String

\c STOSB                         ; AA                   [8086]

\c STOSW                         ; o16 AB               [8086]

\c STOSD                         ; o32 AB               [386]

\c{STOSB} stores the byte in \c{AL} at \c{[ES:DI]} or \c{[ES:EDI]},

and sets the flags accordingly. It then increments or decrements

(depending on the direction flag: increments if the flag is clear,

decrements if it is set) \c{DI} (or \c{EDI}).

The register used is \c{DI} if the address size is 16 bits, and

\c{EDI} if it is 32 bits. If you need to use an address size not

equal to the current \c{BITS} setting, you can use an explicit

\i\c{a16} or \i\c{a32} prefix.

Segment override prefixes have no effect for this instruction: the

use of \c{ES} for the store to \c{[DI]} or \c{[EDI]} cannot be

overridden.

\c{STOSW} and \c{STOSD} work in the same way, but they store the

word in \c{AX} or the doubleword in \c{EAX} instead of the byte in

\c{AL}, and increment or decrement the addressing registers by 2 or

4 instead of 1.

The \c{REP} prefix may be used to repeat the instruction \c{CX} (or

\c{ECX} - again, the address size chooses which) times.

\H{insSTR} \i\c{STR}: Store Task Register

\c STR r/m16                     ; 0F 00 /1             [286,PRIV]

\c{STR} stores the segment selector corresponding to the contents of

the Task Register into its operand.

\H{insSUB} \i\c{SUB}: Subtract Integers

\c SUB r/m8,reg8                 ; 28 /r                [8086]

\c SUB r/m16,reg16               ; o16 29 /r            [8086]

\c SUB r/m32,reg32               ; o32 29 /r            [386]

\c SUB reg8,r/m8                 ; 2A /r                [8086]

\c SUB reg16,r/m16               ; o16 2B /r            [8086]

\c SUB reg32,r/m32               ; o32 2B /r            [386]

\c SUB r/m8,imm8                 ; 80 /5 ib             [8086]

\c SUB r/m16,imm16               ; o16 81 /5 iw         [8086]

\c SUB r/m32,imm32               ; o32 81 /5 id         [386]

\c SUB r/m16,imm8                ; o16 83 /5 ib         [8086]

\c SUB r/m32,imm8                ; o32 83 /5 ib         [386]

\c SUB AL,imm8                   ; 2C ib                [8086]

\c SUB AX,imm16                  ; o16 2D iw            [8086]

\c SUB EAX,imm32                 ; o32 2D id            [386]

\c{SUB} performs integer subtraction: it subtracts its second

operand from its first, and leaves the result in its destination

(first) operand. The flags are set according to the result of the

operation: in particular, the carry flag is affected and can be used

by a subsequent \c{SBB} instruction (\k{insSBB}).

In the forms with an 8-bit immediate second operand and a longer

first operand, the second operand is considered to be signed, and is

sign-extended to the length of the first operand. In these cases,

the \c{BYTE} qualifier is necessary to force NASM to generate this

form of the instruction.

\H{insTEST} \i\c{TEST}: Test Bits (notional bitwise AND)

\c TEST r/m8,reg8                ; 84 /r                [8086]

\c TEST r/m16,reg16              ; o16 85 /r            [8086]

\c TEST r/m32,reg32              ; o32 85 /r            [386]

\c TEST r/m8,imm8                ; F6 /7 ib             [8086]

\c TEST r/m16,imm16              ; o16 F7 /7 iw         [8086]

\c TEST r/m32,imm32              ; o32 F7 /7 id         [386]

\c TEST AL,imm8                  ; A8 ib                [8086]

\c TEST AX,imm16                 ; o16 A9 iw            [8086]

\c TEST EAX,imm32                ; o32 A9 id            [386]

\c{TEST} performs a `mental' bitwise AND of its two operands, and

affects the flags as if the operation had taken place, but does not

store the result of the operation anywhere.

\H{insUMOV} \i\c{UMOV}: User Move Data

\c UMOV r/m8,reg8                ; 0F 10 /r             [386,UNDOC]

\c UMOV r/m16,reg16              ; o16 0F 11 /r         [386,UNDOC]

\c UMOV r/m32,reg32              ; o32 0F 11 /r         [386,UNDOC]

\c UMOV reg8,r/m8                ; 0F 12 /r             [386,UNDOC]

\c UMOV reg16,r/m16              ; o16 0F 13 /r         [386,UNDOC]

\c UMOV reg32,r/m32              ; o32 0F 13 /r         [386,UNDOC]

This undocumented instruction is used by in-circuit emulators to

access user memory (as opposed to host memory). It is used just like

an ordinary memory/register or register/register \c{MOV}

instruction, but accesses user space.

\H{insVERR} \i\c{VERR}, \i\c{VERW}: Verify Segment Readability/Writability

\c VERR r/m16                    ; 0F 00 /4             [286,PRIV]

\c VERW r/m16                    ; 0F 00 /5             [286,PRIV]

\c{VERR} sets the zero flag if the segment specified by the selector

in its operand can be read from at the current privilege level.

\c{VERW} sets the zero flag if the segment can be written.

\H{insWAIT} \i\c{WAIT}: Wait for Floating-Point Processor

\c WAIT                          ; 9B                   [8086]

\c{WAIT}, on 8086 systems with a separate 8087 FPU, waits for the

FPU to have finished any operation it is engaged in before

continuing main processor operations, so that (for example) an FPU

store to main memory can be guaranteed to have completed before the

CPU tries to read the result back out.

On higher processors, \c{WAIT} is unnecessary for this purpose, and

it has the alternative purpose of ensuring that any pending unmasked

FPU exceptions have happened before execution continues.

\H{insWBINVD} \i\c{WBINVD}: Write Back and Invalidate Cache

\c WBINVD                        ; 0F 09                [486]

\c{WBINVD} invalidates and empties the processor's internal caches,

and causes the processor to instruct external caches to do the same.

It writes the contents of the caches back to memory first, so no

data is lost. To flush the caches quickly without bothering to write

the data back first, use \c{INVD} (\k{insINVD}).

\H{insWRMSR} \i\c{WRMSR}: Write Model-Specific Registers

\c WRMSR                         ; 0F 30                [PENT]

\c{WRMSR} writes the value in \c{EDX:EAX} to the processor

Model-Specific Register (MSR) whose index is stored in \c{ECX}. See

also \c{RDMSR} (\k{insRDMSR}).

\H{insXADD} \i\c{XADD}: Exchange and Add

\c XADD r/m8,reg8                ; 0F C0 /r             [486]

\c XADD r/m16,reg16              ; o16 0F C1 /r         [486]

\c XADD r/m32,reg32              ; o32 0F C1 /r         [486]

\c{XADD} exchanges the values in its two operands, and then adds

them together and writes the result into the destination (first)

operand. This instruction can be used with a \c{LOCK} prefix for

multi-processor synchronisation purposes.

\H{insXBTS} \i\c{XBTS}: Extract Bit String

\c XBTS reg16,r/m16              ; o16 0F A6 /r         [386,UNDOC]

\c XBTS reg32,r/m32              ; o32 0F A6 /r         [386,UNDOC]

No clear documentation seems to be available for this instruction:

the best I've been able to find reads `Takes a string of bits from

the first operand and puts them in the second operand'. It is

present only in early 386 processors, and conflicts with the opcodes

for \c{CMPXCHG486}. NASM supports it only for completeness. Its

counterpart is \c{IBTS} (see \k{insIBTS}).

\H{insXCHG} \i\c{XCHG}: Exchange

\c XCHG reg8,r/m8                ; 86 /r                [8086]

\c XCHG reg16,r/m8               ; o16 87 /r            [8086]

\c XCHG reg32,r/m32              ; o32 87 /r            [386]

\c XCHG r/m8,reg8                ; 86 /r                [8086]

\c XCHG r/m16,reg16              ; o16 87 /r            [8086]

\c XCHG r/m32,reg32              ; o32 87 /r            [386]

\c XCHG AX,reg16                 ; o16 90+r             [8086]

\c XCHG EAX,reg32                ; o32 90+r             [386]

\c XCHG reg16,AX                 ; o16 90+r             [8086]

\c XCHG reg32,EAX                ; o32 90+r             [386]

\c{XCHG} exchanges the values in its two operands. It can be used

with a \c{LOCK} prefix for purposes of multi-processor

synchronisation.

\c{XCHG AX,AX} or \c{XCHG EAX,EAX} (depending on the \c{BITS}

setting) generates the opcode \c{90h}, and so is a synonym for

\c{NOP} (\k{insNOP}).

\H{insXLATB} \i\c{XLATB}: Translate Byte in Lookup Table

\c XLATB                         ; D7                   [8086]

\c{XLATB} adds the value in \c{AL}, treated as an unsigned byte, to

\c{BX} or \c{EBX}, and loads the byte from the resulting address (in

the segment specified by \c{DS}) back into \c{AL}.

The base register used is \c{BX} if the address size is 16 bits, and

\c{EBX} if it is 32 bits. If you need to use an address size not

equal to the current \c{BITS} setting, you can use an explicit

\i\c{a16} or \i\c{a32} prefix.

The segment register used to load from \c{[BX+AL]} or \c{[EBX+AL]}

can be overridden by using a segment register name as a prefix (for

example, \c{es xlatb}).

\H{insXOR} \i\c{XOR}: Bitwise Exclusive OR

\c XOR r/m8,reg8                 ; 30 /r                [8086]

\c XOR r/m16,reg16               ; o16 31 /r            [8086]

\c XOR r/m32,reg32               ; o32 31 /r            [386]

\c XOR reg8,r/m8                 ; 32 /r                [8086]

\c XOR reg16,r/m16               ; o16 33 /r            [8086]

\c XOR reg32,r/m32               ; o32 33 /r            [386]

\c XOR r/m8,imm8                 ; 80 /6 ib             [8086]

\c XOR r/m16,imm16               ; o16 81 /6 iw         [8086]

\c XOR r/m32,imm32               ; o32 81 /6 id         [386]

\c XOR r/m16,imm8                ; o16 83 /6 ib         [8086]

\c XOR r/m32,imm8                ; o32 83 /6 ib         [386]

\c XOR AL,imm8                   ; 34 ib                [8086]

\c XOR AX,imm16                  ; o16 35 iw            [8086]

\c XOR EAX,imm32                 ; o32 35 id            [386]

\c{XOR} performs a bitwise XOR operation between its two operands

(i.e. each bit of the result is 1 if and only if exactly one of the

corresponding bits of the two inputs was 1), and stores the result

in the destination (first) operand.

In the forms with an 8-bit immediate second operand and a longer

first operand, the second operand is considered to be signed, and is

sign-extended to the length of the first operand. In these cases,

the \c{BYTE} qualifier is necessary to force NASM to generate this

form of the instruction.

The MMX instruction \c{PXOR} (see \k{insPXOR}) performs the same

operation on the 64-bit MMX registers.