The programs in this file were written by Steve Gibson of

Gibson Research Corp  Box 6024  Irvine, Ca  92716

The programs will determine the hard disk performance of any IBM

compatable computer.  These programs have been tested and found to

be safe and NOT destroy data on any hard disk.  However since this

program is made available on BBS's, no guarantee is made if downloaded

from any board other than the Riverside Premium BBS 312-447-8175 where

I personally entered the code and tested the output.

If you obtain this program from any other source I would recommend that

you take normal precautions (BACKUPS).

The source code for creating the COM files is included and was compiled

under QuickBasic version

The programs included are;

SPINTEST and SPINTIME

SPINTEST determines exactly how many revolutions your hard disk

currently requires for full track transfers.  Dos transfers

a full track whenever programs or files larger than 9k are

read or written... which is most of the time.  SPINTEST

does NOT directly read your drive's interleave, but its

interleaving performance can be easily inferred from

SPINTEST's full track transfer revolution count.

SPINTEST  averages the time required for each of over two hundred full

track reads in order to accurately determine the revolution

count per read, then the number of revolutions required to

read just one track and the controller's full-track data

transfer rate are computed.  SPINTEST only READS data from your

drive so data is never altered.

Over two hundred track reads were used as a result of the

inconsistent disk transfer behavior of AT&T's 6300 machines.

These exhibit a maddening inconsistency in their ability

to transfer disk data.  Sometimes they get the next sector

and often not.  This means that a much looser than normal

interleave generates the best overall throughput for AT&Ts.

For this reason, and to guarantee correct results on any

machines which might behave similary, SPINTEST performs

many track reads and averages the results.

SPINTIME  Since SPINTEST measures track read time, it must assume a

given ratio between elapsed time and disk rotation rate.

Some weirdo clone computers have a clock which runs faster

than normal when in TURBO (8mz) mode, and some lap computers

have tiny hard disks spinning at strange speeds.  If these

do not sound like your situation SPINTEST will deliver

correct readings and you'll not need SPINTIME's confirming

measurements.

But if EITHER of these cases might be you SPINTIME will tell

you for sure.  SPINTEST is only useable when SPINTIME gives st

standard readings.  The single exception to this is for the

AT&T 6300 and 6300 Plus machines.  Due to the overall

problems they experience with disk transfers, SPINTIME may

show a lower-than-3600 RPM reading.  For these machines

this is acceptable.

SPINTIME is interesting since it determines EXACTLY how fast

your hard disk is really spinning.

RUN TIMES

SPINTEST

The time required to run this test will vary between

approximately 14 seconds for a fast 2:1 interleave to

98 seconds for an incorrectly interleaved 26-sector

RLL encoded hard disk.  So simply type:  SPINTEST at the

DOS prompt and wait a minute or two.  The results will

be displayed on the screen.

SPINTIME

This program has two purposes:  To determine the speed of

your system's clock, and to verify that your system's hard

disk is spinning at close to the 3600 RPM standard.

Your system's clock is checked by timing SPINTIME's exact

execution time.  It should require EXACTLY 60 SECONDS to

run, no matter what.  During that time it is busy watching

your hard disk spin, counting every revolution.  After

EXACTLY 60 seconds, it displays your drives exact RPM

(within its measuring resolution).  If SPINTIME's total

execution time is NOT EXACTLY 60 seconds or the displayed

RPM is not close to 3600, SPINTEST's reported revolution

count won't mean anything either.

SPINTIME's real value is to assure you that SPINTEST is

delivering worthwhile answers.

IMPORTANT NOTES

Both programs assume and require DOS 2.0 or higher.  They

operate on your FIRST (C) drive only.  It should not require

any special CONFIG.SYS drivers for operation.

The tests will also be more accurate if you do not have any

memory resident programs active.  Many of these resident

programs "steal" so much time from the computer that a

delicate timing analysis could be upset.  Also turn off

or remove any disk caching software.  Disk caching intercepts

the disk reading performed by the programs and renders their

measurements invalid.  By the way, disk caching program

performance is significantly improved through proper

disk interleaving.

My thanks to Steve Gibson for providing these programs.

As an additional note, Gibson Research has designed a quick and

painless software product which handles the entire chore of optimizing

and resetting a hard disk's sector interleave while leaving all

existing data in place.  The product will also perform several

other functions relating to assuring an MFM or RLL drive's data integrity.

The product is expected to be available for shipping within the next

two weeks for $29.  If you are interested in more information on

this new product or FlickerFree contact

Steve Gibson

Gibson Research Corp

Box 6024

Irvine, CA 92716

Be sure to mention you heard about it from the Riverside Premium BBS

in Riverside, IL  312-447-8175

May all your disks live longer and spin faster.   Spock