.H 1 Draw

Draw is a graphic editor for the Tektronics 4014 terminal.

It enables the operator to draw lines, arcs, circles and to define

and use modules of common shapes or use modules from standard libraries.

.P

The program has a number of simple commands to setup global

parameters, and perform desired tasks, plus an editor subsystem

to create the drawing.

.P

A drawing is a module which consists of a list of primitives

or other module names each with an x,y coordinate pair

which defines the position of the element within the module.

The x,y coordinate specifies the start of a line, the centre

of a circle or the point which will become the zero of

a submodule when it is to be drawn.

A module may include a module which includes another module

to any depth but must not reference a module in the current

path.

.P

The drawings may consist of a number of levels numbered

1 to 9.

Each level is totally distinct and may be worked on

separately or may be combined so that each contain

the same drawings.

This is useful in printed circuit layouts with

the tracks on levels 1 and 2 and the chip outlines

on another level.

.P

A drawing may be written to a file or read from a file.

The file may contain a number of drawings which will

each appear as separate modules.

The file is written in ascii form so that it may

easily be 'adapted' or 'modified' in other editors.

.H 1 Commands

.nf

'! shell command'	shell escape

'c'			clear the screen

'e modulename'		edit a module

'grid'			draw grid

'level'			set printing levels

'lib filename'		read from a library file

'list'			list all module names

'new'			reset and read new file (but keep libraries)

'p modulename'		print a module

'plot modulename'	plot the given module on the flatbed.

'q'			quit draw

'r filename'		read from a file

'rename m1 m2'		change the name of a module

'reset'			clear out all modules

'set "size"'			set standard page size

'size "size"'			set character size

'w filename'		write to a file

'wplot'			produce a file for cyber plotting

.fi

.H 2 "The edit command"

e modulename

.br

.P

The edit command uses the cross hairs of the terminal to specify

the coordinates of a particular sub command.

Each sub command consists of a single letter plus the x,y coordinates

of the cross hairs at the time of striking the command letter.

At any time while the cross hairs are displayed the letter '.'

may be struck to mark the screen to show the point that would be used.

The sub commands for edit mode are:-

.VL 20 2

.LI !

Escape to the command level. Some commands are not permitted (eg. edit).

a second '!' will escape to a shell.

.LI a

Draw an arc.

.br

An arc is drawn through three points starting with the command point

passing through the second point specified by hitting another 'a'

and finishing with the third point again specified by hitting another 'a'.

The arc will be drawn clockwise from the first point

to the third point.

Other characters

will cause the selected point to be marked. It is not necessary for the second

point to actually on the arc itself.

.LI 'c'

Draw a circle.

.br

A circle is drawn with the centre at the command point.

The radius for the circle is then entered as a floating point number

terminated by a <cr>.

.LI 'C\ scale'

Set the default size for new text. Sizes are 3, 2, 1.

.LI 'd'\ modulename

Draw the named module at the specified point but do

not enter it into the module being edited.

.LI 'g'

Draw a grid on the screen. The grid will pass through the origin if the

origin is within the area displayed on the screen otherwise it will be in the

bottom left hand corner.

.LI 'l'

Draw a line.

.br

Start a line which consists on many line segments. Each new segment

will start at the end of the last segment and end a the next point specified

by the striking of a character.

Another letter 'l' will continue the line for a further segment while any other

letter with the exception of '.' will terminate the line at that point.

.LI 'm\ modulename'

Add the named module to the module being edited.

The zero point of the new module will be the point of the command.

.LI 'M'

Move the chosen element to a given point. The destination point is

given by striking any character other then '.' after the M command is issued.

See 'z' for choosing the element.

.LI 'p'

Reprint the module we are currently editing.

.LI 'w [scale]'

\&'Window' move the center of the screen to the point chosen on the drawing.

The picture is scaled by the optional scale factor which defaults to 1.

.LI 'R\ angle'

Set the rotation angle for instantiations of modules.

if the angle is missing the default is taken to be 0.

.LI 's\ string'

Write a string.

.br

A string which is entered after the command letter 's' up to the terminating

<cr> will be entered into the module.

The new string will be added at the current letter size.

The size of the character may be varied by the 'size' command.

.LI 'S\ scale'

Set the default scale for module instantiation. Scale is a floating point

number and if missing will default to 1.0.

.LI 'x'

Mark all reference points with the symbol 'X', it makes deleting easier.

.LI 'z'

Delete the chosen module or module element.

The module or module element whose reference point is the same as

the command point will be deleted. If more than one element is at that

point then the first one encountered in the search will be deleted.

Use the 'x' command to find reference points.

.LI '0'

Reset level numbers.

Reset the current level numbers which are attached to all new

elements added during the edit session following this command.

This command should be followed by one of the following commands.

.LI 1\ 2\ 3\ 4\ 5

.LI 6\ 7\ 8\ 9

Add the specified level into the set of current level numbers.

These commands are cumulative.

.LI 'q'

Quit. Finish the editing session on the current module.

.LE

.H 2 'p'

The named module is displayed.

Only those levels which have been set by the level command will be displayed.

.H 2 'w'

The w command will write the specifications of all modules on to the

named file.

Modules that were defined buy the 'lib' command are not written out

as these are assumed to be library modules.

.H 2 "plot"

The plot command will plot the given module on the flatbed plotter.

If the plotter is not powered on already then turn it on and enter

a null command (ie. a 'cr') to set things up.

.H 2 "the r and lib commands"

Both the 'r' and 'lib' commands read the named files for a list

of module specifications previously produced by the draw program.

The lib command will cause the defined modules to be marked as library elements

to prevent them being written out in the next w command.

Any file that has been

produced by this program may be used as a library without modification.

.H 2 'reset'

Reset clears all current module information to enable work on a new drawing.

.H 2 'new'

The new command will discard all non library modules

and then read the named file.

Only libraries read before the first 'r' command will be saved.

.H 2 'level'

The level command takes a number of single digit arguments

in the range of 1 to 9 and sets the corresponding bit in the level mask

used to determine which parts of a drawing are to be printed.

The level numbers may be in any order with a maximum of 9 arguments allowed

If an error is detected then all modules read in from that file are deleted

and a warning message issued ( this should not happen unless files are hand crafted).

.H 2 'set'

Set the display to a standard size. The sizes are 'a1', 'a2', 'a3', 'a4'.

Draw will set the standard size 'a4' when the program is started.

.H 2 'size'

Set the default character size. Sizes available are '1', '2' and '3' from

largest to smallest.