FMRADIO/2 V2.2

by

Paul Ratcliffe

(c) Nov. 1999

This program is designed for use with the Reveal RA300 FM Radio Card and

also both the old and the new versions of the RadioTrack card manufactured

by AIMS Labs.

The program is freeware and as such:

USE OF THIS PROGRAM IS ENTIRELY AT YOUR OWN RISK.

INTRODUCTION

This document assumes you have already installed the PMRADIO application

and the FMRADIO.SYS device driver - refer to PMRADIO.DOC if not.

The radio is controlled through the IOCtl interface using category

FMRADIO_CAT and the functions described below.

A device handle must be obtained by using DosOpen to open the device

"FMRADIO$". If the device is in use, this will return DEV_INUSE.

As the operation of the device driver is time critical, it is recommended

that you wrap a pair of DosSetPrty() calls around the IOCtl call e.g.

PTIB	ptib;

PPIB	ppib;

ULONG	prty;

DosGetInfoBlocks (&ptib, &ppib);

prty = ptib->tib_ptib2->tib2_ulpri;

DosSetPriority (PRTYS_THREAD, PRTYC_TIMECRITICAL, PRTYD_MAXIMUM, 0);

call DosDevIOCtl as appropriate

DosSetPriority (PRTYS_THREAD, prty >> 8, prty & 0xFF, 0);

Applications should call the GET_VERSION IOCtl as the first thing they do

after opening the device and check for a compatible version. The API is not

guaranteed to remain the same between device driver versions, although it

probably will.

DEFINITIONS

#define	VERSION		0x214

#define	FMRADIO_CAT	0x82

#define	SET_ONOFF	0x41

#define	VOL_UPDOWN	0x42

#define	SET_FREQ	0x43

#define	RESET		0x44

#define	GET_VERSION	0x60

#define	GET_ONOFF	0x61

#define	GET_VOLUME	0x62

#define	GET_FREQ	0x63

#define	GET_TUNED	0x65

#define	DEV_INUSE	0xFF01

#define	INV_FREQ	0xFF02

#define	MIN_FREQ	(87 * 40 + 20)

#define	MAX_FREQ	(108 * 40)

IOCTL FUNCTIONS

GET_VERSION

Entry

Parameters

None

Data

None

Exit

Parameters

None

Data

USHORT - Device Driver version number, currently 0x0214

Returns

0

SET_ONOFF

Entry

Parameters

USHORT - TRUE = Switch On, FALSE = Switch Off

Data

None

Exit

Parameters

None

Data

None

Returns

0

GET_ONOFF

Entry

Parameters

None

Data

None

Exit

Parameters

None

Data

USHORT - TRUE = On, FALSE = Off

Returns

0

VOL_UPDOWN

Entry

Parameters

USHORT - TRUE = Volume Up 1 step, FALSE = Volume Down 1 step

Data

None

Exit

Parameters

None

Data

None

Returns

0

Notes

Altering the volume automatically turns the radio on

GET_VOLUME

Entry

Parameters

None

Data

None

Exit

Parameters

None

Data

USHORT - Volume level

Returns

0

SET_FREQ

Entry

Parameters

USHORT - Frequency in MHz * 40 (radio can be tuned in 25kHz steps)

Data

None

Exit

Parameters

None

Data

None

Returns

INV_FREQ if frequency outside range MIN_FREQ to MAX_FREQ inclusive

0 if valid frequency

GET_FREQ

Entry

Parameters

None

Data

None

Exit

Parameters

None

Data

USHORT - Frequency in MHz * 40

Returns

0

RESET

Entry

Parameters

None

Data

None

Exit

Parameters

None

Data

None

Returns

0

Notes

Sets volume level to 0

GET_TUNED

Entry

Parameters

None

Data

None

Exit

Parameters

None

Data

USHORT - TRUE = Tuned, FALSE = Not tuned

Returns

0

HISTORY

Version 2.2  3-11-99

Support for new version of AIMS Labs. Radiotrack card.

Version 2.1  9-8-97

Allow tuning between 87.5 and 88.0 MHz.

Add "Turn off radio at shutdown" facility.

Version 2.0  19-5-97

Initial release.

OTHER

This program was initially developed under Warp 3 (without Win-OS/2) and

now is developed under Warp 4 using VisualAge C++ 3.0 for OS/2.

If you use this program could you please let me know. Also if you have any

comments, criticisms, bug reports or anything that could be improved.

I will reply to all messages received.

Paul Ratcliffe:	Internet: pmradio@orac.clara.co.uk

Internet: paul.ratcliffe@bbc.co.uk