Version 3.02.09 December 9, 1997

NOTE: This document is also in Beta form right now.

===========================================================================

Special thanks to:

Lance Rose for sharing his results in decoding the format of

SA3 startup files.

Jack Yeazel for the donation of his time in evaluating and

making very helpful suggestions concerning g7to.

Thanks to Brent Hildebrand, the author of Waypoint+ for his

help with the Garmin Protocol in various Garmin GPS

receivers.

===========================================================================

DISCLAIMER(s):

As part of it's functionality, g7to creates MapDoc (*.sa3, sa4 and/or

*.sa5) files from GPS data.  The data may then be displayed overlaid

on a SA3, SA4 or SA5 map.  The data is not real time and should not

be used for critical navigation purposes.

In my opinion, the Lat/Long grid shown in SA3/4/5 is not accurate

enough for critical navigtion applications (and the license agreement

appears to prohibit using maps from SA3 for navigation).  For

instance, in the Corvallis area of Oregon SA3/4 is in error about 150

feet to the East at magnification 16.  For example, plotting a known

Lat/Long on SA3/4 results in a point on the SA map that is ~150 feet

East of it's actual location.  The error varies from place to place

and with the magnitude of the displayed map.  I've heard rumors that

SA3 is up to 1/4 mile off in some areas, but I've not personally

observed an error that large.

There are no guarantees expressed or implied with this g7to and/or

it's documentation.  The package works for me, it may or may not work

for anyone else.  Proper use could corrupt the data stored in your

GPS or on your computer.

The Control point, Proj, Garmin64 and navigate.exe support has not

been fully tested since version 2.05.  I no longer actively use those

formats.

===========================================================================

Document definitions:

download:  Primary data flow is from the GPS to the PC

upload:    Primary data flow is from the PC to the GPS

Unless otherwise stated all references to the GPS-45 apply equally to

all supported Garmin receivers.

To work with g7to the GPS must have it's interface set to GRMN/GRMN.

This program has been tested by the author on the GPS 12XL Software

version 3.51, the GPS II+ Software version 2.04 and the GPS III

Software version 2.02.

Program Features:

G7TO will:

o upload/download Track/Waypoint/Route data to/from a Garmin GPS.

o Data from the GPS may be displayed, redirected to a text file,

and/or sent to a Street Atlas startup file, otherwise known as

MapDocs.

o Versions 3.02.05 and up support Street Atlas 5.0 MapDoc files

and .gpl (track log) files.

o Versions 3.02.05 no longer support Precision Mapping files.

o Versions 3.02.02 and up re-enabled the uploading of almanac data

in the Yuma format.  If 'g7to -i fname' is executed and file

fname contains almanac data in Yuma format then the data will

immediately be sent to the GPS.  This command should be used

alone, i.e.  no other -i or -o processing on the same command

line.

o Versions 3.02.00 and above include support for the 12XL Software

version 3.51 and the GPS III.

o Version 3.01.08 and above provide some capability to read/write

Fugawi exported waypoint and track files.

o Version 3.00.00 and above allow multiple input and multiple

output files on the command line.

o Version 2.08 and above contain some support for the data files

created by WSII, Lowrance's program for communicating with their

GPS's.

o Versions 2.06a and above include some support for Street Atlas

4.0.

o The current position and time can be retrieved from the GPS45.

This function can be set to repeat every 'n' seconds.

o The Garmin internal and external voltages can be read from those

units supporting that portion of the Garmin protocol.

o The Backlight can be turned on and off.  On the GPS II+ , GPS

12XL Versions greater than 3.02 and the GPS III the level of the

backlight can be set.  The values range from 0 which is off to

255 which is full on.  Lower levels, i.e.  below 40 or so, are

hard to see.  A useful starting place is g7to -o g45L40.  The

maximum useful value appears to be about 95.

o On the Garmin 45 and 12XL units the LCD contrast can be set.

Proximity waypoints:

o Proximity waypoints may not be sent to Proj, Garmin64 or .SA3

files.

o The distance parameter stored in the text file is in meters.

o The 12XL Versions 2.02 and up support Proximity Waypoints,

however on versions 3.50 and up proximity waypoints cannot

be uploaded or downloaded via the GRMN/GRMN protocol.

- If a proximity waypoint is sent to a SA4 file, a circle is drawn.

- If a waypoint exists which has the same coordinants as the

center of the circle, it is used as the identifier for the

proximity circle.  If such a waypoint does not exist, one is

created with the name PROXxx where xx is a number representing

the xth waypoint created.

- If the waypoint in the center of the circle is moved so that it

no longer resides at the center of the circle and the file is

then downloaded by g7to a pseudo proximity waypoint will be

created.  In other words, a waypoint is tied to a circle and

becomes a proximity waypoint if and only if the waypoint is at

the exact center of the circle.

- Uploading proximity waypoints to the Garmin 12XL first deletes

all proximity waypoints and adds those in the current upload.

This means that you can have 8 proximity waypoints stored in

the 12XL, send one new proximity waypoint and end up with only

the new one, not 9 in the 12XL.

Lowrance WSII file (.low) support:

Waypoints, icons and routes may be read from a Lowrance WSII data

file 'file.low'.  WSII version 1.1 and 1.2 used with a Global Map

Sport version 2.1 has been tested.  Other versions of WSII software

and Global Map Sport GPS units have not been tested.  The .low file

has been tested with the Windows 95 version of WSII only.

===========================================================================

Supported Input Formats:

o Text -- similar to Gardown7 format.

- For input, GarDown7, Proj and Garmin64 are all read as text

files.

o SA .gpl track log files

o GarDown7 raw format.

o Proj format.

- Track and waypoint only

o Garmin64 format.

- Track and waypoint only

o Navigate.exe routes file, routes.clg.

o Fugawi waypoint and track files when exported as txt files

(?.fwp and ?.ftr respectively)

o Lowrance WSII files 'fname.low' (waypoints, icons and routes

only).

o SA3 startup files (fname.sa3).

- All symbols are output as Waypoints.

- The first 6 characters of an object's name becomes the

waypoint name.  The 7th character is skipped.  Therefore,

it is a good idea to make the 7th character a ';' or other

character that isn't actually part of the useful info.  The

8th->Xth (see below for values of 'X') character becomes

the waypoint comment.  The waypoint time is taken from the

time stamp on the input file.

Note:  Many of the newer Garmin receivers do not fully support the

time on GRMN/GRMN i/o.

X==19 for PROJ output format.

X==20 for Text output.

X==20 for Garmin64 output format.

X==23 for Control Point Record (*80*) format.

o SA4/SA5 startup files (fname.sa4 or fname.sa5 as of g7to

version 3.02.05).  As of version 2.07 the supported items are

lines and areas -- which are sent to the text file as track

data, circles which are sent as proximity waypoints, the

normal waypoints previously supported in SA3 files.

- The SA4 route is supported and routes 0-19 in the Garmin

are supported with up to 30 waypoints in each route.

- The Garmin route 0 is also supported as the SA4 route.  The

start and first points are identical as are the finish and

last point.  This results in a route labeled with numbers

only when viewed in SA4.

NOTE:  When the input is a .sa[345] file the time of all records sent

to the Garmin become the date of the input file.  If you have a

GPS that sends/receives the correct date of the record then it

would be better to use text files as an intermediate storage in

order to preserve the dates of origination.

o The GPS 45 via COM port.  Standard IRQ/IO supported.  The GPS

45 I/O SETUP must be set to GRMN/GRMN and HOST.

===========================================================================

===========================================================================

Supported Output Formats:

o SA startup files (fname.sa3, fname.sa4 or fname.sa5).  The

default object is a small square.  The default text location

is NE of the object.  The '.saX' suffix is not added by the

program, it must be input on the command line.

o SA4/5 startup files (fname.sa4 or fname.sa5).

- Tracks are converted to SA4 lines, proximity waypoints to

circles and routes 0-19 are converted to symbols with a

specific text format.

- Provision is made to change the parameters of lines and

circles via '!'  command lines.

Command line parameters for output of SA4/5 files

Default is ON for the following:

-ry    = turn on route line drawing for routes 1-19

-rn    = turn off route line drawing for routes 1-19

-rwy   = turn on route waypoints for routes 1-19

-rwn   = turn off route waypoints for routes 1-19

-r0y   = turn on route 0 objects to sa4/sa5

-r0n   = turn off route 0 objects to sa4/sa5

-r0ly  = turn on route 0 line drawing to sa4/sa5

-r0ln  = turn off route 0 line drawing to sa4/sa5

-r0wy  = turn on route 0 waypoints

-r0wn  = turn off route 0 waypoints

-r0wy or -ry must be used for routes to be output to sa3 files

-by turns on all sa outputs

i.e. -by is a synonym for -wy -ty -ry -r0y -py -r0wy -rwy

-bn turns off all sa outputs

i.e. -bn is a synonym for -wn -tn -rn -r0n -pn -r0wn -rwn

-r cxx = set route line color

-r txx = set route line thickness

-r sxx = set route line style

o Text file.

o PROJ.EXE and/or NAD2NAD.EXE format (fname.prj).  See the

documentation for PROJ for further information

o Control Point Record (*80*) (fname.x80) for use with the

program UTMS.EXE.  See UTMS documentation for further

information.  !!Output Only!!

o Garmin64 format (fname.g64).  See the GARMIN64 documentation

for an explanation of this format.

o The GPS 45 via COM port, the file name MUST be G45x.  See

below for options on '-i G45x'.

o Navigate.exe routes.clg routes file.

o WSII from Lowrance files (fname.low).  Waypoints only.

Routes, tracks and icons are not written.

o SA .gpl track log files

o Fugawi waypoint and track files when exported as txt files

(?.fwp and ?.ftr respectively)

o If output suffix .wtt is chosen g7to will output the

currently loaded waypoints as a track list.  This is useful

when using waypoints to define a track with Fugawi.

==============================================================================

Command line help available:

==============================================================================

c:\g7to -hi

SA, text, Proj, Garmin64, Fugawi .fwp & .ftr, Navigate routes.clg or GPS

-i <fname> - input file  (?.sa[345], ?.prj ?.g64 ?.x80 ?.fwp ?.ftr)

Use -1, -2, -3 or -4 to set standard COM Port 1-4

To specify the port parameters use:

-5 Port,IOaddr,IRQ (no spaces allowed) where:

Port is 1-4, IOaddr in hex (0xnnn) or decimal, IRQ (3-15) decimal

g7to -5 1,0x3f8,4       defines a standard COM1 port

-i G45Pn -- Send Time/Position record every n seconds (press 'x' to stop)

-i G45A  -- Send Almanac data

-i G45I  -- Send current Time

-i G45D  -- Send software iD

-i G45R  -- Send Routes

-i G45T  -- Send Tracks (-d to sum distances)

-i G45W  -- Send Waypoints

-i G45X  -- Send proXimity waypoints

-i G45V  -- Send Voltages

-e Do not print symbol attributes to text file (! line)

==============================================================================

c:\g7to -ho

-o G45O GPS45 Off; -o G45Cx set contrast x=0 to 15 (tested on 12XL only)

-o G45Lx (x=0-255) 12XL: 1=light on; II+: x=intensity level

-----------------------------------------------------------------

-o <fname> - output file  (?.sa3, ?.sa4, ?.prj ?.g64 ?.x80 ?.fwp ?.ftr)

-a - Text output

-b y/n - Turn on/off all SA output options

-c - 1st-center map Lat|2nd-center map Long (DDdMM.mmmm)

-g - No Lat/Long Grid on SA files

-m - map magnitude

-n - No name and no description

-p - Circle (Proximity) Parameters

-r - Line (route) Parameters

-s - Don't sort waypoints and don't remove duplicate waypoints

-t - Line (track) Parameters      - C-color, T-thickness, S-style

-----------------------------------------------------------------

-w <parms> - set waypoint (Map Symbol) parameters where:

text alignment:  .NW .N .NE .W .E .SW .S .SE           | .  = a

text colors:     ..black ..red ..blue ..green ..yellow | .. = ct

obj  colors:     ..black ..red ..blue ..green ..yellow | .. = co

text sizes:      ..vs ..s ..m ..l ..vl                 | .. = st

obj  sizes:      ..vs ..s ..m ..l ..vl                 | .. = so

default icon:    dxxx where xxx is SA4 obj number      | .  = d

-----------------------------------------------------------------

-qx- (x=0-15) remove certain text from waypoints in SA files. See g7to.doc

==============================================================================

c:\g7to -hd

Codes for use with text files COMMENT;xx;DisplayType

0 = Agricultural                            35 = One Way Up

1 = Airport        18 = Education           36 = Park

2 = Amusement      19 = Emergency           37 = Parking

3 = Arrow, Down    20 = Fishing             38 = Phone

4 = Arrow, Left    21 = Fuel                39 = Picnic

5 = Arrow, Right   22 = Golf Course         40 = Post Office

6 = Arrow, Up      23 = Government Building 41 = Power

7 = Bank           24 = Historic Site       42 = Religious Star

8 = Beach          25 = Home                43 = Religious Cross

9 = Boat Ramp      26 = Industrial          44 = Restaurant

10 = Campground     27 = Information         45 = Sports

11 = Cemetary       28 = Library             46 = Square

12 = Circle         29 = Lodging             47 = Star

13 = Construction   30 = Mountain            48 = State Capitol

14 = Cross          31 = National capitol    49 = Stop Sign

15 = Delivery Stop  32 = One Way Down        50 = Triangle Down

16 = Diamond        33 = One Way Left        51 = Triangle Up

17 = Doctor/Dentist 34 = One Way Right       52 = Zoo

53 = Bike           54 = Bridge              55 = Church 56 = Police

==============================================================================

Typical command lines:

INPUT -- GPS45-wpts, trks, routes, Prox wpts with output .sa3, sa4 and

text.  (a download)

g7to -i G45w -i g45r -i g45t -i g45x -o file.sa3 -o file.sa4 -a >file.txt

will download waypoint, route, track and proximity waypoint data from

the GPS45 and store it in the SA3 startup file file.sa3, file.sa4 and also

write it to the text file file.txt.

Note:  The -i g45 commands can be placed in a single command.  Therefore,

the above command line can be shortened to:

g7to -i G45wrtx -o file.sa3 -o file.sa4 -a >file.txt

NOTE:  The above example reads proximity waypoints, if they exist,

from the Garmin.  It also sends the data to several locations,

one of which is a Street Atlas 3 file.  SA3 does not support

proximity waypoints, so, proximity waypoints, if they exist in

the Garmin will be stripped from the data sent to the .sa3

file.

---------

g7to -o G45 -i file.sa3 -i file.sa4 -a >waypoint.txt

uploads waypoint data to the GPS45 from the SA3 startup file file.sa3

and also writes it to the text file waypoint.txt.  By default,

waypoint data is sorted and waypoints with the same name are removed.

To disable sorting and duplicate name removal, specify -s on the

command line, i.e.

---------

g7to -s -o G45 -i file.sa3 -i file.sa4 -a >waypoint.txt

NOTE:  When the input is a .sa3 or .sa4 file the time of all records

sent to the Garmin become the date of the input file.  If you

have a GPS that sends/receives the correct date of the record

then it would be better to use text files as an intermediate

storage in order to preserve the dates of origination.

If no output file is specified then the output is assumed to be text.

If a COM port is required and none is specified then COM1 is assumed.

---------

g7to -i G45R -o routes.sa4 -a >routes.txt

uploads the routes in the Garmin to a SA4 file and creates an

intermediate text file, routes.txt.

---------

g7to -i waypoint.txt -o waypoint.low

will read file waypoint.txt and create waypoint.low which will

contain the waypoints only.  The waypoint names in waypoint.txt are

ignored and the waypoint comment (after the date string in the text

file) are transferred as the Lowrance WSII waypoint name.

---------

g7to -i track.txt -o tracks.gpl

will read file track.txt and create track.gpl which can be read into

SA as a track log

NOTE:  The maximum number of track points supported by the program

defaults to 4096.  This value can be increased to 32767 by

the parameter -f.  However, each track point read requires

approximately 80 bytes of memory.

g7to -i xxx -o g45 -f 10000

sets the program to allow ten thousand track points

If a value of 0 is used then a temporary file is created and

used to hold the tracks.  This will cause g7to to run much

slower, but track files with up to 65534 points can be

accomodated.

---------

g7to -i waypoint.low

This will read the waypoint and route data from the file waypoint.low

and display it on the screen.  The format is normally 'W xxx..'.  If

a -u is added to the command line the output is the 'L' 'L xxx..'

format.  The 'W' format will allow data from a Lowrance file to be

output to a text file which can be sent to a Garmin unit.  If the

'L' format is used the waypoint icon is appended to the record.

NOTE:  The 'normal' output the name of the waypoint is the first 6

characters of the waypoint name as stored in the .low file.

As Lowrance units allow more than 6 characters, some of the

waypoints in the text file may have the same name.  When

uploaded to the Garmin units the last one uploaded will be ok,

the others will be deleted.  It's a good idea to take a look

at the text file, looking for duplicate waypoint names, before

uploading.  Adding a -u to the command line will output a 'L'

record which has room for 10 characters in the name.  This

format is used only for textfile<->.low file operations.

---------

g7to -i file.txt -o file.sa4 -b

will transfer file.txt to file.sa4.  If routes 1-19 exist in

the input file then lines will be drawn between the route points.

The parameters of the lines can be set by the -r command.

NOTE: a -r implies -b and the -b is not required.

------------------------------------------------------------------------

INPUT -- text format, OUTPUT SA3 Text is blue, Text size medium

g7to -i waypoint.txt -o waypoint.sa3 -c Tblue -s Tm

------------------------------------------------------------------------

INPUT -- .sa3, OUTPUT -- CPR (*80*) format

g7to -i myfile.sa3 -o myfile.x80

will read the SA3 startup file myfile.sa3 and write its contents to

the file 'myfile.x80 in Control Point Record format.  It will also

output the contents of myfile.sa3 to the screen in normal text format.

g7to -i myfile.sa3 -o myfile.x80 -a

------------------------------------------------------------------------

INPUT -- SA file, output textfile, output an attribute command

line each time an attribute changes.

g7to -i waypoint.sa3 -o waypoint.txt

The 'command line' output will be similar to:

! -w coblack -w ctblue -w sovs -w stm -w ane

This line signifies that the object will be black, the text will

be blue, the object will be very small and the text size will be

medium.  The text alignment will be North East of the object.

g7to -i waypoint.sa4 -o waypoint.txt

Line (track) options:

o allowable thickness: VT T M K VK

o allowable colors   : black red blue green yellow

o allowable styles   : normal hatched *highlight

! -t cblue -t s*h -t wvt

This line indicates that lines following this command are blue

(-t cblue), highlighted (-t s*h) and very thin (-t wvt)

o  -t cxx = set line color

o  -t txx = set line thickness (width)

o  -t sxx = set line style

and for route lines which are drawn for routes 1-19 when a -b is

present on the command line:

o  -r cxx = set line color

o  -r txx = set line thickness (width)

o  -r sxx = set line style

Circle options:

! -p ly -p fy -p cblue -p wvt -p ared -p s*h

draw circle with very thin blue line, fill circle with color red

highlighted

o  -p lxx = set circle outer line draw no/yes

o  -p fxx = set circle fill no/yes

o  -p cxx = set circle line color

o  -p txx = set circle line thickness (width)

o  -p axx = set circle area fill color

o  -p sxx = set circle area fill style

o allowable thickness: VT T M K VK

o allowable colors   : black red blue green yellow

o allowable styles   : normal hatched *highlight

o allowable drawline : no yes

o allowable drawfill : no yes

default icon:

When translating from a '16 icon' machine, i.e.  a II+ or 12XL 3.02

or greater or from a '45 icon' machine, currently only the III, you

can chose which SA4 object to use for the 'default' in the machine.

The command line parameter is '-w Dxxx' where xxx is the SA4 object

number.  See the table in G7TO.DOC.

The default icon for the 16 icon machines is a small square.  If you

execute a '-w d12' on the g7to command line when uploading waypoint

from a II+ or 12XL with icons, then this small square will be

translated into a circle in the SA4 map.

For the III, the default icon is a Waypoint.  If you execute '-w

d44' on the command line when transferring waypoints from the III

then all 'Waypoint' icons will be transferred as Restaurants.

Don't output command line for object properties option: '-e':

g7to -i waypoint.sa3 -o waypoint.txt -e

No object info is output (! lines) in the text file.

------------------------------------------------------------------------

INPUT  Lowrance WSII data file

g7to -i waypoint.low -u

will print the waypoints in the file:

L  ALSEA-BAY  N44 25.8961 W124 04.2498 Thu May 29 15:57:42 1997 ALSEA BAY BRIDGE

------------------------------------------------------------------------

The 'sa output only' options -c,-t,-r,-m,-n, and -qx

-c <Lat> -c <Long> - The first time the -c parameter appears on

the command line it is assumed to be the desired latitude of

the center of the .sa output file.  The second time it

appears in the command line it is taken to be the Longitude

of the center of the .sa file.  The input format is

DDDdMM.mmmm

-t set track Line draw parameters (SA4 only):

-t cxx = set line color

-t txx = set line thickness (width)

-t sxx = set line style

-r set route Line draw parameters (SA4 only, routes 1-19 only):

-r cxx = set line color

-r txx = set line thickness (width)

-r sxx = set line style

allowable thickness: VT T M K VK

allowable colors   : black red blue green yellow

allowable styles   : normal hatched *highlight

-ry must be used for route lines to be output to sa4 files

-rp must be used for route waypoints for routes 1-19

to be output to sa4 files

-r0 must be used for route 0 to be output to sa4 files

-r0w or -ry must be used for routes to be output to sa3 files

-r0w must me used to output route 0 waypoints

NOTE: -n is for sa3 only as tracks in sa4 are actual lines.

-m <Magnitude> Magnitude of the output .sa file 4-16

-n - Points from a track file do not contain text information,

and the 'waypoint name' normally becomes the position in

the source file.  For instance, if you have 10 points in

your track source file then the points are 'named' 1-10.

-n suppresses this number and only the default symbol, a

small circle, is displayed.  -n also overrides the -q

option and on a waypoint will pass on only the default

symbol.  This works with GarDown7 or text input, not on a

merge.

NOTE: use of the -q options will cause a loss of data in the SA file.

Do not use a SA file created with a -q option as an input file

without being aware of the data that will be lost.

-q 0  - all items written (default)

-q 1  - comment not written on regular waypoints

-q 2  - comment not written on route waypoints

-q 4  - the symbol display  parameter not written

-q 8  - route name is not written on first route point

The parameter values can be summed to turn off more than one

option.  For example -q 9 (the sum of 8 + 1) would turn off the

writing of comments on regular waypoints and turn off writing the

route name on the first route waypoint

-w <parms> - set waypoint parameters where:

text alignment:  .NW .N .NE .W .E .SW .S .SE           | .  = a

text colors:     ..black ..red ..blue ..green ..yellow | .. = ct

obj  colors:     ..black ..red ..blue ..green ..yellow | .. = co

text sizes:      ..vs ..s ..m ..l ..vl                 | .. = st

obj  sizes:      ..vs ..s ..m ..l ..vl                 | .. = so

NOTE:

The program automatically performs a scan of all input points

in order to calculate a proper center and magnitude of .SA3 and

.SA4 output files.  If a latitude, longitude or magnification

is entered on the command line it will override the calculated

value.

g7to -i fname -o xxx.sa3 -c N44d44.0 -c W122d45.0

This calculation is designed to return a magnification that will be

slightly smaller than might be shown on a single page, therefore,

all points should be displayed.

==============================================================================

Waypoint storage/retrieval accuracy.

The GPS45 sends/receives latitude/longitude data encoded in 4 byte

'long' variable.  However, the last byte of this variable is always

rounded by the GPS45.  Because of this there will be times when the

waypoint downloaded from the GPS45 will differ from that sent to it.

For instance if the waypoint:

W  TESTP2 N44 35.8931 ???????????? Mon Mar 25 15:28:04 1996 TEST WAYPOINT

is sent to the GPS45 it will be returned as

W  TESTP2 N44 35.8929 ???????????? Mon Mar 25 15:28:04 1996 TEST WAYPOINT

This 'rounding' applies to both the latitude and longitude variables.

==============================================================================

Text (.txt) file format

The .txt file created by this program can contain Waypoint data,

proximity Waypoint data, Track data Route start, Route End, Lowrance

WSII file type Icon records and datum records.  This program assumes

WGS84 datum, and therefore the datum record is informational only, it

does not trigger a conversion.  The format for the various record types

is shown below.

Character positions are shown for clarity.

Route begin/end and Map datum records are stripped when the output is a

.sa3 file.

The datum for SA3/4/5 is WGS 84, and this program converts Lat/Lon data

literally, so the GPS must be using WGS 84/NAD83 or accuracy will be

degraded.

Map datum record:

111111111

123456789012345678

D  100  WGS 84

Waypoint, Proximity Waypoint (prox value starts after the 3rd ';') and Track:

11111111112222222222333333333344444444445555555555666666666677777777778

12345678901234567890123456789012345678901234567890123456789012345678901234567890

N  New Track Start

W  YAQBRG N44 37.3259 W124 03.4229 Sun Mar 17 04:02:26 1996 YAQUINA BAY BRDG

P  YAQBRG N44 37.3259 W124 03.4229 Sun Mar 17 04:02:26 1996 YAQUINA BAY BRDG;;;534.5

T  N44 35.9589 W123 15.4792 Sun Jan 21 22:47:51 1996

! -w coblack -w ctblue -w sovs -w stm -w ane

Waypoint Symbols (Icons) for GPS II+

11111111112222222222333333333344444444445555555555666666666677777777778

12345678901234567890123456789012345678901234567890123456789012345678901234567890

W  YAQBRG N44 37.3259 W124 03.4229 Sun Mar 17 04:02:26 1996 YAQUINA BAY BRDG;1;1

The first character after the ';' which follows the waypont comment is a

number between 0-99, of which the SA program currently assigns 0-56,

inclusive.  The definition of the number (Text) is as follows (the Text

indicates it is the value used in a waypoint .txt file):

Garmin

G-III Symbol Name   II+ Icon Text/SA     SA Name

==================  ======== =======     =================

Hunting Area           13        0       Agricultural

Ultra Light             -        1       Airport

Bar                     -        2       Amusement

Drinking Water          -        3       Arrow Down

TrakBack               15        4       Arrow Left

Glider Area             -        5       Arrow Right

Tall Tower              -        6       Arrow Up

Bank                    -        7       Bank

Swimming Area           -        8       Beach

Boat Ramp               5        9       Boat Ramp

Campground             11       10       Campground

Danger Area             9       11       Cemetary

City (Small)            -       12       Circle

Private Field          12       13       Construction

City (Medium)           -       14       Cross

Car                     3       15       Delivery Stop

Soft Field              -       16       Diamond

Skiing Area             -       17       Doctor/Dentist

School                  -       18       Education

Medical Facility       14       19       Emergency

Fishing Area            4       20       Fishing

Gas Station             2       21       Fuel

Golf Course            10       22       Golf Course

Restroom                -       23       Government Building

Waypoint                -       24       Historic Site

Residence               1       25       Home

Heliport                -       26       Industrial

Waypoint                -       27       Information

Waypoint                -       28       Library

Shower                  -       29       Lodging

Scenic Area             -       30       Mountain

Short Tower             -       31       National Capitol

Waypoint                -       32       One Way Down

Seaplane Base           -       33       One Way Left

Shipwreck               7       34       One Way Right

Waypoint                -       35       One Way Up

Park                    -       36       Park

Parking Area            -       37       Parking

Telephone               -       38       Phone

Picnic Area             -       39       Picnic

Waypoint                -       40       Post Office

Waypoint                -       41       Power

City (Large)            -       42       Religious Star

Waypoint                -       43       Religious Cross

Restaurant              -       44       Restaurant

Ball Park               -       45       Sports

Waypoint                0       46       Square

Man Overboard           -       47       Star

Waypoint                -       48       State Capitol

Information             8       49       Stop Sign

Parachute Area          -       50       Triangle Down

Marina                  6       51       Triangle Up

Shopping Center         -       52       Zoo

Waypoint                -       53       Bike

Dam                     -       54       Bridge

Waypoint                -       55       Church

Waypoint                -       56       Police

The following examples should clarify this.

Desire:  SA4 icon to be an Airport.  The .txt number used is 1, so the

waypoint record would be similar to:

W  AIRPRT N54 33.8269 W121 14.1090 Wed May 28 17:44:04 1997 MY AIRPORT;1;1

Desire:  12XL (with icons) or GPS II+ to have the Exit symbol.

The II+ icons are displayed in the II+ user manual as a

4x4 grid which g7to numbers and names:

0 - SQUARE     1 - HOUSE       2 - GAS          3 - CAR

4 - FISH       5 - BOAT        6 - ANCHOR       7 - SUNKEN WRECK

8 - EXIT       9 - DANGER     10 - BOUY        11 - CAMP

12 - CROSSING  13 - DEER       14 - FIRST AID   15 - TRAIL

causing the EXIT symbol to be number 8.  In the table

above, the II+/12XL(3.02&>) symbol 8 translates into the

g7to 'Text' symbol number 49.

Therefore, the waypoint would be similar to:

W  MYWAYP N54 33.8269 W121 14.1090 Wed May 28 17:44:04 1997 MY WAYPOINT;49;1

If this waypoint is sent to an SA4 file the waypoint symbol

would be a 'Stop Sign'.  If it were sent to a GPS III the

waypoint symbol would be 'Information'

Note:  See the default icon flag, '-w dxxx' to change the mapping

between the II+ '0' and the SA4 object.  The default mapping is

to a square which is similar to the II+ default icon's actual

shape.

The character after the ';' which follows the icon type is the icon

display style and it can be a ' ', '1', '3' or '5'.  This represents how

the icon symbol is displayed on the map page.  A blank, ' ', will be

translated to the default, 3.

1 - Symbol alone

3 - Symbol & waypoint name

5 - Symbol & waypoint comment

---

The representation normal waypoints in a SA4 Text string is:

name;comment;icon display

and for route waypoints:

(rr:pp)name;comment;icon display;route name for the first point.

(rr:pp)name;comment;icon display for other than the first point.

where rr is the 2 digit route number and pp is the 2 digit number representing

the nth position in the route.

The icon type is encoded as the SA object symbol.

-----

If a mapnote element or a maptext element is read from an SA4 file the

text format must be:

NAME;COMMENT;icon type;icon style

If this is not the case then text output and uploading to the Garmin

can cause icon translation errors.

===================

Route begin:

11111111112222222222333333333344444444445555555555666666666677777777778

12345678901234567890123456789012345678901234567890123456789012345678901234567890

R  01 ROUTE NAME

Route end:

123456

E  01

==============================================================================

Proj format allowed on input (waypoint is output):

0123456789012345678921234567893123456789412345678951234567896123456789712345

124d04'36.28800"W 44d40'31.30200"N 1996-03-25 15:28 YAQHED YAQUINA HEAD

^           ^ ^

|           | |

must be 'd'    | |

must be '"'----. |

|

TAB or SPACE-----.

Proj format allowed on input (Track point is output):

0123456789012345678921234567893123

124d04'36.28800"W 44d40'31.30200"N

^           ^ ^

|           | |

must be 'd'    | |

must be '"'----. |

|

TAB or SPACE-----.

==============================================================================

Time:  If input is from a sa file, time is taken from the time stamp of

the input (-i) file.  For gardown7 or text input the time is

taken from the input record.  The GPS45 ignores the time stamp on

upload of track information.  The XL series does not store time,

therefore the time values on records from the XL receivers is not

meaningful.

==============================================================================

Distance:  The track distance calculation uses the Sinnot formula.

==============================================================================

.sa3 notes:  Street Atlas 3 has at least 3 objects that can be stored in

the .sa3 file.  These objects are Text, Place and MapNotes.

Places are very similar to waypoints, i.e.  there is a Lat/Long

location and a name.  All waypoints in a .txt file or from the

GPS45 become Places during a conversion to .sa3.

All three .sa3 objects are converted to waypoints during a .sa3

to GPS45 upload and/or a .txt conversion (and therefore would

become Places if the .txt file were used to create a new .sa3

file).

.sa4 notes:

Street Atlas 4 adds three more objects beyond those in sa3:  an

Area, a Circle and Lines.  The perimeter of an Area is converted

to track points by g7to.  The Circle is converted to proximity

waypoints and Lines are converted to tracks.

==============================================================================

Version History has moved to the file HISTORY.TXT

==============================================================================

A sample Almanac data file is shown between the 'Cut HERE' lines.

This sample file was created by

1.  Retrieve the data for week 927 from

http://satnav.atc.ll.mit.edu/gps/current.gpy and save it as file

current.gpy.

2.  Upload current.gpy to the GPS45 by: g45to -i current.gpy

3.  Download the sample file by g7to -i g45a >sample.fil

--------------------Cut HERE----------------------------

**** Week 927 almanac for PRN-01 ***********

ID:                          001

Health:                      000

Eccentricity:                3.756046295e-03

Time of Applicability(s):    3.194880000e+05

Orbital Inclination(rad):    9.549833536e-01

Rate of Right Ascen(r/s):   -8.114623640e-09

SQRT(A) (m^1/2):             5.153515625e+03

Right Ascen at TOA(rad):     2.264179707e+00

Argument of Perigee(rad):   -1.620598674e+00

Mean Anom(rad):              1.528355360e+00

Af0(s):                      3.051757812e-05

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week 927 almanac for PRN-02 ***********

ID:                          002

Health:                      000

Eccentricity:                1.721429825e-02

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.405603409e-01

Rate of Right Ascen(r/s):   -8.560356868e-09

SQRT(A) (m^1/2):             5.153605469e+03

Right Ascen at TOA(rad):    -1.992512226e+00

Argument of Perigee(rad):   -2.322431564e+00

Mean Anom(rad):             -2.754760981e+00

Af0(s):                     -4.119873047e-04

Af1(s/s):                   -3.637978807e-12

week:                        927

**** Week 927 almanac for PRN-03 ***********

ID:                          003

Health:                      000

Eccentricity:                2.428531647e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.516996741e-01

Rate of Right Ascen(r/s):   -8.034620969e-09

SQRT(A) (m^1/2):             5.153605469e+03

Right Ascen at TOA(rad):    -9.127451777e-01

Argument of Perigee(rad):    2.601369619e+00

Mean Anom(rad):             -2.218770683e-01

Af0(s):                      6.103515625e-05

Af1(s/s):                    3.637978807e-12

week:                        927

**** Week 927 almanac for PRN-04 ***********

ID:                          004

Health:                      000

Eccentricity:                4.340171814e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.758659005e-01

Rate of Right Ascen(r/s):   -7.783181211e-09

SQRT(A) (m^1/2):             5.153571777e+03

Right Ascen at TOA(rad):     1.631240249e-01

Argument of Perigee(rad):   -8.987296224e-01

Mean Anom(rad):             -3.376117051e-01

Af0(s):                      5.149841309e-05

Af1(s/s):                    3.637978807e-12

week:                        927

**** Week 927 almanac for PRN-05 ***********

ID:                          005

Health:                      000

Eccentricity:                1.185417175e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.434006214e-01

Rate of Right Ascen(r/s):   -8.537498708e-09

SQRT(A) (m^1/2):             5.153473633e+03

Right Ascen at TOA(rad):    -1.975718141e+00

Argument of Perigee(rad):   -9.392756224e-01

Mean Anom(rad):              4.020318091e-01

Af0(s):                      1.173019409e-04

Af1(s/s):                    3.637978807e-12

week:                        927

**** Week 927 almanac for PRN-06 ***********

ID:                          006

Health:                      000

Eccentricity:                6.813049316e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.588542581e-01

Rate of Right Ascen(r/s):   -7.920330169e-09

SQRT(A) (m^1/2):             5.153619141e+03

Right Ascen at TOA(rad):    -8.777751923e-01

Argument of Perigee(rad):   -2.601424217e+00

Mean Anom(rad):              3.291980326e-01

Af0(s):                      0.000000000e+00

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week 927 almanac for PRN-07 ***********

ID:                          007

Health:                      000

Eccentricity:                9.531974792e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.611252546e-01

Rate of Right Ascen(r/s):   -7.931759249e-09

SQRT(A) (m^1/2):             5.153545898e+03

Right Ascen at TOA(rad):    -9.109022021e-01

Argument of Perigee(rad):   -2.286374092e+00

Mean Anom(rad):              2.317899227e+00

Af0(s):                      7.572174072e-04

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week  -1 almanac for PRN-08 ***********

ID:                          008

Health:                      111

Eccentricity:                9.531974792e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.611252546e-01

Rate of Right Ascen(r/s):   -7.931759249e-09

SQRT(A) (m^1/2):             5.153545898e+03

Right Ascen at TOA(rad):    -9.109022021e-01

Argument of Perigee(rad):   -2.286374092e+00

Mean Anom(rad):              2.317899227e+00

Af0(s):                      7.572174072e-04

Af1(s/s):                    0.000000000e+00

week:                        -1

**** Week 927 almanac for PRN-09 ***********

ID:                          009

Health:                      000

Eccentricity:                6.646633148e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.445630908e-01

Rate of Right Ascen(r/s):   -8.046050048e-09

SQRT(A) (m^1/2):             5.153553223e+03

Right Ascen at TOA(rad):    -3.001417637e+00

Argument of Perigee(rad):    2.387853116e-01

Mean Anom(rad):              3.305518925e-01

Af0(s):                     -1.430511475e-05

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week 927 almanac for PRN-10 ***********

ID:                          010

Health:                      000

Eccentricity:                2.264976501e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.640134573e-01

Rate of Right Ascen(r/s):   -8.331776158e-09

SQRT(A) (m^1/2):             5.153544922e+03

Right Ascen at TOA(rad):     1.192146540e+00

Argument of Perigee(rad):   -2.541696727e-01

Mean Anom(rad):             -2.652863264e+00

Af0(s):                      1.430511475e-05

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week  -1 almanac for PRN-11 ***********

ID:                          011

Health:                      111

Eccentricity:                2.264976501e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.640134573e-01

Rate of Right Ascen(r/s):   -8.331776158e-09

SQRT(A) (m^1/2):             5.153544922e+03

Right Ascen at TOA(rad):     1.192146540e+00

Argument of Perigee(rad):   -2.541696727e-01

Mean Anom(rad):             -2.652863264e+00

Af0(s):                      1.430511475e-05

Af1(s/s):                    0.000000000e+00

week:                        -1

**** Week  -1 almanac for PRN-12 ***********

ID:                          012

Health:                      111

Eccentricity:                2.264976501e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.640134573e-01

Rate of Right Ascen(r/s):   -8.331776158e-09

SQRT(A) (m^1/2):             5.153544922e+03

Right Ascen at TOA(rad):     1.192146540e+00

Argument of Perigee(rad):   -2.541696727e-01

Mean Anom(rad):             -2.652863264e+00

Af0(s):                      1.430511475e-05

Af1(s/s):                    0.000000000e+00

week:                        -1

**** Week  -1 almanac for PRN-13 ***********

ID:                          013

Health:                      111

Eccentricity:                2.264976501e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.640134573e-01

Rate of Right Ascen(r/s):   -8.331776158e-09

SQRT(A) (m^1/2):             5.153544922e+03

Right Ascen at TOA(rad):     1.192146540e+00

Argument of Perigee(rad):   -2.541696727e-01

Mean Anom(rad):             -2.652863264e+00

Af0(s):                      1.430511475e-05

Af1(s/s):                    0.000000000e+00

week:                        -1

**** Week 927 almanac for PRN-14 ***********

ID:                          014

Health:                      000

Eccentricity:                1.507282257e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.695022702e-01

Rate of Right Ascen(r/s):   -8.263201678e-09

SQRT(A) (m^1/2):             5.153700684e+03

Right Ascen at TOA(rad):     1.235849977e+00

Argument of Perigee(rad):    2.941669226e+00

Mean Anom(rad):              2.597313643e+00

Af0(s):                      2.288818359e-05

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week 927 almanac for PRN-15 ***********

ID:                          015

Health:                      000

Eccentricity:                7.051467896e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.809352160e-01

Rate of Right Ascen(r/s):   -7.760323051e-09

SQRT(A) (m^1/2):             5.153680664e+03

Right Ascen at TOA(rad):     2.011469752e-01

Argument of Perigee(rad):    1.633580923e+00

Mean Anom(rad):             -1.165151715e+00

Af0(s):                      4.787445068e-04

Af1(s/s):                    3.637978807e-12

week:                        927

**** Week 927 almanac for PRN-16 ***********

ID:                          016

Health:                      000

Eccentricity:                2.227306366e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.659069777e-01

Rate of Right Ascen(r/s):   -8.308917998e-09

SQRT(A) (m^1/2):             5.153564453e+03

Right Ascen at TOA(rad):     1.242112517e+00

Argument of Perigee(rad):    7.036762685e-02

Mean Anom(rad):             -1.351561069e+00

Af0(s):                      3.147125244e-05

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week 927 almanac for PRN-17 ***********

ID:                          017

Health:                      000

Eccentricity:                9.733200073e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.820976853e-01

Rate of Right Ascen(r/s):   -7.714607619e-09

SQRT(A) (m^1/2):             5.153601562e+03

Right Ascen at TOA(rad):     2.381643802e-01

Argument of Perigee(rad):    2.544379711e+00

Mean Anom(rad):              1.679810137e-01

Af0(s):                     -1.621246338e-04

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week 927 almanac for PRN-18 ***********

ID:                          018

Health:                      000

Eccentricity:                6.929874420e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.440417886e-01

Rate of Right Ascen(r/s):   -8.057479128e-09

SQRT(A) (m^1/2):             5.153539062e+03

Right Ascen at TOA(rad):     2.208091259e+00

Argument of Perigee(rad):    1.664717793e+00

Mean Anom(rad):              2.318376064e+00

Af0(s):                      5.722045898e-06

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week 927 almanac for PRN-19 ***********

ID:                          019

Health:                      000

Eccentricity:                3.311157227e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.280667901e-01

Rate of Right Ascen(r/s):   -8.160339959e-09

SQRT(A) (m^1/2):             5.153656738e+03

Right Ascen at TOA(rad):    -3.045706987e+00

Argument of Perigee(rad):   -2.934807777e+00

Mean Anom(rad):             -4.429571629e-01

Af0(s):                      2.651214600e-04

Af1(s/s):                    1.455191523e-11

week:                        927

**** Week  -1 almanac for PRN-20 ***********

ID:                          020

Health:                      111

Eccentricity:                3.311157227e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.280667901e-01

Rate of Right Ascen(r/s):   -8.160339959e-09

SQRT(A) (m^1/2):             5.153656738e+03

Right Ascen at TOA(rad):    -3.045706987e+00

Argument of Perigee(rad):   -2.934807777e+00

Mean Anom(rad):             -4.429571629e-01

Af0(s):                      2.651214600e-04

Af1(s/s):                    1.455191523e-11

week:                        -1

**** Week 927 almanac for PRN-21 ***********

ID:                          021

Health:                      000

Eccentricity:                1.380538940e-02

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.633303881e-01

Rate of Right Ascen(r/s):   -8.308917998e-09

SQRT(A) (m^1/2):             5.153615234e+03

Right Ascen at TOA(rad):     1.198329210e+00

Argument of Perigee(rad):   -2.914901733e+00

Mean Anom(rad):             -2.260015726e+00

Af0(s):                      8.678436279e-05

Af1(s/s):                    3.637978807e-12

week:                        927

**** Week 927 almanac for PRN-22 ***********

ID:                          022

Health:                      000

Eccentricity:                1.015377045e-02

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.405124187e-01

Rate of Right Ascen(r/s):   -8.560356868e-09

SQRT(A) (m^1/2):             5.153501953e+03

Right Ascen at TOA(rad):    -1.977967143e+00

Argument of Perigee(rad):    2.220358551e-01

Mean Anom(rad):             -2.946165800e+00

Af0(s):                      2.288818359e-05

Af1(s/s):                    3.637978807e-12

week:                        927

**** Week 927 almanac for PRN-23 ***********

ID:                          023

Health:                      000

Eccentricity:                1.213836670e-02

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.664942026e-01

Rate of Right Ascen(r/s):   -8.274630758e-09

SQRT(A) (m^1/2):             5.153609375e+03

Right Ascen at TOA(rad):     1.236110687e+00

Argument of Perigee(rad):   -2.104972124e+00

Mean Anom(rad):             -2.522715569e+00

Af0(s):                      4.768371582e-06

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week 927 almanac for PRN-24 ***********

ID:                          024

Health:                      000

Eccentricity:                7.793426514e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.857048988e-01

Rate of Right Ascen(r/s):   -7.691749460e-09

SQRT(A) (m^1/2):             5.153699707e+03

Right Ascen at TOA(rad):     1.656148732e-01

Argument of Perigee(rad):   -1.889712691e+00

Mean Anom(rad):              4.705268890e-02

Af0(s):                      7.772445679e-04

Af1(s/s):                    7.275957614e-12

week:                        927

**** Week 927 almanac for PRN-25 ***********

ID:                          025

Health:                      000

Eccentricity:                6.714344025e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.374564290e-01

Rate of Right Ascen(r/s):   -8.080336400e-09

SQRT(A) (m^1/2):             5.153559082e+03

Right Ascen at TOA(rad):    -3.034118891e+00

Argument of Perigee(rad):   -2.648527861e+00

Mean Anom(rad):              1.368816853e+00

Af0(s):                     -6.675720215e-06

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week 927 almanac for PRN-26 ***********

ID:                          026

Health:                      000

Eccentricity:                9.894847870e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.580333233e-01

Rate of Right Ascen(r/s):   -7.874613850e-09

SQRT(A) (m^1/2):             5.153723633e+03

Right Ascen at TOA(rad):     2.246636868e+00

Argument of Perigee(rad):   -3.218610883e-01

Mean Anom(rad):              2.138721943e+00

Af0(s):                     -2.670288086e-05

Af1(s/s):                   -3.637978807e-12

week:                        927

**** Week 927 almanac for PRN-27 ***********

ID:                          027

Health:                      000

Eccentricity:                1.239776611e-02

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.416389465e-01

Rate of Right Ascen(r/s):   -8.023191889e-09

SQRT(A) (m^1/2):             5.153628906e+03

Right Ascen at TOA(rad):    -3.016989231e+00

Argument of Perigee(rad):    3.073674202e+00

Mean Anom(rad):             -7.828541398e-01

Af0(s):                      3.814697266e-06

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week  -1 almanac for PRN-28 ***********

ID:                          028

Health:                      111

Eccentricity:                1.239776611e-02

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.416389465e-01

Rate of Right Ascen(r/s):   -8.023191889e-09

SQRT(A) (m^1/2):             5.153628906e+03

Right Ascen at TOA(rad):    -3.016989231e+00

Argument of Perigee(rad):    3.073674202e+00

Mean Anom(rad):             -7.828541398e-01

Af0(s):                      3.814697266e-06

Af1(s/s):                    0.000000000e+00

week:                        -1

**** Week 927 almanac for PRN-29 ***********

ID:                          029

Health:                      000

Eccentricity:                5.497455597e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.544620514e-01

Rate of Right Ascen(r/s):   -7.954617409e-09

SQRT(A) (m^1/2):             5.153367188e+03

Right Ascen at TOA(rad):     2.221245289e+00

Argument of Perigee(rad):   -1.975628257e+00

Mean Anom(rad):              1.104343653e+00

Af0(s):                      2.565383911e-04

Af1(s/s):                   -1.818989404e-11

week:                        927

**** Week 927 almanac for PRN-30 ***********

ID:                          030

Health:                      000

Eccentricity:                5.005836487e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.507948756e-01

Rate of Right Ascen(r/s):   -8.457495149e-09

SQRT(A) (m^1/2):             5.153623047e+03

Right Ascen at TOA(rad):    -1.951116800e+00

Argument of Perigee(rad):    1.620874643e+00

Mean Anom(rad):             -2.815076351e+00

Af0(s):                     -3.814697266e-06

Af1(s/s):                   -3.637978807e-12

week:                        927

**** Week 927 almanac for PRN-31 ***********

ID:                          031

Health:                      000

Eccentricity:                7.476806641e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.605619907e-01

Rate of Right Ascen(r/s):   -7.920330169e-09

SQRT(A) (m^1/2):             5.153670410e+03

Right Ascen at TOA(rad):    -9.092386365e-01

Argument of Perigee(rad):    7.159495950e-01

Mean Anom(rad):              1.066185951e+00

Af0(s):                      0.000000000e+00

Af1(s/s):                    0.000000000e+00

week:                        927

**** Week  -1 almanac for PRN-32 ***********

ID:                          032

Health:                      111

Eccentricity:                7.476806641e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.605619907e-01

Rate of Right Ascen(r/s):   -7.920330169e-09

SQRT(A) (m^1/2):             5.153670410e+03

Right Ascen at TOA(rad):    -9.092386365e-01

Argument of Perigee(rad):    7.159495950e-01

Mean Anom(rad):              1.066185951e+00

Af0(s):                      0.000000000e+00

Af1(s/s):                    0.000000000e+00

week:                        -1

-------------------------------------------------------------------

This program has been tested on several PC's running MS-DOS and/or

Windows 95.  It has also been tested on a Hewlett Packard 200LX palmtop.

The only Garmin unit available for testing this program is my own GPS45,

software revision 2.43, therefore the only upload/download functions in

this program are those known to be supported by the GPS45, Software

Version 2.43.

Versions of G7TO greater than 2.04 have been tested on my Garmin 12XL,

version 2.01, only.  I no longer own a GPS45.

Ron Henderson WA7TAS

crh@proaxis.com